F18A Technology Reference Guide

Introduction

The F18 technology consists of small, simple computers and several types of versatile, software defined I/O.  Our chips are configured as arrays of the F18 computers with appropriate selections of I/O types.

An F18 computer is a physically small, very fast, energy-efficient and inexpensive 18-bit machine.  Each F18 is self-contained as a complete system, with its own RAM, ROM, stacks, registers, and where appropriate external I/O.  The F18 computers in an array operate independently and asynchronously, communicating with each other using efficient internal communication ports.

This book will familiarize you with the architecture, internal structure, and use of the computers and I/O of chips built from F18A technology.  Please refer to the data books for specific chips using this technology to learn their exact configurations, pin-outs, ROM content and electrical characteristics.  The current editions of these, along with many other relevant documents and application notes as well as the current edition of this document, may be found on our website.  It is always advisable to ensure that you are using the latest documents before starting work.

Status of Data Given

The data given herein are PRODUCTION.  Electrical and timing values are illustrative only.  Otherwise, the subject matter of this book is the architecture, instruction set, and programming of the F18A computers and I/O, which are fixed by design.  Future revisions of this book may document upward-compatible enhancements of the F18A programming model.

Documentation Conventions

Numbers

Numbers are written in decimal unless otherwise indicated.  Hexadecimal values are indicated by explicitly writing “hex” or by preceding the number with the lowercase letter “x”.

Node coordinates

Each GreenArrays chip is a rectangular array of nodes, each of which is an F18 computer.  By convention these arrays are visualized as seen from the top of the silicon die, which is normally the top of the chip package, oriented such that pin 1 is in the upper left corner.  Within the array, each node is identified by a three or four digit number denoting its Cartesian coordinates within the array as yxx or yyxx with the lower left corner node always being designated as node 000.  Thus, for a GA144 chip whose computers are configured in an array of 18 columns and 8 rows, the numbers of the nodes in the lower right, upper left, and upper right corners are 017, 700, and 717 respectively.

Register names

Register names in prose may be used with or without the word "register" and are usually shown in a bold font and capitalized where necessary to avoid ambiguity, such as for example the registers T S R I A B and IO or io .

Bit Numbering

Binary numbers are visualized as a horizontal row of bits, numbered consecutively right to left in ascending significance with the least significant bit numbered zero.  Thus bit n has the binary value 2n.  The notation P9 means bit 9 of register P, whose binary value is x200, and T17 means the sign (high order) bit of 18-bit register T.

The F18A Computer

Here is an introduction to the resources within each F18A computer.  All elements are 18 bits wide unless otherwise noted.

Figure 1  F18A Block Diagram

RAM:  18 bits wide, 64 words in each node.

- ROM:  18 bits wide, 64 words in each node.

- I/O logic may be present on any edge node of a chip.

- Communication Ports are configured based on position within the array (four for inner; three ports array edges; two for corners.)  One additional port is optional.

- IO  control and status for communication ports and I/O logic.

- Return Stack consists of 8 registers addressed circularly (no fixed top or bottom.)  Pushing a word onto this stack makes it the new “top” item, overwriting the former “bottom” item.  After popping 8 items, those 8 items are repeated.

- R extends this stack to 9 entries but only 8 are circular.  When used as a loop counter, R decrements toward zero.

- P serves as a “program counter”, holding the address of the next word in the instruction stream.  Within RAM or ROM, it is incremented after each word is fetched, circularly within whichever storage class it currently points to.  When P addresses I/O space, it is not incremented.  Bit P9 enables Extended Arithmetic Mode when set.

- A is a general purpose read/write address or data register.

- B is a write only address register, containing on reset the address of  io  register.

- I is the instruction register into which instruction words containing 1, 2, 3 or 4 opcodes are fetched for execution.

- T is the top word of a 10-element data stack.

- The ALU (Arithmetic/Logic unit) performs unary operations on T, binary operations on S and T, or a multiply step involving S, T, and A as directed by opcodes.  A carry latch is involved in some Extended Arithmetic mode instructions.

- S is the second word of a 10-element data stack.

- Data Stack is 8 circularly addressed registers constituting the third through 10th elements of a 10-element stack.

After Reset

On chip reset, the following registers are initialized:

P is set to the address configured in the chip layout.  For F18A, this will always be either a multiport execute or will be ROM address x0aa.

io is set to the state it would have after a program wrote x15555 into the register.

B is set to the address of io.

Stack pointers are set to the same initial condition on every reset.

Other registers, stack contents, and RAM are not directly affected by reset.

Memory Addressing

| Start | End | Address Space |
| --- | --- | --- |
| x000 | x03F | RAM, 64 words |
| x040 | x07F | RAM x000-x03F repeated |
| x080 | x0BF | ROM, 64 words |
| x0C0 | x0FF | ROM x080-x0BF repeated |
| x100 | x1FF | I/O ports and registers |

Figure 2  Address Map

The F18A decodes a nine-bit memory address as shown in Figure 2.

Address incrementing, when applied to A or P, continues from x03F to x040 but wraps from x07F to x000.  The same process occurs with addresses in the ROM space.  Incrementing does not occur at all when the address lies in the region of I/O ports and registers.  Incrementing never affects bits P8 or P9.  P9 has no effect on memory decoding; it simply enables the Extended Arithmetic Mode when set.

Instruction Set

The F18 instruction set consists of 5-bit opcodes as summarized in Figure 3:

| Opcode |  | Notes | Opcode |  | Notes |
| --- | --- | --- | --- | --- | --- |
| ; | 00 | return | +* | 10 | multiply step |
| ex | 01 | execute (swap P and R) | 2* | 11 | left shift |
| name ; | 02 | jump to name | 2/ | 12 | right shift (signed) |
| name | 03 | call to name | inv | 13 | invert all bits (was -) |
| unext | 04 | loop within I (decrement R) | + | 14 | add (or add with carry) |
| next | 05 | loop to address (decrement R) | and | 15 |  |
| if | 06 | jump if T=0 | xor | 16 | exclusive or (was or) |
| -if | 07 | jump if T≥0 | drop | 17 |  |
| @p | 08 | literal (autoincrement P) | dup | 18 |  |
| @+ | 09 | fetch via A (autoincrement A) | r> | 19 | pop R push T (was pop) |
| @b | 0A | fetch via B | over | 1A |  |
| @ | 0B | fetch via A | a | 1B | fetch from register A |
| !p | 0C | store via P (autoincrement P) | . | 1C | nop |
| !+ | 0D | store via A (autoincrement A) | >r | 1D | pop T push R (was push) |
| !b | 0E | store via B | b! | 1E | store into register B |
| ! | 0F | store via A | a! | 1F | store into register A |

Figure 3  Table of Opcodes

The shaded opcodes may be used in slot 3 of an instruction word (see 2.3.1 below).  Add (+) and multiply step (+*) behave differently in Extended Arithmetic Mode.

Instruction Word Format

Each F18A instruction word is subdivided into four possible slots for opcodes that are normally executed sequentially left to right when viewing the word as conventionally oriented with most significant bit to the left.  Jump instructions may be written in slots 0, 1, or 2, and when they are encountered the remainder of the instruction word is interpreted as an address field.  This results in four possible instruction word formats as shown in Figure 4:

| Slot 0 |  |  |  |  | Slot 1 |  |  |  |  | Slot 2 |  |  |  |  | Slot 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instruction 0 |  |  |  |  | Instruction 1 |  |  |  |  | Instruction 2 |  |  |  |  | Instruction 3 |  |  |
| Instruction 0 |  |  |  |  | Instruction 1 |  |  |  |  | Jump instruction |  |  |  |  | Destination |  |  |
| Instruction 0 |  |  |  |  | Jump instruction |  |  |  |  | Destination |  |  |  |  |  |  |  |
| Jump instruction |  |  |  |  | Unused |  |  | Destination |  |  |  |  |  |  |  |  |  |
| 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Figure 4  Instruction word formats

The n-bit destination address field simply replaces the n low order bits of the current (incremented) value of P at the time the jump instruction executes.  Thus the scope of a slot 2 jump is very limited while a slot 0 jump can reach any addressable destination, and can control P9 to explicitly enable or disable Extended Arithmetic Mode.  Slot 1 and 2 jumps have no effect upon P9, so its selection is unchanged as these jumps are executed.  In the F18A, slot 1 and 2 jumps additionally force P8 to zero.  This means the destination of any slot 1 or 2 jump may never be in I/O space.

The instruction selected for slot 3 must be one of those shaded in Figure 3 above.  When the desired opcode cannot be fit into the current instruction word, either because it may not be used in slot 3 or because it is a jump whose destination requires a longer address field, the remaining slots of the current word may be filled with nop instructions.

In addition to jumps, three other instructions affect the consecutive execution of opcodes within an instruction word:

;  (return) and  ex  (execute) discontinue execution of the current instruction word.

unext  (micronext) conditionally causes execution to loop back to slot 0 (and continue with consecutive slots).  When the loop is complete, execution continues with the slot following  unext .

The arrayForth compiler formats instruction words correctly and automatically.  If you wish to compile your own instruction words, to modify code in memory, or to write your own compiler for the F18A, please refer to section 2.4.4 for details about instruction encoding.

Stack Operation

In all cases of pushing and popping stacks mentioned above, the mechanism is the same:

Pushing data stack:  S is pushed onto the circular stack, replacing its bottom entry, and T is copied into S.

Popping data stack:  S is copied into T unless the instruction states that it sets T to some other value, and the top entry of the circular stack is copied into S.

Pushing return stack:  R is pushed onto the circular stack, replacing its bottom entry.

Popping return stack:  The top entry of the circular stack is copied into R.

Pushing and popping of the circular stacks is accomplished by adjusting pointers.  When popping a circular stack, the previous top item becomes the new bottom item.  When pushing onto a circular stack, the new top value replaces the previous bottom value.

Arithmetic, Logic and Register Manipulation

| Op | Source | Function |
| --- | --- | --- |
| x10 | +* | Multiply step. Signed Multiplicand is in S, unsigned multiplier in A, T=0 at start of a step sequence. Uses T:A as a 36-bit shift register with multiplier in A. Does the following: - If bit A0 is zero, shifts the 36-bit register T:A right one bit arithmetically (T17 is not changed and is copied into T16. T0 is copied to A17 and A0 is discarded.) - If bit A0 is one, S and T are added as though they had both been extended to be 19 bit signed numbers, and the 37-bit concatenation of this sum and A is shifted right one bit to replace T:A. Overflow may occur if S and T are both nonzero and their signs differ; this can only occur through improper initialization of T. This instruction is affected in Extended Arithmetic Mode by including the latched carry in the sum in case (2) above, and by latching the carry out from bit 17 of the sum, but this is not a particularly useful effect and may be changed in later F18 versions. You must arrange that the previously executed instruction has not changed the values of S, T or P9. Use nop preceding Multiply Step if necessary to meet this condition. |
| x11 | 2* | Two-Star. Shifts T left one bit logically (shifts zero into T0, discards T17) thus multiplying a signed or unsigned value by two. |
| x12 | 2/ | Two-Slash. Shifts T right one bit arithmetically (propagates T17 by leaving it unchanged; discards T0) thus dividing a signed value by two and discarding the positive remainder. |
| x13 | inv | Invert. Inverts each bit of T, replacing T with its ones complement. |
| x14 | + | Plus. Replaces T with the twos complement sum of S and T. Pops data stack into S. This instruction is affected in Extended Arithmetic Mode, becoming Add with carry. Includes the latched carry in the sum, and latches the carry out from bit 17. You must arrange that the previously executed instruction has not changed the values of S, T or P9. Use nop preceding Plus if necessary to meet this condition.However, if + is used in slot 3 of an instruction word, the (pre-)fetching of the next instruction word increases execution time of the slot 3 add so that it need not be preceded by nop. |
| x15 | and | Replaces T with the Boolean AND of S and T. Pops data stack. |
| x16 | xor | Exclusive Or. Replaces T with the Boolean XOR of S and T. Pops data stack. |
| x17 | drop | Drops the top item from the data stack by copying S into T and popping the data stack. |
| x18 | dup | Duplicates the top item on the data stack by pushing the data stack and copying T into S. |
| x19 | r> | Moves R into T, popping the return stack and pushing the data stack. |
| x1A | over | Makes a copy of S on top of the data stack by pushing S onto the stack, moving T into S, and replacing T by the previous value of S. |
| x1B | a | Fetches the contents of register A into T, pushing the data stack. |
| x1C | . | Nop. Spends time while making no explicit state changes in registers or stacks. |
| x1D | >r | Moves T into R, pushing the return stack and popping the data stack. |
| x1E | b! | B-Store. Stores T into register B, popping the data stack. |
| x1F | a! | A-Store. Stores T into register A, popping the data stack. |

Figure 5  Arithmetic, Logic and Register Operations

There are sixteen instructions in this class.  Approximate execution times (see section 2.4.1) are 1.5 nanoseconds.

Depending on their arguments, any of these may alter T and/or S, with the sole exception of  nop .

The carry latch is not affected by reset and its state on power-up is unpredictable.

Memory Read and Write

| Op | Source | Function |
| --- | --- | --- |
| x08 | @p | Fetch-P. Pushes data stack, reads [P] into T, and increments P (see 2.2) |
| x09 | @+ | Fetch-plus. Pushes data stack, reads [A] into T, and increments A (see 2.2) |
| x0A | @b | Fetch-B. Pushes data stack and reads [B] into T. |
| x0B | @ | Fetch. Pushes data stack and reads [A] into T. |
| x0C | !p | Store-P. Writes T into [P], pops the data stack, and increments P (see 2.2) |
| x0D | !+ | Store-plus. Writes T into [A], pops the data stack, and increments A (see 2.2) |
| x0E | !b | Store-B. Writes T into [B] and pops the data stack. |
| x0F | ! | Store. Writes T into [A] and pops the data stack. |

Figure 6  Memory Read and Write Instructions

There are eight instructions in this class.  Approximate execution times (see section 2.4.1) are 5.1 nanoseconds when reading or writing internal memory.  The addresses used may be in memory or I/O spaces; some I/O addresses may suspend execution until the conditions necessary for their completion are met.  The notation [X] should be read in the descriptions below as “the memory or I/O addressed by X”.

Transfer of Program Control

| Op | Source | Function |
| --- | --- | --- |
| x00 | ; | Return. Moves R into P, popping the return stack. Skips any remaining slots and fetches next instruction word. |
| x01 | ex | Execute. Exchanges R and P, skips any remaining slots and fetches next instruction word. |
| x02 | name ; | Jump. Sets P to destination address (see 2.3.1) and fetches next instruction word. |
| x03 | name | Call. Moves P into R, pushing an item onto the return stack, sets P to destination address (see 2.3.1) and fetches next instruction word. |
| x04 | unext | Micronext. If R is zero, pops the return stack and continues with the next opcode. If R is nonzero, decrements R by 1 and causes execution to continue with slot 0 of the current instruction word without re-fetching the word. |
| x05 | next | Next. If R is zero, pops the return stack and continues with the next instruction word addressed by P. If R is nonzero, decrements R by 1 and jumps (see 2.3.1). |
| x06 | if | If. If T is nonzero, continues with the next instruction word addressed by P. If T is zero, jumps (see 2.3.1). |
| x07 | -if | Minus-if. If T is negative (T17 set), continues with the next instruction word addressed by P. If T is positive, jumps (see 2.3.1). |

Figure 7  Control Transfer Instructions

There are eight instructions in this class.  Approximate execution times (see section 2.4.1) are 5.1 nanoseconds when the destination is in internal memory.  unext  takes approximately 2.0 nanoseconds.  When P is changed by one of these instructions, the new destination may be in memory or I/O space.  Some I/O addresses may suspend execution until the conditions necessary for their completion are met.

Fine Points

For full mastery of the F18A computer, there are several additional things you should understand.

Execution Time

The F18A is built from asynchronous logic, therefore its instruction times are naturally approximate.  The time required for all activity varies directly with temperature, inversely with power supply voltage (Vdd), and randomly within a statistical distribution due to variations in the process of fabricating the chips themselves.  Additional variations may occur with minor revisions in the version of F18 technology used by a particular chip.  Applications should not be designed to depend upon the speed of activities within F18 computers for timing purposes without taking all of these variables into consideration.

The time required for interaction with entities outside an F18 computer (other nodes, or I/O) naturally depends on the behavior of those outside devices.

The approximate execution times shown in the text above are typical of the F18A.  For any particular chip, see its Product Briefs and Data Books for pertinent preliminary information from prototype chips and, later, statistical data from measurements on production chips.

Prefetch considerations

Any F18A computer may, when it is logically possible to do so, be designed to fetch instructions before they are needed.  We reserve the right to make changes or improvements in this process with each chip we design, and therefore you are advised, in the interest of software portability, not to depend on any particular prefetching strategy for the timing of your program or for avoiding the need to insert a nop between an instruction that alters S, T or P9 and an add or multiply step instruction.  If your requirements justify sacrificing upward compatibility in favor of taking such steps to achieve maximum performance, contact our support staff (hotline mailbox) for recommendations pertinent to specific chips.

Best Practices

Within I/O space, read and write only those addresses defined to be readable or writable to avoid unexpected results.

Reading or writing inappropriate I/O addresses, or writing ROM, may result in a hung computer.

Upward compatibility to future versions of the F18 is not guaranteed.  Think in terms of source, not binary compatibility.  Be aware of the reset state of the  io  register since that will be the default setting for any bits assigned to control future I/O facilities.

Compiling Instructions Yourself

If you are writing a compiler, or wish to compose F18A instructions by hand or to read or modify instruction words in memory, you must be aware that instruction words are encoded as follows:

Compose instruction words as shown in 2.3.1 with opcode values as shown in the “Op” columns of the instruction tables above and exclusive OR the result with the value x15555.  If the word requires a destination address field, mask that field to zero and insert an un-encoded address value consistent with those shown in 2.2 and the specific I/O addresses defined in section 3.  In the particular case of slot zero jumps, it is permissible to set unused bits 10, 11 and 12 to any convenient states.

F18A I/O Facilities

| Name | Addr | Description | Notes |
| --- | --- | --- | --- |
| io | x15D | I/O control and status (see below). Bit assignments depend on the facilities present. |  |
| data | x141 | Up port with no handshake. | 3 |
| ---u | x145 | Up | 1 |
| ldata | x171 | Left port with no handshake. | 3 |
| --l- | x175 | Left | 2 |
| --lu | x165 | Left or Up | 1,2 |
| -d-- | x115 | Down |  |
| -d-u | x105 | Down or Up | 1 |
| -dl- | x135 | Down or Left | 2 |
| -dlu | x125 | Down, Left or Up | 1,2 |
| r--- | x1D5 | Right |  |
| r--u | x1C5 | Right or Up | 1 |
| r-l- | x1F5 | Right or Left | 2 |
| r-lu | x1E5 | Right, Left or Up | 1,2 |
| rd-- | x195 | Right or Down |  |
| rd-u | x185 | Right, Down or Up | 1 |
| rdl- | x1B5 | Right, Down or Left | 2 |
| rdlu | x1A5 | Right, Down, Left or Up | 1,2 |

Figure 8  I/O Address Table

Figure 8 shows all of the defined I/O addresses in the F18A.  Shaded addresses are not present in all nodes; see the numbered notes for more information.

Addresses should only be used in a given node if they are present according to these notes; right, down, left and up ports are present for read and write in all nodes except as noted:

Blank:	Present for read/write in all nodes.

- Absent on top and bottom nodes unless supported by particular I/O circuitry

- Absent on side nodes unless supported by particular I/O circuitry

- Present only when documented as such for specific I/O circuitry; read, write, and handshaking facilities are defined by each class of circuitry.

Input-output capabilities of F18A nodes consist of internode communication ports (2, 3, 4, or 5 depending on array configuration and the node’s position in the array) and, for nodes on array edges, optional GPIO pins and/or other more specialized circuitry such as parallel buses, analog I/O, or serializer-deserializer (SERDES).

The io Register

| Bit | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| reset | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| WRITE | GPIO pin 17 control |  |  |  |  |  | WD |  | phan 9 |  | phan 7 |  | GPIO pin 5 control |  | GPIO pin 3 control |  | GPIO pin 1 control |  |
| Alt write | SR |  |  | A/D mode |  | DB |  |  |  | 9-bit D/A value XORed with x155 |  |  |  |  |  |  |  |  |
| READ | pin 17 | Rr- | Rw | Dr- | Dw | Lr- | Lw | Ur- | Uw |  |  |  | pin 5 |  | pin 3 |  | pin 1 |  |

Figure 9  IO Register Bit Assignments

This 18-bit control and status register is each F18A computer’s interface with its I/O circuitry, if any, and with handshake lines for any communication ports that are present.  The bit assignments are shown for reference here.  Their semantics are defined in detail in later sections.  By default, io is populated with latches that read as the inverse of what was last written to them.  Any io bits that are undefined for a given node because the associated I/O facilities are absent implement this behavior.  A second general principle is that on reset each node’s io register is set as though the value x15555 had been written to it under program control.

Suspension

A node is either executing instructions at full speed or it is suspended in mid-instruction, waiting for a “wake up” condition that depends on the state of another node or of other I/O circuitry.  Since an F18 has no clock, the acts of suspending and waking up consume no energy or time in and of themselves.  Perception of changing conditions takes picoseconds within the chip and, typically, nanoseconds for events external to the chip.  When a node is suspended the power used by its computer is nil, on the order of 100 nanowatts.  When executing, an F18 computer uses power on the order of less than 6 milliwatts depending on the instructions used and the data on which they are operating.

Communication Ports

Communication (comm) ports provide for direct, point to point bidirectional information transfer within a chip.  Each F18A node may have from two to five ports connected.  Comm ports are named Right, Down, Left, Up; the fifth port is not implemented in F18A based chips.

Each comm port is an independent, bidirectional, half duplex 18-bit channel connecting two nodes.  Communication occurs when a sending node writes to a comm port address and a receiving node to which that port is connected reads the corresponding comm port address at the same time.  When a node operates on a port the data transfer occurs at approximately memory speed unless the other node connected to the port is not yet performing the complementary operation; in this case the operating node suspends, waking up when the other node connected to that port is performing the complementary operation.

The status of each port is visible through the bits in  io  labeled R/D/L/Ur- and R/D/L/Uw above.  Dr- is zero if the node at the other end of Down is reading; Lw is one if the node at the other end of Left is writing.

Multiport Operations

As shown in Figure 8 above, the I/O addressing scheme of the F18A permits comm ports to be combined.  Reading or writing Multiport Addresses such as rd-u is permitted and suspends until one or more of the nodes to which the selected ports connect is performing the complementary operation.  The design rules for communication flow using multiport operations are straightforward:  When a node is doing a multiport read, only one of the selected ports may be written by another node at any given time.  When a node is doing a multiport write, every node that intends to read the value written must already be doing so and suspended.  It is not possible to directly determine which port(s) transferred data after a multiport operation by inspection of  io; if it is necessary for a node to make this determination it must be accomplished via protocols.

Port Execution

An F18 may execute instruction streams directly from a comm port.  When the F18 fetches an instruction word from a comm port, P is not incremented and unless the instruction stream transfers control elsewhere subsequent words are fetched from the same port.  @p reads a word from the port and !p writes a word into the port.  Multiport addresses may be used so long as the design rules in the preceding section are followed.

Nodes with GPIO Pins

Figure 10  F18A GPIO Pin Circuit

Under program control, you may place each pin in one of four states by writing a value into its 2-bit GPIO field of io as follows:

| Value | State of pin drive circuitry |
| --- | --- |
| 00 | High Impedance (≈2.8 pF, ≈250 MΩ) |
| 01 | Weak pulldown (≈38 µA in saturation). This is the reset state of all GPIO pins. |
| 10 | Drive to Vss (≈41 mA in saturation) |
| 11 | Drive to Vdd (≈41 mA in saturation) |

Regardless of the drive state selected for a GPIO pin, you may always read the io register to measure the actual voltage on the pin as a logic state (1=pin voltage high) using bits 17, 5, 3 or 1

Any edge node may have up to four fully featured General Purpose I/O (GPIO) pins.  A schematic is shown in Figure 10.  These pins have useful electrical characteristics and are designed to facilitate software defined I/O.  GPIO pins are designated by their bit positions as shown in Figure 9 above (pins 17, 5, 3 and 1.)

Input hysteresis, provided by two Schmitt triggers, protects against noise and other classical problems with signals whose rise and fall times are long relative to computer speeds.  The F18A computers and I/O circuitry are fast and sensitive enough that parasitic resistance, inductance and capacitance effects may be directly observed by reading the pins as external circuits are being driven, and are fast enough that a 1 MHz sine wave is a slow-moving signal.  Refer to the documentation on specific chips for electrical characteristics of their GPIO pins.

Wakeup Pins

When an edge node is configured with a GPIO pin 17 it may be configured and documented for wakeup on that pin.  When this is the case, the pin may be “read” using a comm port address which becomes present for reading:  Up for nodes on the top and bottom edges of the array, or Left for nodes on the lateral sides.  The node suspends until the pin is in the state specified by the WD bit in the io register.  When WD is written as zero, which is the reset state of that bit, a pin read suspends until the pin is logically high.  To suspend until the pin is low, write 1 to WD.  When the read completes, it is required that you must immediately drop the value that was read into T.

F18A multiport read operations may include the wakeup pin as Up or Left.  In this case you must still immediately drop the value read.  To deduce that the pin was the source of the wakeup it is necessary to read io and check the current state of bit 17.

Shared (Read-only) Pins

Some nodes may be configured to observe the voltage on a GPIO pin that they do not directly control.  This is typically done to provide versatility in timing analog to digital conversion.  In these cases, the shared pin may be read as bit 17 of io.  Wakeup may be enabled for reading, writing, or both on a shared pin, as will be documented for the type of I/O node observing the pin.  Wakeup polarity is controlled by WD as with GPIO pins.  See the chip Data Book for details on each node's wakeup configuration as the choice of this configuration may depend on their intended uses.

Phantom Wakeup Signals

A sending node may be configured to drive a “phantom” signal that a receiving node treats as a shared pin.  In this configuration the receiving node operates exactly as described above in 3.4.2.  The sending node controls the phantom signal by writing into bits phan 7 or phan 9; writing 1 makes the signal look high to the receiver, 0 makes it look low, and the reset state of io is equivalent to writing zeroes (low) into these bits.

Parallel Bus Support

F18A Parallel Bus support consists of a cluster of three adjacent nodes, typically laid out on the “bottom” edge of a chip.  The center node has 4 GPIO pins for control signals and each of its parallel port neighbors has I/O circuitry for an 18-bit bidirectional bus interface.  The purposes of all these pins and the direction and timing of signals are defined by software.

The center node is configured to act as sender of a phantom wakeup signal to each of the parallel bus nodes to supplement the comm ports between them to provide versatility in timing.

In each node with an 18-bit bus, the addresses Up and Data are enabled for reading and writing.  The difference between these two addresses is that operations on Up suspend when appropriate (see 3.4.3) while operations on Data do not depend upon the phantom wakeup signal.

Each of the 18 pins of a parallel port node has a subset of the capabilities of a GPIO pin.  When DB in io is set to 1 (its reset state) all 18 pins are in output mode and are driven high or low depending on the state of an 18-bit Bus Register.  When DB is set to zero, all 18 pins are set to high impedance input mode.  Unlike GPIO pins, parallel bus pins do not have Schmitt triggers, nor do they have weak pull-down capabilities.

Writing to Up or Data always sets the Bus Register, regardless of the state of DB.  Writing a 1 or 0 to a bit in this register will, in output mode, drive the corresponding pin to Vdd or Vss, respectively.  Reading from Up or Data in input mode reads the pin states with high reading as 1; in output mode, reading returns the current value of the Bus Register.

Note that parallel buses reset to output mode and that reset does not initialize the Bus Register.  Its value after power up is unpredictable.

Refer to the documentation on specific chips for electrical characteristics of their parallel bus pins.

SERDES Nodes

The high speed Serializer-Deserializer (SERDES) node is equipped with a transceiver for 2-wire (clock and data) synchronous, half duplex communications at a rate of approximately 450 megabits per second.  While a SERDES node might be capable of communicating with something other than another SERDES node, it was not designed with any other objective than communication between our chips.

To receive an 18-bit word, a SERDES node must place the value x3FFFE in T and read the Up port.  The node suspends until a word has been received.

To begin transmitting, a SERDES node must write the first word to Data and must then set bit 17 of io to 1.  Each subsequent word is written to Up ; the node will suspend until the preceding word has been finished and transmission of the new word may begin.  If the node wishes to stop transmitting cleanly, it must wait long enough after writing the last word to Up (or after setting io bit 17 if only one word is to be transmitted) for a full word to have been sent.

Refer to the documentation on specific chips for electrical and timing characteristics of their SERDES nodes and pins.

Analog Nodes

The Analog to Digital converter (ADC) consists of a high speed, free running down-counter whose frequency varies as a function of input voltage between ≈3.6 GHz for Vdd input and ≈5.6 GHz for Vss, as shown in the typical curve in Figure 13.   The A/D Mode field in io controls the counter as follows:

| Value | Mode and input of ADC counter |
| --- | --- |
| 00 | High Impedance ADC input pad (≈2.8 pF, ≈250 MΩ) |
| 01 | Vdd Calibration |
| 10 | Counter disabled to save power (the reset state) |
| 11 | Vss Calibration |

A voltage is measured within the operating range (≈750 mV to ≈1.3v) by calculating the difference between two readings separated by a known time interval.

Figure 11  Analog I/O Circuitry

Any F18A edge node may be configured with an analog input pin and an analog output pin along with the associated circuitry.  A block diagram is shown in Figure 11.

For analog nodes on top and bottom edges, Data and Up are present for reading and writing; for those on sides of an array, Ldata and Left are present.  The procedure for sampling the ADC requires two steps:  First write to the Up/Left or Data/Ldata port as appropriate to signal the VCO to stop for reading, and then read from Data/Ldata or Up/Left as appropriate to obtain the value of the down-counter and restart the VCO.  Reading without writing first will not yield useful values.  An analog node is equipped with a shared wakeup pin (see 3.4.2) and configured to suspend when writing to Up/Left.

The Digital to Analog Converter (DAC) is a programmable current source designed to produce a voltage across a resistance to ground or to source an op-amp.  By writing the xor of a desired value and x155 into the low order nine bits of  io  that value controls the DAC output.  A value of x1FF (written as x0AA) sets the DAC for maximum current; a value of 0 (written as x155, the reset state) sets minimum current, high impedance output.  Typical DAC transfer functions, in mV versus DAC values, into 75, 50, 37.5 and 8 Ohms are shown in Figure 12; as the resistance decreases, the voltage decreases and the function becomes more linear.

Refer to the documentation on specific chips for electrical characteristics of their analog nodes.

Figure 12  Typical DAC Transfer Function

Figure 13  Typical ADC Transfer Function

Programming the F18

If you have not read DB001, the F18A Technology Reference, please do so to familiarize yourself with the computers and their instruction sets before reading this section.

Both environments include a resident Assembler for the F18 instruction set.  As usual, the F18 assembler is a "vocabulary engine" meaning that when the appropriate vocabulary is selected the F18 opcodes, directives, and user defined symbols are available.  On the Win32 saneFORTH system this is vocabulary number 9 while on pF/144 it is number 5.  The full Assembler environment also uses a special Interpreter to access user defined labels which are stored with the object code rather than in the local dictionary.

The assembler may be used in two distinct ways.  The most common is to generate code to reside in and execute from a node's RAM or ROM.  In this case code is laid down at consecutive addresses based on a location counter, and is assumed to execute at the addressing for which it was compiled, and in the node stated for things like the "warm" multiport execution address and for ROM content.

The second way is to generate an instruction word as a data item.  The usual reason for doing this is to make an instruction word that will be used in port execution.  In the past, it was often necessary to hand code such instructions and write them in hex; on the other hand, when it was possible to generate the instructions symbolically, the destination address checking for jumps and calls was based on memory location counter for the instruction stream in which the literal instruction was being stored, not on the port in which it would actually be executed.  This could generate instructions that would not work as expected when executed in the port.  This new method solves both problems.  By default, no location counter is assumed and the assembler will generate jumps and calls that don't depend on the execution address of the instruction word.  Directives may be used to specify an execution address (normally a port) for greater range of possible destinations at the expense of tailoring the instruction word for a particular port.

Other than the common essentials of Forth compilation technology, this environment includes the mechanism to compile code for hundreds of nodes, each of which has ROM that may be used by the application.

Object Code

The assembly process was monolithic in earlier versions of arrayForth.  As of aF-3 we return to incremental assembly, with object code stored in a binary file area from which it may be accessed to load nodes or chips.  This file area survives reboots so it is no longer necessary to reassemble all of the source code every time one needs any part of it.

A set of binary images, called  bins , reside on pF/144 mass storage (serial disk or flash in pF) starting at block  0bin  (4800*3 or 14400 by default).  Each bin is one BLOCK (1024 bytes long) and contains 256 bytes of RAM image (stored as 32-bit numbers), 256 of ROM, and a table of up to 50 labels for RAM and ROM code.  One bin exists for each physical node, and there are additional virtual bins for code that may be used by utilities or placed in different nodes for different applications.  The constant  nnc  gives the number of bins for which space is allocated.  By typing  nnc  you will see on the stack the value  576  which is 144*4; the area is sized for programs using up to two chips, with an extra 288 nodes' worth of virtual bins into which object (for library code distributed by GreenArrays) may be stashed as noted later; see 10.2 Bin Assignments below.

At present, sF's 0bin is at pF's 0bin+2400 on the serial disk image.  This is done to facilitate comparison of results between the two platforms.  When we decide that is no longer necessary, this will cease and both systems will use a single set of object bins.

Because the label tables are persistent in object bins, a programmer with a twisted mind can create much confusion by defining primary Assembler directives or FORTH words as labels, giving the appearance of a broken system.  If this has happened to you, either reassemble ROM for this node or fix it with the tools in the  EXAMINE  utility.

EXAMINE - Object Code Auditing

This minor utility for working with object code is useful during development.  It can examine bin label tables and bin content in binary or dis-assembled, including comparison with reference binaries.  Utilities are provided for initializing a set of bins and for moving quantities of bins.

Two sets of  nnc  bins are available to this utility.  The first, starting at block  0bin , is the object code produced by this machine's Assembler.  The second, starting at  CFORG , is a reference set produced by this or another system (such as colorForth).  Facilities are provided for comparing these and for displaying differences.

-ALL   erases all RAM and ROM object code for all  nnc  bins.  Additional words  -CH0  -CH1  -XTR  -XTR2  erase the four chip-size sections of the bin array.

?ROMS   compares ROM for chips 1 and 0 with reference and displays an array of status indicators for all nodes with with "." indicating empty node in both images, "=" indicating identical ROM content and a highlighted "?" indicating disagreement.

?RAMS   does the same for RAM object for pseudo-nodes 1600 thru 2317, chip 1 and chip 0.

USING (nn)   selects a particular bin for examination.

.RAM   dumps current RAM for selected bin, highlighting each word that differs from reference.

.CRAM   dumps reference RAM for selected bin, highlighting each word that differs from current.

.ROM   dumps current ROM for selected bin, highlighting each word that differs from reference.

.CROM   dumps reference ROM for selected bin, highlighting each word that differs from current.

-ASM (a n)   displays disassembly of the given address range (covering RAM and ROM) in the selected bin and annotating instruction words with labels.  Differences from the reference are highlighted and the corresponding reference disassembly is shown in a second column.

.SYM   displays the label table for the current bin.

COMMIT   burns current RAM for the selected bin into the corresponding reference image.  Use carefully!

In a normal development cycle where accumulated source changes are reconciled in the process of updating the backup, object can be audited in the same way.  To set the current object as reference, use  DISKING  to copy  nnc  blocks from  0bin  to  CFORG .

Getting Object Bins from colorForth

When converting an existing body of colorForth based code to aF-3, it is prudent to use the colorForth object as a reference.  The following steps accomplish this:

- In colorForth,   compile   all of the code of interest.

- Say  bnamed obj-cf  (or other name of your choice) to output file name.

- Say  32768 4800 wback  to write all the bins to a 4800-block file of the above name.  Only the first 864 blocks are relevant, but this simplifies file mapping into the sF system.

- Move this file into the  pf  directory

- On the sF system, in block 149 temporarily replace  OBJ-AF3  with your file's name,  FLUSH  and  RELOAD .

- See block 98.  This is a utility for converting your incoming file, mapped as  11 UNIT , into the aF-3 bin format as one of the sections of  OBJ-REF  which is mapped as  10 UNIT .  To see a text directory of these sections, list block  10 UNIT 60 -  Note that loading block 98 write enables 10 UNIT

- Select one of the 600-block reference slots for your object and maintain the directory accordingly.

- Load block 98 and say  n WAM  where  n  is the starting relative block (multiple of 600) for your data in the  OBJ-REF  file at  10 UNIT .

- In block 149, restore the OBJ-AF3 file name,  FLUSH  and  RELOAD .

- in aF-3, find the definition of  CFORG  and aim it at your new reference image.  FLUSH , RELOAD and proceed.

Assembling Stock Code

While incremental assembly is generally a good thing, it does make object code persistent and as a result a lapse in attention or a simple mistake in numbering can deposit garbage in a bin with lasting effects.

Any time the integrity of the object bin array is in doubt you may re-populate it with known good content on either platform as follows:

- Using the  DISKING  utility, verify the integrity of the aF-3 source code for tools and for all stock F18 code.

- Erase all bins by saying  EXAMINE LOAD  -ALL  .

- Reassemble all GreenArrays code by saying  STOCK LOAD

- Assemble any application code you have added.

- Verify object integrity with  EXAMINE LOAD  ?RAMS ?ROMS

Assembler Syntax and Semantics

F18 coding may be interspersed with high level polyFORTH coding; indeed, it may even be used to generate instruction words as literals within high level FORTH definitions.  This requires clear and explicit bounding between F18 coding and host system compilation / interpretation regimes.

To assemble F18 code simply interpret Forth that includes one of the code bounding words  ASM[  or  A[  when necessary.  The F18 assembler is resident in pF/144 and loaded as needed in saneFORTH.

Assembler State Variables

The Assembler's state is represented by five VARIABLEs:

IWD   18-bit instruction word being built.

'SLOT   The next unused slot [0..3] of the instruction in  IWD .  If 4, further ops are forbidden (inline).

'IW   holds the F18 address at which the instruction in  IWD  will be stored.  If negative, bits 10 and up are set, the low order bits [0..9] are the same as they are in  'IP .  In this case the instruction is being generated inline and will not be stored into a bin directly by the Assembler.

'IP   holds the address of the next word to store into target memory, corresponding with  p  register during execution.  If negative, bits 10 and up are set, and the value of  p  is unknown for inline assembly.  When an address is specified for inline assembly, the address is stored into the low order 10 bits of both  'IP  and  'IW , and bits 10 and up of  'IP  are zero.

'CL   holds the slot number of call opcode in preceding word.  Negative if there isn't one.  Used in tail optimization.  Second volatile cell is slot of last opcode stored.

'##   Volatile flag set zero by # and true after parsing a number, a named literal, or a named call.  When true these things perform their normal behaviors of generating literals or calls.  When false, each of them leaves a number on the Assembler's stack.

Assembler Directives for Code Bounding

Source code compatibility between 32-bit (Win32) and 16-bit (pF/144) host platforms when generating code for an 18-bit computer is, fortunately, simple to assure.  On both platforms anything being prepared to store into memory must be double precision.  All other values are single precision on both platforms.

A special Forth interpreter provides the dictionary searching mechanism to allow access to labels defined in the current or in other nodes' bins in both Win32 and pF/144 implementations.  In addition, this interpreter generates F18 literal references when naked numbers are encountered.

ASM   Selects the Assembler's vocabulary without invoking the full environment, used when necessary in tool building.

# (_ - n)  When running in the Assembler environment, as described below, most numbers occurring in the source code are intended to generate literals for the F18.  When it is necessary to provide a number as an argument to one of the Assembler directives, preceding it by  #  will leave the number on the stack rather than generating a literal.  # also conditions named values, such as the port names, to push values on the stack, and the same is true for named calls.  This usage is shown where appropriate in the following examples.

Directives are used to control the Assembler's assumptions and behavior in generating inline instructions or bins for code.  This section shows appropriate usage for each type of assembly.  In the examples, curly brackets  {  }  surround optional words or phrases, and the vertical bar  |  should be read as "or but not both".

      - Inline Instructions

To build an F18 instruction and leave it on the stack, the following pattern is used:

A[  {# <port> =P}  <opcodes>  ]]

By default, the instruction will be generated with no foreknowledge of the value in  P  during execution.  A port (or other) address may be specified using the  =P  directive.  The stack value produced will be double precision.  When used within F18 code the value may be laid down in the memory image being built using  ,  while when used in high level Forth code the value may be laid down in host memory using  I, .  Each of these words is defined appropriately in each version of the Assembler to compensate for host cell size.  The formal vocabulary is as follows:

A[   Enters the Assembler environment, saving its state and setting up for generation of a single instruction word.  Both interpretable and IMMEDIATE, usable in both  FORTH  and  Assembler environments to generate instruction words in-line for use as literals or in building tables.

]] (-d)   Exits that environment, returning a single instruction word (double precision) and restoring the Assembler's state as it was before  A[  was encountered.  When used in a high level FORTH definition, generates a 2LITERAL with the value of the instruction word.

=P (a)   Sets the assumed address in register  P  when this instruction is executed.  May only be used within an inline instruction definition.

, (d)  lays the double instruction or other value down at the next available address in F18 memory, used when the phrase  A[  ...  ]]  is nested within F18 code (see below).

I, (d)   lays the double instruction or other value down in host memory as a four-byte number, used when the phrase  A[  ...  ]]  occurs in high level FORTH code or data construction.

      - Building a bin

The bin mechanism allows ROM and RAM source code to be maintained and assembled separately.  Ideally assemble the ROM code only once, and incrementally assemble application RAM code as needed.  To build a bin of F18 code, the following general pattern is used:

ASM[  # ccyyxx NODE  {# ccyyxx BIN}  { {{-SYM}ERS}|{-ROM} }

# <addr> org  			always necessary

<coding>

FORTH ... ASM			simple vocabulary switch

]ASM   <host code>   ASM[	complete environment switch

A[ ... ]] {,|lit}		Generate instruction for table/lit

{>ROM}  >BIN  ]ASM

ASM[   Enters the full Assembler environment for F18A without initializing any of its state.  Visible in  FORTH.

]ASM   Exits that environment.

Within  ASM[  ]ASM  the  ASM  vocabulary is selected and a special interpreter runs.  As noted above in 4.2.2, this special interpreter recognizes the words in this section and by default generates opcodes in instruction words, literals and so on.  To put numbers on the Assembler's stack it is necessary to use  #  because if you simply write numbers down they will be assembled as literals, and if you write a label it will be called.  All numbers used with Assembler directives are of the width documented herein.  Numbers to be assembled as literals must be written in double precision form.

NODE (nn)   Specifies which node's position, ROM and RAM to assume, and by default set the target bin to the same.  Reads that node's bin into the local working image.  Required.

BIN (nn)   Overrides the target bin.  Optional.  Bin assignments are currently controlled by GreenArrays; at this time we recommend you compile directly for the intended nodes and not use  bin  yourself.

-SYM   Wipes ROM label table, used before  ERS  in nodes with many unnecessary ROM labels to recover space for application labels.

ERS   Wipes the object memory and label table for the RAM portion of the working image.  Does not touch ROM.

-ROM   Wipes the entire object memory and label tables in the working image.

>ROM   Secures the current label table as ROM labels in the working image.

>BIN   Writes the working image to the current target bin on mass storage.  To write identical content to multiple bins, use  BIN  >BIN  phrases repeatedly before  ]ASM .

reclaim   This deprecated word was used in most colorForth code to prevent crashes due to filling the colorForth dictionary.  Dictionary management in aF-3 uses conventional polyFORTH methods, applicable to any host code or data structures you might build.  The host dictionary is not used for F18 labels in this system.

Later on, when loading code into nodes with the IDE "by hand" or when specifying boot conditions for tools such as the automated IDE loader or the stream generator, object code is identified by its bin number.  By default that is simply the node number unless you have used  BIN  to stash the code elsewhere.

      - Special Interpreter Considerations

There is no problem with encapsulating multiple bins in a single source block, and in fact the source code distributed with aF-3 includes examples of doing this.  However, the special interpreter activated by  ASM[  requires slight changes in common sF/pF practices.

        - Use of EXIT

If  ASM[  interprets an  EXIT  the block continues being processed by the normal FORTH interpreter.  To achieve the usual effect of  EXIT  you must write it twice, as in  EXIT EXIT .

        - Bins Needing more than 1 Block

This can be done in two ways, and the code distributed with aF-3 includes examples.

For the first method, see the code for Async Boot ROM.  The first block loads the second within  ASM[  ]ASM  and the second block must begin with  ASM[  to activate the special interpreter.  Note that the second block does not require any closing bracketing other than an explicit or implicit pair of  EXIT .

For the second, see the Master DMA Nexus (bin 110) of the Ethernet NIC.  The first block begins with all of the normal boilerplate to start a capsule's source, but has no closing bracketing.  The second block begins with  ASM[  to resume using the special interpreter, and ends with the normal closing bracketing.

Location Counter

This is an incremental Assembler.  Opcodes and literals are written into an image of target node memory as they are encountered; the only retroactive action it performs is to store into the destination fields of words containing forward referencing jumps (such as  if ) or calls ( leap ) when those forward references are resolved, or to change a call opcode to a jump when a call is followed by semicolon (tail optimization).  The Assembler keeps track of the current position in target F18 memory with three variables documented earlier:  'IW  'SLOT  and  'IP .

As opcodes are assembled, they are added to the instruction word being built at the address in  'IW  and  'SLOT  is maintained until the word is full or until the next opcode will not fit into the word.  At that time the word being built is padded with  .  (nop) opcodes if necessary, and a new instruction word is started at the address in  'IP .  This leaves  'IW  pointing at the new instruction word and  'IP  pointing at the following location in memory.  The duality of  'IW  and  'SLOT  is necessary to support multiple instructions per word; the duality of  'IW  and  'IP  is necessary to support literals.

When literals are assembled, a  @p  opcode is generated and then the literal value is stored at  'IP , advancing  'IP .  Alternatively you may write  @p  yourself and lay the following words down using , (comma), such as values calculated interpretively or instruction words generated by the assembler's inline nesting feature.

Jumping, calling, and memory operations address a word, not an opcode.  When encountering a colon label, or any other assembler directive defining a place that may be addressed in memory, any code under construction is padded with  .  (nop) opcodes if necessary to align the location counter on a word boundary.  This means that in absence of explicit control transfer opcodes, execution continues across alignment boundaries including the start of a colon labeled definition, a technique we have learned to use often.

The location counter  'IP  is incremented in the same way the hardware increments  P :  The low order seven bits increment without changing the remaining bits of  P .  If you are generating code in RAM you will stay in RAM, wrapping its address space at x80 in terms of the value of  P  and also at x40 for actually addressing the memory image in the sense that the hardware ignores bit 6 of the address.  The compiler will also generate code for ROM, in which case the wrapping points are at x100 and xC0.  The P9 bit (x200) may be set as you wish to specify addressable destinations which will run in Extended Arithmetic Mode.

The location counter is managed using these words:

..   forces word alignment; if an instruction word has been started, fills the rest of the word with nops.  Equivalent to  1 <sl .

org (a)   forces word alignment then sets the compiler's location counter to a given address at which following code will be compiled into the current node's bin.  'IW  and  'IP  are initially equal but will separate after the first opcode has been compiled.

#  Instructs the Assembler that the following number (or label, named literal, named call, or use if  its  [reference to a label in another bin]) should leave a number on the stack instead of assembling a literal or call as appropriate.  For all but named literals the number will be single precision.  If you wish, for example, to  org  to a given address you need to write a phrase like  # x20 org  or  # joe org .  See also 4.2.2 above.

here (-a)   forces word alignment and returns the current aligned location.

, (d)  forces word alignment and lays the double instruction or other value down at the next available address in F18 memory, advancing location counter.

+cy   forces word alignment then turns P9 on in the location counter.  Places in memory subsequently defined will be run in Extended Arithmetic Mode if reached by jump, call, execute or return to those places.

-cy   forces word alignment then turns P9 off in the location counter.

<sl (n)   Ensures that the next slot to be assembled will be less than the given number, forcing alignment in a new word if that is not the case.  Used to solve problems with forward jumps or calls.

Slot assignment is a microscopic aspect of location counter management but is important to an F18 programmer for reasons such as optimizing forward references, aligning code on word boundaries for generation of literal instruction words to send another node through a com port, and the like.  The words  { .  ..  <sl } are your main tools for managing slot allocation.

Control Structure Directives

These are used like those in classical Forth.  The stack effects shown in square brackets reflect the host Assembler's stack; those shown in regular parens reflect the F18 stack at execution time.

      - Simple Forward Transfers

Forward transfer opcodes are assembled into the next available slot and may, unless you are controlling slot allocation yourself, be placed into slots 0, 1 or 2 with 10, 8 or 3-bit destination fields.  The stack "handle" shown as  sa  for the unresolved forward transfer identifies both the location and the slot of the transfer opcode as well as the adjustment for  P  due to any preceding  @p  or  !p  opcodes in the same word.  When  then  resolves a forward transfer, it will abort with error message "Range!" if the transfer is unable to reach the position at which  then  occurs without a larger destination field; when this occurs you must alter the code to resolve the problem.

if [-sa]  If  T  is nonzero, program flow continues; otherwise jumps to matching  then .

-if [-sa]  If  T  is negative, program flow continues; otherwise jumps to matching  then .

zif [-sa]  If  R  is zero, pops the return stack and program flow continues; otherwise decrements  R  and jumps to matching  then .

ahead [-sa]  jumps to matching  then .

leap [-sa]  assembles a call to matching  then .

then [sa]  forces word alignment and resolves a forward transfer.

      - Count-controlled Looping

The F18 hardware supports looping under control of a count in  R .  The number in  R  is zero-based so the number of iterations such a loop makes is one greater than the initial value in  R  and that value will be zero during the last iteration of a loop.  No forward transfers are used by these words and there are no issues with slots; all directives that generate backward transfers will pad the code if needed so that an opcode with the necessary size destination field may be assembled.  The directives are as follow:

for [-a] (n)  pushes  n  onto the return stack, forces word alignment and saves  here  to be used as a transfer destination by the directive that ends the loop.  There are times when it is useful to decompose this directive's actions so that the pushing of the loop count and the start of the loop itself may be separated by such things as initialization code or a label.  In this case you may write a phrase like  >r  <other things>  begin .

next [a]  ends a loop with conditional transfer to the address  a .  If  R  is zero when  next  is executed, the return stack is popped and program flow continues.  Otherwise  R  is decremented by one and control is transferred to  a .

unext [a]  ends a micronext loop.  Since the loop occurs entirely within a single instruction word, the address is superfluous; it is present only so that the form  <n> for … unext  may be written.  The micronext opcode may be compiled into any of the four slots.

      - Arbitrary Control Structures

As with ANS Forth, any desired control structure may be generated based on a few simple directives and flexible semantics; see the ANS Forth standard, or more to the point see the F18 code supplied with arrayForth, for many examples of composite control structures.  The following directives are provided; the same stack notation ( a  for destinations and  sa  for handles to forward references) is employed here.  If necessary you may code  SWAP  to affect the host Assembler's stack.  New words introduced in aF-3 are shown in green.

begin [-a]  forces word alignment and saves  here  to be used as a transfer destination.

while [x - sa x]  equivalent to  if SWAP .  Typically used as a conditional exit from within a loop.

-while [x - sa x]  equivalent to  -if SWAP .  Typically used as a conditional exit from within a loop.

until [a]  If  T  is nonzero, program flow continues; otherwise jumps to  a .  Typically used as a conditional exit at the end of a loop.

-until [a]  If  T  is negative, program flow continues; otherwise jumps to  a .  Used like  until .

again [a]  unconditionally jumps to  a .  The old  colorForth spelling  end  may also be used.

repeat [sa a]  unconditionally jumps to  a  and resolves the forward jump at  sa .  equivalent to  again then .

else [sa - sa]  jumps to matching  then  and resolves preceding forward transfer.

*next [sa x - x]  equivalent to  SWAP next  .

F18 Opcodes

The preferred opcode names, as shown in the F18A Technology Reference, each compile an opcode into a slot:

ex  @p @+ @b @  !p !+ !b !

+* 2* 2/ -  + and or drop  dup pop over a  . push b! a!

The  call  opcode is compiled when an F18 label is referenced.

lit [d]  generates a literal of the given value by inserting a @p opcode and laying the value down in memory.

alit [u]  generates a literal of the given unsigned single precision value by inserting a @p opcode and laying the value down in memory.

<a valid number>  encountered during assembly also generates a literal (in which case it must be written as double precision!) except when preceded by  #  in which case it leaves single or double precision on the Assembler stack as written.

S>D [n - d]  converts a signed number to double precision, sometimes necessary with other directives.

Tail optimization is performed by  ;  if immediately preceded by a call.  In this case, the call is converted into a jump, conserving return stack space and leading to other useful techniques.  Other jumps are generated by control structure directives, described later.

Four of the original opcode names assigned in colorForth, have proven to be poor human factors decisions because their names conflict with standard Forth usage and therefore create "programmer traps."  In aF-3 we have deprecated the four opcodes shown in red above to eliminate these "traps" and renamed them as follows:

inv   replaces  -  for a bitwise ones complement of T.

xor   replaces  or  for a bitwise exclusive OR of S and T.

r>   Replaces  pop  for moving a word from the return to the data stack.

>r   replaces  push  for moving a word from the data to the return stack.

A block of code is provided to add the old colorForth names to the Assembler if you truly wish to use them, however it is not resident by default to save memory.

By default, unused slots are set to return (;) opcodes for object compatibility with colorForth (and with the code in ROM on the chip.)  When the Assembler concludes that it must start a new instruction word within a code stream such that execution may continue from the preceding word into the new one, it must fill any unused slots in the preceding word with nops (.).  You may do this yourself, for example to lay down an inline machine code literal not encapsulated by  A[  and  ]]  with the  ..  directive that's described earlier.

Dictionary Labels

During assembly, a table of labels is built inside each bin image.  Each label is stored as a counted string with maximum of the first seven characters actually saved for uniqueness.  Each label has a 16-bit value.  The following words manage this dictionary:

equ (n _)   creates a new label whose name follows, assigning it the given value.

: (_)   forces word alignment and defines a label at  here  as did red words in colorForth.

its (nn _ - n)   normally assembles a  call  to the following label as defined in the given bin.  However when preceded by  #  the single precision value is placed on the Assembler's stack.

<valid label>   when encountered during assembly, a valid label defined in the working image assembles a  call  to that label.  However when preceded by  #  the single precision value of the label is left on the stack.

Writing a valid <label> is equivalent to writing  # <label> call .

Other Useful Words

Several additional resident definitions facilitate writing source code for F18s:

      - Named Literals

The following words, naming registers, normally assemble literals in the F18 instruction stream, however they simply leave their values (double precision) on the stack when preceded by  # .

io  right  down  left  up  data  ldata

      - Named Literals for Cardinal Directions

When developing systems for our chips, the "floorplan" of assignments for nodes and clusters of nodes often needs to be changed, resulting in moving nodes or clusters from one place to another.  When doing this, one nuisance is the need to change port names when moving a node(s) between odd and even rows or columns.  In addition to being a nuisance it's an excellent way to introduce bugs.  We address this at the levels of both the Assembler and of the Boot Descriptor Language.

The following words, naming registers by cardinal direction, normally assemble literals in the F18 instruction stream, however they simply leave their values (double precision) on the stack when preceded by  # .

east  south  west  north

      - Named Calls

Each of the 15 valid multiport addresses has a named word that normally assembles a  call  to that address.  However when preceded by  #  they simply leave their single precision addresses on the stack:

---u  --l-  --lu  -d--  -d-u  -dl-  -dlu

r---  r--u  r-l-  r-lu  rd--  rd-u  rdl-  rdlu

await  generates a call to the default multiport execute for a node based on its position in the array, also may be conditioned by  #  to return address on the stack.

Module Organization

A convenient convention has been adopted for organizing and packaging code in a modular way, from a single node to a cluster.  This convention allows use of a single block number or name as a "handle" for the module.  The first two or more blocks of the module have fixed functions at fixed offsets.  We recommend that you apply these principles in packaging your own code.  The SRAM Control Cluster Mark 1 may be studied as an example while reading the following sections; the constant  SRAM  is the number of the first block of that module.

Load Block

The first block in a module is a single block which when loaded will cause all of the source code needed by a module to be assembled.  There may be arguments to this block.  When completed the necessary object code will be stored in appropriate bins (which may simply be those belonging to the nodes programmed.)  If by its design a module needs to export addresses or other identifiers, these will be available in the dictionary of that node after it has been assembled.  In a degenerate case this block may actually contain all of the module's source code.  See block  SRAM  for an example.

      - Identifier Scope

F18 assembly is done incrementally, which means that identifiers may not be "forward referenced" symbolically.  Any identifier of any sort must be defined earlier in the assembly sequence than its references.

Boot Descriptors

The second block in a module (load block number plus one) defines the loading requirements for the module, using the high-level language described in section 6.1.1 below.  This language lists the nodes that have to be loaded, indicates what code to load into their RAM, provides for initialization of registers and/or stacks, and provides a starting execution address for each node.  The data are recorded in tables so that the order of declaration does not need to match the actual order in which the nodes are loaded.  See block  SRAM 1+  for an example.

Residual Paths

In rare cases there may be modules that need to be loaded, activated, and used during a boot sequence; SRAM and SDRAM control clusters are examples of these.  Once started it may be difficult or impossible to stop them without resetting the chip and, in the case of SDRAM, perhaps losing the data in the device.  If an application's boot process has this sort of complication then the module in question should provide a new path for use by stream or IDE loaders that can reach and boot all nodes remaining in the chip that are not part of the module which is running and hence now "in the way."  See block  sram 2+  for an example, showing suitable paths for boot nodes 708 (async serial) and 705 (SPI flash.)

Organization of Larger Projects

The appropriate structure for a larger program depends on its size and intended use.  The load block should still be first.  Boot descriptors should start in the second block but may require several.  If IDE operation is appropriate after the program has been loaded, an IDE personalization with path(s) adjusted to access the remainder of the chip may be useful.  Scripts may be necessary to specify boot stream generation, IDE loading, and/or softsim setup as appropriate.  Not all of these elements are appropriate for every application, but if they are small you might wish to place them in this area after the boot stream and residual path allocations, and before the actual source code.  These are merely suggestions and the recommendations may change as the system evolves.

Methods of Loading Code

After all the desired code has been compiled into object bins, it may be loaded for execution or simulation.  There are presently several basic ways in which this may be done:

Manual, interactive loading into the real chip with External or Internal IDE:  Insert code into RAM, do any necessary initialization manually, and call routines to observe behavior, test boundary conditions, look for side effects and so on.  It is impossible to over-emphasize the importance of this practice.  Simple, straightforward unit testing is a fundamental entitlement of a Forth programmer and its use dramatically simplifies debugging later on.  Don't deny yourself this advantage by combining a cluster of nodes full of untested code unless you enjoy coping with serious problems either immediately or some day in the future.

Automated loading into softsim:  Initialize nodes as directed by Boot Descriptors.

Automated loading into the real chip using boot streams:  Use the  STREAMER  utility to construct a boot stream as directed by Boot Descriptors, then present the resulting stream to a boot node or inject it into the chip using the Snorkel.

Fully automated loading into the real chip upon RESET- signal:  Use the  STREAMER  utility to construct a boot stream as directed by Boot Descriptors, then burn it into the front of SPI Flash.

Each of the automated methods uses the common Boot Descriptor Language to define what code, if any, shall be loaded and what additional initialization shall be performed before starting each node.

Which methods are appropriate depends on which host system you are running on and what connection you have to the target system, if any.  sF may use one or two serial COM ports connected to one or two ("host" and "target") chips.  pF/144 may use its 1.8V async interface from node 708 (requires jumpers and cables), or its 1.8V sync interface from node 300 either to the target chip on the same eval board (default jumpers), or to another chip (requires jumpers and cables).  pF/144 uses node 500 to drive the reset pin of the chip and this also requires attention to jumper and cable depending on the target chip.

pF/144 may also inject streams into the chip it's running on via the Snorkel in node 207, and by building a bridge these streams may continue into another chip.  For example, if you're running a basic polyFORTH system that doesn't have  the Ethernet NIC nor the external clock code booted into the chip's nodes, you can add this by loading block 1583.  This is a simple example of using STREAMER to build a boot stream that loads the Ethernet and crystal support, then injects that stream into the chip through the snorkel in node 207.

Boot Stream Load Blocks, in an earlier section, identify load blocks that may be used from saneFORTH for serial booting of chips, or from polyFORTH for building boot streams for insertion into flash or for interactive injection via node 207.  These blocks are present in both systems and are adapted as needed for use in each.  The serial booting mechanism from polyFORTH is not yet supported.

© 2010 GreenArrays, Incorporated.

Specifications are subject to change without notice.

GreenArrays, GreenArray Chips, arrayForth, and the GreenArrays logo are trademarks of GreenArrays, Inc.  All other trademarks or registered trademarks are the property of their respective owners.

© 2010 GreenArrays, Incorporated.

Specifications are subject to change without notice.

GreenArrays, GreenArray Chips, arrayForth, and the GreenArrays logo are trademarks of GreenArrays, Inc.  All other trademarks or registered trademarks are the property of their respective owners.