F18A Assembly Programming and polyForth Reference Guide

Introduction

The F18 technology consists of small, simple computers and several types of versatile, software defined I/O.  Our chips are configured as arrays of the F18 computers with appropriate selections of I/O types.

An F18 computer is a physically small, very fast, energy-efficient and inexpensive 18-bit machine.  Each F18 is self-contained as a complete system, with its own RAM, ROM, stacks, registers, and where appropriate external I/O.  The F18 computers in an array operate independently and asynchronously, communicating with each other using efficient internal communication ports.

This book will familiarize you with the architecture, internal structure, and use of the computers and I/O of chips built from F18A technology.  Please refer to the data books for specific chips using this technology to learn their exact configurations, pin-outs, ROM content and electrical characteristics.  The current editions of these, along with many other relevant documents and application notes as well as the current edition of this document, may be found on our website.  It is always advisable to ensure that you are using the latest documents before starting work.

Status of Data Given

The data given herein are PRODUCTION.  Electrical and timing values are illustrative only.  Otherwise, the subject matter of this book is the architecture, instruction set, and programming of the F18A computers and I/O, which are fixed by design.  Future revisions of this book may document upward-compatible enhancements of the F18A programming model.

Documentation Conventions

Numbers

Numbers are written in decimal unless otherwise indicated.  Hexadecimal values are indicated by explicitly writing “hex” or by preceding the number with the lowercase letter “x”.

Node coordinates

Each GreenArrays chip is a rectangular array of nodes, each of which is an F18 computer.  By convention these arrays are visualized as seen from the top of the silicon die, which is normally the top of the chip package, oriented such that pin 1 is in the upper left corner.  Within the array, each node is identified by a three or four digit number denoting its Cartesian coordinates within the array as yxx or yyxx with the lower left corner node always being designated as node 000.  Thus, for a GA144 chip whose computers are configured in an array of 18 columns and 8 rows, the numbers of the nodes in the lower right, upper left, and upper right corners are 017, 700, and 717 respectively.

Register names

Register names in prose may be used with or without the word "register" and are usually shown in a bold font and capitalized where necessary to avoid ambiguity, such as for example the registers T S R I A B and IO or io .

Bit Numbering

Binary numbers are visualized as a horizontal row of bits, numbered consecutively right to left in ascending significance with the least significant bit numbered zero.  Thus bit n has the binary value 2n.  The notation P9 means bit 9 of register P, whose binary value is x200, and T17 means the sign (high order) bit of 18-bit register T.

The F18A Computer

Here is an introduction to the resources within each F18A computer.  All elements are 18 bits wide unless otherwise noted.

Figure 1  F18A Block Diagram

RAM:  18 bits wide, 64 words in each node.

- ROM:  18 bits wide, 64 words in each node.

- I/O logic may be present on any edge node of a chip.

- Communication Ports are configured based on position within the array (four for inner; three ports array edges; two for corners.)  One additional port is optional.

- IO  control and status for communication ports and I/O logic.

- Return Stack consists of 8 registers addressed circularly (no fixed top or bottom.)  Pushing a word onto this stack makes it the new “top” item, overwriting the former “bottom” item.  After popping 8 items, those 8 items are repeated.

- R extends this stack to 9 entries but only 8 are circular.  When used as a loop counter, R decrements toward zero.

- P serves as a “program counter”, holding the address of the next word in the instruction stream.  Within RAM or ROM, it is incremented after each word is fetched, circularly within whichever storage class it currently points to.  When P addresses I/O space, it is not incremented.  Bit P9 enables Extended Arithmetic Mode when set.

- A is a general purpose read/write address or data register.

- B is a write only address register, containing on reset the address of  io  register.

- I is the instruction register into which instruction words containing 1, 2, 3 or 4 opcodes are fetched for execution.

- T is the top word of a 10-element data stack.

- The ALU (Arithmetic/Logic unit) performs unary operations on T, binary operations on S and T, or a multiply step involving S, T, and A as directed by opcodes.  A carry latch is involved in some Extended Arithmetic mode instructions.

- S is the second word of a 10-element data stack.

- Data Stack is 8 circularly addressed registers constituting the third through 10th elements of a 10-element stack.

After Reset

On chip reset, the following registers are initialized:

P is set to the address configured in the chip layout.  For F18A, this will always be either a multiport execute or will be ROM address x0aa.

io is set to the state it would have after a program wrote x15555 into the register.

B is set to the address of io.

Stack pointers are set to the same initial condition on every reset.

Other registers, stack contents, and RAM are not directly affected by reset.

Memory Addressing

| Start | End | Address Space |
| --- | --- | --- |
| x000 | x03F | RAM, 64 words |
| x040 | x07F | RAM x000-x03F repeated |
| x080 | x0BF | ROM, 64 words |
| x0C0 | x0FF | ROM x080-x0BF repeated |
| x100 | x1FF | I/O ports and registers |

Figure 2  Address Map

The F18A decodes a nine-bit memory address as shown in Figure 2.

Address incrementing, when applied to A or P, continues from x03F to x040 but wraps from x07F to x000.  The same process occurs with addresses in the ROM space.  Incrementing does not occur at all when the address lies in the region of I/O ports and registers.  Incrementing never affects bits P8 or P9.  P9 has no effect on memory decoding; it simply enables the Extended Arithmetic Mode when set.

Instruction Set

The F18 instruction set consists of 5-bit opcodes as summarized in Figure 3:

| Opcode |  | Notes | Opcode |  | Notes |
| --- | --- | --- | --- | --- | --- |
| ; | 00 | return | +* | 10 | multiply step |
| ex | 01 | execute (swap P and R) | 2* | 11 | left shift |
| name ; | 02 | jump to name | 2/ | 12 | right shift (signed) |
| name | 03 | call to name | inv | 13 | invert all bits (was -) |
| unext | 04 | loop within I (decrement R) | + | 14 | add (or add with carry) |
| next | 05 | loop to address (decrement R) | and | 15 |  |
| if | 06 | jump if T=0 | xor | 16 | exclusive or (was or) |
| -if | 07 | jump if T≥0 | drop | 17 |  |
| @p | 08 | literal (autoincrement P) | dup | 18 |  |
| @+ | 09 | fetch via A (autoincrement A) | r> | 19 | pop R push T (was pop) |
| @b | 0A | fetch via B | over | 1A |  |
| @ | 0B | fetch via A | a | 1B | fetch from register A |
| !p | 0C | store via P (autoincrement P) | . | 1C | nop |
| !+ | 0D | store via A (autoincrement A) | >r | 1D | pop T push R (was push) |
| !b | 0E | store via B | b! | 1E | store into register B |
| ! | 0F | store via A | a! | 1F | store into register A |

Figure 3  Table of Opcodes

The shaded opcodes may be used in slot 3 of an instruction word (see 2.3.1 below).  Add (+) and multiply step (+*) behave differently in Extended Arithmetic Mode.

Instruction Word Format

Each F18A instruction word is subdivided into four possible slots for opcodes that are normally executed sequentially left to right when viewing the word as conventionally oriented with most significant bit to the left.  Jump instructions may be written in slots 0, 1, or 2, and when they are encountered the remainder of the instruction word is interpreted as an address field.  This results in four possible instruction word formats as shown in Figure 4:

| Slot 0 |  |  |  |  | Slot 1 |  |  |  |  | Slot 2 |  |  |  |  | Slot 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instruction 0 |  |  |  |  | Instruction 1 |  |  |  |  | Instruction 2 |  |  |  |  | Instruction 3 |  |  |
| Instruction 0 |  |  |  |  | Instruction 1 |  |  |  |  | Jump instruction |  |  |  |  | Destination |  |  |
| Instruction 0 |  |  |  |  | Jump instruction |  |  |  |  | Destination |  |  |  |  |  |  |  |
| Jump instruction |  |  |  |  | Unused |  |  | Destination |  |  |  |  |  |  |  |  |  |
| 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

Figure 4  Instruction word formats

The n-bit destination address field simply replaces the n low order bits of the current (incremented) value of P at the time the jump instruction executes.  Thus the scope of a slot 2 jump is very limited while a slot 0 jump can reach any addressable destination, and can control P9 to explicitly enable or disable Extended Arithmetic Mode.  Slot 1 and 2 jumps have no effect upon P9, so its selection is unchanged as these jumps are executed.  In the F18A, slot 1 and 2 jumps additionally force P8 to zero.  This means the destination of any slot 1 or 2 jump may never be in I/O space.

The instruction selected for slot 3 must be one of those shaded in Figure 3 above.  When the desired opcode cannot be fit into the current instruction word, either because it may not be used in slot 3 or because it is a jump whose destination requires a longer address field, the remaining slots of the current word may be filled with nop instructions.

In addition to jumps, three other instructions affect the consecutive execution of opcodes within an instruction word:

;  (return) and  ex  (execute) discontinue execution of the current instruction word.

unext  (micronext) conditionally causes execution to loop back to slot 0 (and continue with consecutive slots).  When the loop is complete, execution continues with the slot following  unext .

The arrayForth compiler formats instruction words correctly and automatically.  If you wish to compile your own instruction words, to modify code in memory, or to write your own compiler for the F18A, please refer to section 2.4.4 for details about instruction encoding.

Stack Operation

In all cases of pushing and popping stacks mentioned above, the mechanism is the same:

Pushing data stack:  S is pushed onto the circular stack, replacing its bottom entry, and T is copied into S.

Popping data stack:  S is copied into T unless the instruction states that it sets T to some other value, and the top entry of the circular stack is copied into S.

Pushing return stack:  R is pushed onto the circular stack, replacing its bottom entry.

Popping return stack:  The top entry of the circular stack is copied into R.

Pushing and popping of the circular stacks is accomplished by adjusting pointers.  When popping a circular stack, the previous top item becomes the new bottom item.  When pushing onto a circular stack, the new top value replaces the previous bottom value.

Arithmetic, Logic and Register Manipulation

| Op | Source | Function |
| --- | --- | --- |
| x10 | +* | Multiply step. Signed Multiplicand is in S, unsigned multiplier in A, T=0 at start of a step sequence. Uses T:A as a 36-bit shift register with multiplier in A. Does the following: - If bit A0 is zero, shifts the 36-bit register T:A right one bit arithmetically (T17 is not changed and is copied into T16. T0 is copied to A17 and A0 is discarded.) - If bit A0 is one, S and T are added as though they had both been extended to be 19 bit signed numbers, and the 37-bit concatenation of this sum and A is shifted right one bit to replace T:A. Overflow may occur if S and T are both nonzero and their signs differ; this can only occur through improper initialization of T. This instruction is affected in Extended Arithmetic Mode by including the latched carry in the sum in case (2) above, and by latching the carry out from bit 17 of the sum, but this is not a particularly useful effect and may be changed in later F18 versions. You must arrange that the previously executed instruction has not changed the values of S, T or P9. Use nop preceding Multiply Step if necessary to meet this condition. |
| x11 | 2* | Two-Star. Shifts T left one bit logically (shifts zero into T0, discards T17) thus multiplying a signed or unsigned value by two. |
| x12 | 2/ | Two-Slash. Shifts T right one bit arithmetically (propagates T17 by leaving it unchanged; discards T0) thus dividing a signed value by two and discarding the positive remainder. |
| x13 | inv | Invert. Inverts each bit of T, replacing T with its ones complement. |
| x14 | + | Plus. Replaces T with the twos complement sum of S and T. Pops data stack into S. This instruction is affected in Extended Arithmetic Mode, becoming Add with carry. Includes the latched carry in the sum, and latches the carry out from bit 17. You must arrange that the previously executed instruction has not changed the values of S, T or P9. Use nop preceding Plus if necessary to meet this condition.However, if + is used in slot 3 of an instruction word, the (pre-)fetching of the next instruction word increases execution time of the slot 3 add so that it need not be preceded by nop. |
| x15 | and | Replaces T with the Boolean AND of S and T. Pops data stack. |
| x16 | xor | Exclusive Or. Replaces T with the Boolean XOR of S and T. Pops data stack. |
| x17 | drop | Drops the top item from the data stack by copying S into T and popping the data stack. |
| x18 | dup | Duplicates the top item on the data stack by pushing the data stack and copying T into S. |
| x19 | r> | Moves R into T, popping the return stack and pushing the data stack. |
| x1A | over | Makes a copy of S on top of the data stack by pushing S onto the stack, moving T into S, and replacing T by the previous value of S. |
| x1B | a | Fetches the contents of register A into T, pushing the data stack. |
| x1C | . | Nop. Spends time while making no explicit state changes in registers or stacks. |
| x1D | >r | Moves T into R, pushing the return stack and popping the data stack. |
| x1E | b! | B-Store. Stores T into register B, popping the data stack. |
| x1F | a! | A-Store. Stores T into register A, popping the data stack. |

Figure 5  Arithmetic, Logic and Register Operations

There are sixteen instructions in this class.  Approximate execution times (see section 2.4.1) are 1.5 nanoseconds.

Depending on their arguments, any of these may alter T and/or S, with the sole exception of  nop .

The carry latch is not affected by reset and its state on power-up is unpredictable.

Memory Read and Write

| Op | Source | Function |
| --- | --- | --- |
| x08 | @p | Fetch-P. Pushes data stack, reads [P] into T, and increments P (see 2.2) |
| x09 | @+ | Fetch-plus. Pushes data stack, reads [A] into T, and increments A (see 2.2) |
| x0A | @b | Fetch-B. Pushes data stack and reads [B] into T. |
| x0B | @ | Fetch. Pushes data stack and reads [A] into T. |
| x0C | !p | Store-P. Writes T into [P], pops the data stack, and increments P (see 2.2) |
| x0D | !+ | Store-plus. Writes T into [A], pops the data stack, and increments A (see 2.2) |
| x0E | !b | Store-B. Writes T into [B] and pops the data stack. |
| x0F | ! | Store. Writes T into [A] and pops the data stack. |

Figure 6  Memory Read and Write Instructions

There are eight instructions in this class.  Approximate execution times (see section 2.4.1) are 5.1 nanoseconds when reading or writing internal memory.  The addresses used may be in memory or I/O spaces; some I/O addresses may suspend execution until the conditions necessary for their completion are met.  The notation [X] should be read in the descriptions below as “the memory or I/O addressed by X”.

Transfer of Program Control

| Op | Source | Function |
| --- | --- | --- |
| x00 | ; | Return. Moves R into P, popping the return stack. Skips any remaining slots and fetches next instruction word. |
| x01 | ex | Execute. Exchanges R and P, skips any remaining slots and fetches next instruction word. |
| x02 | name ; | Jump. Sets P to destination address (see 2.3.1) and fetches next instruction word. |
| x03 | name | Call. Moves P into R, pushing an item onto the return stack, sets P to destination address (see 2.3.1) and fetches next instruction word. |
| x04 | unext | Micronext. If R is zero, pops the return stack and continues with the next opcode. If R is nonzero, decrements R by 1 and causes execution to continue with slot 0 of the current instruction word without re-fetching the word. |
| x05 | next | Next. If R is zero, pops the return stack and continues with the next instruction word addressed by P. If R is nonzero, decrements R by 1 and jumps (see 2.3.1). |
| x06 | if | If. If T is nonzero, continues with the next instruction word addressed by P. If T is zero, jumps (see 2.3.1). |
| x07 | -if | Minus-if. If T is negative (T17 set), continues with the next instruction word addressed by P. If T is positive, jumps (see 2.3.1). |

Figure 7  Control Transfer Instructions

There are eight instructions in this class.  Approximate execution times (see section 2.4.1) are 5.1 nanoseconds when the destination is in internal memory.  unext  takes approximately 2.0 nanoseconds.  When P is changed by one of these instructions, the new destination may be in memory or I/O space.  Some I/O addresses may suspend execution until the conditions necessary for their completion are met.

Fine Points

For full mastery of the F18A computer, there are several additional things you should understand.

Execution Time

The F18A is built from asynchronous logic, therefore its instruction times are naturally approximate.  The time required for all activity varies directly with temperature, inversely with power supply voltage (Vdd), and randomly within a statistical distribution due to variations in the process of fabricating the chips themselves.  Additional variations may occur with minor revisions in the version of F18 technology used by a particular chip.  Applications should not be designed to depend upon the speed of activities within F18 computers for timing purposes without taking all of these variables into consideration.

The time required for interaction with entities outside an F18 computer (other nodes, or I/O) naturally depends on the behavior of those outside devices.

The approximate execution times shown in the text above are typical of the F18A.  For any particular chip, see its Product Briefs and Data Books for pertinent preliminary information from prototype chips and, later, statistical data from measurements on production chips.

Prefetch considerations

Any F18A computer may, when it is logically possible to do so, be designed to fetch instructions before they are needed.  We reserve the right to make changes or improvements in this process with each chip we design, and therefore you are advised, in the interest of software portability, not to depend on any particular prefetching strategy for the timing of your program or for avoiding the need to insert a nop between an instruction that alters S, T or P9 and an add or multiply step instruction.  If your requirements justify sacrificing upward compatibility in favor of taking such steps to achieve maximum performance, contact our support staff (hotline mailbox) for recommendations pertinent to specific chips.

Best Practices

Within I/O space, read and write only those addresses defined to be readable or writable to avoid unexpected results.

Reading or writing inappropriate I/O addresses, or writing ROM, may result in a hung computer.

Upward compatibility to future versions of the F18 is not guaranteed.  Think in terms of source, not binary compatibility.  Be aware of the reset state of the  io  register since that will be the default setting for any bits assigned to control future I/O facilities.

Compiling Instructions Yourself

If you are writing a compiler, or wish to compose F18A instructions by hand or to read or modify instruction words in memory, you must be aware that instruction words are encoded as follows:

Compose instruction words as shown in 2.3.1 with opcode values as shown in the “Op” columns of the instruction tables above and exclusive OR the result with the value x15555.  If the word requires a destination address field, mask that field to zero and insert an un-encoded address value consistent with those shown in 2.2 and the specific I/O addresses defined in section 3.  In the particular case of slot zero jumps, it is permissible to set unused bits 10, 11 and 12 to any convenient states.

F18A I/O Facilities

| Name | Addr | Description | Notes |
| --- | --- | --- | --- |
| io | x15D | I/O control and status (see below). Bit assignments depend on the facilities present. |  |
| data | x141 | Up port with no handshake. | 3 |
| ---u | x145 | Up | 1 |
| ldata | x171 | Left port with no handshake. | 3 |
| --l- | x175 | Left | 2 |
| --lu | x165 | Left or Up | 1,2 |
| -d-- | x115 | Down |  |
| -d-u | x105 | Down or Up | 1 |
| -dl- | x135 | Down or Left | 2 |
| -dlu | x125 | Down, Left or Up | 1,2 |
| r--- | x1D5 | Right |  |
| r--u | x1C5 | Right or Up | 1 |
| r-l- | x1F5 | Right or Left | 2 |
| r-lu | x1E5 | Right, Left or Up | 1,2 |
| rd-- | x195 | Right or Down |  |
| rd-u | x185 | Right, Down or Up | 1 |
| rdl- | x1B5 | Right, Down or Left | 2 |
| rdlu | x1A5 | Right, Down, Left or Up | 1,2 |

Figure 8  I/O Address Table

Figure 8 shows all of the defined I/O addresses in the F18A.  Shaded addresses are not present in all nodes; see the numbered notes for more information.

Addresses should only be used in a given node if they are present according to these notes; right, down, left and up ports are present for read and write in all nodes except as noted:

Blank:	Present for read/write in all nodes.

- Absent on top and bottom nodes unless supported by particular I/O circuitry

- Absent on side nodes unless supported by particular I/O circuitry

- Present only when documented as such for specific I/O circuitry; read, write, and handshaking facilities are defined by each class of circuitry.

Input-output capabilities of F18A nodes consist of internode communication ports (2, 3, 4, or 5 depending on array configuration and the node’s position in the array) and, for nodes on array edges, optional GPIO pins and/or other more specialized circuitry such as parallel buses, analog I/O, or serializer-deserializer (SERDES).

The io Register

| Bit | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| reset | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| WRITE | GPIO pin 17 control |  |  |  |  |  | WD |  | phan 9 |  | phan 7 |  | GPIO pin 5 control |  | GPIO pin 3 control |  | GPIO pin 1 control |  |
| Alt write | SR |  |  | A/D mode |  | DB |  |  |  | 9-bit D/A value XORed with x155 |  |  |  |  |  |  |  |  |
| READ | pin 17 | Rr- | Rw | Dr- | Dw | Lr- | Lw | Ur- | Uw |  |  |  | pin 5 |  | pin 3 |  | pin 1 |  |

Figure 9  IO Register Bit Assignments

This 18-bit control and status register is each F18A computer’s interface with its I/O circuitry, if any, and with handshake lines for any communication ports that are present.  The bit assignments are shown for reference here.  Their semantics are defined in detail in later sections.  By default, io is populated with latches that read as the inverse of what was last written to them.  Any io bits that are undefined for a given node because the associated I/O facilities are absent implement this behavior.  A second general principle is that on reset each node’s io register is set as though the value x15555 had been written to it under program control.

Suspension

A node is either executing instructions at full speed or it is suspended in mid-instruction, waiting for a “wake up” condition that depends on the state of another node or of other I/O circuitry.  Since an F18 has no clock, the acts of suspending and waking up consume no energy or time in and of themselves.  Perception of changing conditions takes picoseconds within the chip and, typically, nanoseconds for events external to the chip.  When a node is suspended the power used by its computer is nil, on the order of 100 nanowatts.  When executing, an F18 computer uses power on the order of less than 6 milliwatts depending on the instructions used and the data on which they are operating.

Communication Ports

Communication (comm) ports provide for direct, point to point bidirectional information transfer within a chip.  Each F18A node may have from two to five ports connected.  Comm ports are named Right, Down, Left, Up; the fifth port is not implemented in F18A based chips.

Each comm port is an independent, bidirectional, half duplex 18-bit channel connecting two nodes.  Communication occurs when a sending node writes to a comm port address and a receiving node to which that port is connected reads the corresponding comm port address at the same time.  When a node operates on a port the data transfer occurs at approximately memory speed unless the other node connected to the port is not yet performing the complementary operation; in this case the operating node suspends, waking up when the other node connected to that port is performing the complementary operation.

The status of each port is visible through the bits in  io  labeled R/D/L/Ur- and R/D/L/Uw above.  Dr- is zero if the node at the other end of Down is reading; Lw is one if the node at the other end of Left is writing.

Multiport Operations

As shown in Figure 8 above, the I/O addressing scheme of the F18A permits comm ports to be combined.  Reading or writing Multiport Addresses such as rd-u is permitted and suspends until one or more of the nodes to which the selected ports connect is performing the complementary operation.  The design rules for communication flow using multiport operations are straightforward:  When a node is doing a multiport read, only one of the selected ports may be written by another node at any given time.  When a node is doing a multiport write, every node that intends to read the value written must already be doing so and suspended.  It is not possible to directly determine which port(s) transferred data after a multiport operation by inspection of  io; if it is necessary for a node to make this determination it must be accomplished via protocols.

Port Execution

An F18 may execute instruction streams directly from a comm port.  When the F18 fetches an instruction word from a comm port, P is not incremented and unless the instruction stream transfers control elsewhere subsequent words are fetched from the same port.  @p reads a word from the port and !p writes a word into the port.  Multiport addresses may be used so long as the design rules in the preceding section are followed.

Nodes with GPIO Pins

Figure 10  F18A GPIO Pin Circuit

Under program control, you may place each pin in one of four states by writing a value into its 2-bit GPIO field of io as follows:

| Value | State of pin drive circuitry |
| --- | --- |
| 00 | High Impedance (≈2.8 pF, ≈250 MΩ) |
| 01 | Weak pulldown (≈38 µA in saturation). This is the reset state of all GPIO pins. |
| 10 | Drive to Vss (≈41 mA in saturation) |
| 11 | Drive to Vdd (≈41 mA in saturation) |

Regardless of the drive state selected for a GPIO pin, you may always read the io register to measure the actual voltage on the pin as a logic state (1=pin voltage high) using bits 17, 5, 3 or 1

Any edge node may have up to four fully featured General Purpose I/O (GPIO) pins.  A schematic is shown in Figure 10.  These pins have useful electrical characteristics and are designed to facilitate software defined I/O.  GPIO pins are designated by their bit positions as shown in Figure 9 above (pins 17, 5, 3 and 1.)

Input hysteresis, provided by two Schmitt triggers, protects against noise and other classical problems with signals whose rise and fall times are long relative to computer speeds.  The F18A computers and I/O circuitry are fast and sensitive enough that parasitic resistance, inductance and capacitance effects may be directly observed by reading the pins as external circuits are being driven, and are fast enough that a 1 MHz sine wave is a slow-moving signal.  Refer to the documentation on specific chips for electrical characteristics of their GPIO pins.

Wakeup Pins

When an edge node is configured with a GPIO pin 17 it may be configured and documented for wakeup on that pin.  When this is the case, the pin may be “read” using a comm port address which becomes present for reading:  Up for nodes on the top and bottom edges of the array, or Left for nodes on the lateral sides.  The node suspends until the pin is in the state specified by the WD bit in the io register.  When WD is written as zero, which is the reset state of that bit, a pin read suspends until the pin is logically high.  To suspend until the pin is low, write 1 to WD.  When the read completes, it is required that you must immediately drop the value that was read into T.

F18A multiport read operations may include the wakeup pin as Up or Left.  In this case you must still immediately drop the value read.  To deduce that the pin was the source of the wakeup it is necessary to read io and check the current state of bit 17.

Shared (Read-only) Pins

Some nodes may be configured to observe the voltage on a GPIO pin that they do not directly control.  This is typically done to provide versatility in timing analog to digital conversion.  In these cases, the shared pin may be read as bit 17 of io.  Wakeup may be enabled for reading, writing, or both on a shared pin, as will be documented for the type of I/O node observing the pin.  Wakeup polarity is controlled by WD as with GPIO pins.  See the chip Data Book for details on each node's wakeup configuration as the choice of this configuration may depend on their intended uses.

Phantom Wakeup Signals

A sending node may be configured to drive a “phantom” signal that a receiving node treats as a shared pin.  In this configuration the receiving node operates exactly as described above in 3.4.2.  The sending node controls the phantom signal by writing into bits phan 7 or phan 9; writing 1 makes the signal look high to the receiver, 0 makes it look low, and the reset state of io is equivalent to writing zeroes (low) into these bits.

Parallel Bus Support

F18A Parallel Bus support consists of a cluster of three adjacent nodes, typically laid out on the “bottom” edge of a chip.  The center node has 4 GPIO pins for control signals and each of its parallel port neighbors has I/O circuitry for an 18-bit bidirectional bus interface.  The purposes of all these pins and the direction and timing of signals are defined by software.

The center node is configured to act as sender of a phantom wakeup signal to each of the parallel bus nodes to supplement the comm ports between them to provide versatility in timing.

In each node with an 18-bit bus, the addresses Up and Data are enabled for reading and writing.  The difference between these two addresses is that operations on Up suspend when appropriate (see 3.4.3) while operations on Data do not depend upon the phantom wakeup signal.

Each of the 18 pins of a parallel port node has a subset of the capabilities of a GPIO pin.  When DB in io is set to 1 (its reset state) all 18 pins are in output mode and are driven high or low depending on the state of an 18-bit Bus Register.  When DB is set to zero, all 18 pins are set to high impedance input mode.  Unlike GPIO pins, parallel bus pins do not have Schmitt triggers, nor do they have weak pull-down capabilities.

Writing to Up or Data always sets the Bus Register, regardless of the state of DB.  Writing a 1 or 0 to a bit in this register will, in output mode, drive the corresponding pin to Vdd or Vss, respectively.  Reading from Up or Data in input mode reads the pin states with high reading as 1; in output mode, reading returns the current value of the Bus Register.

Note that parallel buses reset to output mode and that reset does not initialize the Bus Register.  Its value after power up is unpredictable.

Refer to the documentation on specific chips for electrical characteristics of their parallel bus pins.

SERDES Nodes

The high speed Serializer-Deserializer (SERDES) node is equipped with a transceiver for 2-wire (clock and data) synchronous, half duplex communications at a rate of approximately 450 megabits per second.  While a SERDES node might be capable of communicating with something other than another SERDES node, it was not designed with any other objective than communication between our chips.

To receive an 18-bit word, a SERDES node must place the value x3FFFE in T and read the Up port.  The node suspends until a word has been received.

To begin transmitting, a SERDES node must write the first word to Data and must then set bit 17 of io to 1.  Each subsequent word is written to Up ; the node will suspend until the preceding word has been finished and transmission of the new word may begin.  If the node wishes to stop transmitting cleanly, it must wait long enough after writing the last word to Up (or after setting io bit 17 if only one word is to be transmitted) for a full word to have been sent.

Refer to the documentation on specific chips for electrical and timing characteristics of their SERDES nodes and pins.

Analog Nodes

The Analog to Digital converter (ADC) consists of a high speed, free running down-counter whose frequency varies as a function of input voltage between ≈3.6 GHz for Vdd input and ≈5.6 GHz for Vss, as shown in the typical curve in Figure 13.   The A/D Mode field in io controls the counter as follows:

| Value | Mode and input of ADC counter |
| --- | --- |
| 00 | High Impedance ADC input pad (≈2.8 pF, ≈250 MΩ) |
| 01 | Vdd Calibration |
| 10 | Counter disabled to save power (the reset state) |
| 11 | Vss Calibration |

A voltage is measured within the operating range (≈750 mV to ≈1.3v) by calculating the difference between two readings separated by a known time interval.

Figure 11  Analog I/O Circuitry

Any F18A edge node may be configured with an analog input pin and an analog output pin along with the associated circuitry.  A block diagram is shown in Figure 11.

For analog nodes on top and bottom edges, Data and Up are present for reading and writing; for those on sides of an array, Ldata and Left are present.  The procedure for sampling the ADC requires two steps:  First write to the Up/Left or Data/Ldata port as appropriate to signal the VCO to stop for reading, and then read from Data/Ldata or Up/Left as appropriate to obtain the value of the down-counter and restart the VCO.  Reading without writing first will not yield useful values.  An analog node is equipped with a shared wakeup pin (see 3.4.2) and configured to suspend when writing to Up/Left.

The Digital to Analog Converter (DAC) is a programmable current source designed to produce a voltage across a resistance to ground or to source an op-amp.  By writing the xor of a desired value and x155 into the low order nine bits of  io  that value controls the DAC output.  A value of x1FF (written as x0AA) sets the DAC for maximum current; a value of 0 (written as x155, the reset state) sets minimum current, high impedance output.  Typical DAC transfer functions, in mV versus DAC values, into 75, 50, 37.5 and 8 Ohms are shown in Figure 12; as the resistance decreases, the voltage decreases and the function becomes more linear.

Refer to the documentation on specific chips for electrical characteristics of their analog nodes.

Figure 12  Typical DAC Transfer Function

Figure 13  Typical ADC Transfer Function

Programming the F18

If you have not read DB001, the F18A Technology Reference, please do so to familiarize yourself with the computers and their instruction sets before reading this section.

Both environments include a resident Assembler for the F18 instruction set.  As usual, the F18 assembler is a "vocabulary engine" meaning that when the appropriate vocabulary is selected the F18 opcodes, directives, and user defined symbols are available.  On the Win32 saneFORTH system this is vocabulary number 9 while on pF/144 it is number 5.  The full Assembler environment also uses a special Interpreter to access user defined labels which are stored with the object code rather than in the local dictionary.

The assembler may be used in two distinct ways.  The most common is to generate code to reside in and execute from a node's RAM or ROM.  In this case code is laid down at consecutive addresses based on a location counter, and is assumed to execute at the addressing for which it was compiled, and in the node stated for things like the "warm" multiport execution address and for ROM content.

The second way is to generate an instruction word as a data item.  The usual reason for doing this is to make an instruction word that will be used in port execution.  In the past, it was often necessary to hand code such instructions and write them in hex; on the other hand, when it was possible to generate the instructions symbolically, the destination address checking for jumps and calls was based on memory location counter for the instruction stream in which the literal instruction was being stored, not on the port in which it would actually be executed.  This could generate instructions that would not work as expected when executed in the port.  This new method solves both problems.  By default, no location counter is assumed and the assembler will generate jumps and calls that don't depend on the execution address of the instruction word.  Directives may be used to specify an execution address (normally a port) for greater range of possible destinations at the expense of tailoring the instruction word for a particular port.

Other than the common essentials of Forth compilation technology, this environment includes the mechanism to compile code for hundreds of nodes, each of which has ROM that may be used by the application.

Object Code

The assembly process was monolithic in earlier versions of arrayForth.  As of aF-3 we return to incremental assembly, with object code stored in a binary file area from which it may be accessed to load nodes or chips.  This file area survives reboots so it is no longer necessary to reassemble all of the source code every time one needs any part of it.

A set of binary images, called  bins , reside on pF/144 mass storage (serial disk or flash in pF) starting at block  0bin  (4800*3 or 14400 by default).  Each bin is one BLOCK (1024 bytes long) and contains 256 bytes of RAM image (stored as 32-bit numbers), 256 of ROM, and a table of up to 50 labels for RAM and ROM code.  One bin exists for each physical node, and there are additional virtual bins for code that may be used by utilities or placed in different nodes for different applications.  The constant  nnc  gives the number of bins for which space is allocated.  By typing  nnc  you will see on the stack the value  576  which is 144*4; the area is sized for programs using up to two chips, with an extra 288 nodes' worth of virtual bins into which object (for library code distributed by GreenArrays) may be stashed as noted later; see 10.2 Bin Assignments below.

At present, sF's 0bin is at pF's 0bin+2400 on the serial disk image.  This is done to facilitate comparison of results between the two platforms.  When we decide that is no longer necessary, this will cease and both systems will use a single set of object bins.

Because the label tables are persistent in object bins, a programmer with a twisted mind can create much confusion by defining primary Assembler directives or FORTH words as labels, giving the appearance of a broken system.  If this has happened to you, either reassemble ROM for this node or fix it with the tools in the  EXAMINE  utility.

EXAMINE - Object Code Auditing

This minor utility for working with object code is useful during development.  It can examine bin label tables and bin content in binary or dis-assembled, including comparison with reference binaries.  Utilities are provided for initializing a set of bins and for moving quantities of bins.

Two sets of  nnc  bins are available to this utility.  The first, starting at block  0bin , is the object code produced by this machine's Assembler.  The second, starting at  CFORG , is a reference set produced by this or another system (such as colorForth).  Facilities are provided for comparing these and for displaying differences.

-ALL   erases all RAM and ROM object code for all  nnc  bins.  Additional words  -CH0  -CH1  -XTR  -XTR2  erase the four chip-size sections of the bin array.

?ROMS   compares ROM for chips 1 and 0 with reference and displays an array of status indicators for all nodes with with "." indicating empty node in both images, "=" indicating identical ROM content and a highlighted "?" indicating disagreement.

?RAMS   does the same for RAM object for pseudo-nodes 1600 thru 2317, chip 1 and chip 0.

USING (nn)   selects a particular bin for examination.

.RAM   dumps current RAM for selected bin, highlighting each word that differs from reference.

.CRAM   dumps reference RAM for selected bin, highlighting each word that differs from current.

.ROM   dumps current ROM for selected bin, highlighting each word that differs from reference.

.CROM   dumps reference ROM for selected bin, highlighting each word that differs from current.

-ASM (a n)   displays disassembly of the given address range (covering RAM and ROM) in the selected bin and annotating instruction words with labels.  Differences from the reference are highlighted and the corresponding reference disassembly is shown in a second column.

.SYM   displays the label table for the current bin.

COMMIT   burns current RAM for the selected bin into the corresponding reference image.  Use carefully!

In a normal development cycle where accumulated source changes are reconciled in the process of updating the backup, object can be audited in the same way.  To set the current object as reference, use  DISKING  to copy  nnc  blocks from  0bin  to  CFORG .

Getting Object Bins from colorForth

When converting an existing body of colorForth based code to aF-3, it is prudent to use the colorForth object as a reference.  The following steps accomplish this:

- In colorForth,   compile   all of the code of interest.

- Say  bnamed obj-cf  (or other name of your choice) to output file name.

- Say  32768 4800 wback  to write all the bins to a 4800-block file of the above name.  Only the first 864 blocks are relevant, but this simplifies file mapping into the sF system.

- Move this file into the  pf  directory

- On the sF system, in block 149 temporarily replace  OBJ-AF3  with your file's name,  FLUSH  and  RELOAD .

- See block 98.  This is a utility for converting your incoming file, mapped as  11 UNIT , into the aF-3 bin format as one of the sections of  OBJ-REF  which is mapped as  10 UNIT .  To see a text directory of these sections, list block  10 UNIT 60 -  Note that loading block 98 write enables 10 UNIT

- Select one of the 600-block reference slots for your object and maintain the directory accordingly.

- Load block 98 and say  n WAM  where  n  is the starting relative block (multiple of 600) for your data in the  OBJ-REF  file at  10 UNIT .

- In block 149, restore the OBJ-AF3 file name,  FLUSH  and  RELOAD .

- in aF-3, find the definition of  CFORG  and aim it at your new reference image.  FLUSH , RELOAD and proceed.

Assembling Stock Code

While incremental assembly is generally a good thing, it does make object code persistent and as a result a lapse in attention or a simple mistake in numbering can deposit garbage in a bin with lasting effects.

Any time the integrity of the object bin array is in doubt you may re-populate it with known good content on either platform as follows:

- Using the  DISKING  utility, verify the integrity of the aF-3 source code for tools and for all stock F18 code.

- Erase all bins by saying  EXAMINE LOAD  -ALL  .

- Reassemble all GreenArrays code by saying  STOCK LOAD

- Assemble any application code you have added.

- Verify object integrity with  EXAMINE LOAD  ?RAMS ?ROMS

Assembler Syntax and Semantics

F18 coding may be interspersed with high level polyFORTH coding; indeed, it may even be used to generate instruction words as literals within high level FORTH definitions.  This requires clear and explicit bounding between F18 coding and host system compilation / interpretation regimes.

To assemble F18 code simply interpret Forth that includes one of the code bounding words  ASM[  or  A[  when necessary.  The F18 assembler is resident in pF/144 and loaded as needed in saneFORTH.

Assembler State Variables

The Assembler's state is represented by five VARIABLEs:

IWD   18-bit instruction word being built.

'SLOT   The next unused slot [0..3] of the instruction in  IWD .  If 4, further ops are forbidden (inline).

'IW   holds the F18 address at which the instruction in  IWD  will be stored.  If negative, bits 10 and up are set, the low order bits [0..9] are the same as they are in  'IP .  In this case the instruction is being generated inline and will not be stored into a bin directly by the Assembler.

'IP   holds the address of the next word to store into target memory, corresponding with  p  register during execution.  If negative, bits 10 and up are set, and the value of  p  is unknown for inline assembly.  When an address is specified for inline assembly, the address is stored into the low order 10 bits of both  'IP  and  'IW , and bits 10 and up of  'IP  are zero.

'CL   holds the slot number of call opcode in preceding word.  Negative if there isn't one.  Used in tail optimization.  Second volatile cell is slot of last opcode stored.

'##   Volatile flag set zero by # and true after parsing a number, a named literal, or a named call.  When true these things perform their normal behaviors of generating literals or calls.  When false, each of them leaves a number on the Assembler's stack.

Assembler Directives for Code Bounding

Source code compatibility between 32-bit (Win32) and 16-bit (pF/144) host platforms when generating code for an 18-bit computer is, fortunately, simple to assure.  On both platforms anything being prepared to store into memory must be double precision.  All other values are single precision on both platforms.

A special Forth interpreter provides the dictionary searching mechanism to allow access to labels defined in the current or in other nodes' bins in both Win32 and pF/144 implementations.  In addition, this interpreter generates F18 literal references when naked numbers are encountered.

ASM   Selects the Assembler's vocabulary without invoking the full environment, used when necessary in tool building.

# (_ - n)  When running in the Assembler environment, as described below, most numbers occurring in the source code are intended to generate literals for the F18.  When it is necessary to provide a number as an argument to one of the Assembler directives, preceding it by  #  will leave the number on the stack rather than generating a literal.  # also conditions named values, such as the port names, to push values on the stack, and the same is true for named calls.  This usage is shown where appropriate in the following examples.

Directives are used to control the Assembler's assumptions and behavior in generating inline instructions or bins for code.  This section shows appropriate usage for each type of assembly.  In the examples, curly brackets  {  }  surround optional words or phrases, and the vertical bar  |  should be read as "or but not both".

      - Inline Instructions

To build an F18 instruction and leave it on the stack, the following pattern is used:

A[  {# <port> =P}  <opcodes>  ]]

By default, the instruction will be generated with no foreknowledge of the value in  P  during execution.  A port (or other) address may be specified using the  =P  directive.  The stack value produced will be double precision.  When used within F18 code the value may be laid down in the memory image being built using  ,  while when used in high level Forth code the value may be laid down in host memory using  I, .  Each of these words is defined appropriately in each version of the Assembler to compensate for host cell size.  The formal vocabulary is as follows:

A[   Enters the Assembler environment, saving its state and setting up for generation of a single instruction word.  Both interpretable and IMMEDIATE, usable in both  FORTH  and  Assembler environments to generate instruction words in-line for use as literals or in building tables.

]] (-d)   Exits that environment, returning a single instruction word (double precision) and restoring the Assembler's state as it was before  A[  was encountered.  When used in a high level FORTH definition, generates a 2LITERAL with the value of the instruction word.

=P (a)   Sets the assumed address in register  P  when this instruction is executed.  May only be used within an inline instruction definition.

, (d)  lays the double instruction or other value down at the next available address in F18 memory, used when the phrase  A[  ...  ]]  is nested within F18 code (see below).

I, (d)   lays the double instruction or other value down in host memory as a four-byte number, used when the phrase  A[  ...  ]]  occurs in high level FORTH code or data construction.

      - Building a bin

The bin mechanism allows ROM and RAM source code to be maintained and assembled separately.  Ideally assemble the ROM code only once, and incrementally assemble application RAM code as needed.  To build a bin of F18 code, the following general pattern is used:

ASM[  # ccyyxx NODE  {# ccyyxx BIN}  { {{-SYM}ERS}|{-ROM} }

# <addr> org  			always necessary

<coding>

FORTH ... ASM			simple vocabulary switch

]ASM   <host code>   ASM[	complete environment switch

A[ ... ]] {,|lit}		Generate instruction for table/lit

{>ROM}  >BIN  ]ASM

ASM[   Enters the full Assembler environment for F18A without initializing any of its state.  Visible in  FORTH.

]ASM   Exits that environment.

Within  ASM[  ]ASM  the  ASM  vocabulary is selected and a special interpreter runs.  As noted above in 4.2.2, this special interpreter recognizes the words in this section and by default generates opcodes in instruction words, literals and so on.  To put numbers on the Assembler's stack it is necessary to use  #  because if you simply write numbers down they will be assembled as literals, and if you write a label it will be called.  All numbers used with Assembler directives are of the width documented herein.  Numbers to be assembled as literals must be written in double precision form.

NODE (nn)   Specifies which node's position, ROM and RAM to assume, and by default set the target bin to the same.  Reads that node's bin into the local working image.  Required.

BIN (nn)   Overrides the target bin.  Optional.  Bin assignments are currently controlled by GreenArrays; at this time we recommend you compile directly for the intended nodes and not use  bin  yourself.

-SYM   Wipes ROM label table, used before  ERS  in nodes with many unnecessary ROM labels to recover space for application labels.

ERS   Wipes the object memory and label table for the RAM portion of the working image.  Does not touch ROM.

-ROM   Wipes the entire object memory and label tables in the working image.

>ROM   Secures the current label table as ROM labels in the working image.

>BIN   Writes the working image to the current target bin on mass storage.  To write identical content to multiple bins, use  BIN  >BIN  phrases repeatedly before  ]ASM .

reclaim   This deprecated word was used in most colorForth code to prevent crashes due to filling the colorForth dictionary.  Dictionary management in aF-3 uses conventional polyFORTH methods, applicable to any host code or data structures you might build.  The host dictionary is not used for F18 labels in this system.

Later on, when loading code into nodes with the IDE "by hand" or when specifying boot conditions for tools such as the automated IDE loader or the stream generator, object code is identified by its bin number.  By default that is simply the node number unless you have used  BIN  to stash the code elsewhere.

      - Special Interpreter Considerations

There is no problem with encapsulating multiple bins in a single source block, and in fact the source code distributed with aF-3 includes examples of doing this.  However, the special interpreter activated by  ASM[  requires slight changes in common sF/pF practices.

        - Use of EXIT

If  ASM[  interprets an  EXIT  the block continues being processed by the normal FORTH interpreter.  To achieve the usual effect of  EXIT  you must write it twice, as in  EXIT EXIT .

        - Bins Needing more than 1 Block

This can be done in two ways, and the code distributed with aF-3 includes examples.

For the first method, see the code for Async Boot ROM.  The first block loads the second within  ASM[  ]ASM  and the second block must begin with  ASM[  to activate the special interpreter.  Note that the second block does not require any closing bracketing other than an explicit or implicit pair of  EXIT .

For the second, see the Master DMA Nexus (bin 110) of the Ethernet NIC.  The first block begins with all of the normal boilerplate to start a capsule's source, but has no closing bracketing.  The second block begins with  ASM[  to resume using the special interpreter, and ends with the normal closing bracketing.

Location Counter

This is an incremental Assembler.  Opcodes and literals are written into an image of target node memory as they are encountered; the only retroactive action it performs is to store into the destination fields of words containing forward referencing jumps (such as  if ) or calls ( leap ) when those forward references are resolved, or to change a call opcode to a jump when a call is followed by semicolon (tail optimization).  The Assembler keeps track of the current position in target F18 memory with three variables documented earlier:  'IW  'SLOT  and  'IP .

As opcodes are assembled, they are added to the instruction word being built at the address in  'IW  and  'SLOT  is maintained until the word is full or until the next opcode will not fit into the word.  At that time the word being built is padded with  .  (nop) opcodes if necessary, and a new instruction word is started at the address in  'IP .  This leaves  'IW  pointing at the new instruction word and  'IP  pointing at the following location in memory.  The duality of  'IW  and  'SLOT  is necessary to support multiple instructions per word; the duality of  'IW  and  'IP  is necessary to support literals.

When literals are assembled, a  @p  opcode is generated and then the literal value is stored at  'IP , advancing  'IP .  Alternatively you may write  @p  yourself and lay the following words down using , (comma), such as values calculated interpretively or instruction words generated by the assembler's inline nesting feature.

Jumping, calling, and memory operations address a word, not an opcode.  When encountering a colon label, or any other assembler directive defining a place that may be addressed in memory, any code under construction is padded with  .  (nop) opcodes if necessary to align the location counter on a word boundary.  This means that in absence of explicit control transfer opcodes, execution continues across alignment boundaries including the start of a colon labeled definition, a technique we have learned to use often.

The location counter  'IP  is incremented in the same way the hardware increments  P :  The low order seven bits increment without changing the remaining bits of  P .  If you are generating code in RAM you will stay in RAM, wrapping its address space at x80 in terms of the value of  P  and also at x40 for actually addressing the memory image in the sense that the hardware ignores bit 6 of the address.  The compiler will also generate code for ROM, in which case the wrapping points are at x100 and xC0.  The P9 bit (x200) may be set as you wish to specify addressable destinations which will run in Extended Arithmetic Mode.

The location counter is managed using these words:

..   forces word alignment; if an instruction word has been started, fills the rest of the word with nops.  Equivalent to  1 <sl .

org (a)   forces word alignment then sets the compiler's location counter to a given address at which following code will be compiled into the current node's bin.  'IW  and  'IP  are initially equal but will separate after the first opcode has been compiled.

#  Instructs the Assembler that the following number (or label, named literal, named call, or use if  its  [reference to a label in another bin]) should leave a number on the stack instead of assembling a literal or call as appropriate.  For all but named literals the number will be single precision.  If you wish, for example, to  org  to a given address you need to write a phrase like  # x20 org  or  # joe org .  See also 4.2.2 above.

here (-a)   forces word alignment and returns the current aligned location.

, (d)  forces word alignment and lays the double instruction or other value down at the next available address in F18 memory, advancing location counter.

+cy   forces word alignment then turns P9 on in the location counter.  Places in memory subsequently defined will be run in Extended Arithmetic Mode if reached by jump, call, execute or return to those places.

-cy   forces word alignment then turns P9 off in the location counter.

<sl (n)   Ensures that the next slot to be assembled will be less than the given number, forcing alignment in a new word if that is not the case.  Used to solve problems with forward jumps or calls.

Slot assignment is a microscopic aspect of location counter management but is important to an F18 programmer for reasons such as optimizing forward references, aligning code on word boundaries for generation of literal instruction words to send another node through a com port, and the like.  The words  { .  ..  <sl } are your main tools for managing slot allocation.

Control Structure Directives

These are used like those in classical Forth.  The stack effects shown in square brackets reflect the host Assembler's stack; those shown in regular parens reflect the F18 stack at execution time.

      - Simple Forward Transfers

Forward transfer opcodes are assembled into the next available slot and may, unless you are controlling slot allocation yourself, be placed into slots 0, 1 or 2 with 10, 8 or 3-bit destination fields.  The stack "handle" shown as  sa  for the unresolved forward transfer identifies both the location and the slot of the transfer opcode as well as the adjustment for  P  due to any preceding  @p  or  !p  opcodes in the same word.  When  then  resolves a forward transfer, it will abort with error message "Range!" if the transfer is unable to reach the position at which  then  occurs without a larger destination field; when this occurs you must alter the code to resolve the problem.

if [-sa]  If  T  is nonzero, program flow continues; otherwise jumps to matching  then .

-if [-sa]  If  T  is negative, program flow continues; otherwise jumps to matching  then .

zif [-sa]  If  R  is zero, pops the return stack and program flow continues; otherwise decrements  R  and jumps to matching  then .

ahead [-sa]  jumps to matching  then .

leap [-sa]  assembles a call to matching  then .

then [sa]  forces word alignment and resolves a forward transfer.

      - Count-controlled Looping

The F18 hardware supports looping under control of a count in  R .  The number in  R  is zero-based so the number of iterations such a loop makes is one greater than the initial value in  R  and that value will be zero during the last iteration of a loop.  No forward transfers are used by these words and there are no issues with slots; all directives that generate backward transfers will pad the code if needed so that an opcode with the necessary size destination field may be assembled.  The directives are as follow:

for [-a] (n)  pushes  n  onto the return stack, forces word alignment and saves  here  to be used as a transfer destination by the directive that ends the loop.  There are times when it is useful to decompose this directive's actions so that the pushing of the loop count and the start of the loop itself may be separated by such things as initialization code or a label.  In this case you may write a phrase like  >r  <other things>  begin .

next [a]  ends a loop with conditional transfer to the address  a .  If  R  is zero when  next  is executed, the return stack is popped and program flow continues.  Otherwise  R  is decremented by one and control is transferred to  a .

unext [a]  ends a micronext loop.  Since the loop occurs entirely within a single instruction word, the address is superfluous; it is present only so that the form  <n> for … unext  may be written.  The micronext opcode may be compiled into any of the four slots.

      - Arbitrary Control Structures

As with ANS Forth, any desired control structure may be generated based on a few simple directives and flexible semantics; see the ANS Forth standard, or more to the point see the F18 code supplied with arrayForth, for many examples of composite control structures.  The following directives are provided; the same stack notation ( a  for destinations and  sa  for handles to forward references) is employed here.  If necessary you may code  SWAP  to affect the host Assembler's stack.  New words introduced in aF-3 are shown in green.

begin [-a]  forces word alignment and saves  here  to be used as a transfer destination.

while [x - sa x]  equivalent to  if SWAP .  Typically used as a conditional exit from within a loop.

-while [x - sa x]  equivalent to  -if SWAP .  Typically used as a conditional exit from within a loop.

until [a]  If  T  is nonzero, program flow continues; otherwise jumps to  a .  Typically used as a conditional exit at the end of a loop.

-until [a]  If  T  is negative, program flow continues; otherwise jumps to  a .  Used like  until .

again [a]  unconditionally jumps to  a .  The old  colorForth spelling  end  may also be used.

repeat [sa a]  unconditionally jumps to  a  and resolves the forward jump at  sa .  equivalent to  again then .

else [sa - sa]  jumps to matching  then  and resolves preceding forward transfer.

*next [sa x - x]  equivalent to  SWAP next  .

F18 Opcodes

The preferred opcode names, as shown in the F18A Technology Reference, each compile an opcode into a slot:

ex  @p @+ @b @  !p !+ !b !

+* 2* 2/ -  + and or drop  dup pop over a  . push b! a!

The  call  opcode is compiled when an F18 label is referenced.

lit [d]  generates a literal of the given value by inserting a @p opcode and laying the value down in memory.

alit [u]  generates a literal of the given unsigned single precision value by inserting a @p opcode and laying the value down in memory.

<a valid number>  encountered during assembly also generates a literal (in which case it must be written as double precision!) except when preceded by  #  in which case it leaves single or double precision on the Assembler stack as written.

S>D [n - d]  converts a signed number to double precision, sometimes necessary with other directives.

Tail optimization is performed by  ;  if immediately preceded by a call.  In this case, the call is converted into a jump, conserving return stack space and leading to other useful techniques.  Other jumps are generated by control structure directives, described later.

Four of the original opcode names assigned in colorForth, have proven to be poor human factors decisions because their names conflict with standard Forth usage and therefore create "programmer traps."  In aF-3 we have deprecated the four opcodes shown in red above to eliminate these "traps" and renamed them as follows:

inv   replaces  -  for a bitwise ones complement of T.

xor   replaces  or  for a bitwise exclusive OR of S and T.

r>   Replaces  pop  for moving a word from the return to the data stack.

>r   replaces  push  for moving a word from the data to the return stack.

A block of code is provided to add the old colorForth names to the Assembler if you truly wish to use them, however it is not resident by default to save memory.

By default, unused slots are set to return (;) opcodes for object compatibility with colorForth (and with the code in ROM on the chip.)  When the Assembler concludes that it must start a new instruction word within a code stream such that execution may continue from the preceding word into the new one, it must fill any unused slots in the preceding word with nops (.).  You may do this yourself, for example to lay down an inline machine code literal not encapsulated by  A[  and  ]]  with the  ..  directive that's described earlier.

Dictionary Labels

During assembly, a table of labels is built inside each bin image.  Each label is stored as a counted string with maximum of the first seven characters actually saved for uniqueness.  Each label has a 16-bit value.  The following words manage this dictionary:

equ (n _)   creates a new label whose name follows, assigning it the given value.

: (_)   forces word alignment and defines a label at  here  as did red words in colorForth.

its (nn _ - n)   normally assembles a  call  to the following label as defined in the given bin.  However when preceded by  #  the single precision value is placed on the Assembler's stack.

<valid label>   when encountered during assembly, a valid label defined in the working image assembles a  call  to that label.  However when preceded by  #  the single precision value of the label is left on the stack.

Writing a valid <label> is equivalent to writing  # <label> call .

Other Useful Words

Several additional resident definitions facilitate writing source code for F18s:

      - Named Literals

The following words, naming registers, normally assemble literals in the F18 instruction stream, however they simply leave their values (double precision) on the stack when preceded by  # .

io  right  down  left  up  data  ldata

      - Named Literals for Cardinal Directions

When developing systems for our chips, the "floorplan" of assignments for nodes and clusters of nodes often needs to be changed, resulting in moving nodes or clusters from one place to another.  When doing this, one nuisance is the need to change port names when moving a node(s) between odd and even rows or columns.  In addition to being a nuisance it's an excellent way to introduce bugs.  We address this at the levels of both the Assembler and of the Boot Descriptor Language.

The following words, naming registers by cardinal direction, normally assemble literals in the F18 instruction stream, however they simply leave their values (double precision) on the stack when preceded by  # .

east  south  west  north

      - Named Calls

Each of the 15 valid multiport addresses has a named word that normally assembles a  call  to that address.  However when preceded by  #  they simply leave their single precision addresses on the stack:

---u  --l-  --lu  -d--  -d-u  -dl-  -dlu

r---  r--u  r-l-  r-lu  rd--  rd-u  rdl-  rdlu

await  generates a call to the default multiport execute for a node based on its position in the array, also may be conditioned by  #  to return address on the stack.

Module Organization

A convenient convention has been adopted for organizing and packaging code in a modular way, from a single node to a cluster.  This convention allows use of a single block number or name as a "handle" for the module.  The first two or more blocks of the module have fixed functions at fixed offsets.  We recommend that you apply these principles in packaging your own code.  The SRAM Control Cluster Mark 1 may be studied as an example while reading the following sections; the constant  SRAM  is the number of the first block of that module.

Load Block

The first block in a module is a single block which when loaded will cause all of the source code needed by a module to be assembled.  There may be arguments to this block.  When completed the necessary object code will be stored in appropriate bins (which may simply be those belonging to the nodes programmed.)  If by its design a module needs to export addresses or other identifiers, these will be available in the dictionary of that node after it has been assembled.  In a degenerate case this block may actually contain all of the module's source code.  See block  SRAM  for an example.

      - Identifier Scope

F18 assembly is done incrementally, which means that identifiers may not be "forward referenced" symbolically.  Any identifier of any sort must be defined earlier in the assembly sequence than its references.

Boot Descriptors

The second block in a module (load block number plus one) defines the loading requirements for the module, using the high-level language described in section 6.1.1 below.  This language lists the nodes that have to be loaded, indicates what code to load into their RAM, provides for initialization of registers and/or stacks, and provides a starting execution address for each node.  The data are recorded in tables so that the order of declaration does not need to match the actual order in which the nodes are loaded.  See block  SRAM 1+  for an example.

Residual Paths

In rare cases there may be modules that need to be loaded, activated, and used during a boot sequence; SRAM and SDRAM control clusters are examples of these.  Once started it may be difficult or impossible to stop them without resetting the chip and, in the case of SDRAM, perhaps losing the data in the device.  If an application's boot process has this sort of complication then the module in question should provide a new path for use by stream or IDE loaders that can reach and boot all nodes remaining in the chip that are not part of the module which is running and hence now "in the way."  See block  sram 2+  for an example, showing suitable paths for boot nodes 708 (async serial) and 705 (SPI flash.)

Organization of Larger Projects

The appropriate structure for a larger program depends on its size and intended use.  The load block should still be first.  Boot descriptors should start in the second block but may require several.  If IDE operation is appropriate after the program has been loaded, an IDE personalization with path(s) adjusted to access the remainder of the chip may be useful.  Scripts may be necessary to specify boot stream generation, IDE loading, and/or softsim setup as appropriate.  Not all of these elements are appropriate for every application, but if they are small you might wish to place them in this area after the boot stream and residual path allocations, and before the actual source code.  These are merely suggestions and the recommendations may change as the system evolves.

Methods of Loading Code

After all the desired code has been compiled into object bins, it may be loaded for execution or simulation.  There are presently several basic ways in which this may be done:

Manual, interactive loading into the real chip with External or Internal IDE:  Insert code into RAM, do any necessary initialization manually, and call routines to observe behavior, test boundary conditions, look for side effects and so on.  It is impossible to over-emphasize the importance of this practice.  Simple, straightforward unit testing is a fundamental entitlement of a Forth programmer and its use dramatically simplifies debugging later on.  Don't deny yourself this advantage by combining a cluster of nodes full of untested code unless you enjoy coping with serious problems either immediately or some day in the future.

Automated loading into softsim:  Initialize nodes as directed by Boot Descriptors.

Automated loading into the real chip using boot streams:  Use the  STREAMER  utility to construct a boot stream as directed by Boot Descriptors, then present the resulting stream to a boot node or inject it into the chip using the Snorkel.

Fully automated loading into the real chip upon RESET- signal:  Use the  STREAMER  utility to construct a boot stream as directed by Boot Descriptors, then burn it into the front of SPI Flash.

Each of the automated methods uses the common Boot Descriptor Language to define what code, if any, shall be loaded and what additional initialization shall be performed before starting each node.

Which methods are appropriate depends on which host system you are running on and what connection you have to the target system, if any.  sF may use one or two serial COM ports connected to one or two ("host" and "target") chips.  pF/144 may use its 1.8V async interface from node 708 (requires jumpers and cables), or its 1.8V sync interface from node 300 either to the target chip on the same eval board (default jumpers), or to another chip (requires jumpers and cables).  pF/144 uses node 500 to drive the reset pin of the chip and this also requires attention to jumper and cable depending on the target chip.

pF/144 may also inject streams into the chip it's running on via the Snorkel in node 207, and by building a bridge these streams may continue into another chip.  For example, if you're running a basic polyFORTH system that doesn't have  the Ethernet NIC nor the external clock code booted into the chip's nodes, you can add this by loading block 1583.  This is a simple example of using STREAMER to build a boot stream that loads the Ethernet and crystal support, then injects that stream into the chip through the snorkel in node 207.

Boot Stream Load Blocks, in an earlier section, identify load blocks that may be used from saneFORTH for serial booting of chips, or from polyFORTH for building boot streams for insertion into flash or for interactive injection via node 207.  These blocks are present in both systems and are adapted as needed for use in each.  The serial booting mechanism from polyFORTH is not yet supported.

© 2010 GreenArrays, Incorporated.

Specifications are subject to change without notice.

GreenArrays, GreenArray Chips, arrayForth, and the GreenArrays logo are trademarks of GreenArrays, Inc.  All other trademarks or registered trademarks are the property of their respective owners.

© 2010 GreenArrays, Incorporated.

Specifications are subject to change without notice.

GreenArrays, GreenArray Chips, arrayForth, and the GreenArrays logo are trademarks of GreenArrays, Inc.  All other trademarks or registered trademarks are the property of their respective owners.

Interactive Testing

Interactivity is a critical, cardinal virtue of Forth as a programming system.  Turning Forth into a batch mode programming system would be a travesty, sacrificing one of its most potent properties and setting the clock back nearly half a century.  When using Forth, a programmer becomes accustomed to having her fingers literally on the fabric of the computer, directly manipulating its memory, registers, and other resources; exercising hardware and software without necessarily having to write (and debug) software simply to exercise those things.

The IDE was implemented as a natural and obvious extension of the umbilical methods used with embedded Forth systems for many decades.  The method used is as non-invasive of hardware and software as is practical in this architecture, requiring no RAM or ROM in a node being tested and touching its registers as lightly as feasible.

With aF-3 there are two different environments.  External IDE allows a host system to manipulates a target chip using synchronous or asynchronous serial interfaces as appropriate; this environment has been in use with arrayForth since the dawn of GreenArrays.  Internal IDE is new with aF-3 and employs the Snorkel/Ganglia mechanism to permit a pF/144 system running on a G144A12 chip to interact in analogous ways with other nodes on the same chip as pF/144 is running on (even some nodes that are part of the pF Virtual Machine!) and, if the chip is bridged to another, on a second chip as well.  There is a technical App Note AN019 on the IDE internals.  This section covers the use of both environments.

External IDE

This utility is implemented for both sF and pF/144.

Terminology

Debugging is done on one or more target nodes, using an umbilical connection which will involve one or more intermediate nodes of one to three kinds (root, wire, and end), depending on the physical location of the target node within the chip and on the path taken to reach it.  For IDE into the asynchronous interfaces of a GA144 including either of the chips on an EVB001 board, the root node for either chip will always be 708 because that is the one and only asynch boot node on a G144A12.  The  asynch IDE may thus be used to debug code in any node except 708.

The inner load block for the asynchronous IDE is  SER  but this is intended to be used as a factor of other utilities.  To work with the host chip, say  HOST LOAD  and to work with the target chip say  TARGET LOAD  after remembering to edit  A-COM A-BPS C-COM  and  C-BPS  (in CONFIG and SERIAL, or in config.txt if you are using a project directory) to define the correct COM ports and line speeds.

Using the External IDE

- Assemble the necessary code into bins and make any necessary connections to the target system.  Set serial port numbers as needed.  sF ONLY:  Connect at least one COM port (usually high speed FTDI) to the target system.  If this is a new port, configure it correctly as described in the Appendix applicable to your platform.  Edit the main configuration blocks,  CONFIG and SERIAL , or the  config.txt  file if you are in a project, to reflect this port number as host or target chip, and select a baud rate that works for the electrical interface in use.  Repeat this process if you are using two connections.

- Examine and, if necessary, change the path lists to make sure you can reach all the places you need to.  See the definitions of  0PA  1PA  2PA  after the block named  SER  for examples.

- Make sure nothing else in the windows environment (including dialog boxes) has any open handles for these device(s).

- To work with the host chip, say  HOST LOAD  on either platform.  sF ONLY:  To work with the target chip say  TARGET LOAD .

- Say  TALK  to reset the target chip (may be prevented) and download root node talker code into the chip.  On some target boards, you may need to cycle power or press a reset button before this step.  If the serial port is already set up and you do not wish to reset the chip, use  Talk  instead.

- Say  .PTH  for a concise display of IDE state.  You should at this point have three defined paths, one to each of the nodes immediately adjacent to the root node, for example:

.PTH

2  709    1 709

1  608    1 608

0  707    1 707  <--- SELECTED

The first column is path number; the second shows which direction it goes (first node away from the root).  Third column shows the number of COM port boundaries between the root and the target node, and the fourth column is the target node to which the path is currently connected.

- To see more about the target node use  SEE  which displays the path state as well as the stack and RAM in the target node.

- Define, tear down, and select paths as you wish, and operate on the nodes in question using the words described below.  Note:  If you wish to prove that all other bootable nodes are completely idle, begin your activities by saying  2 <root> HOOK   (for example, 2 708 HOOK when using async IDE) and then  2 –HOOK  to tear that path down.  The default path 2 can reach all 143 nodes accessible to the IDE.

- Operate on the selected target node as you wish using the basic or enhanced vocabulary.

- If you wish to change the code you are downloading into any node(s), change the F18 source and load it to update the object bin(s).  IDE state is not affected by normal assembly.  Any paths you had wired up, and whichever you had most recently selected, will still be in effect as you can see by displaying  PANEL  again.  Assembly does not communicate with the chip under test.

- When you are finished you may depart the IDE like any other FORTH utility by typing  EMPTY .

External IDE Vocabulary

Words come in two classes:  Those which wire, rip and select paths, and those that operate on the target node.

      - Path Routing Control

To see how the path routing is currently set up, say  .PTH .  Node numbers used with these words are always in the cyyxx form.

PATH (i)   Selects path  i  (0, 1, or 2) so that all subsequent target operations will apply to the target node for that path.  This is necessary when more than one path leads to the same target node.

NODE (nn)  Selects whichever path presently has node  n  as its target, if any.  Note that it is possible to set up more than one path to different ports of the same node; if you have done this,  PATH  will permit you to select the desired port.  Leaves a default path selected if none of them targets the node given.

HOOK (i nn)  Wires path  i  to target node  n  after ripping out any existing wiring for that path, leaving path  i  selected.  If the node in question is not accessible in the route list for that path, leaves the path set for the appropriate adjacent node as target.

-HOOK (i)  Forces ripping out of any wiring for path  i .  Each node that had previously been part of this path is normally left in its default  warm  state (multiport execute); the target node is not affected.

Path  Node  Hook  -Hook   These words do the same things, but silently.

PTB (i-a)  an array of starting addresses indexed by path number.  To adopt a new path, store its address into this table at a time when that path index has been unhooked.

      - Target Operations

Each of these operations applies to the target node of the currently selected path.

UPD   retrieves all ten words of the data stack into the local array  STACK  which is indexed (T, S, and the eight elements in the F18 stack, first element being the one that would next be popped into S).  Displays the stack conditionally (using  ?STK .)

.STK  displays the contents of  STACK  in two rows, in the current  BASE .  First row is the deepest four elements of the F18 stack, followed by the shallower four elements, then S and T.  The shallowest element is immediately left of S; “deeper” moves to the left on that row, then to the right on the row above, with the deepest element on the right side of the top row.  Thus, the eight words of the stack array seem to move circularly.  This is isomorphic with the stack array and makes its behavior clearer.

.STK                      vv--Next Push from S

2AAAA  2AAAA     15     15     (S)    (T)

2AAAA  2AAAA  2AAAA  2ABAA     15     15

^^--Next Pop to S

?STK  displays the stack as does  .STK  if  ?MUTE  is zero.

?RAM  displays all of RAM from the selected node (in the current  BASE ).

?ROM  displays all of ROM from the selected node (in the current  BASE ).

-ASM (a n)  disassembles  n  words of code starting at  a  in ROM or RAM.  Labels are integrated from whatever bin is current.

LIT (d)  removes a double number from the host computer's stack, pushing it onto the data stack of the target node and displaying the updated stack if not muted.

R@ (a-d)  reads RAM, ROM, register or port word at  a  in the target node, pushing the data read,  d  onto the host computer's stack.  In this release, uses and does not restore register  a  in the target node.

R! (d a)   writes a double number from the host computer's stack into target memory (RAM, register, port) word at address  a .  In this release, uses and does not restore register  a  in the target node.

RINS (d)  Executes a given instruction word in the port of the target node.  Usually built with  A[ .. ; ]]

CALL (a)  calls, from the port, the code at address  a  in the target.  If that code returns to the port or re-establishes the normal rest state of the target node, interactive use may continue once the code completes.  No interlocking is done so this method may also be used to start application processing which will not admit to further port execution access.  Interaction with a node actively running an application may also be arranged but at the cost of periodically polling, as described below.

BOOT (a n nn)  loads code into the target node, starting at address  a  in both target and bin for  n  words, from the current binary output area identified by  nn .  This need not be the same node as the target.

FOCUS  forces the target to call only the port on which the current IDE path is talking to it.  This remains in effect until the target is directed to execute elsewhere.

VIRGIN  forces the target to call its default multiport execution address to un-do the effects of  FOCUS  .

IO  DATA  UP  DOWN  LDATA  LEFT  RIGHT  place chip port addresses on the host's stack.

RB!  @B  !B  RA@  RA!  @A  !A  @+  !+  R+  R+*  R2*  R2/  RINV  RAND  ROR  RDROP  RDUP  ROVER  Execute single F18 instructions in target, using target's stack, and update stack display in panel.  Note:  These functions may be omitted in favor of  A[ opcodes ; ]] RINS

Advanced External IDE Uses

When ripping a path out, all wire and end nodes (if any) are left in their default multiport executes.  This is necessary so that, for example, paths may be crossed so long as they are not used concurrently to cause conflicts.  There are occasions, such as when starting nodes that send unsolicited code or data to other nodes, when this is not appropriate.  In those cases the following two functions are useful:

UNFOC  Conditions  -hook  to work as stated above..

FOC  Conditions  -hook  to leave every wire and end node focused back toward the root.  This leaves the path completely impenetrable to anyone other than the root node, and further hooking and starting of nodes will be necessary if that path is ever to be permeable again.

The IDE is factored such that it may become a tool of other utilities.  Examples are the  SELFTEST  and  AUTOTEST  routines, the IDE boot and flash burning tools used for polyFORTH, and even the  HOST  and  TARGET  load blocks.  Until these conventions are fully documented here, please read the code just identified to see how it is done.  In the meanwhile here are general points:

- HOST begins by loading the necessary code, and then sets up the communication environment.

- If the IDE is to be used in an automated fashion, it's usually correct to set ?MUTE nonzero.  This will prevent chatty displays of the routing every time it's changed, and will allow use of remote instructions without updating the stack display gratuitously.

Configuring IDE for Target Environments

As shipped, the IDE is configured for the environment of the EVB002 Evaluation Board.  This will do for many other circuits using our chips, however the target environment may require attention to several things.  Most of these have to do with devices connected to GA144 I/O pins or to code being loaded from flash.

- Flash Code:  In some cases it is necessary to enable flash booting even when planning to work with the chip using the IDE.  For example, some I/O initialization may be always necessary, such as switching the 18-bit parallel Address and Data buses from output to input mode, or placing other pins in states other than their default weak pull-down.  The existence of such code may require adjustment of high timing in the IDE's definition of  RESET  (10 ms as delivered, to accommodate one such board we use in development.)  Additionally, if a watchdog circuit is active in the target environment, this flash boot may need to load code into at least one node to keep that circuit from resetting the chip; this will require changing IDE paths from the defaults.  Finally, in some cases a more extensive amount of the chip or chips is booted but we still want to keep the external IDE usable on nodes not (yet) programmed.  In this  case more extensive IDE path changes will be required.  In either case, it may prove difficult to devise single IDE paths that can reach all nodes one might wish to program.

- Connected Devices:  If our output GPIO pins are high at the time of reset, the amount of time it takes for these to be restored to low state after the GA144 is reset can be rather long if the capacitance on the pin is high.  Our inherent pin capacitance is on the order of 2.8 pF but that is without connection to a PCB or to driven device(s).  The output circuit is typically an RC first order lag, with the weak pull-down transistor looking like a 50k resistor.  The time constant of this circuit is on the order of 500 ns per 10 pF.  This is more important for on-board reset circuits than it is for the external IDE, but it should still be taken into consideration if extremely high capacitances are involved.  Another property with similar results might be the time required by a device to restore its outputs to resting conditions upon removal of an enable or other signal due to resetting the GA144.  In all such cases the RESET low time must be long enough to ensure that code in the chip, particularly boot code, does not awaken at end reset and see a pin high when it should be low.  The default low time in the IDE's definition of RESET is 5 ms as delivered.

Working with Two Chips using External IDE

The IDE may be used to interactively debug software on both chips of the Evaluation Board as though they were a single chip with 288 nodes (two of which are invisible), using just the serial interface for the host chip.

BRIDGE LOAD  compiles a host IDE version capable of operating on both chips.  Initially its environment and function is identical with  HOST  and you should establish connection with the host chip using  TALK  if it isn't already established.

SPAN  extends the  bridge  IDE to encompass both chips.  In order to successfully invoke  SPAN  the edge nodes of the host chip starting with 707, proceeding to the left to 700, and downward to and including 300 must be accessible for programming; it is typically used immediately after loading  BRIDGE  and establishing host connection unless something such as polyFORTH needs to be set up on the host chip first.  SPAN  resets the Target chip and programs node 300 on each chip as a transparent bridge for carrying port communications between them, using 2-wire synchronous communications.  New default paths 0 and 2 are set up to cover both chips.  Thereafter, nodes 300 are dedicated to this purpose until the chips are reset.

With the port bridge built, the up ports of node 400 on each chip are logically connected as though they were a simple COM port. Port read/write communications, such as IDE, are basically transparent across this connection except that data transfers take 100 or more times longer.  The current version of the bridge supports flow control, so unlike the old version node 400 may determine whether the bridge has data for our chip or is ready to send a word to the other chip by examining io:  If the bridge is sending us a word, node 400 can see up writing, and after we write to up from node 400 we will not see up reading again until the word was accepted by 400 in the other chip.  However, you may not infer by seeing up reading that node 400 on the other chip is at this time reading its up port.  Any programming method that depends upon such awareness will have to be implemented in some other way.

The default paths after  SPAN , shown below, facilitate full access to the Target chip with or without the polyFORTH virtual machine present on the Host chip.  Extended node numbering is supported in the form cyyxx where c is zero-relative chip number; thus nodes 000 through 717 are on the Host chip, while nodes 10000 through 10717 are on the Target chip.  The IDE is aware that a connection from 400 to 10400 exists through their up ports.  You may only have one path hooked through the bridge at any time!  Attempting to violate this rule will simply hang.

Default External IDE Paths

After loading HOST, BRIDGE or TARGET these paths exist in the chip to which the serial connection has been made.  Path 1 is available for general use at that time.

After  SPAN  path 0 is generally used for target chip, starting on host and passing thru the bridge to cover the entire target as shown here

... and path 2 is generally used for the host chip.  Path 1 is available for general use.  Path 0 by default avoids passing through the top row of nodes on the host because in polyFORTH environment node 705 is programmed for SPI flash operations.

Because paths 0 and 2 share nodes one should be unhooked before the other is hooked through those nodes.  The port bridge only requires special code in nodes 300 and 10300.

Internal IDE

Terminology

Using the Internal IDE

Internal IDE Vocabulary

Preparing Boot Streams

While the external IDE may be used to load and interact with F18 code in any chip for hardware or software testing without requiring any other hardware or initialization on the part of the chip, higher level modules such as the polyFORTH Virtual Machine may require additional hardware and the loading and initialization of many nodes as well as perhaps external SRAM in a practically simultaneous manner.  This is best done by generating a boot stream to do all of that.

Unless equipped with special ROM or booted using SERDES, our chips must be booted after reset by having a suitable boot stream made available to one of its boot nodes (such as async serial, 2-wire synchronous serial, or SPI flash nodes that are enabled for boot after reset.)  A boot stream consists of one or more boot frames, which are data structures defined and processed by the boot nodes of our chips.  Each boot frame contains zero or more words of data, a starting memory or port address at which the data are consecutively written, and a jump address to which control is transferred by the boot node after the frame has been processed.  Every boot node defines a concatenation address to which a boot frame may jump to process another frame.  After reset, a chip may boot itself from a slave device such as SPI flash memory, or it may wait to receive boot stream(s) on one or more of its enabled interfaces.  For example, a daisy chain of subsidiary chips may be booted by a master chip using frames stored in the master's mass storage or read from its boot device (such as an SPI flash).  As another example, the external IDE works by transmitting boot frames into the chip under test.  arrayForth hosts can build and transmit streams of boot frames into a target chip for efficient booting (the old IDE based boot mechanism has been deprecated in arrayForth-3.)

The Streamer Utility

This utility is a set of tools for generating and packaging boot streams that consist of one or more frames.  In a boot stream, each frame has a header to be interpreted by a boot node, and will end either by returning to a simple function in the boot node such as its  warm  cold  or its appropriate concatenation address; in some circumstances the boot node will be given code to execute, which may read additional data from the boot stream and dispose of it in some way (for example, copying the polyFORTH nucleus into external SRAM) before completing its job and returning to such a place in the boot node, perhaps with a new starting address for the next frame in the case of SPI flash.  Streams may also be generated for insertion into a node attached to the Snorkel, in which case no boot headers are necessary and special methods must be used if that first node is itself to be programmed.

The Streamer operates in two phases.  In the first, it composes the entire stream as instructed, one 18-bit word per 32-bit cell in memory (in pF/144 we do this using unallocated space in the nearly 1 Megaword of extended memory.)  In the second phase, the boot stream is converted into a form suitable for whatever medium is being used (for SPI it is a stream of 18-bit values expressed as a succession of bytes; for async serial each word is inverted and shifted into a 3-byte form equipped for auto-baud of each word.  For delivery through the Snorkel, it is left unchanged.)

A minimal use of the Streamer consists of the following elements:

STREAMER LOAD            Compile the utility

nn STREAM[             Specify root node and clear

{a COURSE}             Override default path for that node

FRAME[                 Begins a frame suitably for root node

boot description --- see 6.1.1, Boot Descriptor Language (BDL)

]FRAME                 Ends a frame suitably for root node and,

if a forward reference was made, resolves

it at the current aligned flash location.

]STREAM                Completes a stream and packs it as needed

Here is the full  STREAMER  vocabulary, followed by the Boot Descriptor Language:

STREAM[ (nn)  If usage from root node nn has been defined, selects the header formulation and final packaging suitable for that node as well as a default path whose address is returned by  ENTIRE  after STREAM[  has been used.  Clears the entire stream buffer.

The node numbers defined for this use are at the time of this writing  207, 708, 300 and 705 .  For node 708 we use asynch serial header and 3-byte asynch word packing.  For node 300 we use synchronous serial header and don't pack the stream.  For node 207 we generate no headers and do no packing for Snorkel injection into the chip.  And for node 705 we use SPI flash headers, compress each 8 18-bit words into 18 bytes (9 16-bit words), and set special end-frame behavior for forward references because at the end of loading each stream node 705 must be instructed to read the next stream starting at a byte boundary.

COURSE (a)  Overrides the default path selected by root to be used as  ENTIRE  at any time.  In a multi-frame stream it will often be necessary to change the path, for example because an earlier frame some of the chip's nodes have been programmed.  It is the nature of boot streams that the first node loaded will be the last one in the path.  The first node in a path must be the number of the node from which it originates.  The address given must be the tick of an ARRAY holding a sequence of nn numbers ending with -1.  Default paths for the standard root nodes are named  df708  thru  df207 .

FRAME[  begins generation of a frame.  All descriptor tables are initialized with their default settings.  The root node is either not programmable at all (node 207) or partly programmable (memory settings and /P are the only valid operations on a boot node itself)

]FRAME  Ends a frame.  Calculates the frame length and generates the appropriate frame header if any, followed by all port and memory pumps to load the nodes in the current  COURSE .

/ROOT (jmp a n bin)  is used outside of  FRAME[  ]FRAME  encapsulation to generate a boot frame for the root node, loading that node with  n  cells from the given  bin  and jumping to the address  jmp .

FORWARD  is used immediately after  /ROOT  when generating a stream for SPI flash to indicate that the program just loaded will access flash before resuming stream processing.  When this is done the address register in the flash loses synchronization with bit stream reading.  In order to start up correctly again, the stream is padded to the next zero modulo 8 18-bit word index in the overall flash image, and the code loaded in the root node is patched to provide the absolute byte address for continuation in a standard manner.  See definition of  FORWARD  and the source code it is used with.

]STREAM  ends a stream.  Converts the stream in place to the appropriate form for the intended node of origin.

.STREAM  dumps the current stream, which must be in unpacked numeric form.

?STREAM  audits an unpacked stream against a reference stream, see below.

?FLASH  audits a packed flash stream against a reference stream, see below.

STREAM ( - a n)  sF ONLY returns origin and length in octets of the converted stream.  Normally used within other tools.

STREAM ( - da n)  pF/144 ONLY returns origin and length in octets of the converted stream.  Normally used within other tools.

Boot Descriptor Language (BDL)

Automated loading touches all nodes in a defined path starting at the relevant root node.  Automated SOFTSIM loading touches all nodes in both chips.  By default, a stream does as little as possible to each node touched.  One item is by necessity pushed onto each stack, and register A is altered.  Register P is set to the appropriate IDLE multiport address for that node, and B is set to the address of  IO  for all nodes not otherwise loaded.  BDL is used to specify non-default treatments using statements that start with the word  +NODE  and continue with phrases describing the needed initialization of memory and/or registers in the node.  Nodes may appear in any order since these statements are simply filling tables for later use.  For the same reason,  +NODE  statements are cumulative in that one  100 +NODE  phrase may set the memory loading and starting address for node 100 while another  100 +NODE  phrase interpreted later may override the starting address specified earlier.  The following words constitute the BDL.  Their use in context is described here and in the chapter on SOFTSIM; examples may be found in the distributed source code.

+NODE (nn)  Selects table entries for node number nn, in cyyxx notation.  The values in the table entries are not changed by  +NODE  and so their values will be default unless a previous  +NODE  phrase has been interpreted for the same node.

/RAM (bin)  Loads all of RAM from the given bin in cyyxx notation.  By default nothing is loaded.  Partial loads may be described using the following words.  Up to three RAM load descriptors may be specified for each node.  Such loads are applied to the node in the order encountered in the boot description, so that if there are overlaps the last descriptor will overwrite earlier ones.  When a RAM load of all 64 words is specified, any previously declared RAM loads are deleted and will not appear in the stream.  Thus it is possible to preset a default background, such as of Ganglia, and override that for nodes that won't be able to serve as Ganglia, without extending stream length.

/SOME (s d n bin)  Loads part of RAM,  n  words from the given  bin , starting at address  s  in the bin, are loaded into the node's RAM at address  d .

/PART (a n bin)  Loads part of RAM, equivalent to   a a n /SOME  .

/B (d)  Specifies an initial value for register  B .  By default B is set to IO.

/A (d)  Specifies an initial value for register  A .  By default a port call instruction is left in A.

/IO (d)  Specifies a value to be loaded into the  IO  register.  By default IO is not altered.

/STACK (<n double values> n)  Specifies up to ten values to be pushed onto the data stack, with the rightmost value on top.  For example  30 20 10 3 /stack  produces the same effect as though a program had executed code  30 20 10 .

/RSTACK (<n double values> n)  Specifies up to nine values to be pushed onto the return stack, with the rightmost value on top.  For example  30 20 10 3 /stack  produces the same effect as though a program had executed code  30 20 10 .

/P (a)  Specifies an initial value for register  P .  Default value is  xA9  which is the routine  warm  in every node's ROM.  When the node is started,  warm  jumps to the appropriate multiport execute for the node's position in the array.  If you wish to leave the boot routine enabled in a boot node that is touched by an automated loading procedure, specify its  P  value as  xAA  as is done by reset for such nodes

!ND ( nn)  Macro to load node nn with code from same bin.

+ND ( a. b. p nn)  Macro to load node nn with code from same bin and initialize registers.

ITS (nn _ - a)  Returns the value of the label whose name follows in bin  nn .

RIGHT LEFT UP DOWN IO LDATA RDATA  Register names returning double addresses in I/O space.

EAST WEST NORTH SOUTH  Register names like the above, but leading in cardinal directions for the node selected by  +NODE .

      - BDL Macros

Several higher level BDL functions are also provided:

/RINF  initializes return stack with all -1s.  Used to support infinite  <op> <op> unext unext  loops.

+wire (s d nn)  places a perfect 1-word wire in node  nn  with source and destination ports  s  and  d  respectively.

+dly ( s d nn)  Sets node  nn  up as a delay line from ports  s  to  d .

Stream Structure

The components used for building frame content differ from those used in the External IDE.  This section includes information for calculating worst case frame length.  For calculations based on node quantity, we assume a bridged pair of chips containing a total of 288 nodes, two of which (300 and 10300) are inaccessible because they are the bridge, and the boot node itself which is loaded in a separate frame; so the frame will visit a new of 285 nodes in the full, 2-chip path.

A frame begins with a 3-word header unless it isn't needed.  The first node receiving data via the frame body is given a focusing call that must be present in the stream; subsequent nodes get their focusing calls from the pump in the preceding node.  So we begin with 1 or 4 words and add to it modules of the following kinds.

      - Port Pumps

A frame begins with one port pump for each node except the last in the path.  The port pump is still five words but uses A rather than B to point to the port at which the pump is directed; thus the port pump clobbers A, and it also has to push one word on each stack.  For n nodes in path after the root, n-1 pumps are required; for two chips n-1 is 284 so 1420 words of pumps add to the 4 above for 1424.  Structure within the stream:

04DAF     @p dup a!  @p  (preceded by focusing call)

12xxx     Literal, the focusing call given to next node

lng-1     Literal, number of words following pump -1

2FAB2     >r !

05A72     begin  @p !  unext

Following the sequence of port pumps, there is a Memory Load module and a Post-Load Initialization module for each node beginning with the last node in the path and proceeding backward along the path to and including the first node after the root.  When a node given a port pump is finished with pumping, the next thing in the stream will be its memory load(s) (if any) and initialization (at least of P and IO.)

      - Memory Loads

Each memory pump is five words long followed by the text to be loaded.  In the typical case there is one memory pump and 64 words of text for a total of 69 words.  Complicating this, we permit combining up to three memory loads for a node, in which case one might expect still a max of 64 words of text but with two additional pumps, bringing the total to 79 words.  285 of these would be 22515 words, for a total thus far of 22939 although a smaller number is more common because normally there is only one memory load in effect for each node.  Structure:

04A12     @p a!  @p

<adr>     Literal, start address of this RAM load

lng-1     Literal, number of words following -1

2E9B2     >r

05872    begin  @p !+  unext

It is possible to describe an even worse case in which there would be three fully overlapping 64-word memory loads for each node, but this is not worth considering.  We do make provision for all nodes to be initially loaded as a background default by such things as the Ganglia, but to avoid letting this run up the size of the frame we check in the RAM load specifiers for full 64-words descriptors and if one is encountered all previous RAM load descriptors for that node are deleted so that they will not be present in the frame.

      - Post-Load Initializations

Initializing  IO  costs 4 words; A and B cost 2 words each and P costs 1.  Initialization of each stack takes 2 words per value.  We could fight for a reduction on the stack initializations but the benefit would appear to be marginal.  The initialization components are structured as follow:

Set IO   04BB2     @p b!

0015D     Literal, address of IO

05BB2     @p !b

value     Literal

Set A    04AB2     @p a!

value     Literal

Set B    04BB2     @p b!

value     Literal

Push R   048B2     @p >r

value     Literal

Push S   049B2     @p

value     Literal

Set P    10xxx     Jump to start address

P is (and must be) always set, so the minimum module is 1 word; for consistency with colorForth, we also always initialize B, to IO if nothing else is specified in the BDL.  To initialize everything possible we have 9 words for the discrete registers and 38 words for all of the stacks, for a total of 47 words worst case and 13395 for two chips.  This brings us to 36334 words so far for a practical worst case 2-chip frame.

That number pushed us across a 64k byte boundary and so we have chosen to reserve the first 128 kB of flash media for boot stream text.

      - Root Node Programming

The root node may simply be programmed as the last step in a complete boot stream.  Earlier in the stream it may be necessary to program the root node to do other things such as for example speeding up the flash clock timing, or copying data from flash to an external SRAM as is done in booting eForth or polyFORTH.

Such intermediate programs for the root may be generated using  /ROOT  which does not employ BDL, or included in the BDL for a stream in which case the streamer generates two frames for the boot path (one for all nodes but the root, and one for the root itself.)  When such a program will be accessing other flash during its operation and then resuming stream processing, the word  FORWARD  should be used after  ]FRAME  in a flash boot stream to pad the stream to the next even byte boundary in flash, and to patch the root node program for a forward reference to the padded restart address.

Only one memory load may be specified for a root node, and it must have the same source and destination offsets if the load is specified by  /SOME .  Although it is possible to initialize more than RAM and P in a root node, this process would be complex and we have not built any particular such mechanism into the BDL.  None of this is relevant for node 207, but it is true for any real boot node.

      - Special Considerations for SPI Flash

The SPI node processes standard boot frame headers which may be concatenated so long as the flash is not being accessed in any other way during booting.  This stipulation is necessary because the end of a boot frame may occur on any even bit boundary within the flash, hence 0, 2, 4 or 6 bits into an addressable byte.  A straight concatenation may begin on the next bit boundary to be read.  A standard BDL frame will actually generate two concatenated boot frames:  One to load all nodes but the root, and the next to load the root node and begin execution.  The second frame is actually generated using  /ROOT  but is derived from the limited BDL allowed for the root node.  Initialize P to the concatenation address  [ 705 ITS spi-exec ]  if the root is not going to be otherwise accessing flash and there is to be another concatenated frame.

In complex flash boots there may be several logical steps, and some of them may involve programming the root node to operate on the flash.  Examples include reading the polyFORTH nucleus from flash and writing it into external SRAM, and accessing a table of parameter values to be written at specified places in various nodes before booting the rest of their memory.  In these cases the bit alignment of the end of the previous boot stream is lost and a new starting address must be written into the root node's memory so that after that step is complete the normal boot code may be started at a byte-aligned place following the end of the previous boot stream.  This address, within the flash, is only known after the boot stream image has been padded, and  FORWARD  is used to patch the root node program with this address information.

To build an SPI boot stream that is unpacked for debugging purposes, use the phrase  0 S-END !  within  STREAM[ ... ]STREAM .

      - Special Considerations for Serial boot

When preparing a 2-chip boot stream for serial delivery through node 708, it's infeasible to visit node 000 on the first chip as part of the main stream.  So, if you need to program this node in any way it must be done as part of the bridge construction.  The 2-chip serial pF stream generator we provide loads a ganglion into node 000.  Copy the BDL for your node 000 here if you need it.

Transmitting Boot Streams

You must generate a boot stream for the particular root node you plan to present it through.  While the BDL may be the same for chips to be booted through various nodes, the text of the boot streams differs because the paths necessarily differ for each root node.

The vocabulary for injecting these streams depends on the system you're running on and on the method to be used.  These methods are as follows:

From saneFORTH

      - Asynchronous Serial

sF uses its external IDE to inject asynchronous boot streams into node 708 of the HOST chip

S-ORG S-LNG  HOST LOAD  !STREAM  resets the chip and pumps the stream just built into it through boot node 708.  Returns when all words have been transmitted.  There is no flow control so completion does not prove success.

!stream  is used when the serial port is already set up and the chip has already been reset.  Used for second and subsequent streams when multiple streams are being injected.

!NUCLEUS  transmits the polyFORTH nucleus from block zero of the pF serial disk when initializing RAM over serial port.

The asynchronous boot stream can also span both host and target.  Upon completion the external IDE remains loaded and, if the stream in question left IDE code in node 708 with proper P setting, the external IDE may still be used with care and due considerations for what paths are feasible after the stream has been loaded.  Use  Talk  instead of  TALK  to use existing serial port and avoid resetting the chip.

From pF/144

      - Internal Streams rooted at node 207

Streams may be injected, at very high speed, directly into the host chip after being built:

!SNORK  pumps the stream just built out through the Snorkel into node 208 or 307 as indicated by the path in effect at the start of the stream.  Returns when all words have been absorbed by at least the first node in the path.

Here is an example that instructs node 715 to emit an approximately 75 MHz signal on pin 715.17, assuming that nothing but ganglia have been loaded into any of the nodes along its path:

1581

0 ( Descriptor test)

1 ASM[ # 715 NODE ERS # 0 org

2    begin begin  !b unext unext   >BIN ]ASM

3

4 0 ARRAY MYP  207 ORGN 210 TO 710 TO 715 TO -1 ,

5

6 207 STREAM[   ' MYP COURSE

7    FRAME[   715 +NODE  0 1 715 /PART

8    -1. -1. -1. -1.  -1. -1. -1. -1.  -1.  9 /RSTACK

9    x20000. x30000.  2OVER 2OVER  2OVER 2OVER  2OVER 2OVER

10                     2OVER 2OVER  10 /STACK  IO /B  0 /P

11    ]FRAME  ]STREAM

12 !SNORK

This example instructs node 500 of the target chip to emit a similar signal on its pin 500.17, assuming that the bridge has already been installed and activated, and that there are no obstructions along the path.  Note the use of  ORGN  to make the hop between nodes 400 and 10400:

1582

0    ( 2-chip test)

1 ASM[ # 10500 NODE ERS # 0 org

2    begin begin  !b unext unext   >BIN ]ASM

3

4 0 ARRAY MYP  207 ORGN 407 TO 400 TO  10400 ORGN 10500 TO  -1 ,

5

6 207 STREAM[   ' MYP COURSE

7    FRAME[ 10500 +NODE  0 1 10500 /PART

8    -1. -1. -1. -1.  -1. -1. -1. -1.  -1.  9 /RSTACK

9    x20000. x30000.  2OVER 2OVER  2OVER 2OVER  2OVER 2OVER

10                     2OVER 2OVER  10 /STACK  IO /B  0 /P

11    ]FRAME  ]STREAM

12 !SNORK

For the next example, we begin with a flash boot that does NOT load anything but polyFORTH and, if desired, the bridge.  This demonstrates that polyFORTH can load the ethernet cluster from within a live chip.  It is a torturous procedure because we must initially do a 9 LOAD for serial clock (HOME 9 LIST and edit is the best way to go)  :

1583

0 ( Add Ether to pF)

1 STREAMER LOAD  1 CONSTANT ?CLK   1 CONSTANT ?ETH

2

3 0 ARRAY MYP  207 ORGN 208 TO 108 TO 110 TO 10 TO 17 TO

4    617 TO 616 TO 116 TO 115 TO 415 TO 414 TO 114 TO 111 TO -1 ,

5

6 207 STREAM[   ' MYP COURSE

7    FRAME[  ( Clock/Ether)  ENIC 1+ LOAD  ]FRAME  ]STREAM

8 !SNORK

9

3983

0 Assumes flash boot only has polyFORTH and, if desired, bridge.

1    This loads and activates the ethernet NIC Mk1 and the 10MHz

2    clock.  Because of the latter several steps are involved:

3

4    1.  Reset and hit space.

5    2.  HOME 9 LIST and edit to use serial clock.  HI

6    3.  AFORTH  1583 LOAD  ... and link should become active.

7    4.  RELOAD  hit space

8    5.  HOME 9 LIST and edit to use Ethernet clock.  HI

9    6.  ETHER LOAD

10

11 ETHER may in fact be loaded on top of AFORTH, or presumably

12    vice versa, but conflicts on double constants may exist.

      - Asynchronous Serial to Another Chip

      - Synchronous Serial to Another Chip

Burning Flash

By default, when generating a stream for node 705, both saneFORTH and pF/144 pack the stream into bytes that may be written to a flash memory starting at its absolute location zero.  This section describes the procedures for writing that stream image into a local or remote flash.

Burning Flash from saneFORTH

Burning flash from pF/144

Code is provided for burning one's own flash (for example, the SPI flash that boots the Host chip on an Evaluation Board) or for burning the flash on another chip with which pF is communicating using external IDE.

      - Burning pF's Own Flash

Burning one's own flash is straightforward; all we need do is to copy the stream image to the front of the flash starting at absolute block zero (normally mapped at 40 DRIVE).  After generating the stream, simply execute the following word to rewrite flash boot on your own system:

WRITE-FLASH  copies the packed boot stream just generated to the boot area of the current system's flash, at absolute block 48000 and regardless of the current  OFFSET .

Warning!  This procedure assumes the flash boot area begins at absolute block 48000 in the  UNITS  mapping that is currently active on the pF/144 system.  If you are using some other mapping you will need to attend to the definition named  )FLASH .

      - Burning Flash on Another Chip

Erasing Flash

Flash needs to be erased before being written.  Unless otherwise noted, all of our operations that write flash take care of the necessary erasure.  If you need to erase flash without writing it, study the code in BULK which uses  FERS  with absolute starting block number and count on the flash (remember that 0 DRIVE starts at block 128 absolute.)

The smallest unit of flash that can be erased on the Evaluation Board is 4K bytes, so erasing always starts on a 4K byte boundary and the number of bytes erased will be rounded up to the nearest 4K as well.

Auditing Streams

When working with new stream structures it's useful to compare the stream just made with a reference.  Mechanism for doing this is included in  STREAMER .  Standard areas involved in these operations are 4740, where a packed flash stream may be saved, and 2340, where an unpacked stream of up to 60 blocks (14760 words) may be saved.

UNPACK (n)  unpacks the given number of blocks of flash stream from 4740 to 2340.

STASH  saves the streamer's current stream buffer to 2340, limited to 60 blocks.

?STREAM  dumps the current unpacked stream buffer in hex, highlighting each word that differs from the reference stream in the area at 2340.  In saneFORTH the reference stream is at 2340 on the serial disk used by pF/144.

?FLASH  dumps the current stream buffer in hex packed for flash, highlighting each word that differs from the current boot flash.  In saneFORTH the reference stream is at 4740 on the serial disk used by pF/144.

.LOADING  available after loading block 1619.  Displays what the most recently generated unpacked stream does based on the content of the STREAMER's tables.

Getting Streams from Flash pF/144 ONLY

The simplest way to obtain a stream made by colorForth is to burn the stream into flash from colorForth, boot to it, and then copy it to the comparison area.  With standard mapping (SPI flash for 24000 blocks at absolute zero, followed by 24000 blocks of serial disk, followed by 128-block flash boot area), this can be done as follows:

- 0 DRIVE DISKING LOAD

- 48000  24000 4740 +  nn BLOCKS  (to place it on serial disk)

- 48000  4740  nn BLOCKS  (to place it on SPI flash disk).

Getting Streams from colorForth

When converting an existing body of colorForth based code to aF-3, it is prudent to use the colorForth stream for loading that code as a reference.  The following steps accomplish this:

- In colorForth,   compile   all of the code of interest.

- Say  bnamed mystream  (or other name of your choice) to output file name.

- Edit the stream generation block to abort with a bad word after the word  stream

- Generate stream, let it abort and note the address and count found there.

- Say  32768 nnc + nnc + 4800 wback  to write a 4800-block file starting with the stream. Only the first few blocks are relevant, but this simplifies file mapping into the sF system.

- Move this file into the  pf  directory

- On the sF system, in block 149 temporarily replace  OBJ-AF3  with your file's name,  FLUSH  and  RELOAD .

- Use utility not written yet and conventions not made yet to capture and save the stream.

- In block 150, restore the OBJ-AF3 file name,  FLUSH  and  RELOAD .  If you are using the new project packaging mechanism, you will probably need to edit your  custom.txt  file instead.

- Additional steps to be determined.

- Remove the change you made to the stream generation block in colorForth.

Simulation Testing with SOFTSIM

SOFTSIM (the Software Simulator) is a program that simulates the actions of Green Arrays computers, in two GA144 chips, at a high level.  The charter of this simulator is to achieve the same results in values but not in timing, either relative to wall time or between nodes, as would a real chip.  There are coarse delays simulated for long operations such as memory read/write, but these are only provided to aid in perspective..  Tutorial follows.

The Display

After starting aF3, type  SOFTSIM LOAD  at the command line. The command prompt window will be enlarged (you may need to select a smaller font in order for this to succeed; we are not permitted to exceed the screen dimensions with that enlargement.)  Here is an example of the display you should see.  In the default layout shown below, there are three sections which help you "drill down" into the chip.  In the upper right corner is a full chip view showing two GA144s as 8x18 arrays of computers, with chip 1 on top.  On the left is an overview showing the major status and registers of a rectangular section of the chip's nodes (4x8).  Between these sections is a detailed view of a focus node.  The lower right-hand part of the display is a normal 25x80 FORTH terminal for control and programming.

The characters in the full chip view are # for nodes in the overview, O for the focus node, and = for all others.  Background of nodes in this view are blue if suspended, yellow if active, and red if the node is presumed "dead" after executing an undefined operation.  These colors also apply to node numbers in the overview and focus node sections.

Meaning of the Data Displayed

For simplicity, the simulation assumes a fixed cycle time.  All instructions begin at a cycle boundary, including resuming after suspension or completing the delay for a long operation.  All parts of the display show conditions as they are before any node has begun executing its next cycle.  This means the opcode shown is about to be executed if the node isn't suspended, and registers/stacks are as that next opcode will find them.  While an instruction fetch is occurring, the opcode reads  fetch  .  Various instructions involve time delays, in which case the opcode shown will be the one that is still executing during the delay (the delay indication differs between overview and focus node views.)

Please remember that this is not actually how the chip works!  The purpose of SOFTSIM is to assist in visualizing and debugging code at a very high level, not to accurately simulate the timing of a chip made of asynchronous computers, each of which will due to process variation be running at a slightly different speed, and responding to stimuli that are synchronized only with the sources of those stimuli, not with any fixed clock.  If your code respects the rules pertaining to multiport read and write operations, it should give the same results in SOFTSIM as it does in the real chip... but not in the same amount of time, and certainly not with synchronization of events between nodes that are not talking to each other or to the same internal or external central points.  If on the other hand your code is pushing those rules, it could appear to work in SOFTSIM but fail in the real chip, or vice-versa.  It is important to always keep this in mind when making use of SOFTSIM.

Likewise, I/O testbeds, discussed later, must be written and used with the understanding that the simulation time scale is intentionally and practically not reliable.  Attempts to stress the chip with external inputs whose speeds approach the chip's limits will easily produce misleading results.  Using the simulator to test logic is fine; explore timing limits with real chips and apply margins to account for variation in chip performance due to process, voltage, temperature and aging.

Where addresses are shown in these displays, they are formatted either as two digit hex numbers for RAM or ROM; or a three character string for I/O addresses.  In the latter case  all  means  rdlu .  Other forms are self-explanatory.

The display shown below represents "cycle" 505 of chip 0 loaded with polyFORTH and the Ethernet NIC, set to use a 10 MHz crystal, but with no I/O testbeds (so external SRAM is not connected and what's being read from it is garbage).  The details of the sections, and the vocabulary for arranging the display as you like, are discussed in the following sections.

"Time" and Cycle Count

Under the full chip views are two numbers.  The first, labeled  ~us , is an extremely rough estimate of the elapsed time since simulated RESET.  The second, labeled  cyc , is the number of simulation cycles that have run since RESET.  Because, as noted above, "cycles" do not exist in the real chip either within a node or between nodes, the number of cycles is an abstraction.  The approximate elapsed time is accumulated at 1.4 ns per "cycle", so it too is an abstraction.  That said, the time is likely within an order of magnitude of reality so it can be useful.

The Overview Section

This section shows a high level summary of what's going on in each of 32 nodes in an 8x4 array.  The array may be moved around on the chips; the nodes currently displayed are shown on the full chip view as the character # .  As an example  let's look at node 008 in the example above.

The lines, starting from 1 on top, indicate:  1) that we are driving pins 17,5,3,1 as shown; 3 high means SRAM read.  2) This is node 008, not running, and the "address bus" is  r-l- .  3) Next op is @b in slot 0.  4,6,8,9,10,11) Registers I (instruction), a, io, r, t, s.  5) no delay, register P is x25 (op came from x24).  7) b register, interpreted symbolically as an address, is  r-l- .  The reason the node isn't running is that it is suspended, waiting on a fetch from RIGHT or LEFT.

Refer to the table in the next section for the complete legend for nodes in this view.

I/O pins or ports are shown on the top and/or bottom of the overview.  GPIO and other normal pins are depicted as 0 or 1 for low or high with color attribute indicating pin mode (white on blue high impedance input; white on grey input with weak pull-down; white on magenta output.)

Each node's depiction shows a list of the values of registers and opcode names representing the current state of that node.  The state depicted is before an instruction is executed; the opcode shown will be executed next, and all the registers are shown as they will be when the opcode begins execution.  When the F18 is suspended waiting for port or pin wake-up, the operation shown is the one that's suspended.  For certain fetch or store operations there is a multi cycle delay, in which case the operation shown is the one whose execution is not yet complete.  If an instruction word is being fetched, the current opcode will read  fetch .  Each node shows twelve lines.  From top to bottom, using node 007 from the above display as an example, they are:

| Row | Example | Name | Description |
| --- | --- | --- | --- |
| 1 | C pppp | Port r/w and pins. | C indicates reading (V) or writing (^) the port leading to the "North". pppp is blank unless the node has pins. +ph and -ph indicate phantom pin high or low. Pin states are shown with pin 17 on left and pin 1 on right. Each pin is shown with its voltage as high (1) or low (0). The mode of the pin is shown by color and background mapping. Output is white on magenta. High impedance input is white on blue. Input with weak pull-down is white on grey. |
| 2 | yxxbus | NODE/Abus | yxx is node number within either chip. As noted above, it will be black on yellow if active, white on blue if suspended, or white on red if locked up. The "address bus" value shown (symbolically if in the I/O range) is that used for the most recent (or ongoing) memory operation. |
| 3 | sOpcod | Slot/Opcode | The slot selector and next/current opcode to be executed in the next cycle. The opcode is fetch if we are doing an instruction fetch. |
| 4 | i03B0A | I | The instruction register |
| 5 | WdpppE> | M/P | W and E indicate writing or reading the "West" and "East" ports using <> to show read/write direction similarly to use of ^ and V for North/South. d is the delay counter. If it's >1, the operation shown will not complete this cycle. ppp is the P register (program counter) value, defining the next address to be fetched/stored by @p !p or instruction fetch. |
| 6 | a00115 | A | 18 bit pointer register A. |
| 7 | b -l- | B | 10 bit pointer register B, symbolic if known. |
| 8 | o102AA | IO | The write-only part of the IO register. |
| 9 | r2AAAA | R | Top of return stack. |
| 10 | t00141 | T | Top of data stack. If carry latch is set, "*" replaces "t". |
| 11 | s0015D | S | Second on data stack. |
| 12 | Cddddd | Pins and/or ports | C indicates reading (^) or writing (V) the port leading to the "South". ddddd if nonblank is the value of the data register/bus/pins. When white on blue the port or device is in input mode. When white on magenta, the port or device is in output mode. |

The Focus Node Section

Between the overview and the full chip view lies a detailed depiction, expanding on the overview, of the current focus node.  Field depiction and color coding are the same except as indicated.  Here is node 8 at the moment of the above example.

The full  cyyxx  node number is shown, highlighted for activity or lock-up.  Registers p, io, b and a are shown on the top line; p and b are shown with symbolic addresses when relevant.  If a is in the I/O range, its full hex value is also shown symbolically.

Pins and data in the next line are the same as those in the top and bottom rows of the overview.

The next line shows the I register with its disassembly.

The address, slot and opcode shown reflect the instruction to be executed in the next cycle, or the previous instruction being continued due to suspension (for I/O) or delay (for operations like memory).  Note that in normal operation P advances after each fetch or store in memory using P, so unless the node is executing from a port, "p" and "loc" will normally differ.  Beneath that line may appear  SUSPENDED , DELAYING  or LOCKUP! .

To the left of these lines is a dot.  If there are any port communications going on, the symbols used for the same purpose in the overview appear on the cardinal sides of the dot.  "Arrows" pointing to the dot are reads, away writes.

Beneath these lines is a dump of 16 contiguous cells of memory showing labels if any, address, value of the data and a disassembly of that value interpreted as an instruction word.  If the address in P is visible, it is highlighted black on yellow.

The right side of this view shows the return stack on top, growing downward, and data stack below, growing upward.  Carry is shown in the same way as in overview:  Label for register t changes to * when carry set.

Operator functions are provided to select a focus node, and to toggle between the current and most recent focus nodes.  For any given node, by default the memory dump tracks P to ensure the next cell of the instruction stream is visible.  Operator functions exist to override this by specifying the starting address which is then frozen, and to release this freezing of the dump origin.

Loading Code to Simulate

When SOFTSIM is loaded, node RAMs are only initialized as they typically appear on power-up (x15555) and the nodes are all set to their P values as of RESET (the simulator presently uses xA9,  warm , for non-boot nodes, for simplicity).  If you allow this to run, all nodes will, in not many ns, be  SUSPENDED (asleep) as is the case with the real chip.

At present, there is only one fully supported method for initializing the chip with code to run.  SOFTSIM directly understands and processes Boot Descriptor Language (see 6.1.1).  So, after loading SOFTSIM, load some BDL.  For example, see block 1674, which loads polyFORTH virtual machine, SRAM cluster, and the Ethernet NIC.  This body of code will not do much without I/O testbeds but it is a place to start.  See below for another example.

Operating the Simulator

At any time, you may type  HELP  to display a help screen with reminders of these operating procedures.  The vocabulary for operating is as follows:

OVIEW  enables the IJKL keys to move the 8x4-node overview rectangle around.  You will see feedback in both the full chip view and the overview area.  Any key other than IJKL exits this dialog.

SEE (nn)  selects a new Focus Node (cyyxx) and remembers the previous one.

OTHER  toggles between the current focus node and the one most recently focused.

MEM (a)  sets new starting address for memory display of current focus node and freezes it.

-MEM  un-freezes memory display of current focus node so it will again track P.

Z  takes a single step of all nodes and updates entire display.

FAST (n)  runs  n  steps updating only the display of "time" and cycles.  Full display update when done.

SUPER (n)  like  FAST  but does not update the display at all until done.

SS  Sets for operator-initiated single steps by striking any key except ENTER, which punches out.

BREAK (a s nn)  sets breakpoint for node nn when next op to execute is address a slot s.

-BREAK (nn)  clears any breakpoint for node nn.

About Breakpoints

Each node may have one and only one breakpoint defined.  When any node is about to execute the instruction from the address and slot given, SOFTSIM will stop before starting to execute the entire cycle in which that instruction would begin (or continue) execution and display that node as the focus node.

If you specify an instruction that may suspend, it will continue to stop simulation before its execution has started and will not re-breakpoint during suspension.  The same is true of operations such as memory fetch/store that can be delayed.  Some thought is justified in choosing breakpoints.

Initialization

Block 1698 is an example of loading SOFTSIM, loading chip 0 with polyFORTH and the Ethernet NIC, using the TB-SRAM testbed that simulates a 2Mb external SRAM that's initialized with the polyFORTH nucleus.  It will run up to the point of waiting for an autobaud space key in node 200.

Another example is block 1737 which loads SOFTSIM, assembles a simple program, loads it into node 000, and sets up the view for simulation.

Testbeds

Testbeds in SOFTSIM are pieces of code that simulate external circuitry well enough to exercise the F18 code.  Several are provided and by default compiled with SOFTSIM.  Each testbed provides a vocabulary to instantiate it for one or more interfaces.  For example,  TB-SRAM  instantiates a 2MB external SRAM interfaced with the 40 pins of nodes 7, 8 and 9.

You may write your own testbeds for use with SOFTSIM.  The linkage is via  nTB  for each node.  This vector is @EXECUTEd after each instruction cycle has been executed on a node but before the end-cycle updating, suspension or delay processing have been done.

Interactive Testing with Softsim

You are invited to study the code of SOFTSIM and to make use of that information if you wish to probe details of a node's state that are not displayed in our views.  The vocabulary for doing this is not documented presently because SOFTSIM in arrayForth 3 is still under development and thus is subject to change.

Reference Material

F18A Code Library

GreenArrays has written, tested and used a considerable Library of generally useful F18A code; this varies from single utility nodes, such as the Perfect Wire, to multi-node clusters such as the Ethernet NIC.  Some of the code is useful anywhere on the chip, while other code such as, for example, the SRAM cluster, depends on the I/O and geometric properties of the nodes for which it is designed.  Every bit of this code is distributed as source.  Where practical, such code is pre-assembled and is distributed with the system as object code in virtual bins (1600 to 3200) so that it may be used readily by tools such as the IDE or included in applications via boot descriptors.  In other cases, such as the Ethernet Cluster or the polyFORTH Virtual Machine, enough nodes are involved that it makes more sense to assemble it for the target nodes when needed by an application being built.

This section documents the code, and its usage, for each such element in the Library.

Perfect Wire (any node)

It's often necessary to use a node as a simple wire to connect ports that are not physically adjacent.  In the simplest case, when the use is permanent (until chip reset) and the data move in only one direction, there is a demonstrably perfect solution:  A single instruction word program that, once fetched from memory, runs entirely in the Instruction Register  I  and is an infinite loop with minimal latency and minimal jitter (one word of every 262144 transferred takes an extra unext time.)

The instruction word may be written as  A[ begin @ !b unext unext ]]  .  This single word is provided as location zero of Virtual Bin 1802 and may be placed in an application by the following descriptor macro:

+wire (s d nn)   Places perfect wire in RAM location zero of the given node.  Initializes  a  and  b  to the source and destination addresses  s  and  d.  Initializes the return stack with nine -1s.

64-word Delay Line (any node)

A node may be programmed to serve as a 64-word delay line with throughput on the order of 8 memory cycles (~40 ns in theory, 52 ns in practice) per data word, and this performance remains the same no matter how many such nodes are placed in series (although latency from input to output when a word is pushed into a quiescent line does depend on the number of nodes).

Each data word is pushed into the first delay line node surrounded by two instruction words, and each data word between nodes and emitted at the far end of a delay line is also surrounded by the same two instruction words.  No Virtual Bins are used because no code in RAM or ROM is involved at all.  Delay line nodes are only committed to this use in the sense that they require special initialization of the data stack (with instructions) and registers a (as any valid RAM address), P (as the source port) and B (as the destination port).  So, at any time, a delay line node may be recommissioned via port execution (by default, we set it up with the source port address in P, but any multiport address may be used if desired.)

To place a delay line node in an application, use one of the following descriptor macros:

+dly (s d nn)   Initializes  P  and  B  of the given node to the source and destination addresses  s  and  d.  Sets  A  to zero.  Initializes the data stack with 5 copies of this instruction pair:

A[ !b @p @ ]]  in T  and  A[ !b !+ !b ]]  in S.

+dx (nn n)   Places  n  delay line nodes starting with node  nn  and moving horizontally (to the East if  n  positive, to the West if  n  negative).  Source and destination ports of all nodes including the first and last proceed in the direction indicated.

+dy (nn n)   Places  n  delay line nodes starting with node  nn  and moving vertically (to the North if  n  positive, to the South if  n  negative).  Source and destination ports of all nodes including the first and last proceed in the direction indicated.

The following code shows one way to insert a datum into the delay line:

: shove ( n)   <port> b!  @p !b A[ !b @p @ ]] ,

!b @p !b ; A[ !b !+ !b ]] ,

At the output end, here's one way to receive a datum:

: suck ( - n)   @ drop  @  @ drop ;

As you can understand by studying the above, this is an excellent example of the remarkable capabilities of the F18A computer.  The "program" for a delay line node is the pair of instructions pushed into its source port and is executed in that port; these two instructions surround the incoming datum.  What this program does is to send, from its own data stack, exactly the same two instructions to the next node; however the datum between those two instructions is the oldest word in this node's RAM, which is replaced by the incoming data word.

Synchronous Port Bridge (300 and others)

An almost-transparent port bridge in bin 1904 uses nodes 300 on two chips to simulate a COM port between the UP ports of nodes 400 on each chip.  The exception about transparency is that a node 400 cannot tell by inspecting its IO register whether the other chip's node 400 is actually reading its UP port.  This bridge code is used by External and Internal IDE, by boot streams, and afterward by applications in 2-chip environments.  For discussion, see 5.1.6 above.

Ganglia Mark 1 (any node)

Ganglia Mark 2 (any node)

By default we like to fill all nodes with this versatile messaging fabric, which is capable of exchanging up to 262k word payloads and replies between any two nodes on one or more chips that can be reached via any path through contiguous nodes running the ganglia and connected by COM ports or equivalent, such as the synchronous port bridge or any other bridging mechanism that supports flow control.  These Ganglia are documented fully in App Note AN017, Ganglia Mark 2.

Object code is stored in Virtual Bins 1714, 1715, 1716 and 1717, one for each of the four physical orientations of F18A nodes.  To fill both chips with ganglia, use the descriptor macro  GANGLIA  as an early step (before any memory load descriptors are processed) in a new frame definition.

SRAM Mark 1 Cluster (7, 8, 9, 107, more)

Code for providing services to, and coordination among, up to three Masters for a 1 MWord external SRAM.  See AN003, SRAM Control Cluster Mark 1, for detailed documentation of usage and of internals.

Snorkel Mark 1 (any SRAM client node)

Code for a programmable DMA channel that may act as one of the Masters for the SRAM cluster.  See AN010, The Snorkel Mark 1, for complete details.

IDE Components (708, 300 + any nodes)

The External IDE loads these components into the nodes it uses to reach into a chip and touch any nodes to which a path of available nodes exists.

      - IDE Asynchronous Interface (708)

Code for node 708 to provide services for external IDE.  See AN019, Interactive Development Environment, for discussion of internals.

      - IDE Synch Interface (300)

Code for node 300 to provide services for external IDE.  See AN019, Interactive Development Environment, for discussion of internals.

      - IDE Special Wire (any node)

Code for the nodes, if any, between the root node and the end node of an external IDE path.  See AN019, Interactive Development Environment, for discussion of internals.

      - IDE End Node (any node)

Code for the final node, if any, between the root and target nodes of an external IDE path.  See AN019, Interactive Development Environment, for discussion of internals.

Boot Stream Components

Boot stream generation is done entirely by distributed utilities.  The components should be changed only if absolutely necessary and then only with great care.

      - SPI Flash Speed-up (705)

The ROM code for SPI flash booting uses, initially, a very slow timing parameter so that it can interoperate with slow devices.  The first boot frame included in a boot stream for use from flash should begin by dropping a short program into node 705 and executing it to adjust this parameter for a higher speed.  This program is provided in Virtual Bin 1608 for use by GreenArrays utilities.

      - Synch Boot Master

This code for node 300, in bin 1901, can transmit boot frames to a a GreenArrays synchronous boot node.  It's used as a step in the set-up of a Port Bridge and for other purposes.

      - Port Bridge Set-up

This code for node 400, in bin 1907, is used ephemerally along with the Synch Boot Master in node 300 and a copy of the code for the Synchronous Port Bridge in node 500.  Once all three of these nodes have been loaded it initializes node 300's I/O, resets the other chip via node 500 on pin 500.17, loads the port bridge code into node 300 of the other chip and starts it up, then loads the same code into our chip's node 300 and starts it as well.  After this is completed nodes 400 and 500 may be re-loaded with application code, filled with Ganglia, or whatever else you desire.

      - SRAM Initialization (specific nodes)

polyFORTH Cluster (specific nodes)

This body of F18 code implements a virtual machine that runs a pseudo-instruction set from external SRAM.  It includes a 2-node basic VM with four nodes of extension coprocessors, serial I/O through nodes 100 and 200 with associated wiring, and SPI flash mass storage.  Dependencies include the SRAM Control Cluster Mk1, Snorkel Mk1 and Ganglia Mk2.  Support is provided for the Ethernet NIC and for the various 10MHz frequency references it supports.  For details about this body of code, please Chapter 10, polyFORTH.

Ethernet NIC Cluster (specific nodes)

This body of F18 code, documented in AN007, implements a 10baseT Full Duplex autonomous NIC that operates on data structures in external SRAM as a virtual DMA device.  The object code is temporarily stored in bins corresponding with the nodes in which the code resides when loaded.  In due time it will be moved to library bins.

ATS Mark 1 (Serial IDE based)

The Automated Testing System is documented in DB007.  The bulk of its F18 test code, in the form of "creeper" packages, is stored in bins 2000..2208, although a couple of components are in bin "row" 1900.

ATS Mark 2 (On-chip based)

Words.

eForth (specific nodes)

Words.

Documented Examples

Words.

      - Practical Example used above

Words.

Additional Test Code

Words.

Bin Assignments

"Bins" are receptacles for object code and are numbered like nodes in the chips.  Bins 000 through 717 contain code that is by default destined for the nodes on the host chip; future releases will vacate this range completely.  Bins 800 through 1517 map onto the target chip.  Bins 1600 through 3117 are reserved for utility and tool code, and are managed by GreenArrays.  Their purpose is to allow this code to be compiled and available for use at any time without interfering with application code.  This table documents the assignments in effect as of the release identified above; yellow highlight indicates conversion to and validation of af-3 source.

1600  IDE async root node

1601  IDE sync root node

1602  IDE wire node

1603  IDE end node

1604  ide all nodes template

1605  Snorkel Mk1

1606  SST25WFxxx Flash node 705 pF

1607  New flash helper 706

1608  spi speedup function

1609  18-bit flash r/w for 705

1610  18-bit flash helper for 706

1611  8-bit flash r/w

1612  Flash ops for new devices

1613  flash erase

1614  SRAM Node 107 interface (Mk1)

1615  - Node 007 SRAM Data Mk1

1616  - Node 008 SRAM Ctls Mk1

1617  - Node 009 SRAM Adr Mk1

1700  polyFORTH Stack (106)

1701  - Stack down (006)

1702  - Stack up (206)

1703  Bitsy (105)

1704  - Bitsy down (005)

1705  - Bitsy up (205)

1706  Serial tx (100)

1707  - Rx (200)

1708  - Interface (104)

1709  - Wire (102 and others)

1710  Flash to sram for 705

1711  - Wire for 605 etc

1712  - <unused>

1713  - Temp SRAM code for 108

1714  Ganglion nodes eee (Mk2)

1715  - nodes eoo

1716  - nodes oee

1717  - nodes ooo

1800  <reserved for eForth/pF>

1801  Ethernet DMA

1802  Small tools (wire, etc.)

1803

1804

1805

1806

1807

1808

1809

1810

1811

1812  Flash to SRAM 8-bit

1813  Node 600 code (temporary)

1814  Flash to sram for 705

1815  - Wire for 605 etc

1816  - SRAM interface for 208

1817  - Temp SRAM code for 108

1900  ats/ide analog

1901  ats/ide sync boot master

1902  ats/ide test frame

1903  ats/ide test frame

1904  Sync Bridge (with flow)

1905  ats/ide uut bridge debug

1906  ats/ide tester bridge debug

1907  framer bridge builder

1908  Sync Bridge (no flow)

1909

1910

1911

1912

1913

1914

1915

1916  ATS empty creeper

1917

2000  ATS Port test

2001  ATS Port test with 2*

2002  ATS RAM extensive test (Jeff)

2003  ATS T&S register test

2004  ATS T&R register test

2005  ATS Data stack registers

2006  ATS Return stack registers

2007  ATS LoVD RAM test (Mark)

2008  ATS Stacks 1 (Mark)

2009  ATS Multiply Step & shift

2010  ATS T,S,A&R Data Path Tests

2011  ATS GPIO pin test (no EVB)

2012  ATS Data stack (Mark)

2013  ATS I/O reg latches

2014  ATS Carry set/clear

2015  ATS @p test (Mark)

2016  ATS R,I to Address data paths

2017  ATS ROM Checksums

2100  ATS Parallel Port Pins

2101  ATS @b !b (Mark)

2102  ATS T-B (Mark)

2103  ATS Port-I (Mark)

2104  ATS Port-I Shorts 1 (Mark)

2105  ATS Port-I Shorts 2 (Mark)

2106  ATS Port-I Shorts 3 (Mark)

2107  ATS Port-I Shorts 4 (Mark)

2108  ATS Port-I Shorts 5 (Mark)

2109  ATS Port-I Shorts 6 (Mark)

2110  ATS P-R-P Call (Mark)

2111  ATS P-R-P Execute (Mark)

2112  ATS SERDES 001 Slave

2113  ATS SERDES 701 Master

2114  ATS Analog Basic checks

2115  ATS 2* (Mark)

2116  ATS 2/ (Mark)

2117  ATS 2*.b (Mark)

2200  ATS + (Mark)

2201  ATS Ret&Data Stacks v2 (Mark)

2202  ATS Data stack v2 (Mark)

2203  ATS + v2 (Mark)

2204  ATS and (Mark)

2205  ATS and v2 (Mark)

2206  ATS xor (Mark)

2207  ATS xor v2 (Mark)

2208  ATS inv (Mark)

2209

2210

2211

2212

2213

2214

2215

2216

2217

2300  <reserved creeper tests 4>

2301

2302

2303

2304

2305

2306

2307

2308

2309

2310

2311

2312

2313

2314

2315

2316

2317

2400

2401

2402

2403

2404

2405

2406

2407

2408

2409

2410

2411

2412

2413

2414

2415

2416

2417

2500

2501

2502

2503

2504

2505

2506

2507

2508

2509

2510

2511

2512

2513

2514

2515

2516

2517

2600

2601

2602

2603

2604

2605

2606

2607

2608

2609

2610

2611

2612

2613

2614

2615

2616

2617

2700

2701

2702

2703

2704

2705

2706

2707

2708

2709

2710

2711

2712

2713

2714

2715

2716

2717

2800

2801

2802

2803

2804

2805

2806

2807

2808

2809

2810

2811

2812

2813

2814

2815

2816

2817

2900

2901

2902

2903

2904

2905

2906

2907

2908

2909

2910

2911

2912

2913

2914

2915

2916

2917

3000  GPS Correlator Mk1 Carr 1st

3001    " " Carrier 2nd

3002    " " Carrier Turnaround-1

3003    " " Carrier Turnaround 2

3004    " " Code

3005    " " Carrier special first

3006    " " Carrier special last

3007  GPS Strm delay fill

3008  GPS Strm delay empty

3009  GPS Osc Monitor (500)

3010  GPS Osc Agent (600)

3011

3012  GPS TEMP input mux strm/data

3013  GPS TEMP Smart answer buffer

3014  GPS TEMP snork/gang sig feed

3015  GPS TEMP Sig gen for meas

3016  GPS TEMP Haleakala sF inject

3017  GPS TEMP absorber

3100  SRAM test helper node 108

3101  GPS Sig Input 708

3102  GPS FIFO Mk3

3103  GPS Instrumentation 705->706

3104  GPS Mem Mk3 608

3105  GPS TEMP Sig Input 709

3106

3107

3108

3109

3110

3111

3112

3113

3114

3115

3116

3117

000  TEMP

001  TEMP

002  TEMP

003

004

005  TEMP

006  TEMP

007  TEMP

008  TEMP

009  TEMP

010  ETH Rx Control

011  ETH Rx Byte swap

012  ETH Rx Pack

013  ETH Rx CRC

014  ETH Rx Frame

015  ETH Rx Parse

016  ETH Rx Timing

017  ETH Rx Jitter buffer & Clean

100  TEMP

101  TEMP

102  TEMP

103  TEMP

104  TEMP

105  TEMP

106  TEMP

107  TEMP

108  TEMP

109  ETH Slave of DMA Nexus

110  ETH Master of DMA nexus

111  ETH Tx Control

112  ETH Wire **

113  ETH Tx Unpack

114  ETH Tx CRC

115  ETH Link States

116  ETH Wire **

117  ETH Rx Pin

200  TEMP

201  TEMP

202

203

204  TEMP

205  TEMP

206  TEMP

207  TEMP

208  TEMP

209

210

211

212

213

214  ETH Tx Delay

215  ETH Negotiation

216

217  ETH Rx Pull-down

300  TEMP

301  TEMP

302

303

304  TEMP

305

306

307

308

309

310

311

312

313

314  ETH Tx Framing

315  ETH Tx Mux

316  ETH Wire **

317  ETH Tx Pin

400  TEMP

401  TEMP

402

403

404  TEMP

405

406

407

408

409

410

411

412

413

414

415  TEMP XTAL Start control

416  TEMP XTAL Sequencer

417  ETH Passive TX Oscillator

500  TEMP

501  TEMP

502

503

504  TEMP

505

506

507

508

509

510

511

512

513

514

515

516

517  TEMP Xtal Osc

600  TEMP

601  TEMP

602  TEMP

603  TEMP

604  TEMP

605  TEMP

606

607

608

609

610

611

612

613

614

615

616  ETH/CLK Osc Agent

617  ETH/CLK Osc Monitor

700  TEMP

701  TEMP

702

703

704  TEMP

705  TEMP

706  TEMP

707

708  TEMP

709

710

711

712

713

714

715

716

717  TEMP

Examples to remember

A simple stream:

1581

0 ( Descriptor test)

1 ASM[ # 715 NODE ERS # 0 org

2    begin begin  !b unext unext   >BIN ]ASM

3 0 ARRAY MYP  207 ORGN 210 TO 710 TO 715 TO -1 ,

4 207 STREAM[   ' MYP COURSE

5    FRAME[   715 +NODE  0 1 715 /PART

6    -1. -1. -1. -1.  -1. -1. -1. -1.  -1.  9 /RSTACK

7    x20001. x30002. x20003. x30004. x20005. x30006.

8    x20007. x30008. x20009. x30010.  10 /STACK  IO /B  0 /P

9    714 +NODE  111. 222. 333. 444. 4 /STACK

10    ]FRAME  ]STREAM

11 !SNORK

.STREAM 512

0  10175  4DAF 121D5    78 2FAB2  5A72  4DAF 12175   208,9

8     72 2FAB2  5A72  4DAF 12115    6C 2FAB2  5A72   210

10   4DAF 12145    66 2FAB2  5A72  4DAF 12115    60   310,410

18  2FAB2  5A72  4DAF 12145    5A 2FAB2  5A72  4DAF   510,610

20  12115    54 2FAB2  5A72  4DAF 121D5    4E 2FAB2   710

28   5A72  4DAF 12175    48 2FAB2  5A72  4DAF 121D5   711,12

30     42 2FAB2  5A72  4DAF 12175    3C 2FAB2  5A72   713

38   4DAF 121D5    2E 2FAB2  5A72  4A12     0     0   714,15

40  2E9B2  5872  9175  4BB2   15D  48B2 3FFFF  48B2   Regs

48  3FFFF  48B2 3FFFF  48B2 3FFFF  48B2 3FFFF  48B2

50  3FFFF  48B2 3FFFF  48B2 3FFFF  48B2 3FFFF  49B2

58  20001  49B2 30002  49B2 20003  49B2 30004  49B2

60  20005  49B2 30006  49B2 20007  49B2 30008  49B2

68  20009  49B2 30010 101B5  49B2    6F  49B2    DE   714

70   49B2   14D  49B2   1BC 101B5 101B5 101B5 101B5   713-11

78  101B5 101A5 101A5 101A5 101A5 101A5 101A5 101A5   710-208

Maintenance

This section covers procedures for maintaining the sF and pF/144 systems.

saneFORTH Maintenance

Bootstrapping

sF is normally started on a Win32 platform via one of three Windows executables, named in the following forms:

- sF<vers>-aF3.exe  the normal production nucleus, with 16 MB of system memory.  This executable must lie in a path whose name contains no embedded spaces because it parses its command line with simple code.  Upon boot, this autoloading nucleus loads block 534 which loads block 9, defines code for accessing the command line, parses it, and INTERPRETs any FORTH code following the first space (which should be the one following the name of the executable).  This program is normally started using a supplied shortcut by the name arrayForth 3 G144A12 whose command line has  AFORTH appended.

- sF<vers>-glow.exe  the normal production nucleus, but with 1.6 GB of system memory to support the GLOW design tools (which are redacted from publicly released systems.)  Normally started using a supplied shortcut by the name  arrayForth 3 with GLOW .  As distributed the shortcut has no interpretive code.

- sF<vers>-panic.exe  is a standard saneFORTH nucleus which does not autoload; you will need to type HI to load block 9.  It has its own shortcut as well, named arrayFORTH 3 PANIC.  It is provided to give a clean saneFORTH environment that may be used, for example, to fix mistakes that have been made in either the 9 LOAD or in the AFORTH code.

Nucleus Maintenance

Because this is a hosted system, the nucleus that is run for saneFORTH is necessarily packaged into the Windows executable.  The sF nucleus source, as well as the Target Compiler and other utilities employed to test nuclei and to generate Windows executables, are proprietary to FORTH, Inc. and/or ATHENA Programming, Inc. and GreenArrays has not been licensed to distribute them.  Therefore these sections are redacted and users of arrayForth 3 are unable to maintain the underlying x86 nucleus.  Special versions, if necessary, must be requested from GreenArrays.

polyFORTH/144 Maintenance

pF/144 is fully equipped to maintain and alter both its Virtual Machine and its polyFORTH nucleus.  These procedures are fully documented in DB006, G144A12 polyFORTH Supplement.  The shadows for pF/144 nucleus source, in the index page at 60, constitute its primary documentation.

Introduction to the substrate polyForth

The polyFORTH ISD-4 Reference Manual is designed to provide a reference source for the most common features of the polyFORTH integrated software development system.  We assume at least an elementary knowledge of Forth, of a level consistent with having studied Starting FORTH and attended a polyFORTH programming course, or the equivalent.  If you are new to Forth, we encourage you to begin learning by reading Starting FORTH carefully, working the problems at the end of each chapter.

This Reference Manual is primarily intended to describe how a programmer can use polyFORTH to solve problems.  This is a rather different goal from explaining how polyFORTH works, but it is a practical necessity for the new user of a polyFORTH system.  This manual is also organized to serve experienced programmers who need to check some point quickly.

We also highly recommend that you spend some time actually reading the polyFORTH source supplied with your system, about 40 pages of program source with extensive embedded documentation.  polyFORTH was designed to be highly readable, and the source listing offers many examples of good usage and programming practice.

We cannot cover “all” polyFORTH commands:  polyFORTH is an extensible system and no two implementations need or use identical components.  What we can do is provide a detailed exposition of the most valuable and most commonly used features and facilities of the system from which your application begins.

FORTH, Inc. supports polyFORTH for a growing number of mini and micro-computers.  Since hardware is unique for each computer, it is not feasible for this document to cover every feature of every computer supported.  This Reference Manual presents features common to all polyFORTH systems.  In discussing features that are hardware-specific, particularly dictionary structure, high-level object format, data base management, and peripheral drivers, an idealized model of a polyFORTH system is used.  Refer to the CPU-specific supplement that accompanies each system for specifics of that system.

## 1.1	Forth Language Features

Many of the following topics are treated in a tutorial form in Starting FORTH.  This section highlights special considerations arising from the polyFORTH implementation.  More detailed technical discussions of subjects covered here will be found in later sections of this manual, especially Section 2.0.

### 1.1.1	Dictionary

The dictionary contains all the executable routines (or “words”) which make up a polyFORTH system.  System routines are entries predefined in the dictionary that become available when the system is booted.  Electives are definitions that are optionally compiled to the dictionary just after booting.  User-defined words are entries that you add subsequently.  System and elective definitions are available to all users in a multi-user configuration, whereas user-defined words are available only to the user.  Otherwise there are no differences in size, speed, or structure.  You may make user words available to other users simply by loading them with the other electives.

The dictionary is the fundamental mechanism by which polyFORTH allocates memory, and by which Forth performs “symbol-table” operations.  Because the dictionary serves so many purposes, you should be sure you understand how to use it.  You may wish to review this material in Starting FORTH.

The dictionary is a linked list of variable-length entries of Forth words and their definitions. It grows toward high memory; each entry points to the one that logically precedes it (see Fig. 1.1).  The next available cell at the end of the dictionary is pointed to by the variable H.  This address will be put on the stack by the word HERE.

The dictionary is searched by sequentially matching names in source text against names compiled in the dictionary.  To speed dictionary searches, the dictionary is organized in eight linked chains of approximately equal length.  A word will be found in one of these chains depending upon a “hash value” computed from its name and the “vocabulary” in which the word is located (see the reference below on vocabularies, and Starting FORTH, p. 219*).  The search follows the selected chain until a match is found or the end of the chain is reached.  Since the latest definition will be found first, this organization permits a word to be redefined, a technique that is frequently useful.

Fig. 1.1

The “top” of a user’s dictionary.  H points to the next available byte; there is a copy of H for each user in a multi-user system.

A “vocabulary” is a subset of the dictionary containing words for some special purpose.  There are three vocabularies present in all systems and these are available to all users on a re-entrant basis:

Word	Description

FORTH	Contains all “system” commands plus the shared re-entrant portion of an application.

EDITOR	Contains commands used to edit polyFORTH text.

ASSEMBLER	Contains commands used only in code definitions (op-codes, etc.).

In addition, each user in a multi-terminal system may have his own private dictionary, which may contain words added to the standard vocabularies.  Each user has a private variable called CONTEXT which tells the text interpreter the order in which to search the vocabularies.  All private dictionaries are linked to the public dictionary, which means that a search will start in the user’s dictionary and thread back through his private definitions before searching through the shared dictionary.  This has the effect of protecting private portions of the dictionary from mistaken or unauthorized use.

Fig. 1.2

Diagram of the logical structure of the polyFORTH dictionary.  Logically consecutive definitions don’t have to be in contiguous memory.

The essential structure of all dictionary entries is the same for all words and is diagrammed in Fig. 1.2.  The first cell, called the link, contains the location of the preceding entry.  This speeds up searches, which start at the “recent” end of the dictionary and work backwards to the “older” end.  By this process, the most recent definition of a word is always found.  In a developed application, where the user is dealing with the highest level of the program, this process optimizes search time.

The second cell of a dictionary entry contains the count of characters in the name field, followed by the first of three or more characters of the name.  The name may be longer, depending on what the value of the user variable WIDTH was when the definition was compiled, and how long the name is.  WIDTH sets the maximum number of characters which will be saved when a name is compiled.  On processors that require addresses to be on even-byte addresses, WIDTH should be set to an odd value.  On most machines, WIDTH works with any value from 3 to 31.

WIDTH is a variable, two of whose bytes are used separately.  The lowest addressed byte holds a character count up to 31, to be used as the maximum character count of the next definition.  The next byte of WIDTH stores a default value (also up to 31), to which the lowest addressed byte is set after each definition.  Most of the polyFORTH nucleus is compiled using three-character names, so WIDTH in the system as furnished is set to the hex value 0303.  If you want to change WIDTH to compile a 31-character name, you may do it two ways.  Using:

31 WIDTH C!

changes the lowest addressed byte only.  After the next definition, WIDTH will again be set to 3.  You may get the same offset by using the ~ (“tilde”) command.  Using ~ preceding a definition sets WIDTH to 31, specifying that the next definition only will be compiled to its full width.  For example:

: CONTEST   ... FORTH words ... ;

~ : CONTEXT   ... FORTH words ... ;

: DABBLE   ... FORTH words ... ;

In this example, three definitions will be created.  The entries for DABBLE and CONTEST will each have names with three letters and a count.  The entry for CONTEXT will contain all seven letters.  The words CONTEST and CONTEXT will not interfere with each other.  If their order in the dictionary were reversed, CONTEXT would never be found by a search because CONTEST would be found and executed by each search for CONTEXT.  As a result, it is important that such name conflicts be resolved by always using  ~ with the most recent of the conflicting pair.

To make a “permanent” change of WIDTH to, let’s say, 31 characters, use:

HEX  1F1F WIDTH !  DECIMAL   (hex 1F is 31 decimal)

This changes both bytes of WIDTH to 31.*  Remember:  WIDTH C! stores a byte into the lowest addressed byte of WIDTH, while WIDTH 1+ C! stores a byte into the highest addressed byte of WIDTH, and WIDTH ! changes both lowest and highest addressed bytes.  Never use 31 WIDTH !, which puts 0 in the default byte of WIDTH, rendering all subsequent definitions invisible (because WIDTH will be thereafter set to 0).

Fig. 1.3

Structure of a dictionary entry.  The actual byte order and position of bits varies somewhat with each CPU; see the polyFORTH ISD-4 CPU Supplement for the correct order for your system.

The use of three characters and a count gives you far more flexibility than a simple limit on the number of characters, and saves a substantial amount of space in comparison with compiling all names to full width, but it does require uniqueness in the length and first three characters.  Any characters other than space, backspace and carriage-return can be used as part of a name field.

Although the name field is arranged in each implementation to optimize each machine’s dictionary search, the general model in Fig. 1.3 is followed whenever possible.  In this example, since the longest allowed name field has 31 characters, and needs only five bits to express a count, the byte containing the count in a dictionary entry has its three most significant bits available for other purposes.  The most significant bit, bit seven, is used as the precedence bit.  When the precedence bit is set, the word is executed by the compiler at compile time.  The precedence bit is set by the word IMMEDIATE, and is used for a few special words, such as compiler directives.  The precedence bit is set to zero for most words.  Bit six is reserved for future use by FORTH, Inc.  Bit five is the “smudge” bit.  When this bit is set, the word is invisible to a dictionary search.  The smudge bit is set by the compiler when starting to compile at the beginning of a colon definition, to prevent unintentional recursive references.  It is reset by the semicolon that ends the definition.  This is accomplished by the word SMUDGE, which toggles the smudge bit of the most recent definition, setting it if it is reset and vice-versa.

In another common arrangement the most significant bit of the cell containing the count and first character remains the precedence bit, but bit seven of the cell is the smudge bit.  Consult your CPU Supplement for your machine’s arrangement.  The first cell after the name field contains a pointer to the code to be executed for the definition. This pointer is called a “code field” and points to machine code whose behavior specifies the type of word.

For a CONSTANT, the code field address refers to code that puts the value of the constant (which is in the second cell after the name field) on the stack:

Fig. 1.4

For a VARIABLE, the code field address refers to code that puts the address of the value (stored just after the code field address) on the stack:

Fig. 1.5

For a defined word, the code field address points to a portion of the address interpreter that will begin following a string of addresses starting just after the code field cell and will continue until it finds the EXIT compiled by ; which terminates that definition.  A diagram of a : definition is shown in Fig. 1.6.

Fig. 1.6

For assembler coded words, the pointer is to the next cell itself, which contains the first instruction of the code that is executed directly:

Fig. 1.7

A problem which can occur is finding the code field address from a given link field address, despite a variable-length, possibly truncated name field.  The predefined word CFA translates a link field address to a code field address, so that you do not need to write this code yourself.

The cells after the code field address are called the parameter field, which is of variable length.  Words such as CONSTANTs and VARIABLEs keep their values in the first parameter field cell, as noted above.  Other definitions may keep several values.  In the latter cases, the length of the parameter field is either determined by the type of definition or kept in one of the early cells in the field.

The polyFORTH words concerned with dictionary management are H and HERE:

Command	Function

H	(A variable.)  Contains the location of the head (next available cell) of the user’s private dictionary.

H 2+	(H 4+ for 32-bit machines.)  Contains the address of the beginning of the user’s private dictionary.

HERE	Pushes the contents of H onto the stack.

ALLOT	Increments H by a number of bytes given on the stack.

REFERENCES

CODE Definitions, Section 6.1
Code Field Addresses, Section 2.7.4
Creating Dictionary Entries, Section 2.7.1
Vocabularies, Section 3.4

### 1.1.2	Parameter Stack

Each multiprogrammed task has a parameter stack in the upper part of its partition, as shown in Fig. 1.8.  The purpose of the parameter stack is to contain numeric operands for polyFORTH operators.  polyFORTH operators expect their parameters on this stack and leave their results there.  The size of a parameter stack is indefinite; its legal domain extends from its start downward to the top of the user’s dictionary, less the space required for PAD and working storage.  These amounts vary, but typically approximately 256 bytes must be reserved for working storage.  The parameter stack rarely grows beyond 10-20 entries in a well-written application.

When numbers are pushed onto or popped off of the stack, the remaining numbers are not moved.  Instead, a pointer is adjusted to indicate the last used (bottom) cell in a static memory array.  On most computers, the stack pointer is kept in a register.

It is important that the stack extend toward low memory for two reasons:

1.	So that a positive address relative to the stack pointer will locate numbers on the stack, and

2.	So that double-cell items are placed on the stack in the same relative position that they have in memory.  Because the stack grows toward low memory, a “push” operation involves decrementing the stack pointer, while a “pop” involves incrementing it.

A number encountered in text by the text interpreter will be converted to binary and pushed onto the stack.  polyFORTH “nouns” (data items such as VARIABLEs and CONSTANTs) are defined to push their addresses or values onto the stack.  Thus, the stack provides a medium of communication not only between routines but between a person and the computer.  You may, for example, place a number or address on the stack and then type a word which acts on this to produce a desired result.  Typing:

12 2400 * 45 / .

pushes the number 12 on the stack, pushes 2400 over it (see Fig. 1.9), replaces both numbers by their product (*), pushes 45 on the stack, divides the product by 45 (/) and types the quotient (.).  All numbers put on the stack are removed, leaving the stack as it was before typing 12.

The basic dictionary provides some words for simple manipulation of single and double-length operands on the stack:  SWAP, DUP, DROP, 2SWAP, etc. (covered in detail in Section 2.1).

Fig. 1.8

Relative location of the parameter stack in both a terminal task and a background task.

The use of the push-down stack simplifies the internal structure of polyFORTH and produces naturally re-entrant routines.  Passing parameters via the stack means that fewer variables must be named, reducing the amount of memory required for named variables (as well as the associated programmer’s housekeeping).

Fig. 1.9

The parameter stack in memory.  S0 points to the “empty stack” location (which is constant for each user), and S (in a register) points to the top item.  A zero is kept in the stack underflow position for “fail-safe” protection.

A pointer to the top (latest entry) of the user’s stack is maintained by the system.  Its actual value may be obtained at any time by use of the word 'S (pronounced “tick-S”).

The cell immediately above the “bottom” parameter stack location always contains a zero.  Just above this is a space which, in most systems, is used for the terminal input buffer.  At the top of this region lies the return stack. This arrangement helps make the system resistant to minor stack underflows.  Severe underflows, however, can cause a system crash.

polyFORTH checks for stack underflow only after attempting to execute a word from the input stream. Underflows that occur during execution will not be detected at that time.

The result of a detected stack underflow is the message:

Stack empty

followed by a system abort.

REFERENCES

Forth Re-entrancy, Section 4.1
Stack Manipulation, Section 2.1
System Abort Routines, Section 2.4.7

### 1.1.3	Return Stack

Each multiprogrammed task has its own return stack, located above its parameter stack in memory (see Fig. 1.8).  The return stack serves the following purposes:

1.	It holds return addresses for nested definitions.

2.	It holds loop parameters.

3.	It saves temporary data, especially file and record pointers in the Data Base Support.

4.	It saves interpreter pointers when loading source text blocks.

Since the return stack has multiple uses, care must be exercised when using it to avoid conflicts.

There are no commands for direct manipulation of the return stack, except for those moving parameters to and from the top of the stack.

The maximum size of the return stack for each task is specified at the time that the task is defined, and remains fixed during operation; a typical size is 64 cells (136 cells on 32-bit machines).

REFERENCES

Loading, Section 3.3.1
Loop Parameters, Section 2.4.4 
Parameter Stack, Section 1.1.2 
Return Stack Use in Data Base Support, Section 8.1.3
Transfers Between Stacks, Section 2.1.3

### 1.1.4	Text Interpreter

The text interpreter serves two critical functions:

1.	It executes the commands that users type.

2.	It executes commands in source blocks stored on disk.

The terminal is the default text source.  The terminal input interrupt handler will accept up to 80 characters into a text buffer called the “Terminal Input Buffer,” or TIB.  Either the acquisition of the eightieth character or an ASCII RETURN character (0DH) will cause the terminal’s task to be activated, under the control of the interpreter which will process the text in the buffer.  For example, typing:

100 LOAD

however, directs the interpreter to the 1024-byte string that is stored in Block 100.  This string is brought into memory from mass storage automatically.  In order to let one block load another, the current block number (the user variable BLK, set to zero for terminal input) and the text interpreter character pointer (the user variable >IN, pronounced “to-in”) are saved on the return stack.  So, in the middle of a LOAD of one block, that block may direct LOADing a different block, and return to complete the first block.

When the text interpreter executes a defining word (such as CONSTANT, VARIABLE, or :) the result is that a definition is compiled into the dictionary.

Because the text interpreter is a colon definition, it is interpreted by the address interpreter.  The text interpreter is defined by a phrase equivalent to:

: INTERPRET   BEGIN  -' IF  NUMBER  ELSE
   DROP  EXECUTE  DEPTH 0<
   ABORT" Stack empty"  THEN AGAIN ;

where:

Word	Function

BEGIN	Marks the beginning of an infinite loop.

-'	Extracts a word from the input string using the delimiter 32 (“blank” in ASCII code).  It then searches the dictionary, returning a true value on the stack if the interpreter does not find a match for the word, false otherwise.

IF	Continues on a true condition, otherwise skips to the word after ELSE.

NUMBER	Attempts to convert the word to a binary number; if the conversion fails (due to illegal characters), it aborts with an error message.

ELSE DROP	Eliminates an unused address left by -'.

EXECUTE	Executes the address of the word found in the dictionary.

DEPTH 0<	Returns a true value if there is a stack underflow condition, false otherwise

Fig. 1.10

Diagram of INTERPRET, the text interpreter.

Word	Function

ABORT" Stack empty"	Aborts with the error message “Stack empty” if there was a stack underflow.

THEN	Marks the place where both branches merge and continue.

AGAIN	Returns to BEGIN to repeat the procedure.

This process is diagrammed in Figure 1.10.

REFERENCES

Disk Blocks, Section 1.2.2
System Abort Routines, Section 2.4.7
Text Interpreter Words, Section 2.6
The LOADing Process, Section 3.3.1
Using EXIT to Leave a Load Block, Section 2.4.6

### 1.1.5	Numeric Input

The word NUMBER is used by the text interpreter to convert strings of ASCII numerals and punctuation into binary integers that are pushed onto the stack.  polyFORTH interprets any number containing an imbedded punctuation character (see below) as a double-precision integer.*  Single-precision numbers are recognized by their lack of special punctuation.  Conversions operate on character strings of the following format:

[ - ] dddd [ punctuation ] dddd ... delimiter

where “dddd” is one or more valid digits according to the current base or radix in effect for the user.  The user variable BASE is always used as the radix.  All numeric strings must be ended by a blank or a carriage return.  If another character is encountered (i.e., a character which is neither a digit according to the base nor punctuation or a blank), an abort will occur.  There must be no spaces within the number, since a space is a delimiter.

The leading minus sign, if present, must immediately precede the first digit or punctuation character.

Any of the following punctuation characters may appear in a number:

Word	Description

,	(comma)

.	(period)

+	(plus)

-	(hyphen, may appear anywhere other than to the immediate left of the most significant digit)

/	(slash)

:	(colon)

All punctuation characters are functionally equivalent.  A punctuation character causes the digits that follow to be counted.  This count may be used later by certain of the conversion words.  The punctuation character performs no other function than to set a flag that indicates its presence.  Multiple punctuation characters may be contained in a single number; the following two character strings would convert to the same number:

1234.56

1,234.56

Punctuation characters do not affect the resulting number.

The user variable PTR is used during the number conversion process to track punctuation.  PTR is initialized to a large negative value, and is incremented every time a digit is processed.  Whenever a punctuation character is detected, it is set to zero.  Thus the value of PTR immediately following a number conversion contains potentially useful information:

•	If it is negative, the number was unpunctuated and is single-precision.

•	Zero or a positive non-zero value indicates the presence of a double-precision number, and gives the number of digits to the right of the right-most punctuation character.

This information may be used to scale a number with a variable number of decimal places.  Since PTR doesn’t care (or, indeed know) what punctuation character was used, it works equally well with American decimal points and European commas to start the fractional part of a number.

The working copy of the number during the conversion process is 32 bits on the stack.  If the number is single-precision (negative PTR), the high-order part of the working number (normally zero) is saved in a location given by the phrase:

'NUMBER 2+	(4+ on 32-bit CPUs)

This information may be recovered to force the number to double precision.  This is useful when dealing with naturally unpunctuated large numbers such as five-digit zip codes.

Most large numbers are punctuated, however, because humans remember them better that way.  Some numbers such as:

229-48-0332
8/03/40
372-8493
3,124,896
8:45:06

will automatically convert appropriately.

REFERENCES

Use of the Text Interpreter for Number Input, Section 2.6.2

### 1.1.6	Address Interpreter

The address interpreter executes previously compiled high-level definitions, which compile to a string of absolute addresses of definitions to be executed in turn.  When a standard Forth colon  definition is invoked, the run-time code for a colon definition (called colon) sets the interpreter pointer I to the parameter field of the definition and executes NEXT.*  NEXT is the most fundamental routine of the address interpreter, which branches to the next routine (address) to be executed as indicated by I.  NEXT performs the following functions:

1.	Moves the current value of I into W, so that W points at the code field address of the word now being executed.

2.	Increments I to point to  the next address.

3.	Performs an indirect jump to the address in W.

4.	Increments W to point to the parameter field of the word being executed.

The efficiency with which these operations can be performed is a measure of a machine’s overhead to run high-level polyFORTH.  On most computers NEXT can be reduced to one or two instructions.  If the word being executed is a code word, the machine code is executed directly.  If it is another : definition, however, the run-time code for : is executed.  The word colon performs the following functions:

1.	Pushes the current value of I onto the return stack (which has a top-of-stack pointer named R).

2.	Moves W (the parameter field address) into I.

3.	Executes NEXT.

At the end of a : definition, the run-time code for ; (the word EXIT) is executed.  EXIT performs the following functions:

1.	Pops the return stack into I.

2.	Executes NEXT.

The time required to execute the run-time code for the : ... ; pair is the overhead to nest high-level definitions.

REFERENCES

:, Section 2.7.4
Compiling Words and Literals, Section 2.8
EXIT, Section 2.4.6
I, Section 2.7.4, 6.4

## 1.2	polyFORTH Operating System Features

polyFORTH is based on a multitasking, multi-user operating system.  Many polyFORTH implementations run in a fully stand-alone mode, in which polyFORTH provides all drivers for the hardware attached to the system.  Some versions of polyFORTH, however, run in a “co-resident” mode with a host operating system.  In the latter case, the drivers that supply I/O services for peripherals such as disk and terminals do so by means of issuing calls to the host system.  Although co-resident systems may be somewhat slower than the standalone versions, they do offer full file compatibility with the host OS, and usually somewhat more flexibility in hardware configuration.  Co-resident versions of polyFORTH usually offer all the system-level features of the native systems (including multi-user support on otherwise single-user systems such as MS-DOS), plus added commands for interacting with the host OS; the latter are documented in the CPU Supplement that accompanies this Reference Manual.

### 1.2.1	Typical Memory Organization

A diagram of memory use in a typical application is shown in Fig. 1.11.  The area in low memory contains the precompiled portion of the program.  The system electives are kept in source form to facilitate changes and additions. Since re-compiling electives and application into memory takes only a few seconds, this costs little.  Re-compiling is only necessary during the boot procedure or after a system crash.

Each terminal task has a partition that contains its stacks, private (or “user”) variable area, PAD (for text strings) and dictionary.  A selected vocabulary may be compiled into this partition to do some particular kind of processing which is a subset of the application but which is not available to other users.  The background task has a much smaller area, with only enough space for its stacks; there is no terminal associated with it.  The routines that the background task executes are located in the shared area or the dictionary of one of the terminal tasks.

Fig. 1.11

Typical memory organization in a polyFORTH system.

### 1.2.2	Disk Block I/O

Disk I/O is handled by polyFORTH in standard blocks of 1024 bytes.  This fixed block size applies both to polyFORTH source program text and to data used by polyFORTH programs.  It is used because it allows I/O on different media with different physical sector or record sizes to be handled by one standard block handler.  polyFORTH applications may put several data records into one block or combine several blocks to form one data record.

Standard polyFORTH systems have at least two 1024-byte block buffers in memory.  This number may be increased (for example, in an application with many tasks). For each buffer, there is a “descriptor table” that contains the block number of the block currently residing in that buffer.  The sign bit of the block number will be set if the block has been changed since it was read into the buffer.

A block is requested by the instruction:

n BLOCK

where n is the logical block number.  The block handler will check the block buffers to see whether the requested block is already in memory; if not, it will fetch the block from disk.

When a block is to be read from disk, it will necessarily over-write a block that is currently in a buffer.  polyFORTH optimizes disk performance by attempting to minimize the number of physical disk reads and writes that must be performed.  This is done by maintaining the list of buffers in an order reflecting how recently the block currently in each buffer was referenced.

•	When searching for a block, the most recently used buffer is checked first.

•	When selecting a buffer to be over-written, the least recently used buffer will be selected.

This method of buffer management is called an LRU (for Least Recently Used) algorithm (discussed in detail in Section 3.2.1).

If the block buffer to be over-written has been updated, the updated block will be automatically written to mass storage before the requested block is read.  Finally, BLOCK will push the address of the first cell of the requested block onto the stack.

There are no explicit disk input or output commands; BLOCK will always return the address of the block buffer in memory, having performed actual I/O only if necessary.  To the programmer, all data is always manipulated in memory.

The command:

UPDATE

marks the most recently referenced buffer as having been updated.  UPDATE is included in those operators in polyFORTH’s text editor and Data Base Support option that change fields.

The disk buffer pool is shared among multiprogrammed tasks.  This is an advantage because it tends to reduce disk access for “popular” blocks and avoids the problem of multiple copies of a block.  In applications where block accesses need to be restricted, you may easily add the appropriate protections.

Please note that native (stand alone) versions of polyFORTH do not require any sort of directory in memory or on disk since block numbers are a direct function of disk address (the exact relationship is designed to suit the particular disk involved).  Applications that involve management of complicated data-file structures sometimes do use a disk directory; this is a feature of the application, however, rather than of polyFORTH.

The only requirement for fitting data records into this scheme is that data record numbers be a fixed function of the block number; then you can define a word that will use BLOCK to fetch the block(s) that contain records requested by a specified record number.  The accessing words in the Data Base Support use BLOCK.

Another related technique is useful for keeping very large arrays on disk.  Sometimes data-record applications are written using a virtual byte array.  An example of a handler for a virtual array of bytes stored beginning at Block 250 would be:

: VIRTUAL ( v - a)   1024 /MOD
   250 + BLOCK + ;

: V@ ( v - n)   VIRTUAL @ ;

: V! ( n v)   VIRTUAL !  UPDATE ;

In the example above, VIRTUAL converts an address for the virtual array into an address that points into a block buffer in memory.  The phrase 1024 /MOD 250 + produces a block number on top of the stack, with an offset into the block just underneath.  The word BLOCK uses the block number on top of the stack and leaves the address of the first byte of the block buffer containing a 1024-byte segment of the byte array.  The + adds the block buffer address to the offset to produce an absolute address into the correct block buffer.  The operation of V@ is straightforward, but note the UPDATE in V!.  Without the word UPDATE, the changed block buffer data would never be written out to disk when the block buffer is reused.

Error checking is not performed in the low-level definition of BLOCK.  Status bits are preserved, however, in order that higher-level routines may check for errors and take appropriate action.  The reason that error checking is not done at the lowest level is that it is easiest to take the appropriate action at the application level.  The action that is appropriate in a data base application, for example, will necessarily differ from the action appropriate in a disk diagnostic.  You should check your system listing for details of your particular implementation.  On most systems, the following information is available:

Variable	Content

PREV	Address of the descriptor table for the most recently referenced buffer.

DISK	Address of the STATUS cell of the task currently accessing disk, if any.  Zero if disk is available.

DISK 2+	(4+ for 32-bit machines.)  Status data from the most recent operation using the disk controller.

'BLOCK	Address of the routine to be executed by BLOCK.

'BUFFER	Address of the routine to be executed by BUFFER.

'BUFFER 2+	(4+ for 32-bit machines). The block number of the block just written.

The buffer descriptor pointed to by PREV contains the address of the buffer, as well as the block number of the block in the buffer.  It also contains the address of the descriptor for the next most recently used buffer, and so on for as many buffers as are currently configured in the system.

REFERENCES

Data Base Support, Section 8.0
DISKING Utility, Section 5.3
The Disk Driver, Section 3.2

### 1.2.3	Multitasking

polyFORTH makes it easy to control multiple tasks, either asynchronous background tasks or independent terminal service tasks.  Terminal tasks have text input/output facilities.  Background tasks have some other function.  A small but crucial set of commands is dedicated to the multitasking facility.

The polyFORTH multitasker consists of one relatively small code routine called PAUSE.  PAUSE contains the idle loop of the system.  When the system is “quiet,” it is running around the PAUSE loop looking for a task ready to be awakened.

The limits on the number of tasks in the system are usually defined by memory size.  Since polyFORTH definitions are naturally re-entrant, tasks rarely require much memory.  The polyFORTH multitasking facilities do not limit the number of tasks.  The tasks are linked in a circular chain (Fig. 1.12) in which a new task may be inserted.  The multiprogrammer follows the chain.   Each task has a “user variable area” which contains a pointer to the user area of the next task to be examined.  This “round-robin” algorithm is extremely fast in execution; since the actual process of changing tasks is also simple and fast, the result is maximized service to all tasks.  Should the constraints of a particular application require a more elaborate queuing scheme, it may be substituted directly for the standard definition of PAUSE.

The single task present after booting is named OPERATOR.  This task services the terminal that is used to type additional loading commands.  Since OPERATOR is the first terminal task up after the boot, it is the logical task to begin system loading and other operations.  However, aside from being first and usually having more memory, OPERATOR does not differ from other terminal tasks.

A task is referred to by name.  Invoking a task’s name puts on the stack the address of a pointer to the first cell of its user area.  This first cell contains the task’s status.  When a task is inactive the first cell typically contains a jump to the succeeding task, whose address follows in the next cell.  Thus, in a quiet system the processor is simply jumping from one task to the next.  When a task needs to be activated, a subroutine call to the “awaken” section of PAUSE is substituted for the jump.

Fig. 1.12

Diagram showing several tasks linked in a polyFORTH “round robin.”

When a task relinquishes control of the CPU, the following actions are performed to inactivate it:

1.	The task’s system pointers I and R are pushed on the task’s parameter stack.

2.	The task’s current parameter stack pointer is saved in the task’s user area.

3.	The task’s status is set to the appropriate instruction to jump to the multiprogrammer code that will activate the task.

There are three ways to enter the PAUSE loop in polyFORTH:

1.	The word PAUSE gives each task the possibility of a turn, and then resumes execution.

2.	The word STOP puts a task to sleep until something outside that task, such as an interrupt routine or another task, stores the “wake” instruction into the task’s status cell.

3.	The assembler code ending WAIT is used to re-enter the pause loop from assembly code, and like STOP, does not automatically re-awaken the task containing the WAIT.

The last step of most I/O routines is to enter the multitasking loop via WAIT to wait for the device interrupt routine to complete its data transfer (or other operation).  By performing multitasking while waiting, service to other tasks is maximized during I/O operations.  Because most I/O operations enter the round-robin, polyFORTH’s multitasker is said to be “I/O driven.”  Note that a task which performs no I/O and does not execute PAUSE will retain control of the CPU.

This method has several advantages.  Since a change of tasks can only happen between Forth words, there is no need to save working registers from high-level programs.  As a result, changing tasks takes only a few CPU instructions on any system.  Secondarily, the programmer has full control and knowledge of when the CPU will and will not be relinquished, which can substantially simplify some coding situations.

On the other hand, the programmer does have the responsibility for putting PAUSE in any CPU-intensive routine that may run more than a few milliseconds without performing I/O.  In general, such situations are rare; timing studies have shown that on a native 8088 polyFORTH in normal operation a task needs to wait over 1 ms for service only 1% of the time.

polyFORTH normally runs with interrupts enabled. Interrupt vectors branch directly to the code which services the interrupting device, without any system intervention or overhead.  The interrupt code is responsible for saving and restoring any registers it needs.

Interrupt code (actual assembler code) is responsible for performing any time-critical actions needed, such as reading a value from an analog device and storing it in a temporary location.  The interrupt routine must also notify the task responsible for the device.  Notification may take many forms, ranging from incrementing a counter to “awakening” the task by storing WAKE in the task’s STATUS cell.  WAKE is a constant containing the code that will cause the task to become active the next time it is polled in the round-robin; on most systems this will be in less than 1 millisecond.  Many interrupt handlers do nothing else.

Any processing which is not time-critical can be done by a task running a routine written in high-level polyFORTH.  In effect, the time-critical aspect of servicing an interrupt is decoupled from the more logically complex aspects of dealing with the consequences of the event.  Thus, it is guaranteed that interrupts will be serviced promptly, without having to wait for task scheduling, and yet as a programmer you have the convenience of using high-level Forth executed by the responding task for the main logic of the application.

The process of activating a task is the converse of the process of de-activating it:

1.	The stack pointer is set to the value in the task’s user area.

2.	The task’s values for R and I are popped off the task’s parameter stack and set.

The task will then execute the next Forth word (pointed to by I).  This is the text interpreter if the task was awakened by a terminal message.

REFERENCES

Multitasking, Section 4.0
polyFORTH Re-entrancy,  Section 4.1
User Variables, Section 4.6

## 1.3	The polyFORTH Assembler

All polyFORTH systems contain an assembler for the CPU on which the system runs.  Although it offers most of the same capabilities of other assemblers, its integration into the polyFORTH environment means it will not be fully compatible with assemblers supplied by the computer’s manufacturer.

A polyFORTH assembler produces exactly the same code as a conventional assembler (which means it runs at full machine speed), but it does it somewhat differently.  The differences are in notation and procedure, and are described below.  The differences occur for two reasons:

1.	To improve transportability of polyFORTH applications between processors by making assembler notation as similar as possible without impairing the programmer’s ability to access and control the processor fully, and

2.	To yield a compact assembler which can be resident at all times to facilitate interactive programming and debugging.

### 1.3.1	Notational Differences

#### Instruction Mnemonics

Most mnemonics specifying instructions are the same as the manufacturer’s.  Occasionally there are differences where the manufacturer uses a prefix or suffix on the mnemonic to describe something we specify as a parameter (e.g., polyFORTH uses MOV B whereas DEC uses MOVB as a separate op-code) or to differentiate instructions which are really different (e.g., Intel uses MOV for both memory/register operations and segment register operations, whereas polyFORTH has different instruction names when segment registers are involved because the internal instruction format is quite different).

#### Addressing Modes

In all computing, there are only a few specific addressing modes (register direct, register relative, memory indirect, etc.).  Notation specifying these has been standardized across all polyFORTHs, to make it easier for programmers working with several different CPUs.  Naturally, this means the notation differs from the manufacturer’s notation; however all modes supported by the processor are implemented in the polyFORTH assembler.

#### Instruction Format

Most assemblers encourage a “four-column” format, with one instruction per line, allowing space for labels, op-codes, addressing operands, and remarks.  In polyFORTH, the op-code itself is a Forth command which assembles the instruction according to operands passed on the stack giving the addressing information.  This leads to a format in which the addressing mode specifiers precede the op-code.

#### Labels, Branches, and Structures

polyFORTH assemblers support structured programming in the same way that high-level Forth does. Arbitrary branching to labeled locations is discouraged; on the other hand, structure such as BEGIN ... UNTIL and IF ... ELSE ... THEN are available in the assembler (implemented as macros that assemble appropriate conditional and unconditional branches).

### 1.3.2	Procedural Differences

#### Resident Assembler

The polyFORTH assembler is resident at all times.  This means that a programmer can assemble code at any time, either from a source block or by typing it in directly from the terminal.  Regardless of where the code comes from, the assembled version will be the same.

#### Immediately Executable Code

In conventional programming, assemblers leave the code in a file, which must be integrated with code in files from high-level language compilers (if any) by a linker before the resultant program can be loaded into memory for testing.  The resident polyFORTH assembler assembles the code directly into memory in executable form, thus avoiding this whole cumbersome procedure.

#### Relationship to Other Routines

The polyFORTH assembler is used to write short, named routines that function just like routines written in high-level Forth; that is, when the name of the routine is invoked, it will be executed.  Like other Forth routines, code routines normally expect their arguments on the stack and leave their results there.  Within a code definition, one may refer to defined constants (to get a value), variables (to get an address) or other defined data types.  Code routines may be called from high-level definitions just as other Forth words are, but cannot themselves call high-level definitions.

#### Register Usage

polyFORTH (like other Forths) runs on a “virtual computer.”  For optimum performance, some of its “registers” are kept in hardware registers, which are permanently assigned.  The CPU Supplement for each polyFORTH system documents the register assignments for that CPU. On each CPU some registers are always designated as “scratch” (meaning they can always be used within a code routine without saving or restoring); those containing polyFORTH pointers must be saved and restored, if needed.  polyFORTH system registers are given names which make references to them in code easy and readable.  Since most polyFORTH code routines can do what they need using the designated scratch registers, there is less need to save and restore registers than in conventional programming.

REFERENCES

Principles of polyFORTH Assemblers, Section 6.4
The CPU Supplement for Your polyFORTH System

## 1.4	System Configuration and Electives

When you first boot a polyFORTH system, it clears the screen and displays a message identifying the product version and date, then greets the user by saying, “hi.”  At this time, the nucleus has been loaded.  This pre-compiled nucleus contains:

1.	Most Forth primitives (stack manipulation, single-precision arithmetic, input and output number conversion, most string operations, etc.).

2.	Disk and terminal drivers.

3.	The multitasker, although at this time only one task (OPERATOR) is defined.

4.	The address and text interpreters.

5.	The Forth compiler and assembler.

The capabilities included in the nucleus are the minimum set required for meaningful programming.  Nonetheless, you can do quite a great deal.

Your response to polyFORTH’s greeting is to type HI.  This command loads Block 9, a “load block” which, in turn, loads some additional capabilities which we refer to as system electives.  These are routines which you are more likely to modify in the course of your work, or to selectively include or omit.  They also include the configuration of the system, specifying such things as the additional tasks defined, type of system printer (serial or parallel), and even the number of disk buffers.

We strongly recommend that you read through Block 9 on your system, and make note of the various capabilities being loaded and specified.

### 1.4.1	Task Definition

Specific details of task definition and control are discussed in Section 4.0 of this manual and the CPU Supplement for your system.  This section covers the organization of your system with respect to allocation of task user areas.

All tasks should be defined as part of the 9 LOAD process, so that their definitions will reside in the shared dictionary of the system.  This is important because once a task has been linked into the round-robin loop it is difficult to remove without breaking the chain, which would halt the system.  Although the tasks must be defined and initialized at this time, the definitions that they will be asked to execute need not be defined until later.

There are three types of tasks:  printer tasks, serial tasks supporting terminals and other serial devices, and background tasks performing application-related functions. Printers are controlled by TERMINAL tasks, as are the serial lines.  Printer tasks differ, however, in that they usually have only a limited provision for keyboard input.

TERMINAL tasks further differ from one another in that CRTs and printers have special control codes that control such functions as “clear screen” (“form feed” on printers), carriage return or new-line, tabbing, highlighting, etc.  polyFORTH provides for such differences by vectoring the commands that perform these functions.  Specific routines are provided to specify these functions for many of the most popular printers and terminals; these are called “personality blocks,” because they reflect the individual characteristics of the devices in question.

The terminal “personalities” supplied with each polyFORTH system are named in Block 10, and the “personalities” for printers are listed in Block 10’s shadow block.  Provision was made in the first 60 blocks of the system for two personality blocks, for a primary terminal and printer. If the pair configured in the system as shipped is inappropriate, you should replace them with an appropriate pair from the set provided.  If your terminal and/or printer aren’t included, you can develop your own by selecting a given personality block for a terminal or printer that is similar to yours and modifying it according to the documentation for your device.

The personality block for a printer should be loaded immediately before the block that defines the printer task.  The terminal personality blocks are designed to be loaded by the user at the terminal in question.  That is, after the terminal task has been PROMPTed, the user at that terminal would type (for example):

ANSI

to load the ANSI-standard set of terminal control functions.  Alternatively, if one or more of the additional terminals configured by Block 9 is always of the same type, the personality can be pre-specified by loading its block just prior to the block that defines the terminals.  If you are doing this, you need to modify the block by removing the EMPTY at the top of the block, and placing the command EXIT before the phrases at the end of the block that stores the addresses of the functions in the vectored words 'TAB, etc.

0 ( Electives)   DECIMAL

1 EDITOR

2 ( Aids)  18 19 THRU   ( Buffers)  44 LOAD   60 HELPS

3 ( 32-BIT)  21 25 THRU   ( Traps)  29 LOAD

4 ( Date/time)  36 37 THRU  40 4* THRU  55 56 THRU  DATE TIME

5 ( Extensions)  27 28 THRU

6 ( Disk Support)  43 46 THRU  57 5* THRU

7 ( Utilities)  10 LOAD

8 ( Editor Support)  51 53 THRU   ( F-83)  48 LOAD

9 ( Fast Display)  54 LOAD

10 : SYSTEM   [ 60 ] +U DATED  ."  Time " TIME ;

11 ( Tasks)  30 31 THRU   ( Serial)  282 LOAD

12 ( Printer) 35 LOAD 34 LOAD  ( VDTs) 168 LOAD 291 LOAD

13 : HELP   SYSTEM ;

14 GILD  ' ?CREATE 'CREATE !  JOE PROMPT

15 EXIT  polyFORTH II ISD-4 pF86/IBM-B  Disk 1  10 July 1986

Fig. 1.13

Typical system electives load block, showing order of defining tasks.

Fig. 1.13 shows a typical load block of system electives.  To summarize the  load sequence for terminals and printers, Block 9 should be:

1.	Task definition words (normally Blocks 30-31).

2.	Hardware drivers for serial port, as needed.

3.	Printer personality block.

4.	Printer task definition.

5.	Terminal personality block (optional).

6.	Terminal task definition(s).

7.	The command GILD, which defines the “top” of the shared dictionary.

8.	PROMPT commands for any terminal tasks that have been defined.

The PROMPT commands, if any, must come after the GILD command in order to attach the terminal tasks’ dictionaries to the full shared dictionary of the system.

REFERENCES

Background Tasks, Sections 4.3, 4.4, 4.5
Defining Terminal Tasks, Section 4.8 and the CPU Supplement
GILD, Section 3.4.4
Multitasking, Section 4.0
Printer Tasks, Section 4.11

### 1.4.2	System Feature Selection

There are many options that are selected during the loading of the system electives.  These include the creation of tasks (discussed in Section 1.4.1), the selective loading of software facilities as needed by your applications, and the installation of features that are normally resident but not always active.  The latter two categories are discussed in this section.

Block 9 of your system as shipped loads all the features documented in this manual, except for utilities which are normally handled as overlays.  Many routines are included for each system that aren’t part of standard polyFORTH, and are appropriate for some applications but not others; these are not loaded in the product as shipped, but you may wish to include them.  You may wish to omit routines that you don’t expect to use, in order to allow more dictionary space for your application.  You may very likely want to add routines developed to support your application (ranging from custom hardware drivers to special math functions) in order to have them always available in the shared dictionary.

The following items (including the section in this manual where a fuller discussion of the feature may be found) are common configuration issues:

Feature	Section	Discussion

LOCATE	1.5.1	This feature compiles the source block from which each definition of any type was compiled, so that you may interactively display the block.  The cost is two bytes per definition.  If you are short of dictionary space you may wish to do without this convenience.  It is installed in the EDITOR load block (12) by the phrase:

' <CREATE> 'CREATE !

Omit this phrase (or disable it by putting it in parentheses) if you don’t want to use this feature.

32-bit Math	2.2	Many applications don’t need all of these operators (especially control applications and those using the 8087 for computation).

Calendars	3.5	There are two calendars:  the month-day form, used when most dates are current, and the m/d/y calendar which is useful when dates may be in any year (e.g., birthdates).  Select the calendar of your choice by loading Block 39 or 40, plus Block 41.  The internal forms are the same for both.

?CREATE	2.6.1	This version of CREATE checks for name conflicts, and issues a warning message if a word being defined may conflict with a previous name.  Such conflicts may be resolved by using ~ (“tilde”).  It is enabled by the phrase,

' ?CREATE 'CREATE !

in Block 9.  If you don’t wish to get these messages, omit the phrase.  If you think you have an inadvertent name conflict in your system electives, you may wish to move this phrase earlier in the block.  It may be used any time after Block 18 is loaded.  If you are loading application features, you may choose to load them before or after this phrase, depending on where you wish the check to be performed.

As an application progresses, you will probably include more and more application functions in the electives as they are fully tested, leaving routines that are still under development to be loaded in a terminal partition as overlays.  Your application should have a main load block, which loads features included with polyFORTH that are special to your application (such as graphics), plus your application drivers and other routines.  This block may be loaded at the end of Block 9 (before the GILD command) in order to include these routines in the shared dictionary.

## 1.5	Documentation and Source Management
	Facilities

In polyFORTH, as in all other languages, the primary responsibility for producing readable code lies with the programmer.  polyFORTH does, however, support the programmer’s efforts to produce easily managed code by providing a number of aids to internal documentation.  In addition to these, we also recommend each Forth programming group adopt editorial and naming standards and conventions which all share.  Although “readability” is fundamentally an aesthetic and rather personal value, adopting a set of standards that all members of a group adhere to will aid significantly in the ability of the group to share code and support one another.

### 1.5.1	Internal Documentation

polyFORTH comes with a comprehensive listing utility, capable of printing indexes, program text, double-sided program text, and program text with associated shadow blocks.  These listing programs are most useful when you follow certain simple conventions in writing your application program (see the section on disk and block layout referenced below).

For each block of program source, there is a corresponding block, called its “shadow,” which contains comments and descriptions of the words defined in the source.  Shadow blocks are a documentation facility intended to help you document applications and use polyFORTH. Shadow blocks reside on disk at a fixed offset from the source code blocks which are being documented.  The offset is a constant called SHADOWS.  SHADOWS is used by the word Q (for “Query”), which alternately displays the block of source code and the shadow block containing comments for the source code, adjusting the EDITOR’s block number SCR accordingly.

Shadow blocks are edited the same way as source code, and form a powerful commenting facility.  polyFORTH source code is often compact, so corresponding shadow block space is at a premium.  Shadow blocks are therefore intended to document intimate code details which are necessary to use or to rewrite important words.  If space is a problem, documentation of usage should take precedence over theory of operation.

Comments embedded in polyFORTH source are enclosed in parentheses.  For example:

( This is a comment)

The word ( must have a space after it, so that it can be recognized and executed.  polyFORTH comments are most often used to give a picture of a word’s stack arguments and results; for example, a high-level definition of the polyFORTH word = is:

: = ( n n - t)   - NOT ;

The dash in the comment separates a word’s arguments from its results.  Certain letters have specific meanings by convention.  The most common are:

Word	Description

n	A single-precision signed integer.

u	A single-precision unsigned integer.

t	A single-precision Boolean value (zero is false and non-zero is true).

a	A single-precision address.

d	A double-precision signed integer.  Thus, in the example above, the word = expects two single-precision integers and returns a truth flag.

Words which have separate interpretive and run-time behaviors should have comments for both sections:

: CONSTANT ( n)   CREATE ,  DOES>
   ( a - n)   @ ;

Electives and resident applications are generally loaded immediately after the system boot, by the command HI.  If the “programmer aid” block is loaded and the appropriate version of CREATE (called <CREATE>) is installed, the source text of any word already compiled from disk may be displayed by typing:

LOCATE word

If the word is not in the dictionary, the response will be:

LOCATE word     word ?

If a word was defined from the keyboard or if the word exists, but was defined before the electives were loaded, the response will be:

LOCATE word     can't

If the word exists and was defined during or after the electives load, the block containing the definition of the word will be displayed.  SCR is set by LOCATE, so that the block’s shadow block can be seen by typing Q followed by a carriage return.

REFERENCES

Behavior of =, Section 2.2.2
Disk and Block Layout and Design, Section 5.2.5
Listing Utility, Section 5.2
Stack Notation Conventions, Section 2.1
Vectored CREATE (<CREATE>), Section 3.1

### 1.5.2	Source Management

The polyFORTH system source is usually followed on disk by additional blocks for applications.  On most systems, particularly those with hard disks available, there is a region of 600 blocks or more for source, followed by a region of the same size for the shadow blocks documenting this source.  Systems with dual floppy disks usually keep the source on the disk in Drive 0 and the shadow blocks on the disk in Drive 1.  Hard disks are usually organized into “volumes” of a convenient size so that you may easily keep a backup copy of your source and shadow blocks on the hard disk, as well as archival copies on floppy disks or other removable media.

polyFORTH includes several utilities to assist in managing the blocks of source for an application.

The DISKING utility provides simple commands for copying ranges of blocks from one disk to another or one place to another on a hard disk.  polyFORTH organizes hard disks into logical volumes of a convenient size for most applications.  Commands are included to copy entire volumes as well as shorter ranges of blocks.  A command is also provided to copy a range of source blocks along with its associated range of shadow blocks.

The AUDIT utility includes facilities for comparing ranges of blocks, displaying non-matching blocks on the screen or printer with the differences highlighted.  This facility is especially useful when several programmers are working on the same application on different computers.

Several useful source management aids are also resident in the system.  The command:

n QX

(for Quick IndeX) displays the first part of Line 0 of a 60-block region of disk starting with block n.  The related commands NX and BX display the next and previous 60-block regions, respectively.  This is useful when you are looking for certain blocks, or for an appropriate block to put a new feature.

The editor contains the command S, which Searches forward over a range of blocks for a specified text string.  This is useful for finding words that cannot be LOCATEd, or finding instances of a word or phrase.

REFERENCES

The AUDIT Utility, Section 5.5
The DISKING Utility, Section 5.3
S Command, Section 5.1.7

# 2.0	BASIC FORTH VOCABULARY

This section defines the major elements of the polyFORTH implementation of the Forth language.  These words are grouped into categories and documented in the following sections.

## 2.1	Stack Operations

Stack operators work on data that are present on the parameter and return stacks.  The words defined in this section use the stack as the major source and destination for their operands.  Many other Forth words also result in modification of the stack.  Other commands are described in the sections of this manual that deal with their primary functions.  Besides the stack operators discussed in this manual, stack manipulation words that relate to assembly language are covered in Section 6.0 and your CPU Supplement.

In this section, several notational conventions have been adopted for clarity.  The item on the top of the stack is referred to as “S”; the item immediately below the top of the stack is referred to as “S+1,” etc.  A double-length item on the top of the stack is referred to as “S+0,1”; a double-length item below it would be “S+2,3,” etc.  This notation reflects usage on all polyFORTH systems in which stacks grow toward low memory.

Operations that use the stack usually require that a certain number of items be present on the stack and then leave another number of items on the stack as results.  Most operations remove their operands, leaving only the results.  To help see the effect of the operation on the number and type of items on the stack, each word in this section has a notation under the stack column.  This same stack notation is used in your system listing to document the stack arguments of system words.

The specific notation of the stack items follows these conventions:

Word	Description

a	A cell-wide byte address.

b	A byte, stored as the least significant 8 bits of a stack entry.  The remaining bits of the stack entry are zero in results or ignored in arguments.

c	An ASCII character stored as a byte (see above) with the parity bit reset to zero.

n	A signed single-precision 2’s complement number.  On 16-bit machines the range is from -32768 through +32767.  On 32-bit machines, the range is from -2,147,483,648 though +2,147, 483,647.  (Note that Forth arithmetic rarely checks for integer overflow.)

u	A single-precision unsigned number, with a range from 0 to 65536 on 16-bit machines, or from 0 through 4,294,967,295 on 32-bit machines.

d	A double precision, signed, 2’s complement integer, with a range from -2,147,483,648 to 2,147,483,647; stored as two stack entries (in most systems the least significant cell is the cell below the most significant cell).  On 32-bit machines, the double-precision range is from -9,223,372,036,854, 775,808 through 9,223,372,036,854,775,807.

ud	A double-precision, unsigned integer, with a range from 0 to 4,294,967,296.  On 32-bit machines, the range is from 0 through 18,446,744,073,709, 551,615.

t	A single-precision Boolean truth flag (zero means false, non-zero means true).

If you type several numbers on a line, the right-most will end up on top of the stack. As a result, we show multiple stack arguments with the top element to the right.

These are examples of special cases which will be explained when used:

Word	Description

l c	Screen position, in lines and columns, respectively.

s d n	Source, destination and count.

y x	A 2-vector for graphics, etc.

f l	First and last limits, inclusive.

Word	Description

l h	Low and high limits (exclusive high limit).

r	Register (for assembler words).

Where there are several arguments of the same type, and clarity demands that they be distinguished, a ' (pronounced “prime”) will be used.  In extraordinary instances, numeric suffixes may be used.

Please remember that these locations are relative to the top of the stack and do not affect any stack items below the lowest stack item referenced by the operation.  For example, the notation ( n n - n) means that the operation uses the top two stack items and leaves a one-item result.  Therefore, if the stack initially contained three items, execution would result in a stack of two items, with the bottom item unchanged and the top item derived as a result of the operation.

### 2.1.1	Parameter Stack Manipulation Operations

This category of stack operations contains words which manipulate the contents of the parameter stack without performing arithmetic, logical, or memory reference operations.

Word	Stack	Function

OVER	( n n' - n n' n)	Duplicates S+1 on the top of the stack (S).

SWAP	( n n' - n' n)	Swaps the top two items on the stack.  S is placed in S+1 and S+1 is placed in S.

DUP	( n - n n)	Duplicates the top entry on the stack.

DROP	( n - )	Removes the top entry from the stack.

'S	( - a)	Fetches the address of the top of the stack, placing this address on the top.  Normal usage is to add to this value an offset such that the resulting address is that many items down the stack.  For example, to push the address of the third item below onto the top of the stack, the following code would be used:

'S 6 +

Word	Stack	Function

'S (cont.)		('S 12 + on 32-bit processors.)  Note that although the actual definition is machine code, OVER is equivalent to:

'S 2+ @

('S 4+ @ on 32-bit systems.)  Pronounced “tick-s.”

2DROP	( d - )	Removes the top double-length item (2 cells) from the stack.

2SWAP	( d d' - d' d)	Swaps the top two double-length items.

2DUP	( d - d d)	Duplicates the top double-length item.

2OVER	( d d' - d d' d)	Pushes the double-length item (S+2,3) onto the top of the stack (S+0,1).

ROT	( n1 n2 n3 - n2 n3 n1)	Rotates the top three items on the stack.

?DUP	( n - n , n n)	Conditionally duplicates the top item on the stack if its value is non-zero. Equivalent to the following:

DUP IF DUP THEN

DEPTH	( - n)	Tells how many items are on the stack. Note that DEPTH will return 2 for each double-precision integer on the stack.

The following words are included for compatibility with the FORTH-83 standard.  Good Forth programming rarely accesses a parameter stack to a depth of more than 4 cells.  Therefore, the following words are available in the FORTH-83 compatibility block, rather than being coded in the nucleus.

Word	Stack	Function

PICK	( n - n')	Copies the nth stack entry onto the stack.  S is entry number zero, S+2 is entry number two.  The phrase 1 PICK is equivalent to the word OVER.

Word	Stack	Function

ROLL	( n)	Moves the nth stack entry to the top of the stack, moving down all the stack entries in between. ROLL numbers the stack in the same way as PICK.

REFERENCES

FORTH-83 Standard Compatibility, Section 2.9

### 2.1.2	Memory Stack Operations

This category of operations allows you to reference memory by using addresses that are on the stack.

Word	Stack	Function

C@	( a - b)	Replaces S with the byte addressed by S.  The byte fetched is stored in the low-order byte of S with the remaining bits cleared to zero.  Pronounced “c-fetch.”

C!	( b a - )	Stores the low-order byte of S+1 at the address in S, removing both from the stack.  Pronounced “c-store.”

C+!	( b a - )	Adds the low-order byte of S+1 to the byte addressed by S, removing both from the stack.  Pronounced “c-plus store.”

@	( a - n)	Replaces S with the item addressed by S.  Pronounced “fetch.”

!	( n a - )	Stores S+1 at the item addressed by S, removing both from the stack.  Pronounced “store.”

+!	( n a - )	Adds S+1 to the contents of the item addressed by S and stores the result in the item addressed by S, removing both from the stack. Pronounced “plus-store.”

2@	( a - d)	Pushes the double-length item that is addressed by S onto the top of the stack (S+0,1). Pronounced “two-fetch.”

Word	Stack	Function

2!	( d a - )	Stores S+1,2 into the double-length item that is addressed by S, removing three cells from the stack.  Pronounced “two-store.”

The following words are in 32-bit systems only:

Word	Stack	Function

H@	( a - n)	Replaces S with the 16-bit (half-word) content of S, sign-extended.  Pronounced “h-fetch.”

H!	( n a - )	Stores the low-order half-word of S+1 into the 16-bit location addressed by S.  Pronounced “h-store.”

U@	( a - u)	Unsigned version of H@.  Pronounced “u-fetch.”

H+!	( n a - )	Adds the low-order signed half-word of S+1 to the 16-bit contents of S.  Pronounced “h-plus-store.”

### 2.1.3	Return Stack Manipulation Operations

The “return” stack is so named because it is used by the address interpreter to keep track of where Forth words will return when they have finished execution.  When a high-level Forth word invokes a lower-level Forth word, the address of the next cell of the high-level word is pushed onto the return stack by the address interpreter.

The return stack is a convenient place to keep frequently used values (because of the words I, I' and J), but it should be cleared before the word reaches an EXIT (EXIT is compiled by ;) or else the address interpreter will return to the “address” on the return stack.  This behavior is occasionally useful.  For example:

: VECTOR   >R ;

will act like the word EXECUTE, but will only execute : definitions.  VECTOR works by pushing a word’s parameter field address onto the return stack.  Therefore, when EXIT pops the return stack into the address interpretive pointer I the address interpreter will begin to execute the word whose address was on the stack for VECTOR.

This section documents those operations which involve both the return stack and the parameter stack.

Word	Stack	Function

>R	( n - )	Removes the item on the top of the parameter stack and puts it onto the return stack.  Pronounced “to-r.”

R>	( - n)	Removes the item on the top of the return stack and puts it onto the parameter stack.  Pronounced “r-from.”

2>R	( n n - )	Pops the top two cells from the parameter stack and pushes them onto the return stack.  This word is the run-time code for DO.  Pronounced “two-to-r.”

2R>	( - n n)	Pops the top two cells from the return stack and pushes them onto the parameter stack. 2R> is the inverse of 2>R.  Pronounced “two-r-from.”

I	( - n)	Duplicates the item on top of the return stack on the parameter stack.  Normally used to obtain a copy of the index of a DO ... LOOP structure.

R@	( - n)	Identical to I; defined for compatibility with FORTH-83 standard. For maximum compatibility, I should be used only within a DO structure to obtain its index, and R@ for general return stack access.  Pronounced “r-fetch.”

I'	( - n)	Pushes onto the parameter stack a copy of the second return stack item.  Used to access either the limit inside a loop or I from a definition invoked inside a loop.  Pronounced “i-prime.”

J	( - n)	Pushes onto the parameter stack a copy of the third return stack item.  Used to access the index of the next outer loop from an inner loop.

REFERENCES

Counting LOOPs (DO), Section 2.4.4
EXECUTE, Section 2.4.8.1

### 2.1.4	Conveniences

Word	Stack	Function

DUMP	( a n - )	Types out a region of cells in memory, given the starting address and length:

addr count DUMP

Output is formatted with the address on the left and up to eight values on a line; the output conversion radix is the current value of BASE.  Two cells are removed from the stack.

.S		Displays the contents of the parameter stack using the current base. Stack contents remain unchanged.

?	( a - )	Fetches from the address in S, and displays the result according to the current conversion radix. Equivalent to the phrase:

@ .

'	( - a)	Searches the dictionary for the word that follows tick in the input stream; leaves the address of the word’s parameter field on the stack.  Aborts if the word cannot be found.  Pronounced “tick.”

## 2.2	Arithmetic and Logical Operations

polyFORTH offers a comprehensive set of commands for performing arithmetic and logical functions.  polyFORTH’s arithmetic is optimized for integer arithmetic, since most processors lack hardware floating-point capability and software floating-point is too slow for most real-time applications.  polyFORTH supplies words to perform fast, precise scaled-integer and fixed-point fraction computations.

Programmers who are new to Forth should review Starting FORTH, Chapters 2, 3, 5, and 7, which provide a comprehensive discussion of Forth’s handling of numbers and arithmetic.

### 2.2.1	Arithmetic and Logical Operators

The basic principles of Forth arithmetic are covered in Starting FORTH, Chapter 2.  More advanced concepts are discussed in Chapter 5.

In order to achieve maximum performance, each version of polyFORTH implements most arithmetic primitives to use the internal behavior of hardware multiply/divide instructions on that particular processor.  Therefore, to find out at the bit level what these primitives do, you should consult the manufacturer’s hardware description.

The following general guidelines may help you use these operators:

1.	The order of arguments to order-dependent operators (e.g., - and /) is such that if the operator were moved to an infix position it would algebraically describe the result.  Some examples:

Forth	Algebraic

a b -	a - b
a b /	a / b
a b c */	a * b / c

2.	All arithmetic words containing MOD (MOD, /MOD, */MOD, etc.) are unsigned; others are normally signed.  The exception to this rule is that M*/ on most systems require a positive divisor.

These operators perform arithmetic and logical functions on numbers that are on the stack.  In general, the operands are removed (popped) from the stack and the results are left on the stack.

Single-Precision Operations

Word	Stack	Function

+	( n n - n)	Adds S (popped) to S+1; leaves the result as S.

-	( n n' - n)	Subtracts S (popped) from S+1; leaves the result as S.

1+	( n - n)	Adds one to the value in S.

1-	( n - n)	Decrements the value in S by one.

2+	( n - n)	Adds two to the value in S.*

Word	Stack	Function

2-	( n - n)	Subtracts two from the value in S.*

2*	( n - n)	Multiplies S by two (arithmetic left shift).*

2/	( n - n)	Divides S by two (arithmetic right shift).

*	( n n - n)	Multiplies S+1 by S (popped); leaves the result as S.

/	( n n - n)	Divides S+1 by S (popped); leaves the quotient as S.

MOD	( n n - n)	Divides S+1 by S (popped); leaves the remainder as S. Operands are unsigned.

/MOD	( n n - r q)	Divides S+1 by S; leaves the quotient as S and the remainder as S+1.  Operands are unsigned.

*/	( n n n - n)	Multiplies S+2 by S+1; divides the result by S; leaves the quotient as S (uses a double-precision intermediate result).

*/MOD	( n n n - r q)	Multiplies S+2 by S+1; divides the result by S; leaves the quotient as S and the remainder as S+1 (gives a double-precision intermediate result).  Operands are unsigned.

Double-Precision Operations

Word	Stack	Function

D+	( d d - d)	Adds the top two double-precision stack values.

D-	( d d - d)	Subtracts the top double-precision stack value (S+0,1) from the next double-precision value (S+2,3); leaves a double-precision result as S+0,1.

Mixed-Precision Operations

These arguments have been determined by experience to be the most generally useful.

Word	Stack	Function

M+	( d n - d)	Adds a double-precision value in S+1,2 to the single-precision value in S; leaves a double-precision result.

M-	( d n - d)	Subtracts a single-precision value in S from a double-precision value in S+1,2.

M*	( n n - d)	Multiplies two single-precision values (S and S+1) to form a double-precision value (as S+0,1).  An alternate “quick and dirty” definition of M* may use M*/ when stack arguments of the form ( d n - d) are needed.

M/	( d n - n)	Divides a double-precision value in S+1,2 by the single-precision value in S; leaves a single-precision result as S; does not perform an overflow check.

U/MOD	( ud u - r q)	Divides S+1,2 by S, leaving a remainder as S+1 and a quotient as S.  This operation is called U/MOD because it assumes that the arguments are unsigned, and it produces unsigned results.

M*/	( d n' n - d)	Multiplies S+2,3 (d) by S+1 (n'), giving a triple-precision result, and then divides the triple-precision result by S (n), leaving a double-precision result.  S may be unsigned; see the stack arguments in your system listing.

T*	( d n - t)	Multiplies S+1,2 by S, yielding a triple-precision result (48 bits on 16-bit machines and 96 bits on 32-bit machines) as S+0,1,2. Used in M*/.

T/	( t n - d)	Divides a triple-precision number in S+1,2,3 by S, yielding a double-precision result.  S may be unsigned.  See the stack arguments in your system listing.  Used in M*/.

### 2.2.2	Logical and Relational Operations

As in the case of arithmetic operations, polyFORTH’s implementation of logical and relational operations optimizes speed and simplicity.  This does imply some limitation on generality in 16-bit systems, although this limitation rarely is an issue in real-time applications.

In order to fully understand the issues, we can represent the entire set of 16-bit integers in three ways, as shown in Fig. 2.1.

A relational which treats a given 16-bit integer as a point on the full signed number line (a) is needed for true arithmetic or algebraic numbers in which the application has carefully determined that there will never be overflow or underflow.

Fig. 2.1

Three ways of representing 16-bit binary numbers

For example, this type of relational is needed to test whether -20,000 is less than +20,000.

A relational which treats all values as unsigned (b) is also needed, primarily to test locations of given addresses.  The number line, or rather “number circle” shown in (c) probably needs more explanation than the other two.  Relational operators which treat numbers in this way have the advantage of being able to act like signed tests around zero and like unsigned tests around 32K.  Thus the relation between two numbers is totally independent of their absolute position on the number circle.

It may seem that the third case is the rarest and least useful.  In fact, though, it turns out that it handles the vast majority of signed comparisons in real applications, and better still, it is much faster in execution and easier to implement than a relational that assumes number line (a).

Consider three points on the number circle, as shown in Fig. 2.2.  It doesn’t matter where these points lie in relation to true zero on the circle.  Since numbers increase in a clockwise direction, point A is considered to be greater than point X.  Point B is considered to be less than point X.  Notice that the maximum range of comparisons in the circle is 32K.  Further away than that, a value will appear to lie in the opposite semicircle and will produce the opposite result.

Fig. 2.2

A circular representation of the range of 16-bit numbers.

Thus a relational which uses this number circle is limited to a 32K range for any single test.  Since most comparisons use an extremely small fraction of the total 65K range of the circle, this limitation is generally safe.

The number circle relational is easy to implement because it can be defined in terms of subtract, e.g.,

: <   - 0< ;

When A is subtracted from X, the arc of the difference is greater than 180 degrees.  That is to say, the result of subtraction will appear to be negative in sign.  When B is subtracted from X, the arc of the difference is less than 180 degrees; this difference will appear positive.  In the first case, the 0< test produces “true,” in the second case “false.”

16 bit versions of polyFORTH ISD-4 use the fully signed model (option “a” in Fig. 2.1) to implement most relationals, as well as MAX and MIN.  32-bit versions of polyFORTH use the circular model.  The operator U< is provided for unsigned comparisons, particularly for memory addresses that can extend over the full 0-65K range.

Single-Precision Logical Operations

Word	Stack	Function

NEGATE	( n - n)	Negates the top value (S is set to -S).  The phrase NEGATE 1- is equivalent to logical negation (1’s complement).

MAX	( n n - n)	Compares S+1 and S (popped); leaves the larger as S.

MIN	( n n - n)	Compares S+1 and S (popped); leaves the smaller as S.

ABS	( n - n)	Replaces the top stack value with its absolute value (S is set to ABS S).

AND	( n n - n)	Places the logical AND of S+1 and S as S.

OR	( n n - n)	Places the logical OR of S+1 and S as S.

XOR	( n n - n)	Places the exclusive OR of S+1  and S as S.  The phrase -1 XOR -1 is equivalent to logical negation.

Double-Precision Logical Operations

Word	Stack	Function

DNEGATE	( d - d)	Changes the sign of a double-precision stack value.

DABS	( d - d)	Takes the absolute value of a double-precision stack value.

DMIN	( d d - d)	Returns the smaller of two double-precision stack values.

DMAX	( d d - d)	Returns the larger of two double-precision stack values.

Comparison and Testing Operations

These operations leave on the stack a number that is based upon a test of the contents of one or more items on the top of the stack.  This number may be interpreted as a true/false value; zero equals false and any non-zero value equals true.  The words below, which perform explicit tests, return -1 for ‘true.’*  Comparison and testing operations generally precede an IF, WHILE, or UNTIL.

In general, the test always replaces the item(s) tested with the results of the test.

Word	Stack	Function

0=	( n - t)	Tests S for a value of zero.  Pronounced “zero-equals.”

0<	( n - t)	Tests S for a value less than zero.  Pronounced “zero-less-than.”

NOT	( t - t)	Equivalent to 0=; tests S for a value of zero.  Used for program clarity to reverse the results of a previous test. For example, the following code would test for a value greater than or equal to zero:

0< NOT

=	( n n - t)	Tests S and S+1 for equality.

<	( n n - )	Tests for S+1 less than S.  Pronounced “less-than.”

>	( n n - t)	Tests for S+1 greater than S.  Pronounced “greater-than.”

D0=	( d - t)	Tests the double-precision item in S+0,1 for zero.  Pronounced “d-zero-equals.”

D<	( d d' - t)	Returns a one if S+0,1 is less than S+4,5, a zero otherwise.  Pronounced “d-less-than.”

The following are written in high-level and are available when the FORTH-83 compatibility block is loaded:

Word	Stack	Function

0>	( n - t)	Returns a one if S is greater than zero.  Pronounced “zero-greater-than.”

D=	( d d - t)	Returns a one if the double-precision number in S+0,1 is equal to the double-precision number in S+4,5.  Pronounced “d-equals.”

You may also use - (minus) or D- as a “not-equal” test, since it will return a non-zero difference if the two single or double-precision numbers are unequal.

REFERENCES

Conditionals , Section 2.3.5
MAX and MIN, Section 2.1.2
Post-testing Loops, Section 2.4.2
Pre-testing Loops, Section 2.4.3
String Comparisons, Section 2.3.5
FORTH-83 Standard Compatibility, Section 2.9

## 2.3	Character and String Operations

polyFORTH contains many words that are used to reference single characters (bytes) or character strings.  Characters may be grouped together and thought of as a string; this group is then operated on as a single variable.  Character strings are supported by the words that are documented in this section.

A standard working area is used to hold most character strings for processing; this area is referred to as PAD.

In addition to the words described in this section, several other words are used to reference character data in different environments, e.g., Data Base Support.  Such words are described in the appropriate sections of this manual.

REFERENCES

Data Base Support, Section 8.0

### 2.3.1	The PAD—Scratch Storage for Strings

PAD is an area of storage of indefinite size that is used to hold strings for intermediate processing.  Each terminal task contains a PAD area.  The word PAD places the address of the first byte in PAD on the top of the stack.

PAD is physically located at a fixed displacement from the current top of the dictionary (H).  The actual displacement is a system function, but will be at least 34 bytes. The largest displacement is the maximum length used for pictured output number conversion. The number conversion area grows downwards from PAD (see Fig. 2.3).

Fig. 2.3

Portion of user’s dictionary showing PAD.

Because PAD is located relative to the dictionary pointer H, the location of PAD changes whenever H changes.  Common operations that affect H include adding definitions; adding data or data areas using ,, C,, or ALLOT; or discarding definitions using FORGET or EMPTY.  This means that information left in PAD before one of these operations will not be addressable following the definition (and may in fact be overwritten by a new definition).

Although PAD does not have a fixed maximum size, a minimum size of 250 bytes is maintained; an error message (“Dictionary Full”) is issued if the dictionary threatens to grow too close to the top of the user area.  The exact current size may be computed by the phrase:

'S PAD -

REFERENCES

, and C,, Sections 2.7.6.1, 2.8.2
ALLOT, Section 2.8.1
EMPTY, Section 3.3.4.1
FORGET, Section 3.3.4.2
Pictured Number Conversion, Section 2.5.2

### 2.3.2	Single-Character Reference Words

The words C@ and C! are used to reference single characters in the same way that @ and ! are used to reference cells.

C@ expects an address on the top of the stack.  This address is replaced with the contents of the addressed byte.  This byte will be placed in Bits 0-7 of the cell on the top of the stack, with the higher order bits set to zero (C@ does not “sign extend”).

C! expects an address on the top of the stack and a character in Bits 0-7 of the cell underneath the byte pointer.  The high-order bits of this lower cell are ignored.  The character is stored in the addressed byte; the address and character cells are removed from the stack.

For example, the following phrase would fetch the first character in PAD to the top of the stack:

PAD C@

REFERENCES

C@, C!, Section 2.1.2

### 2.3.3	String Defining Words

Short messages of a few characters in length may be added to the dictionary by using their ASCII codes with the defining word MSG (pronounced “message”).  A word defined by MSG will, when invoked, type out its characters on the terminal.  For this reason, a word defined by MSG may only be executed by a TERMINAL task.

The principal use of MSG is for sending control information to terminals and other ASCII devices.  Some examples are CR (which sends a carriage-return, line feed, and sometimes nulls for timing to a terminal); PAGE (which sends a “clear screen” or “form feed”); and DRAW (an example of an application command which sends graphics commands to a serial graphics terminal).

The basic form of a MSG is as follows:

HEX MSG NAME nn C, cc C, cc C, ...

The first byte of the string (represented by nn above) gives its length (number of bytes).*  The second byte represents the first letter of the string.  Each cc above represents the ASCII equivalent of one character of the string.

On processors that will not tolerate odd byte addresses (such as the PDP-11 or 68000) odd-length strings should be filled out with a zero at the end; the count may be even or odd, however, and the exact specified number of characters will be typed.

REFERENCES

Example of MSG, Sections 2.7.6.3, 3.7.3
TERMINAL Tasks, Sections 4.8, 4.9, 4.10
Terminal Output, Section 3.7.3
Use of , and C, to Compile Values, Section 2.8.2

### 2.3.4	String Management Operations

polyFORTH contains several words that are used to reference strings, compare strings, and move strings between different locations.  In addition to these words, other words are used to input or output character strings.

Most words that operate on a single character string expect the length of that string to be on the top of the stack, and its address beneath.  Many words that operate on two separate character strings expect three items on the top of the stack, in the format shown in Fig. 2.4.  The exceptions are -TEXT and the byte string operators in the Data Base Support system.

Word	Stack	Function

ERASE	( a n - )	Erases a region of memory (sets it to zeros), given its starting address and length:

addr count ERASE

Two cells are removed from the stack.

Word	Stack	Function

BLANK	( a n - )	Sets a region of memory to blanks (hex 20); S+1 and S are an address and length, as in ERASE.  Two cells are removed from the stack.

FILL	( a n b - )	Fills a region of memory with the least significant byte of S.  S+2 and S+1 are an address and length, as in ERASE.  Three cells are removed from the stack.

MOVE	( s d n - )	Moves the number of bytes in S from a source starting at S+2 to the destination starting at S+1.  MOVE is always the fastest way to move data, but on cell addressed machines, s and d may be restricted to even addresses.  Transfers bytes beginning at low memory and ending at higher memory.  Three cells are removed from the stack.

Fig. 2.4

Format of arguments for most two-string operators.  One length applies to both strings.  The above format is used instead of two separate character counts.

Word	Stack	Function

CMOVE	( s d n - )	Moves the number of bytes in S from a source starting at S+2 to the destination starting at S+1.  Arguments and behavior are as MOVE, but s and d may have odd values.  Three cells are removed from the stack.

Word	Stack	Function

<CMOVE	( s d n - )	<CMOVE has arguments and behavior as CMOVE, but starts transferring at high memory and works toward low memory.  <CMOVE is good for transferring from a data field to an overlapping data field in higher memory.  Three cells are removed from the stack. Pronounced “back c-move.”

Field names which are character strings contained in files may be used to provide operands for string reference operations.

REFERENCES

-TEXT, Section 2.3.5
Character String I/O, Section 2.3.6
Fields, Data Base Support, Section 8.0

### 2.3.5	Comparing Character Strings

Character-string comparisons operate on two separate character strings; this allows the two to be compared by use of the ASCII collating sequence.  The following words are provided:

Word	Stack	Function

-TEXT ( a n a' - n) Compares two strings of the same length. The string beginning at the address in S is subtracted from the string beginning at the address in S+2. S+1 is the number of characters to compare. Returns -1 if the “S+2” string is less than the “S” string, 0 if they are equal, and 1 if the S+2 string is greater than the S string. “Greater than” and “less than” are determined on a cell-by-cell basis. Odd-length strings may not be used with -TEXT because the strings are compared cell-by-cell.

Word	Stack	Function

-MATCH ( a1 n1 a2 n2 Searches for a match for the string a2 n2 in the - a1 n1 t) string a1 n1 (which is presumed to be longer). If a match is found, returns 0 ‘false’ with the address of the first non-matching character and the length of the remainder the string. If no match is found, returns a1 n1 and ‘true.’ Pronounced “not-match.”

For example, you could compare a string whose address is returned by NAME with PAD, testing:

PAD length NAME -TEXT

-MATCH is generally used to find a short string in a longer string.  It is used by the polyFORTH editor.  The use of ‘-’ in the names of -TEXT and -MATCH is intended to approximate the Boolean symbol ¬, as a reminder that both words return ‘true’ if the strings do not match.

REFERENCES

B@ and B!, Data Base Support, Section 8.0
PAD, Section 2.3.1

### 2.3.6	Character String Input and Output

Character strings may be input or output by a TERMINAL task through words defined in this section.  Such input or output is only possible where the terminal or pseudo-terminal device is capable of supporting the required operation.

#### 2.3.6.1	Character String Input

EXPECT awaits a character string from the terminal or other serial device, given the maximum number of characters and the address of the place where they are to be stored.  Input will be terminated by receipt of the specified number of characters or a ‘return’ (0DH), whichever comes first.  For example,

PAD 10 EXPECT

will await ten characters and place them at PAD.

During input, these user variables are used:

Word	Description

CTR	Contains the number of characters yet to come, as a negative value (e.g., -80) which is incremented for each character received.

PTR	Contains the address into which the next character will be placed.

SPAN	Contains the number of characters input.

#TIB	Maximum number of characters to be processed by WORD.

Incoming characters are checked for “return” (0DH) which terminates input, and backspace (08) or DEL (7FH) which causes CTR and PTR to be “backed up” one and a backspace (or equivalent) to be sent to the terminal.  All characters except these will be “echoed” to the terminal.

EXPECT should not be executed if there is no terminal or serial device capable of providing input for the task.

No indication is provided at the terminal that the system is awaiting input as a result of an EXPECT request.  The programmer should indicate this fact through some output message issued prior to the EXPECT request.

In some situations it may be desirable to avoid both the editing of the input string and the echoing.  For this reason, an alternative word is available, called STRAIGHT, which is used exactly like EXPECT but does not perform any editing or echoing.

The command KEY awaits one character and leaves it on the stack.  It uses STRAIGHT, and does not edit or echo.

The conventional place to put incoming strings is the input message buffer, which starts at the bottom of the user’s stack (whose address may be found by the word TIB) and extends upward in memory toward the return stack.  At least 80 bytes are available.  The command QUERY will EXPECT 80 bytes into the input message buffer and perform the necessary housekeeping to use words from the text interpreter to process the text.  QUERY is used in QUIT.

REFERENCES

Character String Output, Section 2.3.6.4
Number Conversion, Section 2.6.2
Terminal Input, Section 3.7.1
TEXT, Section 2.3.6.3

#### 2.3.6.2	Scanning Characters to a Delimiter

WORD is the main work-horse of polyFORTH’s text interpreter.  It fetches characters from the input stream (terminal input buffer or a disk block) starting at the offset given by the user variable >IN, to a specified delimiter.  WORD is used by TEXT, and functions in the same manner as TEXT except for the following differences:

The input characters are placed in storage starting two bytes (four bytes on 32-bit systems) beyond the current dictionary head (HERE 2+ or 4+).  H is not modified.  The first byte of the resulting string contains the actual length of the string, up to the occurrence of the delimiter.  The area where the characters are placed is not initialized, although there will be one trailing blank (not included in the length at HERE 2+) inserted by WORD following the string.  WORD returns on the stack the address of the string.  This is a convenience for the words that conventionally follow it, such as NUMBER.

The space between H and PAD is used by other polyFORTH functions, such as output number conversion.  As a result, when you use WORD to pick up a string from the input stream you should finish working with it or move it to another area (such as PAD) promptly to avoid confusion.  TEXT is generally preferable, unless you need the length of the string.

As an example of WORD’s use, consider the following simple example (COUNT is a standard word whose definition is shown here for convenience):

: COUNT ( a - a' n)   DUP 1+  SWAP C@ ;

: TEST   32 WORD  COUNT TYPE ;

TEST would be used:

TEST ABC     (carriage return)     ABC ok

REFERENCES

Fetching Input Characters to PAD, Section 2.3.6.3
NUMBER, Section 2.6.2.1
Text Interpreter, Section 1.1.4
TYPE, Section 2.3.6.4

#### 2.3.6.3	Fetching Input Characters to PAD

TEXT scans the current input stream until a delimiter is encountered.  These characters are placed in PAD, which TEXT has previously set to blanks for a length of at least 72 bytes.

TEXT expects the delimiter character in the low-order byte of the top of the stack.  TEXT bypasses any occurrences of this character until a non-delimiter character is encountered.  That character is then placed in the first byte in PAD.  Succeeding characters are then moved into the following positions of PAD until a delimiter character is encountered or the length of the string (given by #TIB) has expired, which terminates the operation.

TEXT takes its input from the current input source.  This is normally the terminal input buffer.  During the time that text is being interpreted through a LOAD operation, however, the current input source is the selected block being loaded.

TEXT uses WORD to input the string, which is then moved to PAD.

REFERENCES

PAD, Section 2.3.1
Terminal Input, Section 3.7.1
The LOAD Operation, Section 3.3.1
WORD, Section 2.3.6.2

#### 2.3.6.4	Character String Output

TYPE outputs a character string to the terminal or other serial device (such as a printer).  The character string is output exactly as it appears in storage, with parity bits added by the hardware if required by the terminal in use.

The length of the string, in bytes, must be on the top of the stack with the address of the first byte of the string on the stack beneath it.

For example, you could use the following phrase to print thirty-two characters from PAD on the terminal:

PAD 32 TYPE

During output, user variables CTR and PTR contain the count and address, respectively, of characters yet to be sent.

The command EMIT will type the ASCII of a single character from the stack.  Thus,

65 EMIT

will type an “A.”

REFERENCES

PAD, Section 2.3.1
Terminal Output, Section 3.7.3

#### 2.3.6.5	Compiling Messages

There are two cases in which it is desirable to have text messages compiled in programs:  to issue error messages, and to communicate information during normal operation.  The following words provide compiled strings:

Word	Description

"	Compiles the string following, terminated by another ".  At run time, the address and length of the string will be pushed on the stack.  For example:

: "TEMP" ( n)   68 > IF
   " WARM "  ELSE  " COOL "
   THEN TYPE ;

This will display the message "WARM" if the temperature in the stack is greater than 68, and "COOL" otherwise.

." string"	Compiles a string which will be typed-out when the word that contains it is executed.  For example:

: Greeting   ." Hi there" ;

t ABORT" string"	Compiles a string which will be typed out as an error message if the value on the stack (t) is true when the phrase is executed.  For example:

: ?TEMP ( n)   95 >
	   ABORT" TOO WARM!" ;

In both cases the quote mark serves as the delimiter that marks the end of the string.  Both words use the compiling word STRING.

Note that each of these words has functions to be performed both at compile time and at execute time.  At compile time the address of the execute-time function is compiled, along with the string.  At execute time the behavior differs.  For " the address and length of the string must be pushed on the stack.  For ABORT" the test must be performed.  For both ." and ABORT" the string must be typed out.

These words must appear only inside a definition.

If ABORT" finds its argument to be true (non-zero), it will echo the command most recently interpreted, issue the message, clear both stacks, and return control to the operator.

Examples:

: HI   ." HI THERE" ;

: CHECK ( n - n)   1000 OVER <
   ABORT" TOO BIG" ;

REFERENCES

Error Handling, Section 2.4.7
Report Titles and Headings,  Section 8.8.2
STRING, Section 2.8.7

## 2.4	Program Structures

Forth contains a set of words that may be used to establish program loops and alter the normal sequential execution of words.  Such words are described in this section.  Please note that similar words which also alter the normal flow of execution are defined in the ASSEMBLER vocabulary for use in CODE definitions.

Logic control words must be used within a definition, because they cannot operate properly when typed from a keyboard.  Loops generally must be opened and closed within the same definition.  Loops may be nested to any depth.

Some of the words in this section are called “compiler directives.”  When the compiler sees most words, it compiles cell-wide addresses pointing to the words’ code field addresses.  When the compiler sees a compiler directive, it executes it immediately rather than compiling it.  Forth is extensible, so you may define your own compiler directives.  Exact techniques are in the section referenced below.

REFERENCES

Compiler Directives, Section 2.8.8

### 2.4.1	Infinite Loops

The simplest looping method available in Forth is the BEGIN ... AGAIN loop.  The BEGIN ... AGAIN loop repeats the code between the BEGIN and AGAIN endlessly.  BEGIN ... AGAIN loops are used for control activities which are not expected to stop. Examples of such applications include process control loops and computer-sequenced machinery. BEGIN ... AGAIN is also used in QUIT, the highest level word of an interactive polyFORTH system.

An example of a high-level program to control an industrial process might be:

: REACTION   CONTROLS CLEAR
   BEGIN  DATA ERROR CORRECT  AGAIN ;

This process control loop clears the controls, then enters an infinite loop which continuously collects data, calculates an error quantity, and applies a correction function.  Usually such a program is run asynchronously by a background task, and the operator stops it with a word built from task control words.

Note that the word BEGIN does not compile anything; it simply pushes the address of the next available dictionary location on the stack at compile time.  BEGIN thus marks a location for a later compiler directives’ compile time operations.

REFERENCES

Task Control, Sections 4.5, 4.10

### 2.4.2	Post-Testing Indefinite Loops

BEGIN and UNTIL allow the user to set up a loop which may be executed repetitively in a manner similar to BEGIN ... AGAIN loops, except that a test is performed before the loop repeats.

The form of a BEGIN ... AGAIN construction is:

BEGIN  {words to be executed repeatedly}  {test words}  UNTIL

When execution reaches the word UNTIL, the value on the top of the stack is examined and removed from the stack.  If this value is false (zero), then execution returns to the word that follows BEGIN; if the value is true (non-zero), execution continues with the word that follows UNTIL.

BEGIN loops can be nested.  However, a loop of any type must be nested entirely within any outer loop.

BEGIN ... AGAIN may only be used within a single definition; it may not be executed by direct entry from a terminal.

The ASSEMBLER vocabulary also contains words named BEGIN and UNTIL; these words perform similar, but not exactly equivalent, functions.

REFERENCES

BEGIN ... UNTIL for the Assembler Vocabulary, Section 6.8
BEGIN ... AGAIN, Section 2.4.1
Logic Operations, Section 2.2.2

### 2.4.3	Pre-testing Indefinite Loops

Pre-testing indefinite loops are similar to BEGIN ... UNTIL loops except that the test to leave the loop is performed before the end of the loop code.  The syntax of the Forth pre-testing loop is:

BEGIN  {executed every iteration}  {test } WHILE
   {not executed on the last iteration} REPEAT

WHILE removes the number at the top of the stack and tests it, then leaves the loop if the value is false (zero), skipping the words between WHILE and REPEAT.  If the value on the stack is true (non-zero) then WHILE continues to the next word in the loop.  When the CPU reaches REPEAT, the CPU will go back to the words immediately following BEGIN and repeat the loop.

For an example, consider a word that counts fruit in a mechanical sorter:

: GOOD ( - n)   0  BEGIN  FRUIT ?GOOD
   WHILE  1+ FETCH  REPEAT ;

As long as the machine sees good fruit in the test cell the loop continues, and the machine considers the next fruit.  When the test fails, the fruit remains in the test cell, to be evaluated by some process other than the word ?GOOD.

In situations when both are equally convenient, the BEGIN ... UNTIL loop is faster and requires fewer bytes and is thus preferable to the BEGIN ... WHILE ... REPEAT loop.

REFERENCES

BEGIN ... UNTIL Loops, Section 2.4.2
Logic Operations, Section 2.2.2

### 2.4.4	Counting (Finite) Loops

Forth provides words to allow looping within a specified index range in a manner similar to FORTRAN DO-loops.

The possible forms of a counting loop are as follows:

limit index DO {words to repeat} LOOP
  or	limit index DO {words to repeat} value +LOOP
  or	limit index DO {words to repeat} value /LOOP

The words associated with counting loops are:

Word	Function

DO	Establishes the loop parameters.  This word expects the initial loop index value on the top of the stack with the limit value beneath it.  These values are removed from the stack and stored on the return stack when DO is executed.

Word	Function

LOOP	Causes the index value to be incremented by one and compared with the limit value.  If the index value is equal to or greater than the limit value, the loop is terminated and execution resumes with the next word.  If the index value is less than the return value, control returns to the word that follows the DO that opened the loop.

+LOOP	Resembles LOOP but increments the index by the specified signed value.  On 16-bit machines, +LOOP will handle any 32K range (e.g., -16384 through +16383, 0 through 32767, -32768 through 0, etc.). On 32-bit machines, +LOOP will handle any 64K range (e.g., -32768 through 32767, 0 through 65535, and 65536 through 0).

/LOOP	Resembles +LOOP but uses an unsigned value, and can thus support a range from 0 through 65535 (131070 on 32-bit machines).  Where incrementing values are known to be positive (which is most of the time), /LOOP may be significantly faster than +LOOP.  Pronounced “ramp-loop.”

I	Pushes a copy of the top item on the return stack (the current value of the index) onto the top of the parameter stack.  This word may only be used for this purpose within the definition that opened the loop, not in definitions that the loop invokes, because nested colon definitions cause a return address to be put on the stack on top of the loop index.

I'	Pushes a copy of the second item on the return stack onto the parameter stack. I' is often used in a word called inside a loop to obtain access to I, or within the loop to reference the limit.

J	Pushes a copy of the third return stack item onto the parameter stack. When two DO ... LOOPs are nested, this is the value of the outer index from inside the inner loop.

LEAVE	Sets the limit equal to the current value of the index, forcing the loop to terminate on the next execution of LOOP, +LOOP or /LOOP.

To illustrate the use of loops, the word SUM is defined in such a way that it sums the values of the integers 1 to 100 and leaves the result on the stack:

: SUM   0  101 1 DO  I +  LOOP ;

Note that the limit value is specified as 101, not 100, because the loop terminates when the limit value is reached or exceeded after the index value is incremented.  Loops may be nested to any depth, limited only by the capacity of the return stack.  At each point in a nested loop, the word I refers to the index of the innermost active loop.

+LOOP allows descending index values to be used.  When an index value is descending, however, the loop is terminated when the limit is passed (not merely reached).  When the index value is ascending (i.e., the increment value specified for +LOOP is positive), the loop terminates when the index value is reached, as for LOOP.

To illustrate the use of +LOOP with descending index values, the following definition is equivalent to the first definition of SUM:

: SUM   0  1 100 DO  I +  -1 +LOOP ;

Here the initial value of the index is 100 and the final value is 1.

Since the loop parameters are checked at the end of the loop, any loop will always be executed once, regardless of the initial values of the parameters.  Also, loop parameters are kept on the return stack and they are not affected by loop structures other than DO ... LOOP.

### 2.4.5	Conditionals

These words allow conditional execution of words within a single definition.  They may only  appear within a definition and may not be used in interpretive text or in text executed by direct entry from a terminal.

The general form of usage of these words is:

{test words} IF {true-clause} ELSE {false-clause}  THEN
  or	{test words} IF {true-clause} THEN

When IF is executed, the item on the top of the stack is removed from the stack and examined.  If the value is true (non-zero), execution continues with the words that follow IF (i.e., the true-clause).  If the value of the word is false (zero), execution resumes with the words that follow ELSE (i.e., the false-clause) or with the words that follow THEN if ELSE is not present.

Execution of the true-clause terminates with the word ELSE, if present.  Execution resumes with the word that follows THEN.

Both the true-clause and the false-clause may be any group of defined Forth words.  Either clause may contain DO ... LOOPs, BEGIN ... UNTIL loops, and/or other IF ... ELSE ... THEN structures, so long as the entire structure is contained within the clause.  In other words, a DO or BEGIN loop may be used in an IF clause or an ELSE clause so long as the terminator (whether LOOP, UNTIL, or REPEAT) appears within the same clause.  Similarly, one IF structure may be nested inside another structure of any kind so long as the THEN that terminates the structure appears within the same clause.

An “out of range” error message means too much code is between the beginning and end of a control structure.  It can also mean that one part of the control structure is missing.

REFERENCES

Logic Operations, Section 2.2.2

### 2.4.6	EXIT

EXIT allows the address interpreter to leave a definition at any point.  EXIT is also the word compiled by ; at the end of every : definition.  EXIT pops a value from the top of the return stack into I (the address interpreter’s pointer to the next word to interpret), and then performs a NEXT, to begin execution of the next word in the definition which called the word containing EXIT.  The return stack must be clear of any loop parameters or temporarily stored data before an EXIT can be performed.

A trivial example of EXIT is:

: TEST ( n)   1 .  IF EXIT THEN  2 . ;
0 TEST 1 2
1 TEST 1

Frequently, words containing EXIT will have different stack results, depending on whether the word EXITs or not.  The standard stack notation for such a situation is:

( input-arguments - EXIT-case, normal-case)

EXIT is the only Forth word which permits unstructured programs (program modules with multiple exit points).  It is very bad form to use EXIT more than once in a word, because clarity and readability often suffer.  If you believe you need to use EXIT twice in a word, try factoring that word into several words.

Another property of EXIT is the way it behaves during the text interpretation of disk blocks.  When a disk block is LOADed, the text of the disk block is interpreted by the text interpreter defined in the word INTERPRET.  INTERPRET is an endless loop which exits either by a ' EXIT JMP in WORD, or by executing a word which removes the proper depth of entries from the return stack, and then executes EXIT.  When the INTERPRET in LOAD encounters an EXIT, the EXIT pops the address interpreter’s return stack, and performs a NEXT which executes the word after INTERPRET in LOAD. Thus interpretation of a disk block can be stopped by placing an EXIT after the last word to be interpreted.

REFERENCES

Address Interpreter, Section 1.1.6
LOAD, Sections 3.3.1, 3.3.2
Text Interpreter, Section 1.1.4

### 2.4.7	Abort Routines

There are three routines available for handling abort conditions for terminal tasks, plus two that may be used by any task:

Word	Function

ABORT	Unconditionally terminates execution and returns to the task’s idle behavior.  No message is issued.  May be executed by any task.

t ABORT" text"	If t is true (non-zero), types the specified text at the user’s terminal, clears both stacks, and returns to the task’s idle behavior.  (No further words will be executed.)

Must be used inside a definition; may be executed only by a TERMINAL task.

The definition of ABORT" concludes with the word ABORT.

Word	Function

QUIT	Terminates execution of the current word (and all words above it).  All words are removed from the return stack and the parameter stack is cleared.  No indication is given to the terminal that a QUIT has occurred. Execution will not resume until a new word (or words) is entered from the terminal. QUIT is the default idle behavior for TERMINAL tasks.

STOP	Terminates execution, leaving the task inactive.  The task may only be awakened by an interrupt routine or another task.

NOD	An infinite loop containing STOP. This is the default idle behavior of background tasks.  If an interrupt awakens the task it will simply go inactive again.

Since a background task by definition has no terminal on which to issue messages or to which control may be passed, abort behavior for such tasks must be handled differently, normally by setting a flag and calling STOP.

REFERENCES

Background Tasks, Sections 4.3, 4.4, 4.5
Terminal Tasks, Sections 4.8, 4.9, 4.10

### 2.4.8	Vectored Execution

Although normal Forth usage (as well as good programming practice) emphasizes the “structured programming” modes of sequential, iterative, and conditional execution, it is sometimes desirable to direct Forth to execute a specific function in response to some external stimulus.  This technique may be used, for example, by a report that searches a data base, selecting records according to one of a number of specified criteria; by a bank of push-buttons, each of which is attached to a particular Forth word; or by a routine that computes the address of a function to be executed.

#### 2.4.8.1	Using EXECUTE for Vectored Execution

The word EXECUTE expects on the stack the address of the parameter field of a definition.  EXECUTE will execute the routine by jumping to its code address.

For example:

VARIABLE NUMERAL
: T1   1 . ;
: T2   2 . ;
: ONE   ['] T1  NUMERAL ! ;
: TWO   ['] T2  NUMERAL ! ;
: N   NUMERAL @ EXECUTE ;

If the user types:

ONE N

the computer will type 1.

Typing:

TWO N

will produce 2.

The phrase @ EXECUTE is so common that a special word @EXECUTE is defined to save space and CPU time.  The behavior of @EXECUTE is exactly the same as the phrase @ EXECUTE, with the addition of a check on the contents of the address supplied:  If the address to be executed is zero, @EXECUTE will simply return to the calling definition without performing any operation.  This means that such execution vectors may not require special initialization.

The stack effect of all members of a set of words to be EXECUTEd in a particular context must be the same.  That is, they must all require or leave the same number of items on the stack.  The behavior of @EXECUTE in ignoring zero addresses is appropriate only in cases where there are no stack arguments.

REFERENCES

['], Section 2.8.6

#### 2.4.8.2	Using ASSIGN for Variable Functions

ASSIGN provides a convenient means of storing a code address into RAM.

ASSIGN must be used inside a : definition.  When executed, ASSIGN expects an address (usually that of a VARIABLE) to be on the stack.  ASSIGN will store into that address the address of the cell that immediately follows ASSIGN in the current definition.  Execution of the definition containing ASSIGN then terminates, so that the words following ASSIGN can be executed by a subsequent use of the phrase:

variable-name @EXECUTE

Thus in the structure:

VARIABLE  A
: B   A ASSIGN  X  Y  Z ;

executing B will set the address of the cell that follows the address of ASSIGN in the definition of B (i.e., the address of X) in A but will not execute X, Y, and Z.  Thereafter, the phrases:

A @ EXECUTE   or   A @EXECUTE

will result in the execution of X, Y, and Z.

Using this technique, the example of vectored execution may be simplified to:

VARIABLE NUMERAL
: ONE NUMERAL ASSIGN  1 . ;
: TWO NUMERAL ASSIGN 2 . ;
: N NUMERAL @EXECUTE ;

The use of these words would be the same as in the previous example.

An actual example of the use of ASSIGN (in the standard polyFORTH graphics package) specifies plotting modes:

VARIABLE  MARK

: LINES   MARK ASSIGN

( code to connect the last data point to this one) ;

: POINTS   MARK ASSIGN

( code to draw a + for this data point) ;

: HISTOGRAM   MARK ASSIGN

( code to draw a histogram segment from the last data point 		   to this one) ;

: PLOT   N 0 DO  I PTS @  I MARK
   @EXECUTE LOOP ;

All three “markings” routines expect the Y and X values for the current point, where the phrase I PTS @ supplies the Y value and I gives the X value.  N is the number of points.

The user can now type:

LINES PLOT
  or	POINTS PLOT
  or	HISTOGRAM PLOT

in a natural manner.  The chosen function will remain set until changed.

#### 2.4.8.3	Creating Vectored Execution Tables

Most uses of EXECUTE and @EXECUTE are for implementing a variable function as described in the previous section.  The ability to generate and manage a table of execution addresses is also extremely useful for such purposes as managing a function-button pad, function menu on a graphics tablet, etc.  This section will outline a simple button response application which may serve as a model for similar situations.

Let us assume that the word BUTTON has been defined to wait until a button is pressed and then to return the button number (0-15) of the button (the actual definition of BUTTON would depend on the computer and interface).  Now consider the following:*

VARIABLE BUTTONS   30 ALLOT
: IGNORE ;
' IGNORE BUTTONS !  BUTTONS DUP 2+ 30 MOVE

The above lines create a table with one cell for each button and also initialize all positions to contain the address of an empty definition (which effectively “ignores” an undefined button).

Now we will define a special defining word that will not only create an ordinary : definition but also store the address of its parameter field into a specified cell of BUTTONS:**

: :B ( n)   :   LAST @ @ CFA 2+
   SWAP 2* BUTTONS + ! ;

Now we can create definitions which are attached to certain buttons by using :B with the button number as a parameter.  Each such definition will have a name to allow it to be tested independently of the button pad.  For example,

0 :B ESCAPE   1 ABORT" ?" ;

defines Button 0 to be an “escape” button.

All that remains is to define a routine to monitor the button pad and handle responses:

: MONITOR   BEGIN BUTTON BUTTONS
   + @EXECUTE  AGAIN ;

Typing MONITOR will place the terminal task in an infinite loop that responds to buttons.  Button 0 will cause an abort and return control to the terminal.

In practice, MONITOR may very likely be executed by a background task.  In this case you must use STOP or NOD for halting rather than ABORT" (which requires a terminal).

REFERENCES

ABORT", Section 2.4.7
BACKGROUND Tasks, Sections 4.0, 4.3-4.5
STOP and NOD, Sections 2.4.7, 4.2

## 2.5	Numeric Output Words

Numeric output words allow the output of numeric quantities in ASCII.  This output is generally directed to the terminal.

Numeric output words are divided into two categories:  normal output words and conversion output words.  The latter allow the “picturing” of ASCII text, in a manner that resembles COBOL picturing.

All numeric output words produce ASCII text, which is the ASCII number expressed in the current radix contained in BASE.  The BASE is controlled either through the appropriate radix word (e.g., OCTAL, DECIMAL, or HEX) or by directly setting the current value of BASE.  For example, BASE might be set to binary by:

2 BASE !

REFERENCES

Numbers, Section 1.1.5

### 2.5.1	Standard Numeric Output Words

Several standard words allow printing of single or double-precision signed numbers.  Each prints an output string that consists of the following characters:

1.	If the number is negative, a leading minus sign (hyphen).

2.	The absolute value of the number, with leading zeroes suppressed.  The number zero results in a single zero in the output.

3.	A trailing blank.

The following table lists the standard numeric output words.

Word	Stack	Function

.	( n)	Prints a signed single-precision integer followed by one space. Pronounced “dot.”

?	( a)	Prints the contents of the address on the stack (? is equivalent to the phrase @ .).

Word	Stack	Function

U.	( u)	Prints an unsigned single-precision integer followed by one space.  Pronounced “u-dot.”

U.R	( u n)	Prints the unsigned single-precision integer u with leading spaces to fill a field of width n, right-justified.  This word expects an integer on the top of the stack to specify the length of the output field.  The width of the printed string that would be output by . is used to determine the number of leading blanks printed.  No trailing blanks are printed.  If the magnitude of the number to be printed prevents printing within the number of spaces specified, additional output characters result.

D.		Prints a signed double-precision integer.

D.R		Prints a signed double-precision integer in a specified field-width, as for U.R.

### 2.5.2	Pictured Number Conversion

polyFORTH contains a series of words that allow numeric quantities to be output through use of a pictured format control.  These words allow specification of field sizes, editing characters, etc.

In Forth, the description of these words starts with the low-order portion of the field and continues to the high-order portion.  Although this is the reverse of the method apparently used in other languages, it is the actual conversion process in all languages.  BASE is a user variable containing the current conversion radix.

These words are used to convert numbers on the stack into ASCII character strings which have been edited according to the picture specifications.  These strings are built in an area in memory which immediately follows the end of the dictionary (the address left by HERE).  This area is large enough to accommodate at least 32 characters of output (64 characters on 32-bit machines).  Following the end of picture conversion, the address of the beginning of the string and the count of the number of characters in it are passed to the user.  At this point, the converted string can be printed at the terminal with TYPE or used in some other way.

All of the standard numeric output words use the same region in the user’s partition (the area below PAD).  As a result, these words may not be executed while a pictured output conversion is in process (e.g., during debugging).  Furthermore, the user may not make new definitions during the pictured conversion process, since this would move the area in which the string is being generated.

REFERENCES

Numbers in Forth, Starting FORTH, Chapter 7
PAD, Section 2.3.1
Report Generator, Data Base Support, Section 8.0
Standard Numeric Output, Section 2.5.1

#### 2.5.2.1	Using Pictured Numeric Output Words

These words allow control over the conversion of binary numbers into digits.  This section only describes pictured words which result in digit output; the following sections describe output of non-numeric punctuation such as periods and commas.  Throughout the number conversion process the number being operated on remains on the stack, where it is repeatedly divided by BASE as digits are converted.  The number is finally discarded by #> at the end of the process.

Word	Stack	Function

| <# ( ud - ud) or ( n ud -n ud) | Initializes pictured output of an unsigned double-precision integer. If the output is to be signed, a signed value must lie immediately beneath this integer to control whether or not a minus sign will be introduced into the output string by SIGN (below). |
| --- | --- |

#	( ud - ud)	Adds a digit to the low-order portion of the resulting string. Must be used after <# and before #>.  The first digit added is the lowest-order digit (units), the next digit is the tens digit, etc.  Each time # is used, a digit is generated.

#S	( ud - ud)	Converts digits repetitively until all significant digits in the source item have been converted, at which point conversion is completed.  Must be used after <# and before #>.  #S always results in at least one output character, even if the number to be converted is a zero.

Word	Stack	Function

SIGN	( n ud - ud)	Inserts a minus sign at the current position in the string being converted if the signed value in the third stack position is negative.  This signed value is a single-precision number; if the high-order bit is set, a minus sign will be introduced into the output as the left-most non-blank character.  The magnitude of the signed value is irrelevant.  In order for the sign to appear at the left of the number (the usual place) SIGN must be called after all digits have been converted.

#>	( ud - a n)	Completes the conversion process after all digits have been converted.  This word discards the (presumably) exhausted double-precision number, and pushes onto the stack the address of the output string, with the count of bytes in this string above it.

To aid in understanding the use of these words, consider a definition of the standard Forth word. (“dot”):

: . ( n)   DUP ABS 0  <# #S SIGN #>
   TYPE SPACE ;

DUP ABS puts two numbers on the stack; the absolute value of the number on top of the number itself, which is now useful only for its sign.  0 adds a cell on top of the stack, so that the 0 cell and the ABS cell form a double-precision integer to be used by the <# ... #> routines.

If you want to print a signed double-precision integer with the low-order three digits always appearing, regardless of the value, you could use the following definition:

: NNN ( d)   SWAP OVER DABS  <# # # #S
   SIGN  #>  TYPE SPACE . ;

The SWAP OVER DABS phrase establishes the signed value beneath the absolute value of the number to be printed for the word SIGN.  The sequence # # converts the low-order two digits, regardless of value.  The word #S converts the remaining digits and always results in at least one character of output, even if the value is zero.

From the time when the initialization word <# executes until the terminating word #> executes, the number being converted remains on the stack.  It is possible to use the stack for intermediate results during pictured processing but any item placed on the stack must be removed before any subsequent picture editing or fill characters may be processed.

#### 2.5.2.2	Using Pictured Fill Characters

In addition to pictured numeric output, it is possible to introduce fill characters (or punctuation) into the output string through the use of HOLD and '.'.  When one of these words is used, the appropriate character is entered into the output string at the current position.

Arbitrary fill characters may be inserted in a string being formatted by using the word HOLD.  HOLD requires as a parameter the numeric value of the ASCII character to be inserted.  Thus,

2F HOLD	(value given in hex)

inserts the character / into the output string.

'.' produces a decimal point at the current position in the pictured numeric output.  To illustrate, the word .$ will print double-precision integers as signed amounts with two decimal places:

: .$ ( d)   SWAP OVER DABS  <# # # '.'
   #S SIGN #> TYPE  SPACE ;

If fill characters are likely to be used in several definitions, you may wish to add commands similar to '.'.  The following format is used for such a definition:

: 'name'   char-value HOLD ;

where char-value is the ASCII value of the character in the current radix and 'name' is the name of the word to be defined.  HOLD is defined in such a way that executing 'name' during pictured editing causes the indicated fill character to be introduced into the output.

#### 2.5.2.3	Processing Special Characters

The normal pictured output capabilities described in the preceding two sections are generally sufficient to handle most output requirements.  Certain special cases, however, such as the introduction of commas in a number or the floating of a character (e.g., $), require special processing.  In order to perform certain of these operations, it is necessary to refer to the unconverted portion of a number being printed.

This unconverted portion is a number that is equivalent to the original number divided by 10 (or the current radix) for each numeric digit already generated.  For example, if the initial number is 123, the intermediate number is 12 (following the conversion of the first digit) and 1 (following conversion of the second digit).

The value of this number may be tested and logical decisions may be made based upon its value.  To illustrate, consider the following definitions.  The word D.ENG prints a double-precision integer in engineering format:

0 ( Number Formats)   DECIMAL

1 : ','   44 HOLD ;

2 : (D.ENG ( d)   SWAP OVER DABS  <#  BEGIN

3    ',' #  2DUP D0= NOT WHILE   2 0 DO

4    #  2DUP D0= IF  LEAVE  THEN   LOOP   REPEAT

5   SIGN #> ;

6 : D.ENG ( d)   (D.ENG)  TYPE  SPACE ;

7

8

9

10

11

12

13

14

15

Using techniques similar to those set forth above, you can do almost any kind of numeric output editing in Forth.

REFERENCES

Report Generator, Data Base Support, Section 8.8

## 2.6	Text Interpreter Words

The text interpreter in Forth is used both for terminal interaction and for processing text in disk blocks (either in direct execution or compilation).  The purpose of this section is to discuss ways in which the programmer may use the text interpreter in application routines.

REFERENCES

Text Interpreter, Section 1.1.4

### 2.6.1	Dictionary Searches

For the dictionary to be useful, it must be possible to look up words and their definitions in it.  Forth provides several words, all of which perform a search and then return information about the word.  These searches are used in the text interpreter and colon compiler without modification.  Note that since polyFORTH has variable length names, words are available which find addresses that point both before and after the name.

The following are dictionary search words:

Word	Stack	Function

'	( - a)	Gets the next word from the input stream, and attempts to look up the word in the dictionary. If the word is found, then ' returns the word’s parameter field address, otherwise ' aborts.  This word calls -'.  Pronounced “tick.”

[']		Must be used in a colon definition.  ['] finds the next word in the text and compiles the word’s parameter field address as a literal.  If the next word is not in the dictionary, ['] aborts.  ['] calls -' and LITERAL.  ['] is an IMMEDIATE word (executed rather than compiled by the colon compiler, see the references section).  Pronounced “bracket-tick.”

-'	( - a a' 0 ,  a t)	Gets the next word from the input stream and attempts to look up the word in the dictionary.  If the look-up succeeds, -' returns the parameter field address (a), the dictionary link address (a') and a zero.  If the word is not found, -' returns the address of the string (HERE 2+ or HERE 4+ on 32-bit machines) and a number guaranteed to be non-zero (i.e., ‘true’).  This word is called by the text interpreter and colon compiler.  Pronounced “dash-tick.”

Word	Stack	Function

'HEAD	( - a)	Gets the next word from the input stream, and attempts to look up the word in the dictionary.  If the word is found, then 'HEAD returns the address of the word’s first byte, otherwise aborts. This word calls -'.  Pronounced “tick-head.”

Tick performs a dictionary search for the word that immediately follows it in the current input stream.

The phrase:

' name

when typed at a terminal or executed interpretively in a source block, pushes onto the stack the address of the parameter field of name if name can be found in the dictionary.  If name cannot be found, an abort will occur with the error message:

name ?

The most common uses of ' for dictionary searches are:

1.	To find out whether a word has been defined.

2.	To find the location of a word (for example, to DUMP its contents).

3.	To obtain the address of a CONSTANT (for example, in order to change it).

Note that ' is not immediate.  That is, if you wish to compile a reference to an address, as in item 3 above, you must use ['].  The words ' and 'HEAD always take their operands from the current input stream at the time they are executed.  This is an important characteristic because it makes possible the use of these words in such words as FORGET (which uses 'HEAD to find the location of the word to be “forgotten”).

The order in which the various vocabularies are searched is specified by CONTEXT.  The content of CONTEXT is best examined as a four-digit hexadecimal number whose high-order digit is the index of the first vocabulary to be searched, whose next digit gives the next vocabulary, etc.  An index of zero terminates the search.  Thus, if CONTEXT is 1500, the search order is FORTH, then EDITOR.  A value of 3150 would specify ASSEMBLER, then FORTH, then EDITOR.  In some CPUs, the order is reversed.  For example, 0513 on the Z80 would specify ASSEMBLER, FORTH, then EDITOR.

Fig. 2.5

Dictionary entry, showing the addresses returned from common dictionary search words.

Please note that ' and 'HEAD make use of a lower level dictionary search word:  -' (pronounced “dash-tick”).  The word -' searches for the next word in the input stream, delimited by blanks.  If the word is found, -' returns the parameter field address, the link field address and a zero (or false) on top.  If not found, -' returns two numbers.  The number on top is guaranteed to be non-zero and the other is the address of the string which was delimited by blanks (HERE 2+, or 4+ on 32-bit machines).  The definition of ', using -' is:

: ' ( - a)   -'  ABORT" ?"  DROP ;

The top of the stack after -' is used as a Boolean truth value by ABORT".  If the search succeeds, the DROP removes the link address, leaving the desired word’s parameter field address on the stack.  If the search fails, ABORT" resets the user environment and prints the text placed just above HERE by WORD (which happens to be the word that -' couldn’t find) followed by an error message.

REFERENCES

['], Section 2.8.6
ABORT", Section 2.4.7
IMMEDIATE Words,  Section 2.8.8
Vocabularies, Section 3.4
WORD, Section 2.3.6.2

### 2.6.2	Input Number Conversion

Wherever possible an application should be designed to take advantage of polyFORTH’s naturally interactive capability.  Thus, a hypothetical word SCANS whose function is to perform some user-specified number of scans (an application function) should expect only its parameter on the stack.  Then to perform 100 scans, the user could type:

100 SCANS

Such a usage is natural and convenient for the operator and requires no special programming to handle the input parameter.

There are occasions in which normal Forth syntax is inadequate.  Some examples include:

1.	Parsing a text string that comes from a source other than a terminal, such as magnetic tape.

2.	Entry of numbers that must be in double-precision but are not punctuated (i.e., zip codes).

3.	Entry of numbers that must follow rather than precede the command.

polyFORTH provides several words to enable the user to handle input numbers in a variety of circumstances.  This section describes these methods.

#### 2.6.2.1	Number Conversion Using the Text Interpreter

The word NUMBER is the standard input number conversion routine used by the text interpreter and colon compiler.  It is available to users and performs number conversions explicitly from ASCII to binary.  NUMBER uses the value in the user variable BASE to determine which radix should be used when converting numeric strings to binary.

Normally NUMBER immediately follows WORD.  It expects on the stack the address of the string that is to be converted, with a count in the first byte of the string and one trailing blank following the counted bytes.  NUMBER will attempt to  convert that string to binary and, if successful, will leave the result on the stack.  Its rules for behavior in the conversion are those described in the earlier section on numbers, depending upon the presence of the extended-precision math option.  If the conversion fails due to illegal characters, an abort will occur, with an error message echoing the string followed by “?.”

Thus if the number to be converted is coming from the normal input stream, the complete sequence would be:

QUERY 32 WORD NUMBER

This would leave the converted binary value on the stack.

Note that QUERY inputs to the input message buffer, and automatically takes care of the housekeeping necessary to prepare the input message buffer for the interpreter.

NUMBER may be used to convert a string from another location (e.g., a string which has not been fetched by use of WORD).  If the location of the string does not contain the count in the first byte, you may simply simulate its presence by subtracting one byte from the starting address of the string.  NUMBER does not actually make use of the count; it only adds one byte to the address before beginning.  Thus, for a string whose actual location is given by a word named BUF, the sequence would be:

BUF 1- NUMBER

The requirement that a blank follow the string is more absolute.  If it is not feasible to guarantee this, you may prefer to move the string to PAD, as shown below, or use CONVERT which will not abort out of the application program as NUMBER does when it sees a non-numeric character.

REFERENCES

CONVERT, Section 2.6.2.2
Numbers, Section 1.1.5
QUERY, Sections 2.3.6.1, 3.7.1
WORD, Section 2.3.6.2
Use of NUMBER in INTERPRET, Section 1.1.4

#### 2.6.2.2	Direct Conversion of Strings

CONVERT is useful because it stops when it encounters any non-numeric digit, rather than aborting as NUMBER does.  For this reason, CONVERT is often used when a number is input by a program directly, without using the text interpreter.

CONVERT expects a double-precision integer and byte address, and leaves a double-precision integer and address.  The initial address into CONVERT must point to the byte preceding the first digit of the string of numerals.  This byte is ignored.  The initial double-precision number is usually set to zero.

After CONVERT stops, the address on top of the stack is the address of the first non-numeric character CONVERT encountered.  The double-precision integer will contain data from all the digits that have been converted thus far.

An example of the use of CONVERT is:

: INPUT ( - n)   PAD 8 BLANK  PAD 1+ 5
   EXPECT  0. PAD CONVERT  2DROP ;

This definition initializes a region of PAD to blanks, and awaits up to five digits which will be stored there.  0. provides an initial double-precision value, and PAD provides the address for CONVERT.  The 2DROP discards the address and high-order part of the numbers.

INPUT will not convert input strings with a leading minus sign, because a minus is not a digit.  If negative input is necessary, the above definition can be extended to check the character upon which conversion stopped to see if it is a minus sign, and if it is, start CONVERT again, and negate the result.  For an example of the system use of CONVERT see the definition of NUMBER in the double-precision input block.

CONVERT returns the address of the string’s next byte so that CONVERT may be called in a loop.  NUMBER calls CONVERT in just this way.  An application similar to NUMBER’s is the parsing of a packet of data received over a communications line or in a tape record, in which numeric fields are separated by an arbitrary delimiter such as //.  To skip over such items, or to skip fields that are not of interest, the appropriate count of bytes may simply be added to the address, which is carried on the stack.

In some cases numbers may be in fields of known length but not separated by any delimiter.  In these cases the best solution may be to use CMOVE to move groups of digits to PAD, where they may be converted easily by NUMBER.

REFERENCES

CMOVE, Section 2.3.4
EXPECT, Sections 2.3.6.1, 3.7.1
NUMBER, Section 2.6.2.1
PAD, Section 2.3.1

## 2.7	Defining Words

polyFORTH provides a basic set of words that are used to define objects of various kinds.  As with other features of polyFORTH, the set of such commands may be expanded.  Here we will present those which are standard in all polyFORTH systems, exclusive of the defining words that are part of the data base support option, the assembler defining words (see your CPU Supplement), and MSG.

REFERENCES

MSG, Sections 2.3.3, 2.7.6.3, 3.7.3

### 2.7.1	Creating a Dictionary Entry

A word is defined when an entry is created in the dictionary.  CREATE is the Forth word which creates a dictionary entry.  A dictionary entry as constructed by CREATE is shown in Fig. 2.6.  CREATE is used by :, CODE, VARIABLE, CONSTANT, and other defining words.  CREATE behaves as follows:

1.	The width between the top of the dictionary and the top of the parameter stack is checked to see if at least 250 bytes remain.  If not, there will be an abort with the error message, “Dictionary full.”  At the same time H is adjusted to an even cell address on machines such as the PDP/LSI-11 and 68K that don’t tolerate odd-byte addresses.

2.	WORD fetches the next word of the input string to the top of the dictionary, with a cell left empty for the dictionary link.  The first byte of the copied string contains its length.

3.	The empty cell is filled with a pointer to the previous entry in the proper dictionary chain for the new word.

4.	The user variable LAST is set to point to the head of the chain containing the new word.

5.	Space is allotted for the new word’s name, depending upon the value of WIDTH.  (Truncation of the name may take place at this point).

6.	The value of WIDTH is reset to the default width in WIDTH 1+.

7.	The code field address of the new word is set to point to the run time code of CREATE, which will push the address of the parameter field onto the stack when the word just defined is executed.

CREATE names a location in memory.  Other defining words which use CREATE may reset the new word’s code field address by using the words ;CODE or DOES> to define different run-time behavior.

CREATE is often used to mark the beginning of an array.  The space for the rest of the array is reserved by incrementing H with ALLOT, as in this example:

CREATE DATA   200 ALLOT

The example reserves a total of 100 cells for an array named DATA (50 cells on 32-bit processors).  When DATA  is used in a colon definition, the address of the first byte of DATA will be pushed on the stack by the run-time behavior of CREATE.  The array is not initialized.  If you wish to set all the elements of the array to zero, you may use ERASE as in the following example:

DATA  200 ERASE

This usage is not appropriate for applications that are to be target compiled for ROM, because in such an environment CREATE returns a ROM address.  VARIABLE should be used in such applications, and the count for ALLOT reduced by two to allow for the two bytes ALLOTed by VARIABLE.  On 32-bit machines, the count for ALLOT should be reduced by four.

Fig. 2.6

CREATE is the fundamental builder of dictionary entries. It is used by :, CONSTANT, VARIABLE, and all other Forth defining words.  Since it is a vectored routine you may modify or enhance the behavior of the Forth compiler in constructing dictionary entries.  Your system provides three levels of enhancement:

Word	Source	Description

(CREATE)	nucleus	Primitive level; constructs a dictionary entry as described in Section 2.7.1, whose code address points to code that pushes the parameter field address on the stack.  Does not allot any parameter field space.

<CREATE>	12	Compiles the information used by LOCATE (source block number) in a cell, then calls (CREATE).  Thus the LOCATE field precedes the head of the definition.

?CREATE	21	Searches the dictionary for the word to be compiled and issues a warning message if the word was previously defined, then executes <CREATE>.

In order to “enable” one of these versions of CREATE, you would store its address in the execution vector 'CREATE.  Thus, the phrase:

' <CREATE> 'CREATE !

in Block 9 enables the version that compiles the LOCATE cell.  Since this version changes the size of the head of a definition, a constant called B/H is provided which contains the number of bytes added to the head size. If B/H is 2, in other words, 'HEAD will return an address two bytes before the link field of all words, even nucleus words that don’t have a LOCATE field.  The word -' (Section 3.3.2) always returns the link field address, however.

?CREATE may be enabled in Block 9 by enclosing EXIT within parentheses, i.e., (EXIT).  Since it doesn’t change the actual dictionary entry in any way, it doesn’t require any change in B/H.

REFERENCES

:, Section 2.7.4
CODE, Sections 2.7.5, 6.0
CONSTANT, Section 2.7.3
ERASE, BLANK, FILL, Section 2.3.4
Host Defining Words, Section 7.5
Target Defining Words, Section 7.8
VARIABLE, Section 2.7.2
Vectored Routines, Section 3.1
WIDTH, and the word ~, Section 1.1.1

### 2.7.2	Variables

A VARIABLE is a named memory location whose value may be either fetched onto the stack or stored into, with equal ease.  It may also be treated as the beginning of an array.

The form of the definition of a VARIABLE is:

VARIABLE NAME

This constructs a definition whose name is NAME, with two bytes allotted for a value (four bytes on 32-bit systems).  A single-precision value may be stored into the parameter field of the definition.  For example:

6 NAME !

will store 6 in the parameter field of NAME.

When a VARIABLE is referenced by name, the address of its parameter field is pushed onto the stack.  This address may be used with @ or ! to fetch or store a value, respectively.

The word 2VARIABLE defines a variable whose parameter field is two cells long.  Such a variable may contain one double-precision number or a pair of single-precision numbers (such as x,y coordinates).  2VARIABLE differs from VARIABLE only in the number of bytes allotted.  The operators 2@ and 2! are used with this format.

Similarly, CVARIABLE defines a variable one byte long.  This is only available on machines that tolerate odd-byte addresses, such as the 8086/88.  The operators C@ and C! are used with this format.

Since VARIABLE defines a four-byte item on 32-bit systems, the word HVARIABLE is available on these systems to define two-byte variables.  The operators U@, H@, and H! are available on 32-bit systems to access HVARIABLES.

In summary, to get the value of a VARIABLE on the stack, you invoke its name and fetch instruction. For example, you could type:

varname @
  or	varname 2@

To store into a variable, you invoke the name and a store instruction.  For example:

value varname !
  or 	value1 value2 varname 2!

In a read-only memory environment, VARIABLE is re-defined to allot space in read/write memory rather than its own parameter field; in this case the read/write memory address assigned is compiled into the parameter field. The run time behavior of a variable in read-only memory is the same as a constant.

REFERENCES

@, !, 2@, and 2!, Section 2.1.2
ALLOT, Section 2.8.1

### 2.7.3	Constants

The purpose of a CONSTANT is to provide a name for a value which is referenced often but changed seldom or never.  There are two constructs:

Word	Description

CONSTANT	Defines a single-precision constant.

2CONSTANT	On systems using the 32-bit electives, or on 32-bit machines, defines a double-precision constant whose value may be either a double-precision number or a pair of single-precision numbers.

The procedure for defining constants is to declare:

value CONSTANT name

For example, you may define:

1000 CONSTANT LIMIT
0 5000 2CONSTANT LIMITS
3.141593 2CONSTANT PI

When a CONSTANT is referenced by name, its value (not its address) is pushed onto the stack.  Similarly, when a 2CONSTANT is referenced, its double-precision value is pushed onto the stack.  In the case of 2CONSTANT (used for two values as in LIMITS, above), the values are placed on the stack in the order specified (e.g., 5000 on top, 0 below).  In the case of a double-precision number, the high-order part of the number is on top of the stack.

In order to change a CONSTANT you must first obtain its address.  This is done by using ' when in interpretive mode or ['] when inside a colon definition, in either case followed by the name of the CONSTANT.  The command ! will store in to the address of a CONSTANT thus obtained and 2! into the fetched address of a 2CONSTANT.

For example, you might type this at your terminal:

100 CONSTANT COUNT   500 ' COUNT !

The first phrase creates a CONSTANT named COUNT whose value is 100.  The second phrase changes the value to 500.

CONSTANTs in read-only memory applications cannot be changed, as the value is kept in read-only memory.  Thus you should avoid storing into constants if you are developing read-only memory applications.

### 2.7.4	Colon Definitions

The defining word : has already been discussed by implication in Section 1.2 and numerous examples have appeared in other sections.  The purpose of this passage is to describe the use and behavior of this important defining word.

The basic form of a : definition is:

: name   content ;

Here the : constructs a dictionary entry for the word name.  content represents a list of previously defined words that will be executed in sequence whenever name is invoked.  The ; terminates the definition.

Each of the words : and ; has two types of behavior:  one type at compile time and another at execution time.

At compile time, : constructs the dictionary entry using CREATE and begins compiling.  In addition, : resets the search vocabulary in CONTEXT to the default in CURRENT, and sets the smudge bit so that the word will not normally compile a reference to itself.  The word RECURSE is used where the definition of a word must call itself.

The code field address of the new definition called name is the parameter field address of the run-time code shared by all : definitions.  This code starts the address interpreter executing the words whose addresses form the body of the definition, by pushing the current value of the interpreter pointer I onto the return stack and setting I to the address of the parameter field of the word, which is obtained from the system pointer W.

The ; ends compilation and compiles the address of the code field address of the run-time code for ; (the word EXIT).  This code pops the address on top of the return stack into the interpreter pointer I.  The effect is to return to the calling environment.

Most of the words that make up the content of the definition are not executed during compilation; instead their absolute addresses are compiled in the form given in Fig. 1.6.  The exception to this procedure are the words which are compiler directives or literals.  These generally have both compile time and run-time behaviors, just as : and ; do.

Note:  Every colon definition requires a minimum of three components:  a colon, a name, and a semicolon.  Such a minimum definition will execute properly, but will do no work.  It does have useful purposes; for example as described in FORGET to mark a location in the dictionary as the beginning of an overlay area.

REFERENCES

Address Interpreter, Section 1.1.6
Compiler Directives, Section 2.8.8
EXIT, Section 2.4.6
Overlays, Section 3.3.4
Program Structures, Section 2.4

### 2.7.5	Code Definitions

The form of a CODE definition is:

CODE name   {assembler instructions}   {code-ending}

The word CODE performs the following functions at assembly time:

1.	Constructs a standard dictionary entry for name using CREATE.

2.	Sets the code field address of the word to point to the word’s parameter field.

3.	Selects the ASSEMBLER vocabulary.

With CODE there is nothing analogous to a compiler; assembler words are executed directly, with the effect of assembling machine instructions in the parameter field of the word being defined.  When high-level Forth words are encountered, they are executed directly as well—thus, words such as SWAP and DUP manipulate the stack during assembly.  Macros can be defined as : definitions containing assembler words.  Macros must be defined in the ASSEMBLER vocabulary.

The basic principles of polyFORTH assemblers are covered in Section 6.0.  Assembler mnemonics, addressing modes, and conventions are covered in the CPU Supplement for each CPU on which polyFORTH is implemented.

The most common code ending is NEXT, which is a return to the address interpreter.  All other code endings also go to NEXT after performing additional functions.  Both the form and exact list of code endings must be taken from the CPU Supplement for your CPU; a typical list might include these:

Code Ending	Action

NEXT	Returns to the address interpreter.

WAIT	Forces the current task to enter the multitasking loop pending an interrupt.

' PAUSE 2+ JMP	Awakens the task and enters the multitasking loop.  The task regains control on its next turn.  Commonly used in polling cycles. The 2+ may be smaller or greater depending on the CPU.

' PAUSE JMP	Same as above, but I is first decremented by a cell, so that on return from PAUSE, the current code definition will be re-executed, from the beginning.  This is useful for polling.

' EXIT JMP	Leaves the high-level word which called the code word containing this phrase.

REFERENCES

Assembler Code Endings, Section 6.2
NEXT, Section 1.1.6
WAIT, Section 4.2

### 2.7.6	Custom Defining Words

One of the most powerful capabilities in Forth is the ability to define words that are themselves defining words.  Thus the programmer may create either new data types with characteristics peculiar to the application or new generic types of words.

In creating a custom defining word, the programmer must specify two separate behaviors:

1.	The compile-time behavior of the defining word (the creation of the dictionary entry, compiling of parameters, etc.).

2.	The run-time behavior (what action members of the new class of words perform when invoked).

In the cases discussed in the next two sections, compile-time behavior is described in high-level Forth. Several methods are available for specifying run-time behavior; these also will be covered below.

#### 2.7.6.1	Basic Principles of Defining Words

There are two ways to create new defining words in Forth.  They differ in that, in the one case (using DOES>) the run-time behavior is described in high-level Forth, and in the other (using ;CODE), the run-time behavior is described in assembler code.  The basic principles are the same and will be covered in this section.

The Forth term “defining word” means a word which will create a new dictionary entry when executed.  All words defined by the same defining word share a common characteristic compile-time and run-time behavior.  For example, VARIABLE is a defining word; all words defined by VARIABLE share two common characteristics:

1.	Each has one cell allotted in which a value may be stored.  These bytes may (in a resident system) be initialized to zero.

2.	When executed, each of these words will push onto the stack the address of this one-cell reserved area.

On the other hand, all words defined by CONSTANT have two different characteristic behaviors:

1.	Each has compiled with it a single-precision value which was on the stack when CONSTANT was executed.

2.	When a word defined by CONSTANT is executed, it fetches its value and pushes the value on the stack.

In each of these examples, Behavior 1 relates to the physical construction of the word, which is determined when the word is compiled:  space allotted, values compiled, etc.  Behavior 2 describes what all defined words of a certain type do when they are executed.  All defining words must have some compile-time behavior (1) and some run-time behavior (2).  The general form of the definition of defining word is:

: {name}   {compile-time behavior}   {transition word}
   {run-time behavior} ;

The “transition word” marks the end of the specification of compile-time behavior and begins the specification of the run-time behavior.  There are two “transition words”:

Word	Description

;CODE	Begins run-time behavior described in code.

DOES>	Begins run-time behavior described in high-level Forth.

The exact behavior of these words is discussed in the following sections.  The description of compile-time behavior is the same regardless of which transition word is used.  In fact, if you change the transition word and run-time behavior from DOES> plus high-level to ;CODE plus equivalent code, no change to the compile-time behavior is necessary.

The compile-time portion of a defining word must contain a previously defined defining word to create the dictionary entry.  If one or more parameters are to be compiled or if space for variable data is to be allocated, it is convenient to use a defining word which takes care of that for you.  The most common defining words are:

Word	Function

CREATE	Used when there are no compile-time parameters.

CONSTANT	Used when there is to be one compile-time parameter.

2CONSTANT	Used when there are to be two compile-time parameters.

VARIABLE	Used when one cell of storage is to be allocated.

2VARIABLE	Used when two cells of storage are to be allocated.

CVARIABLE	Used when one byte of storage area is to be allotted.

Every defining word must allow space for some piece of data or code belonging to each member of the new class of words.  For example, when a variable is defined, space is allotted for its parameter field.  If more space is required than that allotted by a standard word (VARIABLE or 2VARIABLE, for instance), the standard form is to use CREATE followed by ALLOT.  If the application is to be target compiled for ROM, the combination of VARIABLE or CVARIABLE and ALLOT must be used.

When used, a defining word may be followed by any number of words such as , (which compiles a single-precision value) or C, (which compiles an 8-bit value).

REFERENCES

, and C,, Section 2.8.2
;CODE, Section 2.7.6.2
ALLOT, Section 2.8.1
CONSTANT, Section 2.7.3
CREATE, Section 2.7.1
Defining Words, Starting FORTH, Chapter 11
DOES>, Section 2.7.6.3
Target Compiling Defining Words, Sections 7.5, 7.8
VARIABLE, Section 2.7.2

#### 2.7.6.2	Defining Code Defining Words

The use of ;CODE allows the user to specify the run-time behavior of a new class of words in assembler code.  Every word in the new class of words will share the same piece of code:  the code that defines the word’s run-time behavior.

The form of a ;CODE definition is as follows:

: name   {compile-time words} ;CODE
	   {run-time code} code-ending

Here name is the word that will be used to create new definitions of this class.  The code address of all such words will be the address of the code that follows ;CODE.

An example of a defining word using ;CODE on an 8086 might be:

: VARIABLE   CREATE  2 ALLOT
   ;CODE  W INC  W INC  W PUSH  NEXT

The part of VARIABLE which creates a new dictionary entry is the phrase:

CREATE  2 ALLOT

CREATE compiles the head of a new dictionary entry, and then 2 ALLOT makes space for the data in the parameter field.  ;CODE stops the colon compilation of VARIABLE, then compiles the address of a high-level word which will reset the code-field address of the word defined by CREATE, so that the code-field address points at the byte assembled by the first W INC.

The run-time code for defining words (which is shared between all words of the same type) must find the unique data in each word’s parameter field.  The first cell of the new word’s parameter field immediately follows its code field.  That is, the code field address precedes and is adjacent to the parameter field (see Fig. 2.7).  For example, when the address interpreter executes a VARIABLE, some code in NEXT sets W to point at the code field address of that particular VARIABLE (not VARIABLEs in general, but a particular VARIABLE).  The code field address of the VARIABLE points to the first byte of the shared run-time code for all VARIABLEs.  The shared code uses the value in W, which points to the byte following the code field address of a particular VARIABLE, to find that particular VARIABLE’s parameter field.

Fig. 2.7

If you wish to examine your system’s definition of VARIABLE, please note that the run-time code is defined in the nucleus, near the beginning of the system source, while the compile-time definition is back in the defining words.

Examine several system defining words before writing your own using ;CODE. The compile-time behavior of ;CODE is to compile a word called ;code which is the last word to execute in the defining word’s compile-time behavior.  The word ;code stores the address of the cell following ;code’s cell (in the defining word) into the most recent definition’s code field address.  The most recent definition will always be the one created by the defining word containing ;code.  Finally ;CODE’s compile time behavior turns off the : compiler and enters the assembler vocabulary.

REFERENCES

Address Interpreter, Section 1.1.6
ALLOT, Section 2.8.1
Assembler Conventions, Section 6.4
CONSTANT, Section 2.7.3
CREATE, Section 2.7.1
DOES>, Section 2.7.6.3

#### 2.7.6.3	Defining High-level Defining Words

New defining words whose run-time behavior is specified in high-level polyFORTH may be created by using a technique similar to that used for ;CODE.  For these definitions, the word DOES> terminates the compile-time portion of the definition and introduces the run-time portion.  The form of a DOES> definition is:

: name   {compile-time words} DOES> {run-time words} ;

As with ;CODE above, name is a word which defines a new definition of this class. In this case, however, the run-time behavior of this class of words is described at high level.

At run time, the address of the parameter field of the word is pushed onto the stack before the run-time words are entered.  This provides easy access to the parameter field.

The standard example of a DOES> definition is MSG, which types short character sequences:

: MSG   CREATE DOES> COUNT TYPE ;

Here is an example of how MSG is used:

HEX  MSG (CR)   2 C, 0D C, 0A C, DECIMAL

The defined word (CR) shares the DOES> definition with all other MSG words, but puts out its own unique character string, using the shared definition.

The values that make up the string are stored in the parameter field of a word that is defined by MSG.  At execution time the address of the string is on the stack to serve as the parameter for COUNT, which returns the length of the string and its byte address as the arguments for TYPE.  A good example of a DOES> word with more elaborate behavior occurs in the data base support system; if you have this option, refer to the definition of FILE.  Other examples of DOES> words may be found in the high-level definition of your assembler.

DOES> works in the following way:

1.	When the : compiler sees a DOES>, the DOES> is executed.  The compile-time behavior of DOES> is to compile the address of a word called ;code (see Fig. 2.8).  The word ;code resets the code field address of the latest definition (the one constructed by the defining word containing DOES>) to point to the cell following the compiled address of ;code.

2.	After the address of ;code, DOES> compiles a subroutine call to the run-time code for DOES>.  The compiler then proceeds to finish compiling addresses in the new defining word.  The use of a subroutine call in the defining word is hardware dependent.  However, all implementations of DOES> compile something in the defining word which will allow the run-time code for DOES> to find the defining word’s high level code without losing the defined word’s parameter field address.  When the new defining word is executed, its last step is to change the code field address of the entry it creates to point to the jump to subroutine created by DOES> in the defining word.

3.	When one of the defined words created by the new defining word is executed, the address interpreter jumps to the subroutine call in the defined word’s defining word.  Then the subroutine call saves the address of the cell following itself in some CPU-dependent way and jumps to the run-time code for DOES>.  The run-time code for DOES> uses the address from the subroutine linkage to find the high-level run-time code of the defining word.  The run-time code for DOES> also pushes the unchanged value of W onto the parameter stack to form the address of the defined word’s parameter field.

Fig. 2.8

REFERENCES

, and C,, Section 2.8.2
;CODE, Section 2.7.6.2
CONSTANT, Section 2.7.3
CREATE, Section 2.7.1
MSG, Sections 2.3.3, 3.7.3
TYPE, Sections 2.3.6.4, 3.7.3

## 2.8	Compiling Words and Literals

A compiling word stores addresses or values into the dictionary and allots space for definitions and data.

A literal is a number that is compiled directly into a definition or in some other unnamed form.  Covered in this section are several Forth words for compiling literals, including LITERAL and ['].

### 2.8.1	ALLOTing Space in the Dictionary

The resident version of ALLOT reserves a specified number of bytes in the dictionary by adding to the dictionary pointer in H.  The dictionary grows from low memory toward the “top” of the downward growing parameter stack.  ALLOT ensures that at least 250 bytes remain between HERE (the next empty dictionary byte) and 'S.  (The actual number of bytes assured by ALLOT is system-specific, but is intended to be sufficient to support all editor operations in the remaining space.)  If not enough space remains, ALLOT aborts the compilation by issuing the error message “Dictionary Full.”  If the minimum number of bytes are available, ALLOT adds the argument on the stack to the address of the next free dictionary byte in the user variable H.  This addition prevents other compiling words from compiling into the portion of memory reserved by the ALLOT.

An example of ALLOT’s use to create a 200-byte array is:

CREATE ARRAY 200 ALLOT

The Target Compiler’s version of ALLOT differs from the resident version in that it allots space in the target system’s RAM, rather than the target  dictionary (which is presumed to be in ROM).

REFERENCES

ALLOTting Target RAM, Section 7.3.1
CREATE, Arrays, Section 2.7.1

### 2.8.2	Use of , and C, to Compile Values

The word , (pronounced “comma”) stores the item at the top of the stack into the next available dictionary location (given by H) and increments H by two bytes (four bytes on 32-bit machines).

The most common use of , is to set values into a table whose starting address is defined by using CREATE.  This word creates a definition that behaves in a manner identical to VARIABLE, in that when the new word is executed, its address is returned.  CREATE differs from VARIABLE only in that it does not allot any space.  Consider this example:

CREATE TENS  1 , 10 , 100 , 1000 , 10000 ,

This establishes a table, the starting address of which is given by TENS, which contains powers of ten from zero through four.  Indexing this table by a power of ten will give the appropriate value.  A possible use might be:

: 10X ( n n - n)   2*  TENS + @ * ;

Given a single-precision number on the stack with a power of ten on top, 10X will multiply the number by the appropriate power of ten to yield the product.

If the table will be referred to in only one instance, you may avoid the memory overhead of a CREATE definition by using HERE to provide the location.  For example, a one-usage table of twos would look like:

HERE   2 ,  4 ,  8 ,  16 ,  32 ,  64 ,
: 10X   ( n n - n)   2*  LITERAL + @ * ;

The word HERE leaves on the stack the current dictionary pointer where the storage of data values starts.  The word LITERAL in a definition picks up a number from the stack (the address in this example) and compiles it.  When this version of 10X is executed, the address is pushed on the stack in the same way as before.

When a single byte of data is sufficient, C, performs for bytes the same function that , performs for cells.  On processors that do not tolerate odd byte addresses (e.g., LSI-11), uses of C, must be for strings of even byte length.  Note that C,’s name implies compiling a character-sized value in the dictionary.

REFERENCES

CODE, Sections 2.7.5, 6.1
CONSTANT, Section 2.7.3
CREATE, Section 2.7.1
LITERAL, Section 2.8.5
MSG, Section 2.3.3

### 2.8.3	The polyFORTH Compiler:  ] and [

The polyFORTH compiler uses ALLOT, , (“comma”), and occasionally C, to lay down addresses in the dictionary for the address interpreter to interpret.  The compiler finds the addresses by using -' (“dash-tick”) repetitively on the input stream.  The address compiled for each word is the address of the word’s code field address.

The compiler must handle two special cases besides address compilation.  The first case occurs when numbers are included in a high-level definition.  The compiler handles numbers much like the standard Forth text interpreter. When a dictionary search fails, the compiler attempts to convert the ASCII string using NUMBER.  When conversion succeeds, the address of a special word called “cell” is compiled, followed by the number in binary.  At run time, “cell” will push the binary value onto the stack. On machines that tolerate odd addresses, when the number is less than 256 the compiler saves space by compiling a special word called byte (instead of cell) followed by the least significant eight bits of the number.

When the numeric conversion fails, the conversion word aborts, printing the text not understood followed by a question mark.

The second special case occurs when a word needs to be executed at compile time by the compiler.  These words are called “compiler directives.”  The words IF, DO, and UNTIL are examples of compiler directives.  After the word is found in the dictionary, the compiler checks the precedence bit in the header of the dictionary entry.  If the precedence bit is set, the word is executed, rather than compiled by the compiler.  If the precedence bit is reset, the address of the code field address of the word is compiled. The precedence bit of a word may be set by placing the word IMMEDIATE after the word’s definition.

The most common use of [ and ] is to leave compile-mode temporarily to perform some operation at compile-time.  For example, suppose you wish to refer to an ASCII code in hex, in a definition containing numbers most naturally thought of in decimal:

: GAP ( n)   10 0 DO  [ HEX ] 0A
   EMIT  LOOP ;

Since the words that control BASE aren’t IMMEDIATE, it is necessary to leave compile-mode to execute HEX to permit compilation of the hex code following.  The Forth word which ends compilation is [ (pronounced “left-bracket”).  The word [ is an IMMEDIATE word which performs an EXIT to leave the compiler and resume interpretation.  This word is used in the definition of ;.

Sometimes when Forth high-level code is necessary, but a dictionary header is not, (as in some power-up code) the word ] is used rather than :.  Similarly, where high-level polyFORTH is necessary but no address for EXIT need be compiled on the end of the definition, (as when compiling endless loops) [ is used instead of ;.  In polyFORTH the phrases [ and 
; RECOVER are equivalent, except that many people find ; RECOVER more readable.

Consider, for example, the response to a “restart” key on an Intel 8086:

HERE ] ." Break"  CR ABORT [
ASSEMBLER BEGIN   SWAP # I MOV   STI
   NEXT   0A INTERRUPT

The HERE pushes the address of the next available byte in the dictionary onto the stack.  ] enters compile mode, and compiles the ." message followed by the references to CR and ABORT.  Note that ABORT will abort the operation of the terminal task which initiated the interrupt (which, on the IBM-PC from which this example was taken must be OPERATOR due to the hardware configuration) and return control to the keyboard.  Immediately following the address of ABORT is the assembler MOV instruction, followed by the rest of the code through NEXT.  The BEGIN pushed the address of the MOV on the stack; this address and 0A (the interrupt vector) are the arguments to INTERRUPT, which stores the address in the interrupt vector.

When the user presses the “Break” key, the interrupt causes a branch through the vector to the MOV instruction, which will set Forth’s interpreter pointer I to the beginning of the high-level phrase starting with .".  The NEXT at the end of the code will start execution of the high-level phrase, terminating with the ABORT.  Since the phrase is only entered in this way (never called from another high-level word, for example) there is no need to begin it with : name and since it terminates in ABORT there is no need for an EXIT (compiled by ;) at the end.

Fig. 2.9

The Forth word which begins compilation is ] (pronounced “right-bracket”).  This word contains the definition of the compiler, and is used in the definition of :.

Please note that it is rarely the case in a multitasking environment that you can know which task is controlling the CPU at the time an interrupt occurs.  The technique used in this example is therefore appropriate only in a narrow range of applications.

REFERENCES

ABORT, Section 2.4.7
Address Interpreter, Section 1.1.6
Colon Definitions, Section 2.7.4 
Compiler Directives, Section 2.8.8
Dictionary Searches, Section 2.6.1
Input Number Conversion, Section 2.5.2
Interrupts, Section 6.11

### 2.8.4	Use of Literals in : Definitions

When the polyFORTH compiler encounters a number in a : definition the number is converted to binary and compiled as a literal.  The compiled form of a literal in a : definition occupies two cells (one cell on 32-bit machines).  The first contains the address of a routine which, when executed, will push the second cell onto the stack.  When Forth is compiling a definition and a number is encountered, this form is automatically compiled.  There are other ways in which a literal in a definition may be generated (which will be discussed in the following section), but this is the most common situation.

On computers that tolerate odd byte addresses, a literal less than 256 will be compiled as a byte.  A byte literal occupies three bytes (the address of the routine plus the actual byte). The compiler automatically determines which form to use.

Since a literal occupies two cells on 16-bit machines, a number that will be used frequently (more than six times) should be defined as a CONSTANT to save space.  There is not much difference between the time required to execute a CONSTANT and a literal.  The space saving is even more significant in the case of 32-bit literals versus the use of 2CONSTANT.

REFERENCES

Explicit Literals, Section 2.8.5
Literal Addresses, Section 2.8.6

### 2.8.5	Explicit Literals

The word LITERAL compiles into a definition the number that was placed on the stack at  compile time.  When the definition is executed, that number will be pushed onto the stack.  The compiled result of LITERAL is identical to that of a literal number, described in the previous section.  LITERAL is useful for compiling a reference to an address or computed number that may be computed at compile time.

Consider the table TENS in the example in Section 2.8.2.  If TENS is to be used only by the definition 10*, a more compact form would be:

HERE  1 ,  10 ,  100 ,  1000 ,  10000 ,
: 10* ( n n - n)   2*  LITERAL + @ * ;

HERE leaves a value on the stack which LITERAL compiles into the definition of 10*.

Both the usage and result of 10* are identical to those of 10X in Section 2.8.3.  Here, however, instead of using CREATE to provide a definition which will return the address of the beginning of the table, HERE is used to supply it at compile time; LITERAL compiles this address as a literal.  The net saving is six bytes of memory.

Another technique is to compile the results of complex calculations that only need to be performed once.  As a trivial example, disk status information is stored in the location DISK 2+.  A word to retrieve that information might be:

: STATE   [ DISK 2+ ]  LITERAL  @ ;

The [ stops : compilation, and ] restarts : compilation.  During this hiatus, the words DISK 2+ are executed, leaving on the stack the address of the status word, which, after compilation is resumed, is compiled into the definition by LITERAL.  Time savings can be large if the calculations are in an inner loop.  (Note:  On 32-bit machines, the words DISK 2+ would be DISK 4+.)

REFERENCES

[ and ], Section 2.8.3
Compilation of Literals, Section 2.8.4

### 2.8.6	Use of ['] to Compile Literal Addresses

The word ['] is used inside a definition to compile as a literal the address of the parameter field of the word that follows it at compile time.  The most common use of ['] is to obtain the address of either a CONSTANT or a 2CONSTANT (on systems that have the 32-bit option or on 32-bit machines).  Consider the following example:

0 500 2CONSTANT LIMITS
: RANGE   ['] LIMITS 2! ;

Given these definitions, the phrase:

0 2000 RANGE

resets the values of LIMITS to 0 and 2000.  The address of the beginning of the double-precision parameter field for LIMITS is compiled as a literal in RANGE and pushed onto the stack when RANGE is executed to provide the address for 2!.

### 2.8.7	Compiling Strings

polyFORTH provides two methods for compiling strings.  The most generally useful word is " (pronounced “quote”).  This word is used only in colon definitions.  It compiles a string, which follows in the dictionary and is terminated with another quotation mark.  When the word containing this string is executed, the address of the beginning of the string is pushed on the stack.  The string is compiled with its count in the first byte (as is typical with strings in polyFORTH).  Therefore, it will frequently be useful to use the word COUNT to fetch the length and address of the actual first byte of the string.

For example, lets consider a word that compares a data string whose address and length are given to a specific string, and returns ‘true’ if it matches:

: ?NO ( a n - t)   " no" 1+ -TEXT ;

When ?NO is executed, " will push the address of the compiled string on the stack.  It is incremented to the beginning of the actual text, leaving the appropriate arguments for -TEXT (which performs the comparison).*

Here is a word which will search for the compiled string in a longer string whose address and count are given:

: ?DUCK ( a n - t)   " duck"
   COUNT -MATCH  SWAP DROP ;

The phrase SWAP DROP discards the address of the first non-matching character in the first string, which is returned by -MATCH.

In either of the examples above you may need to allow the test string to contain an arbitrary mixture of upper and lower case characters.  If so, you should either set or clear the appropriate bit in each byte of the test string to standardize on all upper or all lower case before making your comparison.

Often you will find it more convenient to specify another run-time behavior than returning just the address.  You may always want the results of COUNT, for example, or perform some direct action such as typing the string.  Or, perhaps you want a terminating character other than a quotation mark.  The word STRING will compile a string with a specified delimiter from the input stream to the dictionary.  It has no run-time behavior.  STRING is used by " as well as by .", ABORT" and other similar words.

STRING expects the ASCII code for the delimiter on the stack.  It will compile the string terminated by that delimiter with the length of the string in the first byte.  An example of STRING’s use is the word:

: MESSAGE"   >IN @  CREATE  >IN !
   34 STRING  DOES> COUNT TYPE ;

MESSAGE" creates messages which are named by their first word, for example:

MESSAGE" HELLO THIS IS FORTH! " ok

Executing HELLO will cause the computer to print:

HELLO THIS IS FORTH! ok

The way MESSAGE works is as follows.  The phrase:

>IN @ CREATE  >IN !

Creates a definition whose name is the first word (delimited by spaces) in the string following MESSAGE".  It saves and restores the text interpreter pointer >IN so that the first word will be incorporated in the string as well as being the name of the definition.  The phrase 34 STRING then compiles the string to a terminating " (22H).  The resulting definition is shown in Fig. 2.10 (Section 2.8.9).  The code field address of the new definition points to the code for the run-time behavior provided by DOES> in the definition of  MESSAGE".  When the word HELLO is executed, the 21-byte string in its parameter field will be typed out.  Note that this method will handle strings up to 127 bytes long.

REFERENCES

." and ABORT", Section 2.3.6.5
Defining Words, Section 2.7
String Comparisons, Section 2.3.5

### 2.8.8	Compiler Directives

A “compiler directive” in Forth is a word that is executed at compile time, i.e., during a : compilation.  Many such words exist:  DO; LOOP and +LOOP; BEGIN and UNTIL; IF, ELSE and THEN; literals; and others.  It is rare that a user needs to add compiler directives; it is not difficult, but requires mastery of the use of IMMEDIATE and the use of COMPILE.

COMPILE is frequently used to lay down (in the dictionary) the address of the run-time behavior of a compiler directive.  When COMPILE is executed by the address interpreter, COMPILE compiles the cell immediately following COMPILE’s address cell, and then adds two (or four) to the interpretive pointer so that the compiled address will not be executed. The word IMMEDIATE is used just after a definition.  Its effect is to make the word just defined a compiler directive by setting its precedence bit (usually the high-order bit in the count field).  This bit signals the compiler that the word is to be executed at compile time (when all non-immediate words are being compiled).

Consider the definition of BEGIN:

: BEGIN   HERE ;  IMMEDIATE

The actual behavior of BEGIN is identical to HERE.  The crucial difference is that when HERE appears in a definition, its address is compiled; it will push the value of H onto the stack when the word that contains HERE is executed.  BEGIN, on the other hand, compiles nothing; it pushes H onto the stack at compile time, to serve as the address that UNTIL needs to compile a conditional return to that location.

Any kind of word may be IMMEDIATE.  Sometimes it is useful to make a VOCABULARY IMMEDIATE, so that it may be used to select words inside a definition.

An example of the use of IMMEDIATE vocabulary words is the use of FORTH and HOST in the following definition from Block 51 of the target compiler:

: ;CODE   FORTH COMPILE  HOST ;code
   R> DROP  ASSEMBLER  SMUDGE ; IMMEDIATE

Note that ASSEMBLER is not an IMMEDIATE vocabulary word, and that the IMMEDIATE following ;CODE’s definition will make ;CODE an IMMEDIATE word.

The definition of IF shown in Fig. 2.10 illustrates the definition of a compiler directive which uses COMPILE to lay down a run-time behavior into a : definition.  Note that the run-time definition is a separate word.

REFERENCES

Colon Definitions, Section 2.7.4 
COMPILE, Section 2.8.9
DO, LOOP, Program Structure Words, Sections 2.4,  6.8
Literals, Section 2.8.5
The polyFORTH Compiler, Section 2.8.3
Use of BEGIN, Section 2.4.1
Vocabularies, Section 3.4

### 2.8.9	COMPILE and [COMPILE]

Some compiler directives have only compile-time behavior (such as BEGIN).  Other directives need to perform some actions at compile time and other actions at runtime.  For example, at compile time DO must mark the position to which LOOP, /LOOP or +LOOP will return; at run-time it must push the index and limit for the loop onto the return stack.

Fig. 2.10

The way these functions are managed is to define (usually with CODE) the run-time activity as a separate word and then to have the compile-time definition, which is IMMEDIATE, compile the address of the run-time code (in addition to its other activities).  Fig. 2.10 shows a compiled byte-machine dictionary entry.

The word used to compile the address of the run-time code is COMPILE.  This word is very similar to ['], except that whereas ['] compiles as a literal the parameter field address of the word that follows (so that the address will be pushed onto the stack at run time), COMPILE lays down a pointer to the code field address, so that the word can be executed by the address interpreter.

A very similar word, [COMPILE], is available to compile the addresses of words which are IMMEDIATE (such as compiler directives.)

When executed, this definition does the following:

1.	The phrase COMPILE if compiles the address of the previously defined run-time code if.

2.	HERE pushes an address on the stack.  This address will be used subsequently by THEN or ELSE to set the address for the conditional branch run-time word if.

3.	Finally a zero is compiled to leave space for the conditional branch address.

REFERENCES

['], Section 2.8.6
[ and ], Section 2.8.3
Compiler Directives, Section 2.8.8

## 2.9	FORTH-83 Standard Compatibility

Efforts to standardize Forth have been under way since the late 1970s.  An organization called the Forth Standards Team (or FST) published a widely recognized standard in 1980, called FORTH-79.  In 1983 this was extensively revised, and the result, called FORTH-83, is now the most widely accepted standard.

FORTH, Inc. has been a participant in these standardization efforts since the beginning.  As shipped, polyFORTH ISD-4 is extremely close to the FORTH-83 standard, however, there are several significant incompatibilities:

1.	polyFORTH adheres to a consistent usage wherein all addresses of Forth words passed on the stack are parameter field addresses.  In FORTH-83, some functions return and use parameter field addresses, while others (', ['], and EXECUTE) return and use code addresses.

2.	Finite loop structures and related words behave differently in FORTH-83 in order to provide a full 65K range in LOOP and +LOOP on 16-bit machines.  Specifically, inside the loop the return stack parameters are modified, so that I and J references compute the index values (rather than simply copy them).  There is no equivalent of I'.  Moreover, LEAVE exits the loop at once (rather than adjusting loop parameters so that the loop will terminate on this cycle).  The FORTH-83 versions of these words compile full branch addresses instead of byte offsets, costing an extra byte but preventing problems for long definitions.  Also, DO compiles the loop exit address for use by LEAVE.  In summary, the FORTH-83 versions are slightly slower and longer, but more general.

3.	In FORTH-83, integer division is floored, meaning that the division operators round, rather than truncate, when there is a remainder.  This provides a continuous function through zero, at some cost in performance.  The actual performance difference depends upon the CPU.

# 3.0	SYSTEM FUNCTIONS

This section describes words used to load, organize, and manage polyFORTH applications, as well as the standard system devices (disk, terminal, and clock).

## 3.1	Vectored Routines

Since there are several system functions users often wish to change (because of changing hardware or application requirements) without recompiling the nucleus, pF/x provides “execution vectors” containing the addresses of current versions of these functions.  There are two groups of vectored routines:  those controlling system-wide functions, and those controlling terminal-dependent functions (i.e., those whose behavior differs between different kinds of CRT or between keyboard/display and printer).  For each vectored function, there are at least three Forth words:  the function itself (which performs a @EXECUTE on the vector), the vector itself, and at least one routine to be executed.

The following table summarizes the vectored routines controlled on a system-wide basis:

Function	Vector	Primitive	Description

BLOCK	'BLOCK	(BLOCK)	Returns the address of a specified block.  See references for details.

BUFFER	'BUFFER	(BUFFER)	Returns the address of an available buffer, identified as containing a specified block.  See the references.

CREATE	'CREATE	(CREATE)	Creates a dictionary entry. See the references for additional versions.

Function	Vector	Primitive	Description

NUMBER	'NUMBER	(NUMBER)	Converts a string at a given address to binary on the stack.  See the references.

The following table summarizes the routines vectored through user variables for differing task-specific functions.  'IDLE is used by all tasks; the remainder control CRTs and printers and are relevant only for TERMINAL tasks.  See the section references below for further details.

Function	Vector	Primitive	Description

ABORT	'IDLE	QUIT	“Idle” behavior for a task—the routine it executes following an abort.

TYPE	'TYPE	(TYPE)	Types a string, given address and count.

EXPECT	'EXPECT	(EXPECT)	Accepts a string, given address and count.

CR	'CR	(CR)	Performs a new-line function.

PAGE	'PAGE	(PAGE)	Clears the screen or issues a form-feed on the printer.

TAB	'TAB	(TAB)	Positions a terminal’s cursor.

MARK	'MARK	(MARK)	Types highlighted text.

CLEAN	'CLEAN	(CLEAN)	Clears to end of line.

TYPE and EXPECT require some further discussion.  Both words expect an address and count on the stack, and both store these parameters in the user variables PTR and CTR respectively.  EXPECT negates its count so CTR starts negative and will count up to zero.  EXPECT also sets the user variable SPAN to zero where it will be incremented, to leave a record of how many characters were received.

You may write custom versions of vectored routines—for example, to handle a special kind of disk.  To substitute a user-written routine in a vector, follow the examples given for CREATE in the section referenced below.  In addition to those additional versions of CREATE, the system electives include error-handling versions of BLOCK and BUFFER (Block 48).

REFERENCES

BLOCK, Sections 3.2.2, 3.2.6, 3.2.8
BUFFER, Sections 3.2.3, 3.2.6, 3.2.8
CREATE, Section 2.7.1
EXPECT, Sections 3.7.1, 3.7.2
NUMBER, Section 2.6.2
Support of Special Terminal Functions, Section 3.7.5
TYPE, Sections 3.7.3, 3.7.4 
User Variables, Section 4.6
Vectored Execution, Section 2.4.8

## 3.2	The Disk Driver

This section discusses the words used to access and manage disk blocks and block buffers in Forth.

### 3.2.1	Overview of polyFORTH Disk Access

The polyFORTH disk access scheme is intended to be simple and to require a minimum of effort to use.  The polyFORTH disk driver makes data on disk directly accessible to other Forth words by placing disk data into a buffer in memory, returning on the stack the address of the buffer containing the requested data.  Forth routines thus access disk data using the same techniques as with other memory accesses.  Since disk data always appears to be in memory, this disk access scheme is a form of “virtual memory” for program source and data storage.  Basic principals of disk usage in Forth are discussed in Starting FORTH, Chapter 10.  If you are a newcomer to Forth, you should review this material before proceeding further.

Another consideration in the design of the polyFORTH disk driver is to make disk access as fast as possible.  For this reason, data is read from disk to memory or written from memory to disk only when necessary, because disk operations are very slow compared to memory operations.

The disk is partitioned into 1024-byte data areas called “blocks.”  This standard unit has proven over many years to be a useful increment of mass storage.  As a unit of source text, for example, it contains an amount of source which can be comfortably displayed on a CRT screen; as the basis for a data base system, it contains a useful number of records of typical sizes.

Each block is addressed by a block number.  On native polyFORTH systems the block number is a fixed function of the block’s physical position on the disk.  Absolute addressing of the disk both speeds the driver’s execution and simplifies the knowledge a programmer must have to use the disk.  Absolute addressing also eliminates most of the need for disk directories and indexes.

A program ensures a block is in memory in a “block-buffer” by executing the word BLOCK.  BLOCK uses a block number from the stack and returns the address of the first byte of that block in memory.  For example:

9 BLOCK U.

will return an address such as:

46844 ok

where 46844 is the address of the first byte of the buffer containing Block 9.*  If a block is already in memory, BLOCK will not re-read it from disk.  Although BLOCK uses a disk read to get data if it is not already in memory, BLOCK is not merely a read command.

If BLOCK must read a requested block from disk, it uses BUFFER to select a buffer to put it in.  BUFFER frees a block buffer, writing the block buffer’s previous data to disk if the data is marked as having been changed since the block was read into memory.

BUFFER takes a block number on the stack and returns the address of the first byte of an available block buffer, to which this block number has been assigned.  For example:

127 BUFFER U.

will get a block buffer, assign the block number 127 to the buffer, and then type out the address of the buffer’s first byte:

36084 ok

BUFFER will always select the Least Recently Used buffer.  Use of this “LRU” buffer management algorithm in polyFORTH can substantially improve throughput in a disk-intensive multi-user application.

Fig. 3.1

Although BUFFER may write a block if necessary, it will not read data from disk.  When BUFFER is called by BLOCK to assign a buffer, BLOCK will follow the selection of a buffer by actually reading the requested block from disk into the buffer.

The following example displays an array of 100 16-bit signed integers starting at the beginning of Block 1000, five numbers per line:

: SHOW   100 0 DO  I 5 MOD 0= IF CR 
   THEN 1000 BLOCK I 2* + ?  LOOP ;

The phrase I 2* + converts the loop counter from 16-bit cells to bytes, as internal addresses are always byte addresses, and adds the resulting byte offset to the address of the block buffer returned by BLOCK. The word ? fetches and types out the cell at that address.

BUFFER may be used directly in situations where no data from the disk need be saved.  Examples include initializing a region of disk to a default value such as zero or a high-speed data acquisition routine writing incoming values directly to disk from a memory array 1024 bytes at a time.

You may determine the number of block buffers in your system by typing:

NB ?	4 ok

As shipped, polyFORTH returns 4 (8 on 32-bit systems), the default number of block buffers.  The number of buffers may be changed easily.  Applications with several users using disk heavily may run slightly faster with more buffers.  Your CPU Supplement will give details on changing the size of the buffer pool.

The command UPDATE marks the data in a buffer as having been changed, so that it will be written to disk when the buffer must be used for another block.  UPDATE works on the most recently referenced buffer, so it must be used immediately after the operation that modifies the data.*

The following example uses BUFFER to clear a range of blocks to zero:

: ZEROS ( f l)   1+ SWAP DO  I BUFFER 
   1024 ERASE  UPDATE  LOOP ;

To take another example, assume that an application has defined A/D to read a value from an A/D converter.  To record up to 512 samples in Block 700, you could use:

: SAMPLES ( n)   512 MIN 0 DO  A/D
   700 BLOCK I 2* + ! UPDATE  LOOP ;

In this example, the phrase 512 MIN “clips” the specified number of samples at 512 (the next example will show how to record data in a range of blocks).  As in the example of SHOW above, the phrase I 2* converts the loop counter (in samples) to a byte offset added to the address of the start of the block, returned by BLOCK.  BUFFER cannot be used in this case, as we must preserve previous samples written in the block.

Because BLOCK maps disk into memory, “virtual memory” applications are simple.  The first step of a virtual memory application is to write a word transforming application addresses to physical addresses.  For a virtual byte array, such a definition is:

: VIRTUAL ( v - a)   1024 /MOD  250 +
   BLOCK + ;

The 1024 is the number of bytes per disk block and 250 is the block number where the virtual array starts.  See the reference below (virtual arrays) for a detailed explanation of this technique.

Fetch and store operations for this virtual memory scheme are defined as:

: V@ ( v - n)   VIRTUAL C@ ;

: V! ( n v)   VIRTUAL C!  UPDATE ;

BLOCK does not normally perform any error checking or retries at the primitive level, because an appropriate error response is fundamentally application-dependent.  Some applications processing critical data in non-real-time (e.g., accounting applications) should attempt retries,* and if these fail, stop with an error message identifying bad data.  Other applications running continuously at a constant sampling rate (e.g., data loggers) cannot afford to wait, and should simply log errors.

The low-level routines save error data obtained from the disk controller in the cell at DISK 2+ (on 32-bit machines at DISK 4+) so you may add appropriate error handling code in your application.  By making error response completely user-programmable, polyFORTH makes it possible to ensure an appropriate response in any application environment.  A standard error handling facility is provided in Block 48.  This may serve as an example for application use.  This facility retries failed disk operations a fixed number of times before aborting with an error message.

Native versions of polyFORTH do not put source blocks in named files.  The block structure suffices for polyFORTH source and for simple disk-based applications.  A data base support option supporting named files consisting of ranges of blocks is included with polyFORTH.  This provides programming tools to define and maintain more complex data bases on disk.

Often the hardware provides several disk drives with removable media.  In this situation, it is often convenient for a user to work as though the disk’s first block was block number zero, even though the disk is on Drive 2, for example.  By convention, therefore, when a block number is processed by BLOCK or BUFFER, an offset is added to generate the physical block number.  The offset is set by the word PART, which uses a disk drive number (disks are numbered starting with Drive 0, as in most hardware).  For example, the phrase:

1 PART

sets the user’s value for the offset so when the user types 9 LIST, the block listed will be from Drive 1.

Certain variables and arrays exist in memory to allow the system to manage disk access.  These are available to be used by the programmer.  Variable names for 32-bit systems are given in parentheses.

Word	Description

OFFSET	A user variable containing the physical block number of a user’s first block number.  Set by PART and UNIT.

NB	A system variable containing the system’s number of block buffers.  Typical values in NB range from two to eight.  Multi-user systems tend to run faster with more block buffers, if there are more users than block buffers.

Word	Description

NB 2+	(NB 4+ on 32-bit systems.)  Contains the address of the first byte of the block buffer lowest in memory.  In polyFORTH, the traditional place for block buffers is in high memory, above application memory and all user partitions.

DISK	A facility variable used to ensure only one task at a time may have access to the block buffers.  DISK is set by GET and RELEASE in BLOCK and BUFFER. DISK is also used by the disk interrupt routine to find the user area of the task using the disk.

DISK 2+	(DISK 4+ on 32-bit systems.)  Contains hardware status information, and is often only a copy of the disk controller’s status register(s) after the last read or write operation.  On many systems, DISK 2+ will be zero if no errors occurred.  Some disk controllers return several bytes of status information.  Check your CPU Supplement for the handling of your disk.

'BUFFER 2+	('BUFFER 4+ on 32-bit systems.)  Contains the absolute block number most recently written to disk, set by BUFFER.

PREV	Contains the address of the descriptor of the buffer most recently used by BLOCK or BUFFER.  This value is used by UPDATE to mark the buffer descriptor to indicate a block buffer will need to be written to disk, as well as to start the search for a block in memory or a buffer to be used.

REFERENCES

?UPDATED, Section 3.2.3
32-Bit Block Number Conventions, Section 3.2.7
Data Base Support, Section 8.0
Facility Variables, Section 4.7
User Variables, Section 4.6
Virtual Arrays, Section 1.2.2

### 3.2.2	Using BLOCK for Disk Access

BLOCK is the basic disk access word in Forth.  BLOCK moves data between disk and memory, forming a “window” in memory, which shows the data on disk.  While BLOCK contains a read operation, BLOCK only reads from disk when the data is not available in memory.  Blocks are only written to disk when an UPDATEd block buffer must be overwritten to free a block buffer.

BLOCK expects a block number on the stack, and returns on the stack the address of the first byte of the data in memory.  For example:

65  250 BLOCK 3 + C! UPDATE

stores an “A” (decimal 65) in the fourth byte of Block 250.  UPDATE marks the block buffer containing Block 250 “updated” so that it will be written to disk before the buffer is re-used.  If the system goes down before the buffer is re-used, the data will not be saved to disk.  The word FLUSH writes all updated buffers to disk.  If no changes are made to the block buffer (when fetching data, for example) the UPDATE should be omitted.

The following is a detailed discussion of the mechanics of BLOCK.  Please refer to the Dijkstra diagram in Fig. 3.2 and to your system listing.

The word BLOCK performs all high-level functions and calls the hardware level routine (BLOCK).  BLOCK adds OFFSET to the block number on the stack, gains exclusive access to the block buffers by the phrase DISK GET, and then vectors execution through the variable 'BLOCK to the hardware-dependent routine (BLOCK).

The hardware-dependent portion of BLOCK is vectored through 'BLOCK so a new version for a different disk controller can be inserted while polyFORTH is running, by a phrase such as:

' <BLOCK> 'BLOCK !

where <BLOCK> is the name of the new version of (BLOCK).

(BLOCK) first uses the word ?ABSENT to determine whether the block is already available among the buffers.  If the block is already in memory, a superfluous disk read should not be performed.  Therefore, ?ABSENT scans PREV, checking the block numbers of all blocks in memory.  If the block is already in memory, ?ABSENT takes the place of (BLOCK).  If the block is in memory, ?ABSENT uses the block number on the stack, leaves the buffer address on the stack, marks the buffer as the most recently accessed (by moving the number of the block buffer into PREV), and then exits from (BLOCK) back into BLOCK, which then releases DISK.  If the block is not present, ?ABSENT is invisible:  it does not affect the stack, does not mark the buffer, and returns normally to (BLOCK).

Fig. 3.2

Dijkstra diagram of BLOCK.  The shaded area describes the behavior of (BLOCK), containing the hardware-dependent functions.  Custom definitions used with 'BLOCK should call (BLOCK) or its equivalent.

The next step of (BLOCK) is to vector execution through 'BUFFER to (BUFFER).  As with (BLOCK), (BUFFER) is hardware-dependent.

(BUFFER) finds an old block buffer, frees it (writing the data to disk if necessary), and leaves the buffer address of the freed block buffer on the stack.  See the next section for a detailed discussion of (BUFFER).

The next step of (BLOCK) is totally hardware-dependent:  it reads the desired block whose block number is on top of the stack into the buffer whose address is just below the top of the stack.  Note that this step is only performed when BLOCK already knows, via ?ABSENT, that the data must be read from disk, and is not already in memory.

Usually the disk addressing calculation word is shared by this read operation and the write operation in (BUFFER).  The block number and buffer addresses are left on the stack by the address calculation word.

The final step of (BLOCK) is the word ESTABLISH.  ESTABLISH makes the oldest buffer (at the end of the buffer chain) into the newest buffer (by storing the address of the buffer descriptor into PREV), and marks the new buffer with the block number of the second stack entry.  ESTABLISH uses a block number and buffer address, and leaves only the buffer address.  (BLOCK) then returns to BLOCK, which gives up control of the disk system by the phrase DISK RELEASE.

REFERENCES

(BUFFER), Section 3.2.3
Adding a Disk Driver, Section 3.2.8
Virtual Arrays, Sections 1.2.6, 3.2.1

### 3.2.3	Using BUFFER to Select a Block Buffer

The word BUFFER is called by BLOCK to obtain a buffer when a block must be read from disk.  It may also be called in any application needing “write-only” access to a block buffer.  Examples of the latter include initializing an entire block to some value (e.g., zeroes or spaces) or copying a block’s worth of data from a memory array.  BUFFER should not be used when writing individual bytes or cells into a buffer when there is input or output (and hence multitask access) between writes.

BUFFER expects a block number on the stack and returns the address of a buffer marked as containing that block.  The contents of the buffer are undefined.  BUFFER will write the selected buffer’s data to disk if necessary to free the buffer for this use.

Fig. 3.3

Dijkstra diagram of BUFFER.  The shaded area describes the behavior of (BUFFER), containing the hardware-dependent functions.  Custom definitions used with 'BUFFER should call (BUFFER) or its equivalent.

BUFFER is a shell routine vectoring execution through the system variable 'BUFFER to the hardware-dependent routine (BUFFER).  This vectoring enables new disk drivers to be installed while polyFORTH is running.  The hardware-dependent function performed in (BUFFER) is a disk write; disk reads are performed by a similarly vectored hardware-dependent portion of BLOCK.  This vectoring may also be used to modify other aspects of disk write behavior, such as adding an error checking function like the one in Block 48.

If the new version of (BUFFER) is called <BUFFER>, the installation of <BUFFER> would be achieved by the phrase:

' <BUFFER>  'BUFFER  !

The logical flow within BUFFER is diagrammed in Fig. 3.3.  The principal routines used by BUFFER are:

Word	Stack	Function

?UPDATED	( - a) or	Selects the Least Recently Used buffer, by an

( - a d)	algorithm given below, and checks its "update bit” to see whether it has been marked as changed. Returns the address of the selected buffer, now marked as “empty,” and the block number in case a write is to be performed.  ?UPDATED must be followed by hardware-dependent words to perform the write.  If a write is not required ?UPDATED will exit directly from the calling definition, which is normally (BUFFER).

ESTABLISH	( d a - a)	Moves the least recently used buffer, selected by ?UPDATED to the top of the LRU chain (i.e., making it the most recently referenced buffer), and marks it as containing the requested block.

Block numbers used internally by these words are 32 bits long, to allow for disk controllers handling more than 32,768 blocks; see the references below for a more detailed discussion of 32-bit block numbers.

The method used for selecting a block buffer is called the “LRU Buffer Manager” because it selects the Least Recently Used buffer to be over-written.  This attempts to minimize the number of physical disk accesses performed, assuming the least recently referenced block is the least likely to be needed again soon by any of the currently running tasks.  Use of this algorithm can significantly improve performance in a disk-intensive application.  A general statement of this algorithm is as follows:

1.	BLOCK always checks whether a block is in memory before requesting a buffer preparatory to performing a read operation.  The search for the desired block always starts with the most recently referenced buffer, assuming it is the most likely target, and proceeds through the buffers looking at successively older buffers.

2.	BUFFER will always select the least recently referenced buffer, assuming that it is the one of least interest.

The implementation of this algorithm maintains the necessary information about each buffer in a table whose starting address is returned by the word PREV.  The structure of this table is represented in Fig. 3.4 (details of the organization of this table are system-dependent; refer to your listing and CPU Supplement).

Fig. 3.4

Diagram showing details of the first portion of a PREV table in a typical polyFORTH implementation.  There are as many sets of buffer description data (8 bytes per buffer in this example) as there are buffers.  The exact structure of this table, including the order of items within a buffer descriptor, is system-dependent.

PREV contains the address of the most recently referenced buffer’s data.  Within that buffer’s data packet is the address of the next most recently referenced buffer’s data.  The least recently referenced buffer is identified by a zero in this location.  Any reference to a buffer through BLOCK or BUFFER will cause that buffer to be moved to the top of the chain.

Fig. 3.5

Example showing the use of the LRU buffer pointers in a typical implementation with four buffers.  In this case a search for a block in memory will start with Buffer 1, the most recently referenced buffer (whose descriptor is pointed to by PREV), and proceed through 0, 3, and 2.  BUFFER would select Buffer 2, the least recently referenced buffer.

REFERENCES

32-Bit Block Number Conventions, Section 3.2.7

### 3.2.4	Marking Buffers Updated with UPDATE

When the contents of a block buffer have been changed, the change must be followed by the word UPDATE to ensure the change will be recorded on disk.  UPDATE does not write data to disk.  UPDATE marks a block buffer.  When the word BUFFER frees a marked block buffer for re-use, it must write the marked block buffer’s contents to disk.  UPDATE requires no arguments because UPDATE marks whatever block buffer was most recently requested by either BLOCK or BUFFER.  This buffer is the one referenced in PREV.  The following example illustrates the use of UPDATE in a virtual storage array (the array begins in Block 250).  Note that UPDATE is used only in the “store” operation.

: VIRTUAL ( v - a)   1024 /MOD  250 + 
   BLOCK + ;

: V! ( n v)  VIRTUAL C!  UPDATE ;

: V@ ( v- n)   VIRTUAL C@ ;

In the example above, VIRTUAL converts an address for the virtual array into an address pointing into a block buffer in memory.  The phrase:

1024 /MOD 250 +

produces a block number on top of the stack, with an offset into the block just underneath.  The word BLOCK uses the  block number on top of the stack and leaves the address of the first byte of the block buffer containing a 1024-byte segment of the byte array.  The plus adds the block buffer address to the offset to produce an absolute address into the correct block buffer.  The operation of V@ is straightforward, but note the UPDATE in V!.  Without the word UPDATE, the changed block buffer data would never be written out to disk when the block buffer is reused.

UPDATE sets the sign bit of the block number in the buffer descriptor whose address is in PREV.  The sign bit is the update bit of the corresponding block buffer. Because the data in the buffer has changed, if the buffer must be freed, the data must be written to disk to prevent data loss.

No words which enter the multitasking loop should be allowed between BLOCK and UPDATE or between BUFFER and UPDATE.  If this is allowed, another task may use BLOCK or BUFFER to change which block buffer is the most recently used block buffer.  Similarly, a block buffer address should not be used after entering the multitasking loop because another task may have changed the buffer’s contents.  BLOCK and BUFFER  enter the multitasking loop only before they access the disk, so UPDATE can be performed before another task gains control of the disk.  Words entering the multitasking loop are:  words containing PAUSE or STOP, almost all input/output words, and any assembly code word ending in WAIT.  If the CPU enters the multitasking loop before doing an UPDATE, some other task may overwrite the data in the buffer, or reset which buffer is most recent by changing the buffer pointer in PREV.

For example, if the word V! from the virtual array was redefined for debugging as:

: V! ( n v)   VIRTUAL  DUP CR ?  !  UPDATE ;

the results would be unpredictable in a multitasking system because the words ? (equivalent to a @ .) and CR both use TYPE.  TYPE is an output word that enters the multitasking loop.

A possible way to correct such an error is to place the phrase DISK GET immediately after the occurrence of BLOCK or BUFFER, and DISK RELEASE immediately after the occurrence of UPDATE.

For example, to correct the “debugging” version of V! given above:

: V! ( n v)   VIRTUAL  DISK GET  DUP CR ?
   !  UPDATE  DISK RELEASE ;

REFERENCES

GET and RELEASE, Section 4.7

### 3.2.5	Other Buffer Management Words

Several other words are available to aid in buffer management.  These are:

Word	Stack	Function

IDENTIFY	( n)	Marks the most recently used buffer with the block number on top of the stack, after adding the contents of OFFSET.  IDENTIFY is used in COPY.

FLUSH		Ensures all updated buffers are written to disk by freeing all of the buffers, using BUFFER in a DO LOOP.

SAVE-BUFFERS		Writes all UPDATEd buffers to disk, leaving them still in the buffers with their UPDATE flags cleared.

EMPTY-BUFFERS		Erases all of the block buffers to disk without saving them.  EMPTY-BUFFERS works by resetting all the update bits in PREV and performs a FLUSH to free all the buffers.

REFERENCES

BUFFER, Section 3.2.3
COPY, Section 5.1.8
FORTH-83 Standard Compatibility, Section 2.9
OFFSET, Section 3.2.1

### 3.2.6	Disk Error Checking

Sometimes the disk hardware generates errors.  The appropriate response to a disk error depends upon the application.  A disk diagnostic, for example, may only want to count errors.  An application where background tasks (which cannot issue error messages) use disk may set a flag to be detected by a terminal task. Two predefined forms of error checking exist in polyFORTH:  The word SWEEP (which resides in the DISKING utility and checks for read errors), and the disk error handling block.  See references below for further discussion of SWEEP.

The disk error handling block has facilities for finding and correcting hardware errors.  The redefinitions available in this block of the hardware routines for BLOCK and BUFFER check the error flags in DISK 2+ (DISK 4+ on 32-bit processors) and abort with a “Read error” or “Write error.”  Loading the disk error handling block installs the new definitions.  If you wish to use this feature, the block must be loaded by Block 9.

The error handling routines use (BLOCK) and (BUFFER), and are device-independent to a degree; however, any changes to the buffer management code will have to be reflected in the disk retry code.

Word	Function

| /BLOCK/ and /BUFFER/ | Behaves like (BLOCK) and (BUFFER) except that /BLOCK/ and /BUFFER/ abort with an error message if too many retries occur. /BLOCK/ and /BUFFER/ are designed to abort safely, so /BLOCK/ does not leave the block in memory, and /BUFFER/ does not free a buffer, but does remove the update bit. A background task can’t execute ABORT" to issue an error message. If background tasks are accessing disk, /BLOCK/ and /BUFFER/ must be changed to return the buffer address without aborting and/or record the error occurrence so, if desired, a terminal task might issue an error message. |
| --- | --- |

Many disk controllers don’t report the sorts of “soft” errors retries may correct, and others do retries themselves.  For controllers reporting soft errors, polyFORTH’s low-level routines automatically perform retries.  As a result, these routines will only detect un-recoverable errors.

For diagnostic purposes you may find it convenient to temporarily define and install a version of (BLOCK) that displays disk controller status data from DISK 2+ (DISK 4+ on 32-bit machines).

: [BLOCK] ( n - a)   (BLOCK)  DISK 2+ @
   BASE @  SWAP HEX U.  BASE ! ;
   ' [BLOCK]  'BLOCK !

REFERENCES

32-bit Block Number Conventions, Section 3.2.7
BACKGROUND Tasks, Sections 4.2, 4.3, 4.4
DISKING Utility, Section 5.3 
Disk Diagnostics, Section 5.3.4
SWEEP, Section 5.3.4

### 3.2.7	32-Bit Block Number Conventions*

Some disk systems have capacities in excess of the range 15-bit block numbers can handle (up to 32,767).  An IBM-AT, for example, can support a full 30 Mb hard disk plus a 1.2 Mb floppy, plus a 320 Kb floppy using 15-bit block numbers, but could not add a second 1.2 Mb floppy or a RAM disk.

polyFORTH’s standard words BLOCK, BUFFER, LIST, and LOAD all handle single precision numbers.**  However, the polyFORTH disk routines facilitate the handling of double-precision block numbers by designing the entire block and buffer management scheme around 32-bit block numbers (the high order bit is the UPDATE bit) and defining the single-precision words “on top of” double-precision versions.  The following are defined:

Word	Stack	Description

2BLOCK	( d - a)	Takes a 31-bit block number and returns the address of the buffer containing that block.

2BUFFER	( d - a)	Takes a 31-bit block number and returns the address of a buffer identified by that block number.

2OFFSET	( - a)	Returns the address of the USER variable containing the 31-bit offset used to “bias” block numbers.

Word	Stack	Description

2IDENTIFY	( d)	Marks the most recently used buffer with a 31-bit block number, after adding the contents of 2OFFSET.

There are no 31-bit versions of LIST and LOAD.  Source blocks are restricted to a 32 Mb range of logical block numbers.  However, since 2OFFSET is added before the actual disk access, the actual range may be larger.

Merely plugging in a large disk drive, however, doesn’t automatically provide access to its entire contents.  At a minimum, tables in the disk driver specifying the number of heads, tracks, and sectors must be adjusted.  If the polyFORTH nucleus doesn’t contain a driver for the particular controller being used, new driver routines may be required.  Since these issues are hardware dependent, consult your CPU Supplement and program listing for configuration information.

REFERENCES

?ABSENT and ESTABLISH, Section 3.2.2
?UPDATED, Section 3.2.3
Adding a Disk Driver, Section 3.2.8
Disk Retry Block, Section 3.2.6
Error Handling, Section 2.4.7
IDENTIFY, FLUSH and EMPTY-BUFFERS, Section 3.2.5
Structure of PREV, Section 3.2.1
UPDATE, Section 3.2.4

### 3.2.8	Adding A Disk Driver

Disk controllers vary considerably.  Nevertheless, certain principles remain constant.  The guiding principle behind disk driver design is to minimize both the number of disk accesses and the time of each access.  This section is intended to show how the basic arrangement of a polyFORTH disk driver helps these ends, as well as certain low-level interfacing conventions and a technique for using more than one disk controller in a polyFORTH system.

#### 3.2.8.1	The Behavior of Hardware-Dependent Code

The words (BLOCK) and (BUFFER) are hardware-dependent words whose addresses must reside in 'BLOCK and 'BUFFER for use by the higher-level words 2BLOCK and 2BUFFER (and, hence, BLOCK and BUFFER).  (BLOCK) ensures that the data from the requested disk block is in a block buffer in memory.  (BLOCK) only reads the data from disk if the data is not already in memory.  When (BLOCK) needs an empty buffer to move disk data to, it calls (BUFFER) to obtain the address of a free buffer.  (BUFFER) determines whether a buffer has been changed since it was read from disk (it checks a flag set by the word UPDATE).  If the data has been changed, (BUFFER) writes the data back to disk.  In either case (BUFFER) returns the address of a free block buffer.

Wherever possible, BLOCK and BUFFER should share words.  Fairly often, the addressing word mapping block numbers into physical track and sectors can be shared.  Sharing words simplifies the driver and minimizes the amount of code to be debugged.

In the following sample skeleton definitions of (BLOCK) and (BUFFER), d represents a 31-bit block number and a represents a block buffer address.  Note that the read section in (BLOCK) and the write section in (BUFFER) use and leave similar arguments so code can be shared more easily between the read and write operations.  ?ABSENT exits (BLOCK) if a block is already in memory.  ?UPDATED exits (BUFFER) if the next buffer to be freed does not need to be written to disk.

: (BLOCK) ( n - a)   ?ABSENT
   'BUFFER @EXECUTE 'S 2+ 2@ ( - d a d)
   read-words  ( - d a)  ESTABLISH ;

: (BUFFER) ( d - a)   ?UPDATED
   ( - a d)  write-words  ( - a) ;

REFERENCES

32-bit Block Number Conventions, Section 3.2.7
Vectored System Routines, Section 3.1

#### 3.2.8.2	Servicing Disk Interrupts

The interrupt routine for a native polyFORTH disk driver is usually very simple.  The interrupt routine performs the most time-critical operations required (e.g., acknowledging the interrupt and reading status bits if this cannot be postponed) and then stores WAKE into the status of the task using the disk to notify the task that the operation is complete.  The interrupt routine finds the task by using the task address in the facility variable DISK.  The value of DISK is set when the phrase DISK GET is executed in BLOCK or BUFFER (see reference below, Facility Variables).

Usually a disk operation is accomplished by giving a controller a sequence of commands.  Disk controllers usually interrupt as they complete a command (or a sequence if the controller is “intelligent”).  The segmentation of the machine code words for a command sequence is dependent on where the code must pause to await an interrupt.  For a controller needing separate commands to move to a track, find a sector and transfer data, the following phrase might form the “read-words” of (BLOCK):

ADDRESS  TRACK SECTOR READ

The controller would interrupt after TRACK, SECTOR, and READ to indicate it had completed each of those operations.  All three of the operations are assembly coded words, each of which sends a command sequence to the controller and then enter the multitasker’s round robin (via the code ending WAIT) for an indefinite time.  Each time the task is awakened by the interrupt routine, the next word is executed. Waiting for interrupts in the multitasker allows other tasks fairly frequent opportunities to get work done.  Interference between tasks trying to use the disk simultaneously is eliminated by the use of the word GET in the high-level definitions of BLOCK and BUFFER.

REFERENCES

GET, Facility Variables, Section 4.7
WAIT, Sections 4.2, 6.2
WAKE, Section 4.2

#### 3.2.8.3	Interleaving the Disk’s Data Format for Speed

If the data on the disk is ordered properly, a disk driver may run four (or more) times as fast as with a sequentially ordered disk.  Depending on how the disk controller works, two forms of data interleaving are possible.  If the controller can only transfer sectors which are a fraction of a block, then the sectors of each block may be interleaved with the sectors of other blocks.  If the controller can transfer pieces of data the size of a block, then blocks themselves may be interleaved to speed sequential block access.  On some disk drives it may be worthwhile to perform both types of optimization (usually it is not).

Ideal timing permits the driver code to finish its data transfer and housekeeping just in time to enable the disk interrupt before the next piece of data becomes available.  Calculation of a good data spacing consists of choosing the smallest average distance between data segments (sectors or blocks) still resulting in a physical access time greater than the amount of time required by the data transfer.  Usually the best pattern is determined by formatting a disk in several ways and then benchmarking with both random and sequential block access (with mixed read and write operations).  The information required to determine whether such optimization may be desirable is obtained from the disk controller manual.  The time between the availability of two sectors in a block may be computed from the rotational latency of the drive and average “seek” times.  This  figure may then be compared with an estimate of the time required by the software to get from one block or sector request to the next, assuming sequential block requests and normal system loading.  If the software time is longer than the hardware time, some form of interleaving may be desirable.

The goal of the optimization is to be able to read a track’s worth of blocks in one revolution.  The worst case is reading only one sector per revolution.  A good test for overall performance is to SWEEP the disk, and then compare the time for the SWEEP with the number of revolutions.  The sweep time divided by the number of blocks per cylinder should (ideally) equal the number of cylinders covered.

REFERENCES

SWEEP, Section 5.3.4

#### 3.2.8.4	REPORTING DISK STATUS TO polyFORTH

The hardware driver code must store disk status information in DISK 2+ (DISK 4+ on 32-bit systems) and subsequent bytes as needed.  Two conventions for this information are in common use.

One convention declares if no errors occurred then DISK 2+ is zero.  Usually drivers using this convention perform an exclusive OR between a mask and the status data from the disk controller.  If multiple cells of status are available, they are logically combined so common hardware failures may be isolated.  This method has been used most often where the controller design is such that the disk status has no bits set unless errors have occurred.

The other convention copies status data directly from the disk controller to DISK 2+.  If multiple cells of status data are available, they are copied to a region starting with DISK 2+.  Each method has advantages and disadvantages.  The “zero means no errors” method simplifies an application’s disk error checking, and allows the disk retry block to be standardized and simplified in Forth systems with several different disk controllers.  This method also has a higher machine code overhead, and requires more documentation to isolate types of errors.  The “copy-status” method is simpler to implement, faster, and more directly understandable, but sometimes much more difficult to apply in systems with multiple controllers.  Follow the convention already on your system, if possible.  Remember to document the arrangement of error flags in DISK 2+ in the shadow blocks.

The most recent block number written is stored in 'BUFFER 2+ ('BUFFER 4+ on 32-bit systems) by the word ?UPDATED.

#### 3.2.8.5	Assembling a System With Multiple Controllers

Several parts of a polyFORTH system must be modified in order for it to smoothly manage a computer with several types of disk.

1.	BLOCK must be modified to access all the disk over a contiguous range of block numbers (discussed later in this section).

2.	Removable media should be in physical Drive 0 of multiple-drive systems, so the system can be booted from a backup copy of polyFORTH if necessary.

3.	The bootstrap must be adjusted to accommodate item 2.

4.	VOLUME, the disk size in the disking utility, is usually set to the size of the media containing the polyFORTH system to make backing up the system media convenient.

5.	The word PART may need to be modified to use a look-up table.

On most polyFORTH systems, specifications 2 and 3 are already met, and need not be changed.  Specifications 4 and 5 are trivial and will not be discussed further.  Meeting specification 1 simply is difficult, and is the subject of the remainder of this section.

The two disk driver words (BLOCK) and (BUFFER) must be changed.  Pre-existing Forth disk drivers almost always make use of a block number range starting at 0 and going up to some number of blocks dependent on the size of the disk.  Ideally, one would wish for a method to combine unmodified disk drivers in a simple consistent way.

The two basic problems a disk-driver-combining method must solve are:

1.	Choosing the correct driver.

2.	Ensuring the driver receives the correct block numbers.

A particularly nice way to solve both problems is to define a word called PLEAT.  PLEAT accepts a block number and a maximum possible block number.  If the block number is less than the maximum block number (i.e., acceptable to the disk driver), then PLEAT returns the block number, and a Boolean “false” for program control.  If the block number is greater than, or equal (i.e., not acceptable), then PLEAT returns the block number, less that controller’s maximum acceptable block number, with a Boolean “true” for program control.  By subtracting the current controller’s maximum block number, PLEAT eliminates all those possible block numbers, so the next driver can begin its block numbers at zero.  If the block number is within the driver’s maximum, then the driver gets the block number unchanged.  Although the example is high-level, a real driver would have PLEAT written in assembler.

In the following example, three different disk controllers are controlled by three separate drivers called <BLOCK>, [BLOCK] and *BLOCK*.  The drivers have meaningful block number input  ranges of 0 to 500, 0 to 3000, and 0 to 9000 respectively.  The “l” in the stack arguments of PLEAT stands for “limit.”

In (BLOCK), ?ABSENT determines if a hardware operation is necessary—if the block is already in memory, ?ABSENT performs all the functions of (BLOCK).  The phrase:

'BUFFER @EXECUTE

returns the address of a usable block buffer (and writes out the contents if necessary).  The OVER brings a copy of the block number over the buffer address.  The phrase 500 PLEAT leaves the block number and a 0 on the stack if the block number is less than 500 (this controller’s maximum block number is 499).  When the IF uses the 0 left by PLEAT, IF transfers control to the controller’s access word, <BLOCK>.  If the block number were greater than 500, PLEAT would subtract 500 from the block number to prepare the block number for the next controller selection, for [BLOCK].

: PLEAT ( n l - n-l 1 , n 0 ) 2DUP U< 
   IF  DROP  0  ELSE  -  1  THEN ;

: (BLOCK) ( n - 1)   ?ABSENT  'BUFFER 
   @EXECUTE OVER ( - n a n) 500 PLEAT
   IF  3000 PLEAT  IF  *BLOCK*   ELSE
   [BLOCK] THEN ELSE <BLOCK> THEN
   ESTABLISH ;

PLEAT can be used in the definition of (BUFFER).  (BUFFER) has different stack arguments than (BLOCK) {( - a) rather than ( n -a)} because the block number used in (BUFFER) is generated by the word ?UPDATED.  PLEAT must be situated so it can use the block number left on the stack by ?UPDATED.

Therefore, the construction using PLEAT in (BUFFER) must deal with the write words of the drivers, instead of the whole  individual versions of (BUFFER):

: (BUFFER) ( - a)   ?UPDATED ( - a n)
   500 PLEAT  IF  3000 PLEAT IF  *WRITE-
   WORDS*  ELSE  [WRITE-WORDS]  THEN ELSE
   <WRITE-WORDS>  THEN ;

Otherwise the application of PLEAT to constructing a (BUFFER) is the same as PLEAT’s application to the construction of a (BLOCK).

## 3.3	LOADING polyFORTH SOURCE BLOCKS

Most compiled languages have a three-step process of constructing executable programs:

1.	Compile the program to an object file on disk.

2.	Link this program to other previously compiled and/or assembled routines.

3.	Load the result into memory.

This often-lengthy procedure has a negative effect on a programmer’s creative effectiveness.  polyFORTH supports fully interactive programming by shortening this cycle to a single, fast operation:  compiling from source to executable form in memory.  This process is accomplished by the word LOAD.

### 3.3.1	The LOAD Operation

The word LOAD specifies the interpretation of source text from a disk block.  LOAD expects the block number of the Forth block to be LOADed on the top of the stack:

number LOAD

The current contents of OFFSET will be added to the block number.  The resulting absolute block  number is stored in the user variable BLK, used by Forth’s text interpreter.  At this time interpretation of text input from the current input source is suspended and input is taken from the specified disk block.  >IN, the character counter, starts at 0, and will be incremented by the action of the text interpreter until it reaches 1023 or an EXIT command is encountered, whichever comes first.

When all processing specified by the disk block has been completed (assuming that no errors were encountered in processing the block), execution resumes with input from the source that was in control when the LOAD was encountered.  Note that LOAD sets the base to DECIMAL before returning.

The block number specified for the LOAD command must be a single-precision integer in the range 0 < n < 32767.

Where a block contains definitions, the result of a LOAD operation will be to compile these into the dictionary.  Remember, though, that this is the result of the execution of defining and compiling words as they are processed by the text interpreter.

The process of LOADing disk blocks is identical to processing the same information entered from the terminal, with all information in the same disk block entered at once (i.e., there will be no embedded carriage returns).

During the interpretation of a disk block, the processing of text input under program control proceeds as normal, substituting the disk block for the terminal, with the following exceptions and differences:

1.	If a TEXT operation is executed, the entire remainder of the disk block will be read into memory, starting at the first position of PAD, unless the count expires or the delimiter is found.  Since there is no guarantee a delimiter will be found at all, this should be avoided.

2.	Use of WORD is equivalent to use of TEXT and follows the same restrictions as given for TEXT.

The block to be LOADed may itself contain a LOAD command, at which point the LOADing of the first block is suspended.  When this occurs, the block number of the current block, the current character pointer for the interpreter operating in this block and the address interpreter pointer are saved on the return stack pending loading of the requested block.  This nested LOADing process may continue indefinitely, subject to return stack size.

Where a group of blocks is to be LOADed, they should all be specified by LOAD commands contained in a single block, as opposed to serial nesting, i.e., having each block load the next block in sequence.  This is because each nested LOAD command requires several cells of additional space on the return stack, as explained below, and the chances of a return stack overflow are significantly greater when each block loads the next block in sequence.  From a management viewpoint, moreover, it is best to see all the blocks needed for a well-defined portion of an application grouped together in one place.

The command THRU can load a group of sequential blocks.  For example, if Blocks 260 through 270 needed to be loaded, THRU could be used:

260 270 THRU

THRU is an elective loaded from the “programmer aids” block.

A LOAD operation may also be compiled in a definition.  The requested LOAD is done when the definition is executed.  Following the completion of the LOAD, execution of the definition containing the LOAD request will resume at the word immediately following the LOAD.

If, during the LOADing process, a Forth error is detected, an error message is produced and all LOADing ceases.  Both the return stack and the parameter stack are cleared and Forth reverts to terminal input.  At this point, typing L will display the block being LOADed when the error was detected, with the editing character pointer CHR set to point just after the last word examined (i.e., the one producing the error).

During loading, all text interpreter input is taken from the specified disk block.  All output, however, proceeds to its normal destination.  Thus, the use of .( or other output commands will send output to the terminal of the task executing the LOAD.

REFERENCES

.(, Section 2.3.6.5
PART, Section 3.2.1
EXIT, Section 2.4.6
L, Section 5.1.1
TEXT, Section 2.3.6.3
WORD, Section 2.3.6.2

### 3.3.2	Use of the Return Stack by LOAD

Each invocation of LOAD places four items on the return stack; they contain the following information:

1.	The logical instruction counter for LOAD.

2.	The input source in effect when the LOAD was issued (i.e., the contents of BLK).

3.	The character position in the input source (i.e., the contents of >IN).

4.	The address interpreter pointer (I) for INTERPRET.

The word LOAD calls the word INTERPRET, which calls the word EXECUTE for each word found in the dictionary.  At some point during the INTERPRETation of a block of source text, a LOAD may be executed.  The new, nested LOAD pushes the above four items on the return stack, and then proceeds to INTERPRET its block of text.  A LOAD may terminate in one of two normal ways:

1.	When the word -' in INTERPRET has exhausted the input stream.

2.	When the EXECUTE in INTERPRET executes the word EXIT.

Either occurrence forces Forth out of the endless loop in INTERPRET, and thereby allows LOAD to continue past INTERPRET and restore all the system data the LOAD earlier stored on the return stack.

If the programmer places items on the return stack during loading and fails to remove them before the end of the block, the four items removed from the return stack as LOAD finishes will be an incorrect set.  This will cause an unpredictable error, depending upon the exact values of the items.  In these circumstances, it is possible loading would continue, using an improper disk block.

REFERENCES

EXIT, Section 2.4.6
INTERPRET, Section 1.1.4

### 3.3.3	Named Program Blocks

The defining word CONSTANT may be used to give names to important blocks, such as blocks which in turn load other blocks to form a utility or application.  Such a block is often called a “key block.”  For example,

120 CONSTANT OBSERVING

will be used as:

OBSERVING LOAD

The above has the effect of loading Block 120 and any other blocks specified to be loaded by that block.

Alternatively, the word LOADS creates a definition where the LOAD operation happens automatically.  In this case,

120 LOADS OBSERVING

defines OBSERVING so you can merely type:

OBSERVING

to load this application.

Normally, LOADS is the more convenient way to name key blocks.  Use of CONSTANT is appropriate when you want to use the name in other ways, such as:

OBSERVING LIST

We recommend the use of a key block for each entire application.  Enter definitions using either of the methods described above for named blocks as well as any other brief application-wide definitions.  Then you can see at a glance which of your application blocks are loaded and in what order.  Your polyFORTH system contains several such key blocks, usually named in Block 10.

Use of this technique is much safer than “chaining” blocks, because “chaining” blocks can cause a return stack overflow.  Generally one block on a system names all the key blocks in a system, and is LOADed immediately after booting.  In polyFORTH systems, this is usually Block 10.

Note:  There is a special danger with named blocks.  They can be successfully LOADed when in any number conversion base.  For this reason, named blocks should have a DECIMAL command in the first line to guard against accidental loading with an incorrect base.

REFERENCES

CONSTANT, Section 2.6.3
LOAD and the Return Stack, Section 3.3.2

### 3.3.4	Overlays

Because of Forth’s compilation speed, there is rarely need for a dynamic run-time overlay capability.  Many resident applications have several functionally independent subsets, however, and it is conventional to organize these as mutually exclusive overlays, any one of which may be loaded into each terminal’s private dictionary.  This is done by explicit command.  Once LOADed, such an overlay will remain resident until replaced by another.

Examples of such overlay categories in a business environment might include order entry, payroll, and general ledger.  In a scientific laboratory system there may be several different data acquisition and analysis modes.

This section covers two techniques for managing such overlays.  To replace the contents of an entire task dictionary with a new overlay, we recommend use of the word EMPTY.  To create additional levels of overlays within the task dictionary, such that when an overlay is loaded, it will replace its alternate overlay beginning at the appropriate level, we recommend use of the word FORGET.  This section also discusses the option of allowing an overlay to reset the boundary between system and private definitions.

#### 3.3.4.1	Single-Level overlays:  EMPTY

The command EMPTY empties a user’s private dictionary.  In polyFORTH, most overlays (such as DISKING and PRINTING) begin with the word EMPTY at the top of the load block.  For example:

0  ( DISK UTILITY)   EMPTY   DECIMAL
1  35 LOAD   37 LOAD
2    etc. ...

Any application definitions which are not meant to be replaced by the overlay should have been loaded as system electives (i.e., by Block 9) not as part of the task’s private dictionary.  System electives, of course, are not affected by EMPTY.

The complete definition of EMPTY is:

: EMPTY   H 2+ @ H !  GOLDEN CONTEXT 20
   MOVE ;

Note:  H 2+ is H 4+ and 20 is 36 in 32-bit systems.

The mechanism used by EMPTY to remove only those definitions in the task dictionary without affecting the system dictionary involves an array named GOLDEN.  The system has one GOLDEN array containing the default values for CONTEXT and CURRENT plus the eight system “link heads”—that is, eight cells, each pointing to the last definition in each of the eight chains in the system portion of the dictionary.

Part of the function of EMPTY is to store the values of the system pointers into the user’s private link head array.  Thereafter, the first word to be compiled into each chain will be linked to the last word in the appropriate chain in the system portion of the dictionary, rather than being linked to  the end of whatever chains may have previously existed in the task dictionary.

This function is accomplished by the phrase:

GOLDEN CONTEXT 20 MOVE

in the definition of EMPTY.

The other function of EMPTY is to reset the value of H.  H is used at compilation time to point to the next available cell in the dictionary where a new definition may be placed.  H is reset to the beginning of the user’s dictionary.  Thus the phrase:

H 2+ @ H !

resets H to the beginning of the task dictionary, so that thereafter any new definitions will be written over any previous definitions residing in the task dictionary.

REFERENCES

CONTEXT, Section 3.4.3
GOLDEN, Sections 3.3.4.3, 3.4.4

#### 3.3.4.2	Multi-Level overlays:  FORGET

The word FORGET is used to discard the most recent portion of a task’s dictionary.  The command:

FORGET NAME

will discard the definition NAME and all words defined after NAME in a user’s partition.  The user’s dictionary pointer H, as well as the task’s private link heads for the dictionary chains, will be reset to the last definition in the vocabulary before NAME.

Since H is reset, the dictionary is truncated spatially as well as logically.

FORGET has two uses:

1.	To discard only part of your definitions.  For example, when testing, you may reload only the last block, not your entire application.

2.	To create additional levels of overlays.

Suppose your application includes an overlay called GRAPHICS whose load block begins with the command EMPTY.  Once GRAPHICS is loaded, you want to be able to load either of two additional overlays, called COLOR and B&W, to load after GRAPHICS, thus creating a second level of overlay.  Here is the procedure to follow.

1.	Define a “null definition” as the final definition of GRAPHICS, using any word you want as a dictionary marker.  For example:

: OVERLAY ;

Preferably, such a null definition will be placed at the bottom of the GRAPHICS load block.

2.	Place the appropriate FORGET phrase on the first line of the load block of each level-two overlay.  For instance,

( COLOR)   FORGET OVERLAY   : OVERLAY ;

Thus, when you execute the phrase:

COLOR LOAD

you “forget” any definitions which may have been compiled after GRAPHICS and restore the null definition of OVERLAY to serve as a marker in the event you want to load an alternate level two definition, such as B&W.

By using different names for your null definitions, you may create any number of overlay levels.  One of the simplest choices of overlay markers is the word HELP, since the first definition in many applications is the definition of the help screen displayed at the beginning of the application.

REFERENCES

Help Screens, Section 1.5

#### 3.3.4.3	Resetting the Pointers for an “Empty” Dictionary

Application words intended to be available to all users in the system are normally loaded from the electives load block so they will become system definitions.  The command GILD actually creates the division between system definitions and task definitions as may be seen near the end of the electives load block.  GILD is defined as follows:

: GILD   CONTEXT GOLDEN 20 MOVE
   HERE H 2+ ! ;

The first part of this phrase copies the values of the eight link heads plus copies of CONTEXT and CURRENT in the OPERATOR user area, at the moment the above phrase is executed, to the GOLDEN array.  Since the phrase is executed after all system definitions have been loaded, the GOLDEN array will henceforth point to the ends of the system-definition chains.

Note that it is important that CONTEXT is FORTH when this is done, so the default CONTEXT (after EMPTY) will be FORTH.

The second part of the above phrase stores the present value of H (the dictionary pointer) into OPERATOR’s H 2+ as a reference for the future as to where the “empty dictionary” should begin.  (Note:  On some systems the empty dictionary location is in H 4+; see your CPU Supplement.)  In certain cases it is useful to have an overlay reset the boundary between system and task definitions by executing the above phrase.  An example in polyFORTH is the Data Base Support option, whose load block concludes with GILD.

This technique has the advantage of making additional capabilities available to all users on the system without recompiling.  Each user will be required to execute EMPTY to reset the beginning of his private dictionary to the end of the extended system dictionary.  This can also be done automatically by PROMPTing them.

REFERENCES

GOLDEN, Section 3.4.4
PROMPT, Section 4.10

## 3.4	Vocabularies

Vocabularies are mutually exclusive collections of definitions residing concurrently within the dictionary.  Up to eight vocabularies may exist at any one time.  Dictionary searches proceed from one vocabulary to another in a specified sequence of up to four vocabularies.  This mechanism allows you to control which vocabulary or vocabularies are to be searched.  Within each vocabulary the search is from newest to oldest.

Vocabularies have three principal uses:

1.	In the resident system, to segregate special-purpose words such as those in the ASSEMBLER, to allow them to have the same names as standard Forth words.

2.	In the Target Compiler, to segregate target versions of FORTH, ASSEMBLER, and EDITOR words from the resident versions.

3.	In applications running in the resident system, to protect against accidental misuse of words only intended to be available to programmers.

### 3.4.1	Vocabulary Selection

The standard vocabularies provided by Forth are:

FORTH
ASSEMBLER
EDITOR

FORTH is the standard fundamental vocabulary.  ASSEMBLER contains all assembler mnemonics, addressing modes, and other special assembler commands.  EDITOR contains the editing commands for editing source text in blocks.

The use of separate vocabularies makes it possible, for instance, for the word I to be defined to supply a loop index in one context and insert a string in another context or name a register in yet another.

There are two actions on the dictionary relevant to vocabularies:  searches and additions.  The sequence of vocabularies to be searched is specified by the contents of the user variable CONTEXT.  The vocabulary to which any further definitions are to be linked is specified by the contents of the user variable CURRENT.

You may change the contents of CONTEXT by simply naming the desired vocabulary.  For example, the word:

ASSEMBLER

changes CONTEXT so future searches will begin with the ASSEMBLER vocabulary.  CONTEXT is automatically set to ASSEMBLER by the defining words CODE and ;CODE.

Similarly, you may employ the word:

EDITOR

to set CONTEXT to begin by searching the EDITOR vocabulary.  Several of the EDITOR commands are found in FORTH and automatically set CONTEXT to the EDITOR vocabulary.

CONTEXT is automatically reset to the contents of CURRENT by the defining word :.  For example, assume FORTH is “current” (as is typical) and a user has just finished modifying a block using EDITOR commands (CONTEXT is now EDITOR).  When the user loads the block, CONTEXT switches back to FORTH when the first : is encountered.

The contents of CURRENT may also be changed.  The word DEFINITIONS sets CURRENT to the “context” vocabulary.  For example, the phrase:

EDITOR DEFINITIONS

first sets the value in CONTEXT to be the EDITOR vocabulary, then sets CURRENT also to EDITOR.  Thereafter any future definitions will be linked according to the EDITOR vocabulary.  When the system starts up, or following an EMPTY, the default vocabulary for CONTEXT and CURRENT is FORTH.

### 3.4.2	Creation of a Vocabulary

Each of the three standard vocabularies is associated with an index, as follows:

1 is FORTH
3 is ASSEMBLER
5 is EDITOR

For internal reasons, a vocabulary index may be any odd number from 1 to FH.

If you look up the definitions for the commands associated with the standard vocabularies, you will find:

HEX	0001 VOCABULARY FORTH
	0015 VOCABULARY EDITOR
	0013 VOCABULARY ASSEMBLER  DECIMAL

(The order of the digits is reversed for some processors.)

When viewed as a hexadecimal number, each of the four 4-bit “nibbles” of the constant represents a vocabulary index.  Thus the variable CONTEXT may contain as many as four vocabulary indexes.  This feature allows vocabularies to be chained; that is, after the system searches the vocabulary specified by the index in the right-most nibble, it will then search the vocabulary specified by the nibble to the left, and so on.  The search continues until the word is found, a 0 nibble is encountered, or all four nibbles have been used, whichever comes first.

Although VOCABULARY is a defining word, it does not define a vocabulary, but rather a “vocabulary-specifying command” (e.g., FORTH, EDITOR, ASSEMBLER).  The word VOCABULARY associates a vocabulary-specifying constant (e.g., 0015) with each of these commands.  This constant, when placed in CONTEXT, specifies the order the appropriate vocabularies are to be searched.  When placed in CURRENT, the first vocabulary to be searched specifies the vocabulary to which new entries will be linked.

As mentioned above, the order of the digits is reversed for some processors.  To determine which order is used by your system, either refer to your listing, or set and display CONTEXT by typing:

FORTH HEX CONTEXT @ U.

If the result is 1 (the leading zeros don’t show), searches begin with the vocabulary whose index is specified by the right-most nibble (vocabulary 1, or FORTH).  If the result is 1000, your system first searches the vocabulary whose index is specified by the left-most nibble.

In all systems (regardless of the direction in which the nibbles are examined), the search order depends on the context vocabulary as indicated in this table:

Command	Search Order	Constant

FORTH	1.	FORTH	0001 or 1000

EDITOR	1.	EDITOR	0015 or 5100
	2.	FORTH

ASSEMBLER	1.	ASSEMBLER	0013 or 3100
	2.	FORTH

Although the same constant that is stored into the variable CONTEXT is also stored into CURRENT (resetting CONTEXT according to CURRENT), only the nibble in the first-search position is used to indicate which vocabulary will receive new dictionary entries (see the definition of CREATE in your system listing and the references below).

The actual creation of a new vocabulary occurs after a new vocabulary-specifying command has been defined and its associated constant has been placed in CURRENT with the phrase:

specifying-command DEFINITIONS

As new definitions are compiled a new “vocabulary” will be created.

If you are not target compiling, it is possible to add up to five vocabularies of your own (using the odd digits 7-F), provided you write definitions linking any new vocabulary to an existing vocabulary, especially FORTH, (otherwise searches will find nothing).  The principal use of additional vocabularies is for “sealed” vocabularies in a resident system.  Most other situations where vocabularies might be considered are better handled by one of these methods:

1.	In an application running on the resident system, to define two classes of words which may have the same names but different meanings, we recommend the use of overlays.

2.	In a target compiled application where the Forth interpreter is not present, there is no value in having separate vocabularies since there are no searches.

3.	In a target compiled application in which the Forth interpreter is present, words which must be protected against inadvertent misuse should be made headless during target compilation, thereby rendering them unfindable in a search.

REFERENCES

CREATE, Section 2.7.1
Sealed Vocabularies, Section 3.4.5
Vocabularies in the Target Compiler, Section 7.2

### 3.4.3	Hashed Dictionary Searches

Among the user variables in each terminal task’s user area, between CONTEXT and CURRENT, reside eight consecutive cells called the “link heads.”  Each link head contains the address of the most recent definition added to one of eight linked lists that comprise the dictionary (see Fig. 3.6).

The space in the dictionary is sequentially allotted, with new entries having greater addresses than older entries.  Entries from all the linked lists are mixed together in the dictionary.  In systems with multiple tasks, the private space for each task is allotted and named in the public dictionary when electives are loaded immediately after booting.

As new definitions are created, they are linked to one of the eight linked lists.  The selection of which linked list depends on a combination of the current vocabulary index and the first letter of the word being defined.  The vocabulary index is added to the ASCII character, then the least significant bit of the 4-bit nibble is masked out to yield an even cell offset to the array of link heads.

Fig. 3.6

Dictionary Link Heads

Thus a single list may contain words from as many vocabularies as have been defined (up to a maximum of eight).  Yet any two words with identical names, belonging to separate vocabularies, will be linked to separate lists.  This arrangement more evenly distributes the number of definitions linked to the various chains, and makes it possible to search an entire vocabulary for a given word by actually searching only one-eighth of the total dictionary.  This greatly reduces search time and thus, compilation time.

As stated earlier, each terminal task user area contains its own copy of the eight link heads.  When a terminal task’s dictionary is empty, these heads point to the end of the lists that comprise the public dictionary.  As words are added, each task’s link heads will point to the new definitions which physically reside within the tasks’ private dictionaries.

### 3.4.4	The GOLDEN Array

The system maintains an array, called GOLDEN, which contains a copy of the eight link heads (see Fig. 3.7) that point to the ends of the system dictionary lists (see Fig. 3.6).  Thus “system definitions” are those linked through GOLDEN and are available on a common, reentrant basis to all users.  The GOLDEN array begins with a system value of CONTEXT (specifying the FORTH vocabulary), continues with the eight link head cells and ends with the system value for CURRENT (again FORTH).

Thus, the definition of EMPTY contains the phrase:

GOLDEN CONTEXT 20 MOVE

which copies the system link heads (as well as CONTEXT and CURRENT) into the user’s private link heads, thus resetting each of the link pointers for the task dictionary back to the end of each of the lists in the system dictionary.

The word GILD contains the reversed phrase,

CONTEXT GOLDEN 20 MOVE

which copies the user’s private link heads into the system’s link heads.  In effect, this relocates the border between system definitions and the user’s definitions.  Thus, any private definitions which a user has loaded prior to the above phrase become public to other users (provided that they first execute EMPTY to reset their private link heads).  GILD also resets the physical bottom of the dictionary so that the user can no longer forget the “public” section of the dictionary.

Fig. 3.7

The GOLDEN Array

GILD is found near the end of the electives block (usually Block 9).  Normally, when any definitions are meant to be available to all users, their blocks should be loaded from the electives block, above GILD.

REFERENCES

EMPTY, Section 3.3.4.1
FORGET, Section 3.3.4.2

### 3.4.5	Sealed Vocabularies

The vocabulary mechanism offers the potential for an exceptionally powerful security technique.  You can implement this by setting up a special application vocabulary consisting of a limited number of commands guaranteed to be safe for users.  You then ensure no application word can change CONTEXT, and that CONTEXT is set so the text interpreter will only search the application vocabulary.

This has the effect of sealing a task into its limited vocabulary and rendering all other words “unfindable.”  This is how a sealed vocabulary is constructed:

1.	Define a new vocabulary for the findable words.  For example:

HEX 0017 VOCABULARY APPLICATION

This vocabulary must be linked to FORTH at compile time so that APPLICATION words may contain references to FORTH words.  This is done by including FORTH (index 1) in the search order as shown in the example.

2.	Place all definitions to be available to users in the application vocabulary by declaring:

APPLICATION DEFINITIONS

before compiling such definitions.

3.	Define SEALED like this:

: SEALED   0007 CONTEXT ! ;

so once SEALED is executed, only definitions in Vocabulary 7 can be accessed.  Thus SEALED might be executed as the last item in the application load block (in a resident system), and then only when the application has been completely tested.

Note that CONTEXT cannot be changed by a reference to its name, because the variable CONTEXT is defined in the FORTH vocabulary which is sealed from search, as are all words (such as !) that would enable a knowledgeable user to change CONTEXT.

An example of the load block for a sealed application might be:

0
 1 : SEALED   0007 CONTEXT ! ;
 2 HEX   0017 VOCABULARY APPLICATION
 3 APPLICATION DEFINITIONS
 4 : GO   ... ;
 5 : STOP   ... ;
 6 : TURN   ... ;
 7 SEALED
 8
 9
10
11
12
13
14
15

## 3.5	Calendar Support

Forth supports two optional output formats for the date:

mm/dd/yy	and	dd mmm yyyy

The latter uses a three-letter month-name abbreviation and assumes the current year, which may be reset in the calendar block.  An application may use whichever of these is most compatible by loading the appropriate calendar block in Block 9.  The calendar option currently installed can be seen by typing SYSTEM after polyFORTH has been booted and the options loaded.  The two calendars are located in your system in consecutive blocks.

The three-letter abbreviation type of calendar is useful because it is simple, easy to read, and does not require double-precision integers.  The abbreviation calendar is inconvenient for use in applications entering dates for a variety of years.

For both calendars, the internal date format is the same:  a single-precision integer containing the number of days elapsed since January 1, 1900.  This format is called a “Modified Julian Day” (MJD).  An MJD is a compact representation (2 bytes vs. 6 or 8).  Also, arithmetic between MJDs can be performed directly, without complex conversions.  Dates prior to March 1, 1900 will not be set or displayed correctly because only centuries divisible by 400 are leap years and the special conditions for 1900 are not tested for by the calendar.  To obtain the day of the week from an MJD, simply take that number modulo 7; a value of 0 is Sunday.  For example:

4/24/88 M/D/Y 7 MOD .

gives 4 (Thursday).

The week-day number may be used to index a text array for output.

### 3.5.1	Date Input

1.	dd mmm Input Format

The day/month calendar is a convenient, readable format appropriate for applications where the majority of dates entered are current (e.g., transaction or measurement dates).

Where this option is used, the MJD conversion is performed by the month-name abbreviation, which is actually a Forth command.  The months are:

JAN	MAY	SEP
	FEB	JUN	OCT
	MAR	JUL	NOV
	APR	AUG	DEC

These words expect the day-number on the stack; they will add offsets for the current year and month to leave the correct MJD on the stack.  For example,

25 NOV

will leave on the stack the MJD for the twenty-fifth of November of the current year.  To set the year type:

year A.D.	(e.g., 1988 A.D.)
	dd mmm NOW	(e.g., 25 NOV NOW)

The current calendar year is the default.  It is set in Block 9.  The year must be edited every January.

2.	mm/dd/yy Input Format

The mm/dd/yy format is most convenient for applications where dates are frequently entered for years other than the current one (e.g., birth dates).

The date typed in the mm/dd/yy format is converted to a double-precision integer on the stack by the standard input number conversion routines.  A leading zero is not required on the month number but is required on the day number if it is less than ten.  Years entered as 00 to 99 are treated as being in the twentieth century; twenty-first-century dates may not be entered by this option.

The double-precision number thus entered must be given as a parameter to the date input conversion routine M/D/Y, which computes the MJD.  For example:

8/03/40 M/D/Y

returns the MJD for August 3, 1940.

Normally M/D/Y is incorporated in the application command that accepts the date.  For example, given the definition:

: HIRED ( d)   M/D/Y DATE ! ;

then the phrase, 1/15/87 HIRED would convert the date 1/15/87 to an MJD and store it in the variable DATE.

REFERENCES

MJD, Section 3.5
Number Conversion, Section 2.5.2

### 3.5.2	Date Output

All date output commands expect an MJD (Modified Julian Date) on the stack.  This will be formatted into an output string that has an appearance similar to the input string for the input format of the calendar option selected.

The basic output command is:

mjd .DATE

In systems with the mm/dd/yy calendar installed, .DATE works this way:

mjd .DATE	5/16/88 ok

Systems with the day-month option have another version of the output format:

mjd .DATE	16 MAY 1988 ok

The actual formatting is done by the word (DATE).  (DATE) returns the address and length of a formatted string (the arguments appropriate for TYPE).  These arguments may also be used for other purposes, such as other forms of output.  To print only the day and month (not year) using the day-month calendar, for example, one could define:

: .DAY ( n)   (DATE) 4 - TYPE ;

This would print today’s date this way:

TODAY @ .DAY	16 MAY ok

REFERENCES

MJD, Section 3.5
TODAY, Section 3.5.3

### 3.5.3	System Date Management

The system date is stored as a single-precision modified Julian date.  The system date is near TICKS in the system variables region of memory.  The address of the system date is given by the word TODAY.  Thus, TODAY @ will return the value of the system date in modified Julian form (add .DATE for display).

To set the system date, type:

date NOW

The date need only be set after power-up.  When the system is loaded or when the system “help screen” is displayed by the command SYSTEM, the date is displayed for verification.

Note that rollover of the date at midnight is not programmed to occur automatically, because this would add tremendous overhead to the clock interrupt routine.  Instead, the date is checked for rollover and corrected whenever the time is read by @TIME.

REFERENCES

MJD, Section 3.5
Time Overflow at Midnight, Section 3.6.6

## 3.6	Clock Support

Assuming the presence of a hardware clock, polyFORTH provides standard time-of-day support words.  These words allow the user to set and to print the current time of day.

The internal units of time maintained by polyFORTH are clock ticks, the value of each one depending upon the frequency of the hardware clock.  Part of the standard clock support word-set includes a ratio for converting internal units to milliseconds, and most application-level words operate in milliseconds.

The clock electives also allow each task to establish a timer used to deactivate a task for a user-specified time interval.

### 3.6.1	Internal Time Representation

The clocks on most computers are simple time bases which generate a pulse at a regular interval.  Most often the pulse is used to cause a “clock interrupt” or “tick.”  When a clock interrupt occurs, the computer leaves the program it was running, and runs a routine to increment a counter.  The computer then resumes running the program it was running before it was interrupted.

The polyFORTH clock interrupt routine is made as simple as possible to reduce the clock overhead.  Usually the polyFORTH clock interrupt routine merely increments a double-precision variable called TICKS.  All other functions (setting the clock, hour/minute calculation, time overflow at midnight) are performed by clock words that access TICKS at irregular intervals (from applications, for example).

The word TICKS pushes onto the stack the address of a double-precision integer containing the current time of day in units of actual clock ticks (milliseconds, sixtieths of a second, or whatever, depending on the actual clock hardware).  The stack effect of TICKS is ( - a).

The most important hardware-dependent variable in the design of clock routines is the number of ticks per second.  The two most common rates are power-line frequency, 50 or 60 ticks per second (a popular timebase), and 1000 ticks per second.  With a 60 Hz clock, the clock can run about 820 days before TICKS will turn over to zero.  A 1000 Hz clock can run about 49 days before TICKS will turn over.

The word @TIME returns the current double-precision value of TICKS {stack effect: ( - d)}.  It disables the clock interrupt while picking up the two cells of the counter, to ensure a tick doesn’t occur between fetches.  @TIME should always be used to fetch the time of day.  In addition to protecting against an interfering interrupt, it also checks for midnight, and updates TODAY as needed.

REFERENCES

Time Overflow at Midnight, Section 3.6.6

### 3.6.2	Setting the Clock

On systems with clock hardware, polyFORTH provides a facility to set the system clock.  The time of day is set by the word HOURS.  HOURS is executed with the current time of day on the top of the stack.

Some hardware clocks cannot be turned off when the new value of TICKS is stored.  Thus, potentially, on these machines a clock interrupt could occur between the operations which store the new low and high order cells of TICKS. This is not usually significant because the new low-order value is stored first.  On extremely rare occasions, the new low-order cell of TICKS will be 65535, and the two-cell “store” operations will be broken by an interrupt, resulting in loss of the “carry”, and giving a substantial clock error if the clock interrupt cannot be disabled.  On systems with 60-cycle clocks this time occurs for 2 microseconds (the approximate time to store a cell on many machines) every 18 minutes, and the error can occur once a day. All systems that can will circumvent this possible error by momentarily disabling the clock interrupt.

The current time of day is entered as a double-precision integer.  In the following example, hh is the hour (0-23) and mm is the minute (00-59).

hh:mm HOURS

If the time of day given to HOURS contains an invalid hour or minute specification, the results are unpredictable and times printed by the system may be meaningless.

HOURS separates the hours and minutes out of the double-precision number, and computes the corresponding number of ticks (depending upon the hardware clock frequency).  The result is stored into TICKS.  HOURS can be easily user-modified to allow setting the time to the nearest second.

REFERENCES

Time Overflow at Midnight, Section 3.6.6

### 3.6.3	Timed Events

polyFORTH provides the ability to use the system clock to time events, both in the sense of specifying when something will be done and measuring how long something takes.

The word MS causes a task to suspend its operations for a specified number of milliseconds, during which time other tasks can run.  For example if you have an application word SAMPLE which records a sample, and you want it to record a specified number of samples, one every 100 ms (ten times per second), you would write a loop like this:

: SAMPLES   ( n)   0 DO  SAMPLE
   100 MS  LOOP ;

Since MS does its timing using the system clock, the accuracy of the interval measured depends on the resolution of that clock.  If the above example were run on a version of polyFORTH (such as the Native system for the IBM-PC) which sets the clock to “tick” at 1 ms intervals, the 100 ms will be very close to 1 ms accuracy.  The polyFORTH running on the IBM as a co-resident OS with MS-DOS, on the other hand, is limited by the fact that DOS requires the clock to tick only every 55 ms.  With this resolution, the “100 ms” interval may be anywhere between 56 and 110 ms.  As a general statement, the error on an interval will be approximately the number of milliseconds in one clock tick.

To obtain better accuracy, you may use a clock with shorter intervals.  If you need to respond promptly to an external event, the best way is to associate an interrupt directly with the event.

REFERENCES

Interrupts, Section 6.11; The CPU Supplement
PAUSE, Section 4.2

### 3.6.4	Measuring Elapsed Time

polyFORTH supports two words allowing the user to measure the  elapsed time interval between two events in milliseconds:

Command	Action

COUNTER	Returns on the stack the low-order cell of TICKS.

TIMER	Repeats COUNTER, then subtracts the two values, and displays the time interval since the previous execution of COUNTER, in milliseconds.

For example, the following code would be used to measure the execution time of 1000 executions of the user word xxxx:

: MEASURE   COUNTER  1000 0 DO
   xxxx LOOP  TIMER ;

This measurement also includes the time required to handle the DO ... LOOP itself.  To arrive at a precise value, you may compute the overhead by running the following loop:

: OVERHEAD   COUNTER  10000 0 DO
   LOOP  TIMER ;

Then subtract one tenth of this time from the time you obtained with MEASURE to get the actual time for 1000 executions of xxxx.

To figure times with an arbitrary number of iterations, divide the figure given by TIMER by the number of iterations.  To do the arithmetic in Forth, type:

t 100 n */ .

where t is the time given by TIME and n is the number of iterations.

This yields the number of 1/100s of a millisecond per iteration.  Note the maximum clock error per iteration (in the same units) may be calculated by:

1000 100 Hz */ iterations /

where Hz is the clock frequency (interrupts per second).

High-precision benchmarks with nested loops are left as an exercise for the reader.

### 3.6.5	Time of Day Output

The current time of day may be printed by the word .TIME.  An example of the use of .TIME is:

: CLOCK   BEGIN PAGE  @TIME .TIME
   30000 MS  AGAIN ;

This word prints the time as a five-character string followed by a blank. The time of day is printed only to the most recent minute.  The appropriate phrase to print the current time of day is:

@TIME .TIME

.TIME calls the word (TIME).  The word (TIME) expects a time of day, in internal representation, on the stack. It converts this double-precision number into a string of the format hh:mm and leaves the address and length of this string on the stack.  The stack effect is ( d - a n).  (TIME) is used to format a time of day for output using TYPE or for some other application use.  (TIME) may be used to print a stored time of day or to print the current or stored time with the report generator.

Although it is possible to fetch the internal time representation with the phrase:

TICKS 2@

a much better way, which automatically handles time overflow at midnight, is the word:

@TIME

Since @TIME handles the time rollover at midnight, the phrase @TIME .TIME should precede the system date display to ensure accuracy after midnight.

REFERENCES

Time Overflow at Midnight, Section 3.6.6

### 3.6.6	Time Overflow at Midnight

In polyFORTH, when the clock interrupt routine increments through midnight, no special action is taken and the current time of day becomes 24:00.  As the clock interrupts continue, TICKS continues to be incremented, causing an invalid time to be maintained.

Of course, the polyFORTH clock interrupt could easily be programmed to prevent this, but execution overhead would increase because of the need for constant checking for day rollover.  polyFORTH uses a different scheme to minimize overhead yet ensure correct dates.

The way polyFORTH resets a clock after an overflow past midnight is with the word @TIME.  @TIME returns the internal representation (stored in TICKS) on the stack, so it is convenient to use @TIME in any routine using the contents of TICKS.  For example:

@TIME .TIME

is equivalent to:

TICKS 2@ .TIME

except that the first time that @TIME is used in a new day, @TIME automatically adjusts the value in TICKS to be the correct time for the new day, and increments the MJD in the variable TODAY.

REFERENCES

MJD, Section 3.5

## 3.7	The Terminal Driver

Forth supports a variety of means to perform I/O with a terminal, printer, or other serial-type I/O device.  In addition, a simplified method is provided to make use of cursor positioning, and other hardware-dependent features, without forcing the use of particular models of terminal.

The general scheme of the TYPE and EXPECT interrupts is explained in this section, with a Dijkstra diagram (Fig. 3.8) of a representative implementation .

REFERENCES

EXPECT, Section 3.7.1
TYPE, Section 3.7.3

### 3.7.1	Terminal Input Commands

What follows is a table of specialized words that handle input from serial devices:

Word	Stack	Function

EXPECT	( a n)	Gets n characters from the task’s serial device, echoes each and places them in memory beginning at a.  The process will also stop if EXPECT sees a carriage return.  An example of use is

PAD 5 EXPECT  12345 ok

EXPECT is used for most terminal input.  EXPECT will “back up” over previously input characters when it receives an ASCII BS (8) or RUBOUT (7F).  When the character pointer points to a, the original address, EXPECT stops backing up and will thereafter echo an ASCII BELL (7) for each RUBOUT or BS it receives. The number of characters that have been input by EXPECT is available through the phrase:

SPAN @

STRAIGHT	( a n)	Gets n characters from the task’s serial device without echoing them, and places them in memory beginning at a.  STRAIGHT ignores carriage returns, backspace characters, and all special characters.  STRAIGHT is most often used to transfer binary data over a serial link.

KEY	( - b)	Accepts exactly one byte of data from a serial link.  KEY calls STRAIGHT so it doesn’t echo.  KEY is sometimes used for input prompting and in serial protocols.  KEY is also often useful to interactively determine the numeric value of a character:

KEY .	67 ok

Word	Stack	Function

?KEY	( - b/0)	Checks whether a character has been received on the task’s serial device since the last call to EXPECT, STRAIGHT, KEY, or ?KEY.  If so, the value of the character received is returned; if not, a zero (“false”) is returned.  The returned value is primarily used as a truth flag; if you have interest in the actual character value you should use a phrase such as:

?KEY ?DUP IF ...

Aside from returning the key value, ?KEY does not retain the value.  Use of KEY following ?KEY will await another keystroke.

REFERENCES

String Operations, Section 2.2

### 3.7.2	Basic Principles of Terminal Input

Most terminal input is accepted by the word EXPECT.  EXPECT is a routine which vectors execution to some version (potentially different for every terminal task) of (EXPECT) through the user variable 'EXPECT, which contains the address of the parameter field of (EXPECT).  EXPECT takes an address and length from the stack, and inputs a string of characters to memory starting at the byte whose address was on the stack.  An example of the use of EXPECT is:

PAD 5 EXPECT	12345 ok

This example puts five (12345) characters into memory starting at PAD.  If a carriage return is typed, the string that was input will be truncated to the number of characters preceding the carriage return.  In the following discussion, note that the standard text input interrupt has no provision for an “escape from program” function.  This omission is intentional, so applications can be written to be totally “user-proof.”

On most systems, EXPECT and STRAIGHT share interrupt code (see Section 3.7.1 for a discussion of STRAIGHT).  In addition, systems dealing with X-ON and X-OFF must be capable of taking action when those characters are received.  Thus, serial interrupt routines must often deal with seven cases of possible input (see Fig. 3.8):

1.	Normal characters (for EXPECT).

2.	Carriage Return (0DH)—resume execution (for EXPECT).

3.	Backspace or delete (08H or 7FH) with a partially filled input area (for EXPECT).

4.	Backspace or delete with an empty input area (for EXPECT).

5.	Binary serial input (for STRAIGHT).

6.	X-OFF (13H, CTRL-S)—turn off output stream.

7.	X-ON (11H, CTRL-Q)—turn on output stream.

The first case, normal characters, is processed by the following steps:

1.	The character is input (usually from a device to an accumulator).

2.	The address of the terminal’s task’s user area is calculated so that the interrupt routine can access the task’s user variables, especially CTR, PTR, and SPAN.

3.	Is the user variable CTR less than zero?  If it is, then the system is performing input.  Positive values in CTR indicate that the system is performing output.  In this case (normal EXPECT), the result is always yes.

4.	Is the STRAIGHT flag set?  If the straight flag is set, the CPU may jump directly to step #10—see the case about STRAIGHT.

5.	The parity bit is cleared.

6.	The character is tested to see if it is BS or DEL (this test fails for the normal case of EXPECT).

7.	The character is tested to see if it is CR (this test fails).

8.	The character is echoed.

9.	One is added to the user variable SPAN.  SPAN is a count of the number of characters input.

10.	The character is stored to the address in the user variable PTR.  PTR is short for PoinTeR.

11.	One is added to PTR so the next character will be stored properly.

12.	One is added to the user variable CTR.  CTR contains the two’s complement of the number of characters that may yet be stored without overrunning the input area.  Adding one brings CTR closer to zero.  CTR is short for CounTeR.

Fig. 3.8

D-Diagram of <TYPE> and <EXPECT> for PDP-11 polyFORTH with X-ON/X-OFF processing.

13.	If CTR is zero (the input area is full), then WAKE is stored in the status area of the terminal’s task, to awaken the terminal task.

14.	The CPU returns from the interrupt.

The second case, a carriage return, is processed by the following steps:

1.	1 through 6 are identical to the normal character case, above.

7.	The character is tested to see if it is a carriage return.  The code is shared, but in this case the test succeeds.

8.	A space is echoed.

9.	CTR is set to zero.

10.	WAKE is stored in the status area of the terminal’s task, to awaken the terminal task.

11.	The CPU returns from the interrupt (shared with normal case).

The third case, backspace or delete with a partially filled input area, is processed by the following steps:

1.	1 through 5 are identical to the normal character case.

6.	Tests the character to see if it is BS or DEL.  This test is shared with the normal characters case, but succeeds in this case.

7.	SPAN, the number of characters input, is tested to see if it is zero (this test fails).

8.	A backspace is echoed.

9.	PTR, which points to the next empty byte of the input area, is decremented by one.

10.	CTR, which contains the two’s complement of the number of bytes remaining empty in the input area, is decremented by one.

11.	SPAN, the number of characters input, is decremented.

12.	The CPU returns from the interrupt (shared with normal case).

The fourth case, backspace or delete with an empty input area, is processed with the following steps:

1.	1 through 5 are identical to the normal character case.

6.	Tests the character to see if it is BS or DEL.  This test is shared with the normal characters case, but succeeds in this case.

7.	SPAN, the number of characters input, is tested to see if it is zero.  This test is shared with the BS-or-DEL-and-input-area-partially-full case, but succeeds in this case.

8.	An ASCII BELL is echoed, to inform the typist there are no more characters to delete.

9.	The CPU returns from the interrupt (shared with normal case).

The word STRAIGHT is a special modification of EXPECT.  STRAIGHT takes the same stack arguments as EXPECT, but completely ignores the content of the data it moves.  STRAIGHT is often used when transferring binary data over a serial link.  On most systems, STRAIGHT’s interrupt routine is integrated with EXPECT’s interrupt routine by testing for a “STRAIGHT flag” in step 4 of EXPECT’s “normal character” case.  The STRAIGHT flag must be in the user variable area, so no interference occurs when several terminal tasks are in operation.

For case 5, the steps that STRAIGHT’s interrupt routine perform are:

1.	1 through 3 are as in EXPECT’s normal character case.

4.	Tests a “STRAIGHT Flag” in the user variable area.  This test succeeds. In systems where STRAIGHT and EXPECT do not share interrupt code, this step is not performed.

5.	The CPU jumps to step #10 of the “normal characters” case of the EXPECT interrupt, skipping all of the input testing steps.  In systems where STRAIGHT and EXPECT share no interrupt code, Steps 10 through 14 are independently reproduced.

In case number six (the “X-OFF” case), the system is performing output when it receives an X-OFF character (13H).  The steps performed are:

1.	1 and 2 are identical to the normal character case.

3.	CTR is found to be non-negative, meaning that the system is performing output.

4.	The parity bit is cleared.

5.	The input character is found to be X-OFF (13H), meaning the device receiving output is getting full.

6.	A type-interrupt-disable flag in the terminal task’s user area is set.  When the type interrupt checks this flag, the type interrupt routine does nothing (a character is not transferred).

7.	The CPU returns from the interrupt (shared with the normal case).

In case number seven, the “X-ON” case, the system was performing output, and the output was turned off when the receiving device got full and sent an X-OFF. Now the receiving device has digested its data and it has sent an X-ON to tell the computer to resume transmission.  The steps performed are:

1.	1 and 2 are identical to the normal character case.

3.	CTR is found to be non-negative, meaning the system is performing output.

4.	The parity bit is cleared.

5.	The input character is found not to be an X-OFF (13H).

6.	The input character is found to be X-On (11H), meaning output may be resumed.

7.	The type-interrupt-disable flag is reset, so the type interrupt will transfer characters.

8.	If CTR, the count of characters to output, is zero, then return from the interrupt (don’t do Step 9).

9.	Output a character, and restart the character transfer process.

REFERENCES

Vectored Routines, Section 3.1

### 3.7.3	Terminal Output—High Level Discussion

All terminal output occurs by means of the word TYPE.  TYPE is a routine vectoring execution to some version (potentially different for every terminal task) of (TYPE) through the user variable 'TYPE.  'TYPE contains the address of the parameter field of (TYPE).  TYPE uses an address and a count, storing the address into the user variable PTR, and storing the count into CTR.  After the stores, on systems with interrupts, TYPE initiates the TYPE interrupt routine <TYPE>, and enters the multitasking loop.

TYPE is used by three other routines of interest.

Word	Description

>TYPE	Uses an address and a count (like TYPE) but copies the string into PAD before typing it out.  >TYPE is used in the EDITOR to type lines from the block buffer being edited.  Because >TYPE calls TYPE, which enters the multitasking loop (as all input/output should), there is no guarantee that the contents of the block buffer will remain unchanged as the string is typed, so >TYPE copies a string into the user’s PAD and types the string from there.

MSG	A defining word used to output short, unchanging strings of characters to the terminal.  MSG is used on most systems to define (CR), (PAGE), and other short bursts of control characters.  MSG is used as follows:

MSG name nn C, cc C, ... cc C,

where name is the name of the new word, nn is a byte telling how many characters to output, and each cc is the value of an ASCII character.

EMIT	Outputs a single character from the least significant byte of the top of the stack, and then pops the stack.  EMIT is often useful for initial “cut and try” definitions.  After a sequence of characters is proven useful, it may be redefined using MSG, usually taking less space.

REFERENCES

String Output, Section 2.3.6.4
Vectored Execution, Section 2.4.8

### 3.7.4	Terminal Output—Low Level Discussion

On most systems, TYPE makes use of an interrupt routine.  The TYPE interrupt routine (or TYPE, on systems without interrupts) transfers a character each time the terminal interface interrupts the CPU (or is ready).  The terminal interface interrupts when it is ready for another character.  In order to transfer a character, the TYPE interrupt routine must:

1.	Find the correct task’s user area so the TYPE interrupt can find CTR (the user variable containing the count of the number of characters to output), and PTR (the user variable pointing to the next character to output);

2.	Output the character pointed to by the task’s PTR;

3.	Add 1 to the task’s PTR;

4.	Subtract 1 from the task’s CTR;

5.	If the task’s CTR is zero, wake the task, and disable the TYPE interrupt.

Some systems use the X-ON/X-OFF convention.  The X-ON/X-OFF handshaking sequence allows terminals with slow display-update logic to interface with fast baud rate interfaces over 3-wire transmission lines.  The handshaking sequence is as follows (see Fig. 3.8):

1.	The terminal’s data buffer (which may be as small as three characters) approaches fullness, forcing the terminal to send an X-OFF character (ASCII DC3, 13H);

2.	The computer receives the X-OFF and stops transmitting;

3.	The terminal empties its update buffer, and then sends an X-ON character (ASCII DC1, 11H).

4.	The computer receives the X-ON, and resumes transmission.

In a polyFORTH system not using X-ON/X-OFF handshaking, the TYPE and EXPECT interrupt routines are completely independent. In systems using X-ON/X-OFF, the EXPECT interrupt must turn the TYPE interrupt on and off.

In the implementation diagrammed in Fig. 3.8, <EXPECT> (the EXPECT interrupt routine) controls <TYPE> (the TYPE interrupt routine) by means of a flag in Bit 7 of the user variable SPAN.  The bit is set by <EXPECT> when <EXPECT> receives an X-OFF character.  When <EXPECT> receives an X-ON, the bit is reset, and if the user variable CTR is greater than zero, indicating more characters to TYPE, then <EXPECT> jumps directly into <TYPE>.

<EXPECT> must enter <TYPE> because the TYPE interrupt occurs when the interface is finished putting out a character to the terminal.  Since <TYPE> ignored the last TYPE interrupt (because of the “type disable” bit in SPAN), the interrupt/output … interrupt/output sequence must be restarted by outputting a character again.

REFERENCES

The EXPECT Interrupt Routine, Section 3.7.2

### 3.7.5	Support of Special Terminal Features

Each terminal task defined by TERMINAL has unique user variables, including a port address, or other device and system specific interrupt vectoring.  The design of polyFORTH assures that each terminal task has an associated, particular terminal.  Each terminal may have different control character sequences for the following functions:

CR	PAGE	TAB	MARK	CLEAN

These functions have their addresses stored in user variables, so public programs (such as the EDITOR) can use private definitions of CR, PAGE, TAB, MARK, and CLEAN by directed execution.

Two user variables are defined to track the location of a terminal’s cursor:  L# and C#.  L# contains the number of the line where printing is occurring (0-23 on most CRTs).

C# contains the column number where the next character will be placed (0-79 on most CRTs).  Most special terminal functions read or change L# and C#.

On most systems, these terminal functions are defined as the most recent definitions when the terminal task was created by TERMINAL.  When a terminal task is created, the default values of 'CR, 'PAGE, 'TAB, and 'MARK are compiled into the task’s initialization table, so they can be copied into the task’s user variable area when the task is constructed.  The initialization table is provided so systems can be target compiled into read-only memory.  Often it is convenient to initialize all terminals of a particular type by loading the new terminal definitions just before creation of all the terminal tasks servicing that particular type of terminal.

Note the convention followed by the names of the public functions, vectoring user variables, and device-specific primitives.  All systems-defined vectored routines follow this naming convention.  The terminal functions’ behavior is as follows:

Public	Vector	Primitive	Stack	Function

CR	'CR	(CR)		Adds one to L#, sets C# to zero, and sends a terminal command equivalent to a teletype’s carriage-return line-feed sequence.

PAGE	'PAGE	(PAGE)		Sets L# and C# to zero then clears the screen and homes the cursor on CRT-type terminals, and starts an empty page for all others.

TAB	'TAB	(TAB)	( l c)	Sets L# and C# with line and column numbers from the stack (with values between 0-23 and 0-79, respectively, for many CRTs) and positions the cursor appropriately, taking these values from the upper left-hand corner of a CRT-screen.  For example:

5 0 TAB

positioning the cursor at the first character of the sixth line printed by the terminal.

Public	Vector	Primitive	Stack	Function

MARK	'MARK	(MARK)	( a n)	Uses the same arguments as TYPE (an address and a count). .MARK performs the same function as >TYPE except MARK also highlights the text it types.  MARK goes through the following steps:

1.	Copies the text to PAD, so text from a block buffer can be printed without multitasking interference.

2.	Sets the terminal’s highlighting (underline or reverse video are preferred where possible) or, on terminals that do not highlight, emits a caret (5EH).

3.	Types the text.

4.	Unsets the terminal’s highlighting.

CLEAN	'CLEAN	(CLEAN)		Clears to the end of the line.

Your polyFORTH system is shipped with several terminal configuration blocks.  In addition, the default terminal configuration block is near the section of source code defining the screen editor. See these blocks and their shadow blocks for more detailed information.

REFERENCES

>TYPE, Section 3.7.3
Target Compilation of Tasks, Section 7.9
Terminal Tasks, Sections 4.8, 4.9, 4.10
Vectored Execution, Section 2.4.8
Vectored Routines, Section 3.1

## 3.8	The Forth Bootstrap

There are two basically different kinds of read/write memory:  volatile and non-volatile.  Volatile memory loses its information when its power is cut.  Most semiconductor memory is volatile.

To combat the problem of loading software into a powered-up computer, computer designers place a small, non-volatile read-only memory in the computer.  This ROM contains code which is executed when the CPU powers up.  Usually the code reads Track 0, Sector 0 of Disk 0 into memory, and then jumps to the beginning of the data it has read into memory.  The ROM is called “the system’s bootstrap ROM.”  The Data on Track 0, Sector 0 is usually called the “disk bootstrap.” The term “bootstrap” is derived from the expression “It pulls itself up by its bootstraps.”

polyFORTH is designed so the disk bootstrap and any associated device drivers reside in Block 0.  The polyFORTH disk bootstrap reads the pre-compiled nucleus into memory and jumps to the first byte of it.

The word RELOAD is a convenience available on some polyFORTH systems allowing a programmer to simulate a “cold start” without physically touching a power switch or reset button.  There are two ways RELOAD works:

1.	RELOAD enables the bootstrap ROM (if necessary—sometimes the bootstrap ROM can be “turned off” so it is not accessible), then RELOAD jumps to the power-up entry point in the bootstrap ROM.

2.	On some systems, the Forth system is in ROM, and RELOAD jumps to the first byte of ROM.

Whenever possible, RELOAD is designed not to erase block buffers, change the date variable (TODAY) or change the clock variable (TICKS).

Usually, if a bootstrap installation requires a special command, the command is called BOOT and is available in the DISKING utility.

REFERENCES

DISKING Utility, Section 5.3
Power-up Initialization, Section 7.11

# 4.0	MULTITASKING

Multitasking allows a computer to appear to be doing many things at once.  In particular, the pF/x multitasker is intended to provide service to multiple programs required to operate without any fixed timing relationship (i.e., asynchronously).

This section explains how tasks are constructed, how they are controlled and how the CPU is shared between them.

Two types of tasks are detailed:  background tasks and terminal tasks.  Both types are fundamentally identical.  A terminal task can be thought of as an extremely elaborate background task tailored to service a terminal and run the polyFORTH development environment.  Tasks in some applications require a subset of the facilities available in the development environment.  Terminal tasks can be easily “pruned” of unnecessary facilities.

REFERENCES

Target Compilation of Tasks, Section 7.9

## 4.1	Forth Re-entrancy and Multitasking

When more than one task can share a piece of code, the code has a property called “re-entrancy.”  Re-entrancy is a valuable property, because when tasks can share code, memory is conserved.

Non-re-entrant routines contain sections subject to change as the program runs.  Self-modifying code is not re-entrant. Routines with “private” variables are not re-entrant.  Re-entrant routines can have private constants (a constant’s value does not change).  Re-entrant routines can always be programmed into a read-only memory.

Forth routines are naturally re-entrant.  Most Forth routines keep their intermediate results on either the parameter stack or the return stack.  Programs to handle text or other tables can be designed to keep their tables in the section of random access memory allotted for each task.  A facility is provided to allow a programmer to define public routines to access variables, but still retain re-entrancy by accessing private versions of these variables in each task (such variables are called “user variables”).  Forth routines can be made completely re-entrant with very little effort.

Because Forth routines are naturally re-entrant, pF/x allows tasks to share routines in a single “public” dictionary.  This practice conserves large amounts of memory.  In most applications, all system and application routines can be shared (with the minor exception of the I/O instructions on certain processors).  Therefore, as few as 2048 bytes of read/write memory per terminal can support a satisfactory program development environment.  Terminal tasks not requiring a private dictionary (such as printer spooling tasks) can operate with 300 bytes.  The minimum size for a useful task with no private dictionary is about 250 bytes.  Some applications (PBX’s, process control, and some communications systems) naturally use large arrays of small tasks, with each task running a simple shared program.

REFERENCES

User Variables, Section 4.6

## 4.2	Principles of Operation

The polyPF/x multitasker is designed to fulfill several goals:

1.	Provide asynchronous execution of code.

2.	Be convenient to use.

3.	Be fast in execution.

4.	Be simple to understand.

5.	Use a minimum amount of memory.

6.	Be independent of particular hardware configuration (e.g., a clock is unnecessary).

The pF/x multitasker satisfies 2, 3 and 4 above by consisting of only about 13 words.  Number 5 is a consequence of Forth’s basically re-entrant structure (see “Forth Re-entrancy,” referenced at the end of this section).  Numbers 1 and 6 are assured by the way Forth schedules tasks.  Forth services tasks when an executing task stops to await some form of I/O.

Simplicity and high performance are assured because control is passed from one task to another only at known, programmer-controllable points, and always between Forth words.  This principle greatly simplifies the context-switching operation (thus reducing overhead) and simplifies the programmer’s task of writing routines for a multi-user environment.

The rest of this section is a detailed discussion of the scheduling algorithm, its associated words and some useful techniques.  For a discussion of other words, see their associated sections.

A “round-robin” algorithm (see Fig. 4.1) schedules processor time.  Each task has a turn in control until it executes the high-level words PAUSE or STOP, or the assembler code ending WAIT. Most words performing asynchronous hardware operations (e.g., TYPE, EXPECT, BLOCK, and BUFFER) contain a WAIT or a jump to PAUSE, so that while a task is waiting for an I/O operation to be completed other tasks can use the CPU.  Since Forth is naturally very fast, tasks tend to spend much of their time awaiting I/O.  Tasks that are performing extensive computations may be prevented from impacting overall system performance by using PAUSE in a few regularly executed words.  The PF/x multitasker is said to be “I/O driven.”

The following words control use of the CPU by tasks:

Word	Function

PAUSE	Suspends the task that calls PAUSE to allow all other tasks one turn in control of the CPU.

STOP	Puts the task that calls STOP to sleep until that task is awakened by an interrupt routine or by some other task.

WAIT	An assembler code ending that behaves exactly as STOP.

The round robin is implemented as an endless loop of jump instructions (see Fig. 4.1).  Each task has its own jump instruction, which transfers control to the jump instruction of the next task.  When a task is going to awaken, the task’s jump instruction is replaced by an instruction to transfer control to the machine code to awaken the task.  This special instruction is usually called WAKE, and is often a trap instruction.  The round robin is sometimes called “the PAUSE loop,” “the idle loop,” or “the multitasking loop.”  The address of the first byte of a task’s jump instruction is pushed on the stack by the high-level Forth word STATUS.  In assembly code, the address of the current task’s STATUS is available in a register or cell called U.  The address used by the jump in STATUS is located at STATUS 1+ on most 8-bit processors, at STATUS 2+ on most 16 and 32-bit processors.

Fig. 4.1

Schematic of the relationship between the word PAUSE and the multitasking round robin.  PAUSE does not enter the round robin at a fixed place.  PAUSE always runs the following task.

Interrupt routines are often used to awaken tasks.  For example, whenever the interrupt routine for EXPECT sees a carriage-return, the interrupt routine awakens the terminal task associated with the terminal.  A common way to perform complex non-critical interrupt servicing is to have the interrupt routine perform all time-critical operations, and then store WAKE into a task’s STATUS.  When the round robin gets around to the task, the task resumes execution after the previous STOP, and runs until it executes another STOP or WAITs for an I/O operation. Tasks typically perform asynchronous operations such as data reduction and logging.  An example of a data reduction loop being run by a dedicated task might be:

: COLLECT   BEGIN ACCEPT DATA REDUCE
   STORE STOP AGAIN ;

The interrupt routine that services the data source for this example will awaken the task out of the word STOP, by storing WAKE in the task’s STATUS when data is ready to be accepted.  The specific method for doing this is discussed in the CPU Supplement for each system.

The most general case of changing turns in the round robin occurs when control of the CPU is relinquished from high-level Forth, with arrangements for the task to automatically awaken and resume execution on its next turn. PAUSE performs this most general case.  PAUSE can be embedded in complex calculations which perform no I/O and which might otherwise cause a particular task to control the CPU for undesirably long periods.  Consider the following collision orbit calculation for example:

: POSITION   X STEP  Y STEP ;

: ?COLLIDE   30000 0 DO  POSITION HIT
   PAUSE LOOP ;

In this example the word STEP is assumed to have been defined to perform the calculations for integrating the next step in the target X or Y coordinate.  HIT expects the coordinate of the target (computed by POSITION) on the stack and performs appropriate course corrections.  Since all of these computations are time consuming and since other functions must be running concurrently, it is desirable to give up the CPU for one turn around the round robin for each step in the integration.  Inserting the PAUSE in the loop accomplishes this.

The related words, STOP and WAIT (see Fig. 4.1) share much of the code for PAUSE.  These are the steps that PAUSE performs:

1.	The WAKE instruction is stored into STATUS, replacing the round-robin jump.  This step ensures that the task executing PAUSE will awaken on its next turn.  On many machines, WAKE is equivalent to a subroutine jump to the code for step 5, below.

2.	The system pointers I and R are saved by being pushed onto the current task’s parameter stack.  This portion of the code is the entry point for STOP and WAIT, which by skipping step 1 do not automatically resume execution of the current task on the next turn.

3.	The parameter stack pointer (S, in assembler) is saved in a reserved location in the user area.  At the completion of this step, all non-recoverable unshared task data for the address interpreter has been saved.  Since tasks only relinquish the CPU between Forth words, other registers do not have to be saved.

4.	The CPU jumps to the location whose address follows STATUS (which is the next task’s STATUS), and proceeds to jump through the circular round-robin loop until a WAKE instruction is encountered.  The WAKE transfers control to step 5.

5.	The address of the new task’s STATUS is stored in U.  In machines that use a JSR for the WAKE instruction, the address of STATUS can be calculated using the address that WAKE left on the subroutine linkage stack.  Some way of obtaining the STATUS address is always available.

6.	Using U to find the task, the parameter stack pointer is restored, and then I and R are restored from the new task’s parameter stack.

7.	A jump to the next task is stored into the current task’s STATUS to make the current task’s state “don’t awaken.”

8.	Finally, NEXT is executed, which invokes the Forth word indicated by the new task’s I.

REFERENCES

Assembler Code Endings (WAIT), Section 6.2
Forth Re-entrancy and Multitasking, Section 4.1

## 4.3	Defining a BACKGROUND Task

BACKGROUND tasks have a parameter stack, a return stack, and space for variables which are not shared (these are called “user” variables).  BACKGROUND tasks do not service a terminal or have a private dictionary.

The word BACKGROUND allocates memory in the dictionary and sets up a two-cell task definition table for a BACKGROUND task (see Fig. 4.2).  BACKGROUND is a defining word which expects the sizes of the user variable area (sometimes called “the user area”), the parameter stack, and the return stack.  All sizes are in bytes.  An example of BACKGROUND’s use is:

16 64 48 BACKGROUND SCRIBE

This defines a task whose name is SCRIBE, which has 16 bytes of user area, 64 bytes of parameter stack and 48 bytes of return stack.  Task names should be job assignments (as here) or proper person’s names.

In the example above, 16 will be the number of bytes reserved for the user area.  The smallest possible user area is 16 bytes, and contains the task JMP in STATUS (one or two bytes), the address of the next task in the round robin (two bytes), the stack pointer (two bytes) and S0, the bottom of the parameter stack (two bytes).  To determine the minimum size for your system, see your CPU Supplement.  The three extra cells in SCRIBE’s user area could be used by SCRIBE’s program to keep a sample count, a virtual array address, and a device I/O address for a data logging application.

The 64 in the example indicates the parameter stack can have a maximum size of 32 single-precision integers or 16 double-precision integers.  (On 32-bit systems the maximum size would be 16 single-precision or 8 double-precision integers.)  Note that there must be sufficient space to store I and R on the parameter stack, as well as whatever arguments are present when the task enters the multitasking loop.

Fig. 4.2

Layout of memory allotted for a background task.  The task definition table is compiled by BACKGROUND.  The executable portion of the task is compiled by BUILD.

The 48 indicates the program can nest Forth words, loop parameters, etc. to a depth of 24 return stack entries (12 entries on 32-bit systems).

The total size of SCRIBE in this example is 130 bytes, excluding the dictionary head, because a cell containing the address of SCRIBE’s STATUS is added by the word BACKGROUND.  In addition, a cell is compiled just after the STATUS address that tells where the new task’s S0 should be.  The data to calculate S0 is available when BACKGROUND is executed to compile the task definition table, but need not be preserved after the new task has been fully initialized.  Thus the cell for the value of S0 is included in the space allotted for the parameter stack. The runtime behavior of words created by BACKGROUND is to return on the stack the address of the task definition table, the first cell of which contains a pointer to the first byte of the task’s STATUS.  Thus, the location of SCRIBE’s STATUS is found by the phrase:

SCRIBE @

If a task has an associated interrupt routine (as terminal tasks do, for example), the interrupt routine may be defined or linked to the task when the task’s memory is allotted by an application word performing BACKGROUND plus whatever additional functions are needed at this time.  When many generically similar tasks need to be defined, a word combining the functions of memory allotment and interrupt-routine-linkage is the most convenient way to define them.  The combination is convenient because memory allotment produces the addresses needed for linking the interrupt-routine to a task.

BACKGROUND allots space for a task in the dictionary, but does not link the task into the round robin, or make the task run a program.  This will be described in the next section.

When target-compiling systems with tasks in read-only-memory, BACKGROUND must be executed as part of the target compilation process. BACKGROUND must be part of target compilation because BACKGROUND allots memory in RAM and creates the table in ROM that will be used after power-up in the target system to build the task’s user area and stacks in RAM.

In a resident environment, tasks are usually defined and initialized at the same time, during the loading of system electives (controlled by Block 9).

REFERENCES

Initializing a BACKGROUND Task, Section 4.4 
I and R—Address Interpreter, Section 1.2.5
Making a BACKGROUND Task Run a Program, Section 4.5
Target Compiling Multiprogrammed Applications, Section 7.9

## 4.4	Initializing a BACKGROUND Task

When pF/x starts up, at least one task exists.  This task is a terminal task called OPERATOR.  If no other task exists, OPERATOR’s jump address will contain the address of OPERATOR’s STATUS.  In this way, the round robin first consists of a single jump instruction, which jumps to itself.

The word BUILD initializes the user area for a new background task and links the task into the round robin.

1.	BUILD copies the complete jump instruction, including the address of the next task, from the STATUS of OPERATOR into the STATUS of the new task.

2.	BUILD stores the address of the new task’s status in OPERATOR’s jump address, so that the CPU jumps from OPERATOR to the new task to the task which OPERATOR formerly preceded.

3.	BUILD gets a copy of the value for S0 (the bottom of the new task’s parameter stack) calculated by BACKGROUND and compiled just after the STATUS address, and stores the new task’s value for S0 into the user variable S0.

In a resident system tasks should be defined and initialized when the electives are loaded from Block 9 by the command HI.  Conventionally, task definitions are in Block 36, possibly also 37.  It is not good practice to define a task in a private terminal partition, since an EMPTY would “forget” the task and break the round robin.  Definitions in the public dictionary cannot be forgotten, and thus the tasks defined there are safe.

Here is an example of how to define a task, and then link it into the round robin.  When the electives are loaded, the task is defined by the phrase:

16 64 48  BACKGROUND SCRIBE

In a target-compiled system, task initialization is usually part of the power-up sequence.  In a resident system, task initialization is usually performed during the electives load, just after task definition:

SCRIBE BUILD

SCRIBE leaves the address of the task definition table on the stack.  This is used by BUILD to set up the task’s stack and user area in RAM.

Although the task exists and is part of the round robin, note that it is not yet running a program.  The words the task executes may be defined much later.

REFERENCES

Defining a BACKGROUND Task, Section 4.3
Making a BACKGROUND Task Run a Program, Section 4.5

## 4.5	Controlling a BACKGROUND Task

After a task has been defined, with memory allotted to it by BACKGROUND, and is linked into the round robin by BUILD, the new task is asleep.  This is necessary, because the task still has not been given the values for I and R that determine what word(s) the address interpreter will execute for the task.

The word that makes a task run a program is ACTIVATE.  ACTIVATE expects on the stack the address of a task definition table, as returned by executing a task name.  ACTIVATE clears the parameter and return stacks of the task, and then awakens the task with a value of I that points to the word immediately following ACTIVATE.  ACTIVATE must be used in a : definition.  The following example performs all initialization required to run a BACKGROUND task.  The words following ACTIVATE must end in STOP or an endless loop.

Assume that a background task was defined and initialized during the electives load, by the following phrases (described in previous sections):

16 64 48 BACKGROUND SCRIBE
SCRIBE BUILD

When the application is loaded, the following definition will use a specified task to record data samples onto disk:

: RECORD ( a)   ACTIVATE
   BEGIN  COLLECT >DISK  PAUSE  AGAIN ;

The above version of RECORD would be used in the following phrase:

SCRIBE RECORD

The words in the definition of RECORD between ACTIVATE and ; represent a definition that performs a hypothetical data logging activity.  The word ACTIVATE uses the address of a task definition table from the stack (put there by executing the task’s name SCRIBE) and forces that task to execute the words following ACTIVATE in the colon definition containing ACTIVATE.  In the above example BUILD is kept separate from RECORD because a task can only be built once, but can be activated many times.

For convenience in the following discussion, a “slave” task is the task ACTIVATEd by some other task.  The “master” task is the one executing ACTIVATE to activate the slave task.  These are the actions taken when the phrase SCRIBE RECORD is executed:

1.	The slave task (called SCRIBE in the example) has its return stack emptied.  If the slave task were put to sleep by PAUSE, the value of the slave’s return stack pointer would be placed on the slave’s parameter stack.  ACTIVATE simulates PAUSE’s action, by storing the address of the bottom cell of the slave’s return stack into the bottom cell of the slave’s parameter stack.  The address of the bottom cell of the slave’s return stack is calculated from the address of the slave’s STATUS cell.  These cells are adjacent (see Fig. 4.2).

2.	The address in the master task’s I when ACTIVATE is called is put in the slave task’s stack where it would have been saved by PAUSE.  Thus when the slave awakens its I will point to the address of COLLECT in the definition of RECORD (no address is compiled for the BEGIN).

3.	This step essentially leaves the slave’s parameter stack empty except for the new values of I and R stored there.  The slave’s parameter stack pointer is set to point to the slave’s new value for I on the parameter stack.  The slave’s parameter stack pointer is saved in the slave’s user area until the slave begins executing.

4.	A WAKE instruction is stored into the slave’s status cell.

5.	The master task now finishes running ACTIVATE.  The last step the master task must perform is to leave the colon definition containing ACTIVATE without executing any more of that definition’s code.  Therefore, the last step of ACTIVATE is to jump to EXIT instead of NEXT.

Note that code following an ACTIVATE must never reach the ; because the EXIT which is compiled by a semicolon will attempt to pop an empty return stack.  Therefore, the code following an ACTIVATE must end either in an endless loop (as in the example) or in the word STOP.

Since neither the master nor the slave task ever execute the EXIT compiled by RECORD’s ;, it is common to follow such a ; with the word RECOVER.  RECOVER backs up the dictionary pointer, removing the un-needed EXIT.

Another example of a control word, this time using the word STOP, is:

: HALT ( a)   ACTIVATE STOP ;

Because ACTIVATE forcibly resets a task’s execution environment (empties both stacks and sets I), the definition of HALT given in the example above will forcibly STOP a specified task.  Although HALT is occasionally useful, it is not a predefined Forth word because a slightly different definition is useful for terminal tasks.

REFERENCES

BEGIN, Section 2.3.1
Definition of HALT for a Terminal Task, Section 4.10
PAUSE, STOP, and WAIT, Section 4.2

## 4.6	User Variables

In pF/x, many tasks can share the code for the address interpreter, I/O drivers, etc.  Each task will have different data for these facilities:  the EDITOR, for example, needs to remember which line the user is editing.  The fact that all users have private copies of the variables SCR and CHR, which control the editing cursor, enables them to edit concurrently without conflicts.

pF/x accesses private variables through a facility called USER variables.  USER variables are accessible to programs residing in the public dictionary, yet they are not shared between tasks.

There are two ways to define USER variables.  This is the “absolute” form:

0 USER STATUS   10 USER S0   12 USER 'IDLE

The number preceding the word USER is an offset in bytes from any task’s STATUS.  The name of the variable follows USER.  The compile-time behavior of USER is like a constant:  the number is compiled immediately after the head.  The run-time behavior is to add the number to the system pointer U to produce an address in the task’s private user variable area.  U contains the address of the first byte of the task’s round-robin jump instruction.  The run-time code for user variables is written in assembly language and uses the U register (or location), rather than the word STATUS, but a high-level definition of USER could be written as:

: USER ( n)   CREATE ,  DOES> ( a)
   @  STATUS + ;

The second method of defining USER variables is the “relative” method, using the defining word +USER.  This is most appropriate when a group of user variables is being defined.  +USER expects on the stack an offset in the user area plus a size (in bytes) of the new USER variable being defined.  A copy of the offset will be compiled in the definition of the new word, and the size added to it and left on the stack for the next one.  Thus, when specifying the series, all you have to do is start with an initial offset and then specify the sizes.

For example, the standard pF/x USER variables region on an 8086 begins like this:

0   6 +USER STATUS   2+ 2 +USER S0
   2 +USER 'IDLE ...

What is compiled is equivalent to the previous example.  However, it is convenient to see explicitly that the task status area is 6 bytes long (two bytes for the jump and four for the “follower” task address), followed by two more bytes (for the stack pointer “save” location) and then S0, etc.

When you are finished defining +USER variables, you should DROP the offset.  If you are defining an additional group of user variables, you may pick up the offset from the last one by a phrase such as:

' STATUS C@

(' STATUS W@ on 32-bit systems) replacing the initial 0 in the previous example.

The absolute method (USER) is most appropriate when re-naming an existing user variable; for example, if TERMINAL tasks in your application will not be using the polyFORTH editor, you may use the space occupied by its user variables for application user variables.

Terminal tasks generally have about 128 bytes (256 on 32-bit systems) of user area, devoted to I/O, task management, dictionary management, private text interpretation, and the editor.  The minimum size for a background task’s user area is 16 bytes (see your CPU Supplement) for task management.  The task management bytes usually consist of:

1.	A jump instruction to the next task’s jump instruction.

2.	A save area for the task’s stack pointer while it is inactive.

3.	The address of the bottom of the parameter stack (S0).

A task may need to initialize another task’s user variables, or read or modify them.  The word HIS allows a task to access another task’s user variables.  HIS takes two arguments:  the address of the task of interest, and the address of the executing task's user variable of interest.  For example:

2 CHUCK BASE HIS !

will set the user variable BASE of the terminal task named CHUCK to 2 (binary).  HIS subtracts the STATUS address of the executing task to get the offset and then adds the offset to the STATUS address of the desired task.

HIS is useful for initializations.  The sample data logger developed in Sections 4.3, 4.4, and 4.5 would be more useful if the user could specify the number of samples and did not have to specify the task to run the data logging.  The user variable definitions and COLLECT are included as examples of the use of user variables.  COLLECT reads data into a block buffer.  SHELF contains the block number of the next free disk block.

A/D is assumed to be defined to call a code routine to request a sample from an analog-to-digital converter and WAIT for an interrupt signaling the completion of the conversion.  The interrupt code will read the sample and leave it in a buffer where A/D will fetch it and leave it on the stack.

10 USER SHELF   12 USER #SAMPLES

: COLLECT   #SAMPLES @ 0 DO  A/D
   SHELF @ BLOCK I 2* + ! UPDATE  LOOP ;

: RECORD  ( n n2)
   512 MIN  SCRIBE #SAMPLES HIS !
   SCRIBE SHELF HIS !
   SCRIBE ACTIVATE  COLLECT  STOP [

This version of RECORD would be used with the block number and number of samples as its argument; the task address is built in (SCRIBE).  The following phrase would record 500 samples into Block 30:

30 500 RECORD

Note that in this more sophisticated version the PAUSE to enter the multitasker is replaced by the BLOCK in COLLECT, which contains a call to PAUSE.

Also note the use of [ instead of ; in RECORD avoids the compilation of an EXIT (; compiles an EXIT) which will never be executed.  See Step 5 of ACTIVATE (Section 4.5).

The map of user variables in Table 4.1 shows the user variables required by a fully interactive PF/x terminal task. Your CPU’s implementation may require variables not listed—see your implementation’s system and user variables block.

Although the actual order varies, all systems possess these user variables.  You can obtain the absolute address of Location 0 of a task’s user area by typing:

task-name @

See Block 198 for the exact organization of your user variables.  Generally, the most-used user variables have smaller offsets, so tasks needing only part of the user variables can have a minimal-sized user variable area.  Note also that the user variables that SEND moves from the master to the slave terminal task are grouped together.  Names in parentheses are used merely as identification for the reader; they are not Forth words.  To address these locations, use one of the Forth words in the table (such as SCR) and add or subtract the appropriate number of bytes.

Table 4.1

Map of a Typical User Area

Name	Content

STATUS	Indicates whether the task is ready to become active.

(Follower)	Address of the next user in the multi-tasking chain.

(S)	Stack pointer, saved from when the task was last active.

S0	Pointer to the bottom of the parameter stack and the start of the message buffer (for terminals).

'IDLE	Address of the “idle behavior” routine for tasks.

SPAN	Length of the string actually received by EXPECT.

CTR	Counter for character-oriented I/O

PTR	Pointer to the address for the current I/O.

The following are needed only for terminal tasks.

Name	Content

'KEY	Contains the most recent character received since the last EXPECT or STRAIGHT operation or 0 if none.

DEVICE	Terminal device address or other device information.

'EXPECT	Address of the task’s EXPECT routine.

'TYPE	Address of the task’s TYPE routine.

'CR	Address of the terminal “new-line” routine.

'PAGE	Address of the terminal screen clear or form-feed routine.

'MARK	Address of a routine to mark the editor’s cursor on the terminal.

'TAB	Address of a routine to position the terminal’s cursor.

'CLEAN	Address of the task’s “clear to end of line” routine.

Name	Content

C#	Task’s current cursor position (column).

L#	Task’s current cursor position (line).

TOP	Top of task’s screen scrolling area (line#).

H	Dictionary pointer (to the next available byte).

(H 2+)	Pointer to the “empty” dictionary location.  (H 1+ on 8-bit processors and H 4+ on 32-bit processors.)

OFFSET	Offset automatically added to drive-dependent block numbers to calculate absolute block numbers.

BASE	Number conversion base (eight for octal, ten for decimal, sixteen for hex).

#TIB	The number of characters remaining to interpret in the input stream.

BLK	Number of the block being interpreted (zero denotes a terminal).

>IN	Pointer to the text interpreter’s current position in the input message buffer.

CONTEXT	Index of the vocabulary to be searched, followed by eight cells for vocabulary heads.

CURRENT	Index of the vocabulary into which new definitions will be put.

SCR	Current block number.  Used by LIST and most EDITOR words.

CHR	Current character position in EDITOR.

EXTENT	Contains the number of characters in a block that are affected by character editor functions (normally 64).

WIDTH	Maximum width of a name, in bytes.

(WIDTH 1+) Default width of a name.

The following user variables are added by the Data Base Support option:

Name	Content

R#	Current record number.

F#	Address of the file definition area for the current file.

L/P	Number of lines per page on the task’s output device.  More than 60 indicates a printer.

P#	Current page number for the report generator.

RPT	Address of the report heading routine for the report generator.

REFERENCES

Step 5 of ACTIVATE, Section 4.5
WAIT, Section 4.2
Data Base Support, Section 8.0

## 4.7	Sharing Resources with GET and RELEASE

Some system resources must be shared between tasks without giving any one task permanent control.  Disk units, tape units, printers, non-re-entrant routines, and shared data areas are all examples of resources limited to use by only one task at a time.

PF/x controls access to these resources with two words that resemble Dijkstra’s semaphore operations.  (Dijkstra, E.W., Comm. ACM, 18, 9, 569.)  These words are GET and RELEASE.  An example of their use is:

: BLOCK ( n - a)   OFFSET @ + DISK GET
   block  DISK RELEASE ;

In the example above, the word BLOCK requires private use of the disk controller and block buffers while it performs the actual work of ensuring that a disk block is placed in a block buffer.  The phrase DISK GET waits in the PAUSE loop to obtain private access to these resources.  The phrase DISK RELEASE releases these resources, without awakening another task.

The word DISK in the example above is an example of a “facility” variable.  A facility variable is a normal VARIABLE, but it must be in the public dictionary.  When a facility variable contains zero, no task is using the facility represented by the variable.  When a facility is in use, then its facility variable contains the address of the STATUS of the task that owns the facility.  The word GET waits in the multitasking loop until the facility is free or owned by the task which is running GET.  High-level code for GET would be:

: FREE ( a - a t)   @  DUP 0=  SWAP 
   STATUS =  OR ;

: GET ( a)   BEGIN  PAUSE  FREE UNTIL
   STATUS SWAP ! ;

GET checks repeatedly whether a facility is free.  In conventional operating systems, this time is called a “busy-wait,” and heroic measures are taken to conserve it.  Experience has shown that queued resource schedulers (the “fast” conventional solution) take more time and are more prone to deadlock than assembly-coded versions of GET.  GET is actually written in code, and the overhead rarely exceeds two or three machine instructions.  Maintaining a queue is almost always slower.

RELEASE checks to see whether a facility is free or already owned by the executing task.  If so, RELEASE stores a zero into the facility variable. Using the definition of FREE above, a high-level definition of release would be:

: RELEASE ( a)   FREE IF  0 SWAP !  ELSE
   DROP  THEN ;

Note that GET and RELEASE can be safely used by any task at any time, without endangering the system’s integrity.

pF/x does not have any safeguards against deadlocks, where two (or more) tasks hang up because each wants a resource that the other has.  For example:

: 1HANG   DISK GET  TAPE GET ... ;

: 2HANG   TAPE GET  DISK GET ... ;

If 1HANG and 2HANG are run by different tasks, the tasks could eventually deadlock.

The best practice to avoid deadlocks is to avoid GETting two resources at the same time.  In the disk/tape case, for example, one would use BLOCK, move data to a local buffer and then to tape.  In almost all cases there is a simple way to avoid concurrent GETs.  However, a poorly-written application might have the conflicting requests occur on different levels of nest, hiding the problems until a conflict occurs.

Remember that it is better to design an application so it GETs only one resource at a time—deadlocks are impossible in such a system.

## 4.8	Defining a TERMINAL Task

The word TERMINAL allots memory to a terminal task (see Fig. 4.3) and creates the task definition table for a task that controls a serial port.  A TERMINAL task can also have a private dictionary, though many don’t use one.  The serial port is the critical difference.  The return stacks of a system’s terminal tasks have a standard size, and space is allowed for input message buffers and private dictionaries.  Because terminal tasks can have their own dictionary, they have user variable space for dictionary management, disk access, editing and private interpretation of both blocks and terminal input.

TERMINAL uses a machine-dependent set of arguments to describe terminal I/O arrangements and task-size.  This example is from the PDP-11 (there may be several examples in your system listing):

OCTAL  175610  310  DECIMAL
   2048  TERMINAL  CHUCK

The first number is a port address, the second an interrupt vector, the third (in decimal) is the total number of bytes to be allotted to a terminal task which will be called CHUCK.  The task size argument is required by all implementations of TERMINAL, but the other arguments vary.  Note that the names of tasks are proper names (as here) or job assignments.

Besides allotting space, TERMINAL may also set up interrupt routines for TYPE and EXPECT for the terminal task.  The exact method is dependent upon the serial interface hardware.  TERMINAL also compiles a task definition table that contains initialization values for certain user variables.

Fig. 4.3

Layout of memory allotted for a terminal task.  Read-only memory section is compiled by TERMINAL.

The initialization table compiled by TERMINAL contains values for the task address, S0, 'CR, 'PAGE, 'TAB, 'MARK, 'TYPE, 'EXPECT, CLEAN, and usually a hardware-dependent field DEVICE.  Some of these values are copied to the terminal task’s user variables when the task is initialized by the word CONSTRUCT, which is described in the next section.

S0 will be copied to the task’s user variable area by CONSTRUCT, along with the “tick” variables ('CR, 'PAGE, etc.) which all contain the address of the parameter field of a routine to perform a terminal function (see Special Terminal Functions, Section 3.7.5).  Note that the “tick” variables are set using the most recently compiled versions of the terminal-dependent functions.  If, for example, an ADM-3A terminal task has been defined with a special (CR) routine, and a printer task follows it with no (CR) routine of its own, the printer task will be set up using the ADM-3A’s (CR) routine.

The task definition routines look for words with a name following the parenthesized convention (NAME).  For example, the routine to perform a “clear screen” operation should be called (PAGE).  Its address will be stored in 'PAGE when a task is set up and PAGE will execute it.  For example, 'TAB will be set to the address for the ADM-3A’s TAB if the ADM-3A version of (TAB) is nearest to the top of the dictionary.  Where several of the same model of terminal are to be connected to one Forth computer, a good approach is to define their terminal-dependent functions once and then establish the terminal tasks contiguously in memory.

DEVICE contains the data required by <TYPE> and <EXPECT> (the terminal interrupt routines) to address the specific terminal associated with the terminal task.  Typical values in DEVICE might be a port address in a memory-mapped I/O machine, a port number for a multiplexer, or an input instruction followed by a return from a subroutine.

When building a task definition table in read-only memory, TERMINAL must be executed at target compilation time, because TERMINAL builds the ROM table used to construct the task after power-up in the target system.

REFERENCES

CONSTRUCT, Section 4.9
Support of Special Terminal Functions, Section 3.7.5
Terminal Drivers, Sections 3.7.2, 3.7.4

## 4.9	Initialization of a TERMINAL Task

After a terminal task has been defined by the word TERMINAL, the terminal task must be initialized by the use of the word CONSTRUCT, and then be made to run a program by the use of ACTIVATE.  CONSTRUCT is similar to BUILD (for background tasks).

Terminal task initialization consists of linking the task into the round robin and setting the task’s user variables.  The only remaining initialization required before a task is actually running is to set the values of I and R for the address interpreter, which will be performed by ACTIVATE.  An example of defining and initializing a terminal task is:

OCTAL  175610 310  DECIMAL
   4096 TERMINAL CHUCK
   CHUCK CONSTRUCT

The word CONSTRUCT performs four separate functions:

1.	It links the new terminal task into the round robin (in the same way as BUILD).

2.	It copies non-device-dependent initialization from OPERATOR’s user variables to the new task’s user variables.

3.	It copies device-dependent data from the task definition table compiled by TERMINAL into the user variable area of the new task.  Typical device-dependent data includes DEVICE and the addresses of the terminal-specific routines for TYPE, EXPECT, CR, PAGE, TAB, MARK, and CLEAN.

4.	It sets the task’s private dictionary pointers H 2+ (H 4+ on 32-bit processors) and H.

When terminal definition table resides in read-only memory, CONSTRUCT must be executed after power-up in the target system.

REFERENCES

Controlling a Terminal Task, Section 4.10
Defining a Terminal Task, Section 4.8
I and R, Section 1.2.5

## 4.10	Controlling a TERMINAL Task

After a terminal task has been defined and has its user variables initialized, the new terminal task must be made to run a program.  This is done by ACTIVATE, which controls TERMINAL tasks in the same way as BACKGROUND tasks.

The usual cautions about using ACTIVATE apply:

1.	ACTIVATE must be in a : definition.

2.	ACTIVATE uses a task address from the stack, and starts the task addressed to executing the words following ACTIVATE.  The task must never reach the ; of the definition containing ACTIVATE.

3.	The task executing ACTIVATE exits from the definition containing ACTIVATE, without executing any of the words following ACTIVATE.  See the reference on ACTIVATE, Section 4.5 for more information.

Since most terminal tasks exhibit the characteristic behavior of awaiting input from the keyboard and executing commands thus received, these special words are available which specify this behavior:

Word	Stack	Function

QUIT		Endlessly performs the behavior of EXPECTing keyboard input and INTERPRETing it.  This is the program being run by most Forth terminals.

PROMPT	( a)	Starts a terminal task whose address is on the stack performing its characteristic behavior (QUIT) having first initialized its dictionary links to GOLDEN by using EMPTY and displaying some sort of “prompting” information such as the system “help screen.”  Terminal tasks that are PROMPTed end up running QUIT.

SEND text	( a)	Used by one terminal task to forward a command string from its input message buffer to another terminal task (whose address is on the stack).  The other task will interpret the commands.  SEND uses ACTIVATE to control the specified task, and leaves that task performing QUIT.

A simple example of a word using ACTIVATE to stop execution of a terminal task is:

: HALT ( a)   ACTIVATE QUIT ;

HALT uses a task address and forces the task addressed to run QUIT.  The task that runs HALT leaves the definition of HALT at ACTIVATE, and does not execute QUIT or the EXIT compiled by ;.

An example of the use of PROMPT is:

CHUCK PROMPT

where CHUCK is the name of a terminal task.  PROMPTing should be done after GOLDEN is set, at the completion of loading of the electives block.  PROMPTing should be done after GOLDEN is set because PROMPT performs an EMPTY and EMPTY uses GOLDEN.  An example of the use of SEND is:

CHUCK  SEND 7 EMIT

which will force the task named CHUCK to interpret the phrase 7 EMIT which will ring the bell on CHUCK’s terminal.  SEND copies the input message buffer and a subset of the user variables of the SENDing task to the slave task, and then uses ACTIVATE to force the slave task to interpret its new input message buffer.  The dictionary heads are copied to the slave task so that the slave task may use any words that are available to the SENDing task.  Note that this convenience implies that you may not FORGET a definition that the slave is executing.

REFERENCES

ACTIVATE, Section 4.5
EMIT, Section 3.7.2
EMPTY, Section 3.3.4.1
HALT for BACKGROUND Tasks, Section 4.5
PAUSE, STOP, and WAIT, Section 4.2
The GOLDEN Array, Section 3.4.4

## 4.11	Printer Tasks

It is convenient to dedicate a task to the time consuming purpose of printing reports and making listings.  Such a task is a terminal task with a hard copy peripheral attached.  In many instances, the peripheral has no input facilities at all, being some sort of printer.  If the peripheral interfaces differently than a standard terminal (using a parallel port, for example), the printer task will have a private definition of TYPE.

Printer tasks rarely require more than 300 bytes of memory, unless a printer task is performing a target compilation to produce a printed log.

Printer tasks are controlled by a word that uses SEND.  If the printer task were named TYPIST, then the word would be defined:

: PRINT   PRINTER GET  TYPIST SEND ;

and used (for example):

PRINT  0 240 INDEX  9 240 SHOW  OK

The OK at the end form-feeds a page and RELEASEs the facility variable PRINTER.  A facility variable (see references below) is necessary to prevent the printer task from being accidentally re-commanded in the middle of a listing.

The definition of PRINT shown above will wait until the printer executes an OK.  If the printer task does not execute an OK, the above definition will wait forever.  Another definition which does not wait is:

: PRINT   PRINTER DUP RELEASE
   @ ABORT" Not available"
   PRINTER GRAB  TYPIST SEND ;

REFERENCES

Device Dependent User Variables, Sections 4.8, 3.1
Facility Variables, GET and RELEASE, Section 4.7 
SEND, Section 4.10
Terminal Drivers, Sections 3.7.3, 3.7.4

# 5.0	UTILITY FUNCTIONS

A complete programming environment must provide support for all aspects of the software development process.  In polyFORTH, many of these functions are integral to the system:  multiprogrammed operating system, compiler, interpreters, general purpose command set, and debugging aids such as DUMP and .S.  These have all been discussed in previous sections.  Forth’s assembler is documented in the next section, as well as in the CPU Supplement for your processor.  The purpose of this section is to document a number of separate utilities provided with the system.  An additional DOCUMENTOR utility is provided with the Data Base Support option, described in Section 8.0.

## 5.1	Editing Capabilities

This section documents the commands that are used to edit a program block.  Before any of the commands described in this section may be issued, however, it is necessary to select a program block for editing.  Then, to obtain access to the EDITOR vocabulary, type EDITOR.  Note that for convenience, T and L are in the vocabulary FORTH, and use of either of these will automatically select EDITOR for you.

REFERENCES

Disk and Block Layout, Section 5.2.5
Selecting a Program Block, Section 5.1.1

### 5.1.1	Block Display

To display a block and at the same time select it for future editing, type:

n LIST

where n is the logical block number of the desired block.

Once selected, the current program block may be (re)displayed (and the EDITOR selected) by the following command:

L

The number of the block will be displayed on the first line; the block will be displayed as sixteen lines of text.

Each line of the block will be numbered on the left-hand side, as 0-15 or 0-F depending on whether the user is currently in DECIMAL or HEX.  These line numbers do not actually appear in the stored text but rather are provided by the program for your easy reference.

The characters “ok” will appear at the end of the final line of the block, indicating that the display is complete and that Forth is ready for another command.  Regardless of the position in which they appear in the display, these characters do not appear in the actual text of the block.

The current block number is kept in the user variable SCR.  SCR is set by LIST or LOCATE; all editing commands operate on the block that is specified by SCR.

Before a program block has been used, it contains data of an undefined nature.  The command WIPE will fill the block with ASCII spaces.  The block is considered “unused” whenever the entire first line (first 64 characters) of the block are all ASCII spaces.

For convenience, three additional block display commands exist:  N, B, and Q.  N (Next) adds 1 to SCR, and then displays the next block.  B (Back) subtracts 1 from SCR, to go back and display the previous block.  Q adds or subtracts the shadow block offset into SCR, so that typing Q alternates the block display between a source block and its shadow block.  The words N and B are added to the FORTH vocabulary when the EDITOR is loaded.  The word Q is added when the “programmer aids” option block is loaded.

REFERENCES

Shadow Blocks, Section 1.4
Shadow Documentation, Section 5.2.3

### 5.1.2	String Buffer Management

The EDITOR contains two string buffers that are used by most of the editing commands.  These are called the “find” buffer and the “insert” buffer.  The find buffer is used to save the string that was searched for most recently by one of the three character editing commands F, D, or TILL.  It is at least sixty-four characters in length.  The insert buffer is also at least sixty-four characters long.  It contains the string that was most recently inserted into a line by the character editing commands I or R, or the line most recently inserted or deleted by the line editing commands X, U, or P.  The command K interchanges the contents of the find and insert buffers, without affecting the text in the block.

The existence of these buffers allows multiple commands to work with the same string, understanding which commands use which buffers will enable you to use the EDITOR more economically.  The convention is this:  Commands which may accept a string as input will expect to be followed immediately either by a space (the delimiter following the command) and one or more additional characters followed by a carriage return, or by a carriage return only.

In the former case, the string will be used and will also be placed in the string buffer that belongs to the command (find or insert).  In the latter case (carriage return only), the string that is currently in the appropriate buffer will be used (and will remain unchanged).

For example, the character editing command:

F WORDS TO FIND

will place WORDS TO FIND in the find buffer and will find the next occurrence of the string WORDS TO FIND.  Subsequent use of the command F immediately followed by a carriage return will find the next occurrence of WORDS TO FIND.  The following table summarizes buffer usage:

Buffer:	Find	Insert

Commands:	F	I
		S	R
		D	X
		TILL	U
		K	P
			K

### 5.1.3	Line Display

Any single line of the current block (whose number is in SCR) may be selected by using the following command:

n T

where n (which must be in the range 0-15) is the line number to be selected.

The T (Type) command sets the user variable CHR to the character position of the beginning of the line.  This value may later be used to identify the line to be changed, using the commands defined in the following section.  Since CHR is used to store the cursor position for the character editing commands, using T (i.e., initializing CHR) specifies that any search will start at the beginning of that line.  The new cursor position is marked in some convenient way.

The contents of the string buffers and of the block are unchanged by the use of T.

### 5.1.4	Line Replacement

The command P (“Place”) will replace an entire line with the text that follows it, leaving that text in the insert buffer, or with the current content of the insert buffer (if P is followed by a carriage return).

The line number used by the P command is computed from the value that is in CHR.  P is normally used after the T command, as illustrated by the following example:

4 T	(command)
^THIS IS THE OLD LINE 4	(response)
P THIS IS THE NEW LINE 4	(command + text)

A P followed by two or more spaces and a carriage return will fill the line with spaces.  This is useful for blanking single lines.

P immediately followed by a carriage return will replace the line by the current contents of the insert buffer.  Thus, a line may be placed in several locations in a block by the use of:

1.	P followed by text (the first time).

2.	Alternate use of T (to select the line and confirm that this is the line to be replaced) and P followed by a carriage return.

### 5.1.5	Line Insertion or Move

The command U (Under) is used to insert either the text that follows or the current contents of the insert buffer into the current program block under the line in which the current value of CHR falls.  Normally U is used immediately after the T command, where the line number specifies the line under which a new line is to be inserted.

The handling of text and the insert buffer is the same for U as for the command P.

The word M in the editor brings lines in to the block you’re currently displaying.  If you wish to move a series of lines from one block to another, first list the source block.  Note the block number and the first line number.  Next, list the destination block and select the line just above where you want the line you’re going to bring in to be inserted.  Now enter blk# line# M.  The source line will be inserted below the current line.  It won’t be removed from the source block.  The current line will be one below where it was before.  Additionally the source block number and line number will still be on the stack but the line number will be incremented.  This sets you up to do another M without entering any additional arguments.  The word M checks the stack to be certain it contains only two arguments.  It ABORTs if the depth isn’t two.  This saves you from accidentally knocking the last line off your block by inadvertently entering an M.

### 5.1.6	Line Deletion

You may use the command X to delete the current line (i.e., the line in which the current value of CHR falls).  You will normally use X immediately after a T command that specifies the line to be deleted.

When a line is deleted, all higher-numbered lines are “rolled up” by one line and Line 15 is cleared to spaces.  In addition, the contents of the deleted line are placed in the insert buffer, where they may be used by a later command.  Thus X may be combined with T followed by P or U to allow movement of one line within a block.  The following sequence would move Line 9 to Line 4, changing only the ordering of Lines 4 through 9.

9 T  X  3 T  U

Note that if a line is being moved to a position later in the block, the X operation will change the positions of the later lines.  To move the current Line 4 to a position after the current Line 13, use the following command sequence:

4 T  X  12 T  U

Line 12 is specified as the insert position since the X operation moves the current Line 13 to the new Line 12.

### 5.1.7	Character Editor

The EDITOR vocabulary also includes commands to permit editing at the character level.  Except in the case of F and S, the character editor’s commands work within a specified range, controlled by the user variable EXTENT.  EXTENT is normally set to 64, so that the range will be confined to the current line of the current block.  The line is selected by the regular EDITOR command:

line# T

A cursor (indicated by a ^ or some form of highlighting) marks the position within the line at which insertions will take place and from which searches will begin.  The T command sets this cursor to the beginning of the line.

Insertions will cause characters at the end of the line to be lost; they will not spill over onto the next line.  Deletions will cause blank fill on the right end of the line.

EXTENT’s value may be set to 1024 in some applications (such as word processing) when it’s desirable to allow edits to propagate through the entire block.

In the list of commands below, the word “text” indicates a string of text.  If the text is omitted, the current contents of the find buffer will be used (for the commands F, S, D, and TILL) or the current contents of the insert buffer will be used (for I).  If text is present, it will be left in the appropriate buffer.

The maximum length of a string is determined by the length of the two string buffers being used, at least 64 characters.  In all cases the string is terminated by a carriage return or a caret.  If a string that is too long is typed, the string will be truncated to the buffer’s size.

The following commands are available:

Command	Function

F text	Finds a match on text anywhere after the current cursor position in the current block and leaves the cursor positioned at the end of the matching text.  The search begins at the present cursor position and continues until a match is found or the end of the block is encountered.  When text is not found, issues an error message and leaves the cursor where it was before the operation.

n S text	Searches for text anywhere after the current cursor position in the current block, and continues the search through all subsequent blocks up to but not including block number n.  If a match is found, the block containing the matching text is displayed with the cursor positioned at the end of the matching text (as in F).  If the search succeeds, the ending block number is left on the stack, so that when you are ready to continue the search you may simply type S.

D text	Deletes the matching text or the contents of the find buffer and leaves the cursor where the deletion occurs.  When text is not found, issues an error message and leaves the cursor where it was before the operation.

TILL text	Deletes all text from the current cursor position up to and including the text or contents of the find buffer (which must be within the boundary set by EXTENT, normally on the current line)  When text is not found, issues an error message and leaves the cursor where it was before the operation.  If text is found beyond EXTENT, the current line will be blanked to the right of the cursor location to the limit of EXTENT.

I text	Inserts the text or the contents of the insert buffer at the current cursor position.  Any characters that spill off the end of the line when EXTENT is 64 will be lost. Any characters that spill off the end of the block when EXTENT is 1024 will be lost.  The cursor is left positioned at the end of the inserted text.

R text	After text has been found (by the F or S commands, for example), replaces the found text with the specified text or the contents of the insert buffer, if no text is specified.

Command	Function

K	Exchanges the insert buffer and the find buffer.  Useful for correction of bad deletions.

### 5.1.8	Block COPY Command

The COPY command allows a block to be copied, in its entirety, into a different block.  It has the following format:

s d  COPY

where s is the source block number and d is the destination block number.

COPY moves the entire block; it does not change the contents of the old block.  Note that COPY uses the word IDENTIFY to change the block number to which a block buffer will be written.

## 5.2	Program Listing Utility

The PRINTING utility is used to list program source blocks on disk.  The output is designed for 8-1/2" x 11" paper suitable for keeping in a conventional 3-ring binder.  The utility is loaded with the following command:

PRINTING LOAD

### 5.2.1	Index Listings

INDEX will produce an index listing that shows the first line of each block in a given range of blocks.  It is therefore very helpful to place a parenthesized comment in Line 0 of each block to describe the block’s contents in the index.

Generation of an index is specified by the following command:

start end+1 INDEX

where start is the starting block number and end+1 is the ending block number plus one.  The index will be formatted 60 lines to a page.  As many full pages will be generated as necessary to cover the requested block range.  This command sends the index to the console screen.  Preceding the same command with the word PRINT routes the index to the printer.

Since you can print 60 lines on a standard page, the large-scale organization of a polyFORTH system source disk tends to follow groups of 60 blocks.  For example:

Block	Contents

0-59	polyFORTH Electives
	60-179	polyFORTH Utilities
	180-239	polyFORTH System Source
	240-299	polyFORTH Target Compiler and Drivers

Because of different disk capacities, the layout of your polyFORTH system disk may be different from the example shown above.

### 5.2.2	Program Block Listings

To list the contents of entire blocks, use the following command:

start end+1 SHOW

SHOW lists three blocks per page, with the top block number evenly divisible by three.  Only entire pages will be printed, in sufficient quantity to cover the requested block range.  If either start or end+1 is a number not evenly divisible by three, the printout will contain more blocks than requested, to fill out the pages.  On a partially used page, unused blocks will not be listed although space will be left for them.  Pages with no used blocks will be skipped entirely.  An unused block is defined as one with only ASCII spaces in the first line (first 64 characters), as for example a block that has been WIPEd.

To list a single page, use the following command:

blk# TRIAD

where blk# is the block number of any block on the desired page.

### 5.2.3	Shadow Documentation Blocks

The shadow block system is intended to be an explanatory “shadow” of an entire polyFORTH system source code.  Shadow blocks are intended specifically to document words with unusual usages and difficult-to-decipher coding tricks.  Shadow blocks can be easily and concurrently edited with code, because the single-letter command Q (for question) flips the editing block number from a code block to its corresponding shadow block and back.  The best time to write shadow blocks is at the time the code is written.

At FORTH, Inc. we find it very useful to print source code and the shadow documentation blocks which document that source code on facing pages (when inserted into a binder, for example).  This requires that the shadow blocks be printed on the back of the page preceding the source blocks.

The following procedure will allow you to print this type of double-sided listing on your printer.

1.	Insert the program disk in Drive 0 and the shadow disk in Drive 1.  This step is unnecessary if the shadow and program blocks are on the same disk.

2.	Print an index of the blocks to be listed, following the procedure of Section 5.2.1 (“Index Listings”).  Do not terminate your command with the word OK and do not tear off the paper when the index printing has completed.

Example command:

PRINT 0 60 INDEX

3.	Print a listing of the program blocks you want, as described in Section 5.2.2 above.

Example command:

PRINT 9 45 SHOW OK

will print program Blocks 9 through 44 inclusive.

If you prefer, you may combine Steps 2 and 3 in the same command string, like this:

PRINT 0 60 INDEX  9 45 SHOW OK

4.	After the listing has finished, space out one additional blank page by typing:

PRINT OK

Tear off the paper after that blank page.  Remove the paper stack which has been feeding empty pages into the printer.

5.	Turn over the printed listings you have just finished and feed the beginning (top) of the paper into the printer.  When fed in correctly, the back of the last page of the index you printed in Step 2 above should be the next surface to be imprinted.

6.	Now print the shadow blocks for the program blocks just printed.  The shadow block for any particular program block should be in the block whose number is the sum of the constant SHADOWS plus the program block number.  If you follow this convention in writing your own shadow blocks, Forth will compute the shadow block numbers for you.  Type in the command to print shadow blocks using the same block numbers that you have just printed the listing for, and the correct blocks will be printed.

Example command for a particular computer with an offset of 162 blocks between source and shadow (the offset on yours may be different):

PRINT 9 45 SHADOW OK

will print Blocks 171-206 inclusive.

If you have positioned the paper correctly, the first shadow block should be printed on the back of the last page of the index.  When the printout is finished and separated into pages, you should find that program and corresponding shadow blocks face each other as you turn the pages.  Shadow block listings are useful because they document code conveniently.  A useful by-product of the shadow block system is that undocumented code is conspicuous, and there is no physical space for unnecessary documentation.

Two standard styles of shadow blocks exist.  The first is a glossary-style format in which the words defined in the code block are explained.  The code words are on the left margin with all other text indented three spaces.  The second form is standard English text and is used for other explanations.

The command PAIRS prints source and documentation blocks in pairs of triads.  Each shadow block is printed to the right of its corresponding source block, three pairs per page.  PAIRS, given the starting and ending block number +1, prints the entire range of blocks and shadows.  For example:

PRINT 0 300 PAIRS

prints source and shadow blocks from Block 0 to Block 299.

REFERENCES

Q, Section 5.1.1
Shadow Blocks, Section 1.4

### 5.2.4	Double-Sided Listings

Sometimes, for very large applications, it is useful to print source blocks on the front and back of every page.  For this purpose the words FRONT and BACK are defined in the printing utility.  To obtain a listing with program blocks on both sides of the paper, follow this procedure:

1.	Print an index of the program blocks using the procedure of Steps (1) and (2) in Section 5.2.3 above.  Make sure to leave an empty page before starting the printout.  Do not end the index printing command with OK.

2.	Start the program listing with the FRONT command.  This word lists every other triad of blocks, skipping over the blocks which will be printed on the facing pages.  The command format is:

PRINT start end+1 FRONT OK

where start and end are the starting and ending block numbers of the block range to be printed.

3.	After printing has stopped, type:

PRINT OK

twice to leave two blank pages at the end, then tear off the listing, at the printer’s tear-off line.

4.	Remove the blank paper feed, and insert the top of your printed listing after turning it over as in Step (5) of Section 5.2.3.  Position the paper so that the next page to print will appear on the back of the first triad page.

5.	Print the remaining triads using a command of the format:

PRINT start end+1 BACK OK

where start and end are the same as in Step (2) above.

This will print the program triads skipped during the FRONT printing.  Your listing, when separated into sheets and put into a binder, will be a front-and-back consecutive listing of the blocks of source code.

### 5.2.5	Disk and Block Layout Design

There are several conventions for block and disk design that will make your application programs more readable.  While these conventions are not dictated by the nature of Forth, we recommend them as good programming practice.

Disk Design

1.	Begin Line 0 of each block with a parenthesized comment that describes the contents of the block.  The comment is then conveniently listed by the PRINTING utility’s INDEX.

2.	In a single block put only source text for words that are related to some one function or isolatable portion of a function.  Do not put unrelated words in the same block.

3.	Do not overpack a block.  Leave several blank lines for expansion.  There is seldom an advantage to conserving blocks.

4.	Begin sub-applications on block boundaries that are divisible by three.  The PRINTING utility conveniently puts blocks on a listed page, three blocks per page.  This fits nicely on 8-1/2" x 11" paper. By starting sub-application block groups on these block boundaries, your printed listing is separable.

5.	Begin major portions of your application on boundaries that are divisible by sixty. Thus, each page generated by INDEX will cover one such portion.

6.	Write shadow blocks for your application as you program it.  Six months later they may save weeks of work.

7.	If your application is a resident utility, make the application’s key load block a named block (add a constant to Block 10), and add the application to the system help screen in Block 11.

8.	The first shadow block (for Block 9) should have a block number divisible by 3, in order to be printed properly (this is already done for you in Block 14).

When the application is listed the application starting blocks (modulo 60) and the sub-application starting blocks (modulo 3) will appear at the tops of the pages.

Block Design

1.	Do not define more than one word on a line.  An exception might be two or three related constants or variables or a couple of very brief related colon definitions.

2.	Start colon definitions at the beginning of a line.  A colon definition should never run over a block boundary. A typical colon definition uses three lines or less.  Longer definitions can often be factored with groups of words re-defined as individual words, with improved readability and testability of the code.  Although Forth allows unlimited nesting of loops, rarely should it be necessary to nest more than two levels without redefining the innermost loops as words.

3.	Leave three spaces after the name that is being defined in a colon definition, to set it off from the definition.

4.	Break colon definitions up into phrases, separated by double spaces, so that each phrase describes a particular operation:

: DOUBLE   X @  2*  X ! ;

5.	If a definition takes more than one line, indent three or more spaces on the second and succeeding lines.

6.	Separate instructions in CODE definitions with three spaces.  For example:

CODE KEY ( - c)   BEGIN   F7 INP
	   2 # ANA 0= NOT UNTIL  F6 INP
	   A L MOV   0 H MVI HPUSH JMP

7.	Use standard stack notation in the stack effects comment:

Word	Description

a	An address.

b	A byte (as returned by C@).

c	A byte containing an ASCII character (as returned by C@).

d	A double-precision signed integer.

ud	An unsigned double-precision integer.

n	A signed single-precision number.

t	A single-precision Boolean truth flag.

u	An unsigned single-precision number.

i	An integer used as an index.

Defining words are more readable when stack argument comments are in both the defining section and the run-time section; for example:

: COEFFICIENT ( n)   CREATE ,
	   DOES> ( n - n)   @ * ;

REFERENCES

Documentation Aids, Section 1.4
INDEX, Section 5.2.1
Stack Arguments Notation, Section 2.1
TRIAD, Section 5.2.2

## 5.3	DISKING Utility

The utility named DISKING allows the user to copy ranges of blocks, backup an entire disk, and (on some systems) format a new disk (not all disk controllers support this capability).  There is some variation in the actual commands from one system to another; consult the “Help screen” for your DISKING utility for specific details, or list the source blocks.

To load this utility, type:

DISKING LOAD

polyFORTH has a constant named VOLUME which normally gives the number of blocks on one floppy disk, cartridge, etc.  On some systems with a large disk, however, VOLUME may be set to other sizes depending on the particular system.  Consult your CPU Supplement for details.

### 5.3.1	Use of BLOCKS and +BLOCKS

The basic word for copying a range of blocks from one part of a disk to another is BLOCKS.  BLOCKS takes three arguments: the source and destination starting block numbers, and the number of blocks to be copied.  Thus,

60 560 120 BLOCKS

copies Blocks 60-179 to 560-679.

BLOCKS will correctly handle blocks which overlap either forward or backward without overwriting any blocks.

+BLOCKS is the same as BLOCKS, but copies shadow blocks as well; they are assumed to be offset from source blocks by the value of the constant SHADOWS.

### 5.3.2	Special Commands

The word OBLITERATE is defined to blank a region of disk.  Its usage is:

start  end+1  OBLITERATE

which will fill all blocks from start to end entirely with ASCII spaces.

### 5.3.3	Comparing Disks

An additional facility is provided to ensure that a range of blocks is copied correctly.  The word MATCHES compares ranges of blocks to discover if they are identical.  MATCHES takes three arguments:  The source and destination starting block numbers and the number of blocks to compare.  Thus:

60 560 120  MATCHES

matches blocks 60-179 to 560-679.  MATCHES will print the block numbers of any destination blocks which do not match their corresponding source block.

Note that MATCHES is not called automatically by BLOCKS.

REFERENCES

BACKUP, Section 5.3.2
BLOCKS, Section 5.3.1
Disk Diagnostics, Section 5.3.4

### 5.3.4	Disk Diagnostics

A simple read-only disk diagnostic is provided. It reads consecutive blocks and reports any read errors encountered.  It may be used at any time and does not destroy any disk information. It is used thus:

l h+1 SWEEP	(Checks blocks l through h for read errors.)

If an error is detected, the block number is typed out, along with the disk status.  Comparing the latter (in the appropriate number base) with the disk controller manual’s identification of error bits yields information as to the actual type of error.

Error checking versions of BLOCK and BUFFER can be made available by loading your system’s “Disk Error Handling Block.”

REFERENCES

Disk Error Checking, Section 3.2.6

### 5.3.5	Disk Formatting

Systems that need a disk formatting facility usually have it.  This feature is system dependent; check your CPU Supplement for details.  If provided, it is used by invoking the single command:

INITIALIZE

This command may take parameters, but normally formats an entire disk.  For systems with two removable disks, it is always Drive 1 that is formatted (to prevent inadvertently formatting the master).  For systems with  fixed and removable disks, the removable disk is the default, and a separate command is provided to format the fixed disk; see the CPU Supplement for details.

## 5.4	DEBUG Utility

There are two debugging utilities that let you decompile, breakpoint, or trace any high-level definition.  Typing DEBUG LOAD will provide access to decompilation routines.  Typing STEPPER LOAD will provide access to the above plus breakpoint (trap) and single-stepping routines.  The utilities are divided in this way because STEPPER is relatively large, and should only be loaded if needed.

### 5.4.1	Definition Decompiling

The source code of a definition is not always available, e.g., if it is in the precompiled nucleus.  If you try to locate the source code for ABORT", for example, the following will occur:

LOCATE ABORT"  ABORT" Can't

You can reconstruct the source code (with some limitations) by typing DEBUG LOAD and using the SEE name command, which decompiles the colon definition named name.  Typing SEE ABORT" will produce:

COMPILE [4358]  34 STR___ ;  IMMEDIATE

Compare this to the original definition of ABORT":

: ABORT"  COMPILE abort"  34 STRING ;
   IMMEDIATE

The word abort" is “headerless,” which means it was compiled without a link or a name field and thus only its address is available to the decompiler. Therefore the “run-time” abort" has been replaced with its pfa (the number you see may be different from 4358).  Likewise, STRING has been replaced with STR___ because it was compiled with a WIDTH of 3.

SEE uses the command a PROBE, which begins decompilation at a specific address a.  Decompilation is terminated either when the end of the definition is found, or at 128 bytes (64 cell addresses) beyond the destination of the most forward reference (such as an IF) found in the definition.  The end of a definition is signaled by a semicolon (if it exists—e.g., RECOVER has not been used); or by an unconditional exit (such as ABORT or QUIT) or an unconditional backward branch (such as AGAIN) that lies beyond the farthest forward reference previously encountered.

You may often want to execute PROBE directly.  For example, you could use PROBE to examine the “headerless” abort" at the given (parameter field) address.  Typing:  4358 PROBE will produce:

?R@ HER_ 2+ COU__ TYP_ SPA__ COU__
TYP_ CR BLK 2@ DUP {if 4} SCR 2! ABORT

The expression {if 4} is produced by an IF ... THEN clause and means “branch forward 4 bytes conditionally.”  Compare this code to the original definition of abort":

: abort"  ( t)  ?R@ HERE 2+ COUNT TYPE
   SPACE COUNT TYPE CR  BLK 2@ DUP IF
   SCR 2!  THEN  ABORT ;  RECOVER

Another usage of PROBE is to determine the action of an execution vector.  For example, typing 'IDLE @ PROBE results in:

SUP_____ 0 STA__ ! QUE__ INT______ ." ok"
   CR {else -14}

In this case, {else -14} is produced by AGAIN and means “branch backward 14 bytes unconditionally.”  The original code is:

: QUIT  SUPPLANT  0 STATE ! BEGIN  QUERY
   INTERPRET  ." ok"  AGAIN ;  RECOVER

Most flow-of-control structures ultimately compile the run-time primitives if or else.  For example, the original source code for INTERPRET is:

: INTERPRET  STATE @EXECUTE
   BEGIN  -' IF  NUMBER
   ELSE  DROP EXECUTE  
   DEPTH 0< ABORT" Stack empty"
   THEN  AGAIN ; RECOVER

The decompiled version, resulting from SEE INTERPRET, is:

STA__ @EXECUTE
-' {if 5} NUM___ {else 22} DRO_ EXE___
DEP___ 0< ABORT" Stack empty"
{else -35}

Of course, not all definitions can be decompiled; SEE DUP produces the message:

Not a colon definition

PROBE however will attempt to decompile any definition; typing ' TYPE @ PROBE produces:

[950] [39914] [11234] ...

Here PROBE has misinterpreted machine code as a sequence of headerless definitions.  As stated above, in this case decompilation stops after 64  headerless terms.

### 5.4.2	Breakpoint Setting

Breakpoints are useful for finding “intermittent” bugs, that is, bugs which only appear under (un)certain operating conditions.  To access the breakpoint or trapping routines, type:  STEPPER LOAD.  The basic command is TRAP name, which sets a breakpoint to occur on the next execution of name.  Subsidiary commands allow moving and removing the breakpoint, and single-stepping through the definition of name.

Care must be taken when using breakpoints and single-stepping in a multitasking environment.

For example, suppose you wish to investigate the word CAST as it is called by the word Q.  If you type SEE Q to decompile it, you will get:

SCR @ SHA____ CAS_ LIS_ ;

To stop (“trap”) execution at CAST you “set a breakpoint” with the command TRAP CAST, which will decompile CAST:

SWA_ OVE_ /MO_ 1 XOR ROT * + ;

and set the breakpoint to execute (“fire”) on the next execution of CAST.  Having done this, to examine CAST you might type:

9 LIST	(sets SCR to 9)

followed by:

Q

Q will call CAST, and the trap will fire:

9 321 <-Top nxt-> SWA_

When the trap fires, the stack contents are displayed to the left of the brackets, and the next instruction to be executed is displayed to the right.  In this case, the 9 is from SCR @ and the 321 is the value of SHADOWS.  The next instruction to be executed is SWAP, which is the first instruction in CAST.  To review the remaining instructions in CAST, you may type SEEN, which repeats the most recent decompilation:

SWA_ OVE_ /MO_ 1 XOR ROT * + ;
   TrOk

Notice the change in the Forth prompt from ok to TrOk.  This is a reminder that TRAP uses a special text interpreter.  From here, you can change the arguments to CAST, DUMP memory, or take any other (reasonable) action.  You can even execute QUIT to return to the normal text interpreter.  Or you can use CONT (continue) which will continue execution of CAST (and the remainder of the original command, Q in this case).

TRAP works by installing the word tcode directly in the location to be trapped (after first saving its contents).  When tcode runs, it restores these contents and returns to the special text interpreter.  TRAP displays the decompilation before installing tcode, but subsequent uses of SEEN will show the new version.  Thus TRAP CAST shows:

SWA_ OVE_ /MO_ 1 XOR ROT * + ;

and SEEN will display:

tcode OVE_ /MO_ 1 XOR ROT * + ;

The command RESTOR is used to remove the current breakpoint or trap.  Thus, in this example, typing RESTOR SEEN will display:

SWA_ OVE_ /MO_ 1 XOR ROT * + ;

If you have trapped a word but have not yet identified a bug, you may wish to move the trap and continue execution.  The command NEST name will move the trap to the word name and continue execution without leaving the special text interpreter.  For example, suppose you have trapped Q but then decide that the bug you are looking for must be in CAST.  Typing TRAP Q shows the decompilation:

SCR @ SHA____ CAS_ LIS_

Then executing Q displays:

<-Top nxt-> SCR

You can then type NEST CAST to move the trap to CAST and continue execution without leaving the special text interpreter:

SWA_ OVE_ /MO_ 1 XOR ROT * + ;
9 321 <-Top nxt-> SWA_

NEST combines TRAP and CONT and leaves you exactly where TRAP CAST would have left you—ready to continue or QUIT.  Don’t try to use NEST without first TRAPping a word.

### 5.4.3	Single-Stepping Through a Definition

To access the single-stepping utility, type STEPPER LOAD (if not already loaded).  This utility provides facilities to step through a definition, with or without skipping over intermediate words, and to cycle through DO loops.

For example, suppose you have determined that there is a bug in a definition (such as CAST) but you don’t know exactly where.  You have trapped the definition and would like to execute it one word at a time, examining the stack after each step.  As shown in the previous section, you would type TRAP CAST followed by 9 LIST and Q, at which point the screen would show:

9 321 <-Top nxt-> SWA_

Now typing the command SS (“single-step”) will execute the next word (SWAP) and display the following:

321 9 <-Top nxt-> OVE_

The top two numbers are swapped and the next instruction to be executed is OVER.  Subsequent uses of SS will  display:

321 9 321 <-Top nxt-> /MOD

then	321 9 0 <- Top nxt-> 1

then	321 9 0 1 <-Top nxt-> XOR

Now suppose you want to skip ahead in the definition.  There are two commands used for this:  GOTO name and GO.  The command GOTO name will prepare for execution of all steps up to but not including name; the command GO will then perform the execution.  The skip operation is divided into these two steps in case you need to skip over one or more occurrences of name to get to the one you want.  In this example, suppose you want to execute all words up to but not including the + instruction.  You would type:

GOTO +

Then typing GO will execute up to the + instruction, and display:

9 321 <-Top nxt-> +

Continue single-stepping with SS (on the IBM-PC you can use the <alt-F10> key):

9 321 <-Top nxt-> +	( press the <alt-F10> key)
330 <-Top nxt-> ;	( press the <alt-F10> key)
Trapping done.

Single-stepping through a semicolon takes you directly to the normal text interpreter.  You may use CONT instead to return to the (pending) Q command.

Single-stepping through a loop is made easier with the CYCLE command.  Use of CYCLE at some point inside a DO loop will cause execution of all remaining instructions in the loop, return to the beginning with index incremented, and execution of all instructions up to the starting point, i.e., one complete cycle.  For example, here is a simple definition containing a DO ... LOOP:

: DOUBLE  1  10 0 DO  DUP . 2*   LOOP  DROP ;

Typing TRAP DOUBLE will display:

1 10 0 2>R DUP . 2* {loop -9} DRO_ ;

Then executing DOUBLE will display:

<-Top nxt-> 1

Type SS:	1 <-Top nxt-> 10
Then SS again:	1 10 <-Top nxt-> 0
Then GOTO 2* followed by GO:	1 <-Top nxt-> 2*

If you type SS at this point, you will single-step through the 2* instruction to LOOP.  If instead you type CYCLE, you will “cycle” all the way through the loop, that is, you will execute the 2*, follow the loop back to the DUP, and execute all instructions in the loop, stopping again just before the 2* instruction.  You may CYCLE through the loop as many times as you wish; typing CYCLE four times in this case would produce:

2 <-Top nxt-> 2*
4 <-Top nxt-> 2*
8 <-Top nxt-> 2*
16 <-Top nxt-> 2*

To exit from the loop, GOTO the first word past the LOOP instruction:

GOTO DROP

Followed by: GO.  The display will show:

32 64 128 256 512 1024 <-Top nxt-> DRO_

From here, you can CONT or QUIT or continue single-stepping.

Below are summarized all of the commands in the DEBUG and STEPPER utilities:

Word	Description

SEE name	Decompiles and displays the definition of name as far as possible if it is a colon definition, otherwise issues an error message.  Name fields are reconstructed to their compiled WIDTH; headerless words are replaced by their pfa’s.

SEEN	Repeats the decompilation display of the most recent word decompiled by SEE.

a PROBE	Begins decompilation at address a. Decompilation is terminated when the end of the definition is found, or at 128 bytes (64 cell addresses on 16-bit machines) beyond the destination of the most forward reference found, whichever occurs first. The end of a definition is signaled by a semicolon, or by an unconditional exit or an unconditional backward branch that lies beyond the destination of the farthest forward reference previously encountered.

Word	Description

TRAP name	Decompiles and displays the definition of name, and sets a breakpoint to occur on the next execution of name.  When the breakpoint or trap occurs, the current stack contents and the next instruction to be executed are displayed.  Execution in TRAP mode is controlled by a special text interpreter, as signified by an TrOk response rather than ok.

CONT	Continues execution of all remaining instructions in a definition that has been TRAPped.

RESTOR	Removes the current breakpoint or trap, restoring the current definition.

NEST name	Moves a trap from its current position to the word name, displaying the decompilation of name.  The new name may be at the same level in a definition or lower.  At least one execution of TRAP must precede usage of NEST.

SS	Single-steps through the currently TRAPped definition.  Each use of SS displays the current stack contents and the next instruction to be executed.

GOTO name	Prepares for execution of all steps from the current one up to but not including name.  GOTO name may be used several times in succession to skip over multiple occurrences of name.

GO	Executes all steps from the current one up to the step pointed to by GOTO.

CYCLE	When in single-step mode inside a DO loop, CYCLE causes execution of all remaining instructions in the loop, return to the beginning with the index incremented, and execution of all instructions up to the initial starting point (i.e., one complete cycle).  Each use of CYCLE displays the current stack contents and the next instruction to be executed.  To exit the loop, GOTO the first word past the LOOP instruction.

## 5.5	AUDIT Utility

The AUDIT utility (accessed by AUDIT LOAD) compares two blocks or ranges of blocks and can highlight the differences between corresponding blocks.  The block ranges may be on the same or different parts of disk, and may have the same or different relative block numbers.  The utility is designed as an extension to the standard polyFORTH editor and essentially lets you edit two blocks at the same time.  AUDIT is often used to compare a source part against a backup part to identify changes.  It is also used to coordinate the separate efforts when more than one programmer is working on an application.

As an example, suppose you have the following on disk in Block 100, Part 2:

0 ( Miscellaneous)

1 : STRING ( n)   -2 ALLOT  WORD C@  2+ 1+ ALLOT ;

2

3 : ABORT"   COMPILE abort"  34 STRING ;   IMMEDIATE

4

5 : ."   COMPILE dot"  34 STRING ;   IMMEDIATE

6

7

8

9

10

11

12

13

14

15

Now suppose you have an “improved” version in Block 100, Part 4:

0 ( Miscellaneous)

1 : STRING ( n)   -2 ALLOT  WORD C@  2+ 1+ ALLOT ;

2

3 : ABORT"   COMPILE abort"  [ASCII] " STRING ;

4    IMMEDIATE

5 : ."   COMPILE dot"  [ASCII] " STRING ;   IMMEDIATE

6

7

8

9

10

11

12

13

14

15

Here’s how you would set up to compare Part 2 against Part 4:

2 PART  100 LIST  4 TARGET

After LISTing a block to make it current, the phrase n TARGET highlights all the character positions where the current block differs from the correspondingly numbered block in Part n.  In this case, 4 TARGET highlights all the differences to Block 100 on Part 4:

0 ( Miscellaneous)

1 : STRING ( n)   -2 ALLOT  WORD C@  2+ 1+ ALLOT ;

2

3 : ABORT"   COMPILE abort"  34 STRING ;   IMMEDIATE

4

5 : ."   COMPILE dot"  34 STRING ;   IMMEDIATE

6

7
 8

9

10

11

12

13

14

15

The highlighted text shows that there are some differences between the “current” block (the one you can see) and the “other” block (the one you can’t). To list the other block with its differences compared to the first, type W:

0 ( Miscellaneous)

1 : STRING ( n)   -2 ALLOT  WORD C@  2+ 1+ ALLOT ;

2

3 : ABORT  COMPILE abort" [ASCII] " STRING ;
 4    IMMEDIATE

5 : ."   COMPILE dot"  [ASCII] " STRING ;
 6    IMMEDIATE

7

8

9

10

11

12

13

14

15

W (like Q) toggles between the current block and the other one.  Whichever block you are looking at is the current one.  W always highlights the differences between the two blocks.  The command O will toggle between blocks without highlighting.  If you have edited or re-listed the current block and lost the highlighted differences, use V to see the current block with highlights. To summarize:

Differences are	Differences are
	not highlighted	highlighted

Current block:	L	V
Other block:	O	W

IMPORTANT:

The highlighting method uses space at PAD to hold one of the blocks and therefore interferes with the editor’s Find and Insert buffers.  Thus if you wish, for example, to delete a line in the current block (with X) and insert it into the other block, you must use the O (non-highlighted) command rather than the W (highlighted) command to toggle to the alternate block.

Once block differences have been examined, if you decide that you prefer the current block to the other block, you can KEEP the current block.  This copies the current block to the other block and destroys the previous other block:

KEEP	(The two blocks are now identical )

If you prefer the other block (the one you can’t see), you can TOSS the current block.  This copies the other block to the current block and destroys the previous current block:

TOSS	(The two blocks are now identical )

Once editing of this pair of blocks is complete, type G to continue the “audit.”  The G command will compare corresponding blocks, stopping and displaying the next difference, that is, the first subsequent block which differs between the two parts.

Auditing normally stops at the end of the part.  You can limit the last block to be audited with the n TO command, which halts auditing at the current block n:

320 TO	(Auditing stops at Block 320.)

You can also audit within a single part, or in different block numbers across parts.  The command n m MATCHING performs an audit comparing blocks starting at n to blocks starting at m, both in the current part.  For example, if you want to compare blocks starting at 100 to blocks starting at 150 within Part 2, you would type:

2 PART  100 150 MATCHING

The most general command is n m p AGAINST, which compares blocks starting at n in the current part to blocks starting at m in Part p.  For example, to compare blocks starting at 100 on Part 2 to blocks starting at 200 on Part 4, you would type:

2 PART  100 200 4 AGAINST

## 5.6	PROMS Utility

The PROMS utility is designed to write (“burn”) most popular PROM devices on a GTEK PROM programmer, models 7128 or 7228.  The PROMS utility is called GTEK on some systems.  Before using PROMS you should have created a PROM image on a contiguous range of disk blocks.  This is usually done with the target compiler (documented separately).  You must also have created the serial terminal task REMOTE, typically defined (on the IBM-PC) as follows:

COM1  4096 CHANNEL REMOTE  1200 BAUD
REMOTE CONSTRUCT

The model 7228 runs also at 2400 baud.

The PROMS utility, like other utilities, is compiled and accessed by typing:

PROMS LOAD

There will be a brief pause while the utility initializes the GTEK programmer, you will see the message:

SETTING BAUD RATE

You may also see the message:

Prom HELP  Timeout

This means that either the GTEK is not correctly connected or that the REMOTE task is not correctly initialized.  Otherwise, you will see the message:

Prom 2716  Start 0  At 0  Long 2048
   Bus 1 Image 300

This means the GTEK is programmed with defaults to:

1.	Burn a type 2716 PROM,

2.	starting at 0 bytes offset in the disk image;

3.	placing the data at 0 bytes offset in the PROM;

4.	burning a data array 2048 bytes long (default is the PROM size);

5.	using a bus width of 1 byte (8 bits);

6.	and getting the data from the disk image starting at block 300.

The default PROM and bus width are on Line 15 of the load block. This line, as shipped, reads:

CR 1 >2716   HELP

The 1 selects the “bus width” where 1 means that the PROM has an 8-bit (1-byte) wide data bus, and 2 means that two PROMs will be used in parallel for an effective 16-bit (2-byte) data bus (see EVEN and ODD below).  Most applications will use bus width 1.

### 5.6.1	Burning a New PROM

The 1 >2716 default command selected a 2716 PROM.  To change PROM types or to see a list of available PROMs, type the command PROMS, which will display a list similar to the following:

( PROM types:

prom	type	menu
size	name	char )

2048 PROM >2716  B	 2048 PROM >27C16 L
 4096 PROM >2732  C	 4096 PROM >27C32 M
 4096 PROM >2732A D
 8192 PROM >2764  E	 8192 PROM >27C64 O
16384 PROM >27128 F	32768 PROM >27256 Z

Your list may differ.  PROM is a defining word, so that each line in the list creates a PROM type with a type name as shown, an associated size in bytes which is used to set LONG, and an associated menu character which is sent to the GTEK to initialize it properly.  To select a different PROM, type:  n typename, where n is the desired bus width in bytes and the PROM typename is one of those in your PROMS list.  For example:

1 >2732A.

Every PROM selection command sets START and AT to zero and LONG to the number of bytes in the PROM (the argument to PROM in the list above).  A START of zero means “start the burn with the first (zeroth) byte of the disk image.”  An AT of zero means “start by burning the first (zeroth) byte of the PROM.”  Use of non-zero values for START and AT is discussed below.

The starting block of the disk image is set with the command n IMAGE where n is the block number; for example:

300 IMAGE

The default image starting block is set to the constant NEW in the PROMS load block :

NEW IMAGE

The constant NEW is defined with the system electives and is normally such that the image is located near the upper end of the first system part.  You may need to change IMAGE to suit the size of your PROM.

Assuming you want to burn from the start of the disk image to the start of the PROM, none of these defaults needs to be changed.  Just insert a blank (erased) PROM in the GTEK and type BURN.  You may have to wait several minutes for the PROM to burn, depending on the type and size.  When it is done, you will hear a “beep” and will see the “ok” prompt.  BURN stops immediately if it detects an unerased byte or a defective PROM.  To summarize:

1.	Select a PROM type, such as:  1 >2716.

2.	Select a disk image starting block, for example:  300 IMAGE.

3.	Insert a blank PROM into the GTEK and type BURN.

4.	Repeat Step 3 for multiple copies.

### 5.6.2	Copying a PROM

To copy an existing PROM, you can first read it into the disk image and then burn the image into a new PROM (or PROMs).  To read a PROM, select the PROM type, set the disk image starting block (making sure there is enough space on disk for the entire image), insert the PROM in the GTEK, and type READ.  For example:

1 >2716  300 IMAGE  READ

This can take several minutes.  READ reads the PROM image into the disk image blocks, destroying the previous image.  READ also resets the variables controlled by START and AT to zero and LONG’s variable to the length of the PROM in bytes.

Once the image is read, insert the new (erased) PROM and type BURN.  You can now burn as many PROMs as you like from the current disk image.  If you wish to verify that the new PROM is correct, you can compare it to the disk image with the command MATCH.  In summary, here’s what you do to copy a PROM:

1.	Select a PROM type, such as:  1 >2716.

2.	Select a disk image starting block, for example:  300 IMAGE.

3.	Insert the original PROM and type:  READ.

4.	Insert the new PROM and type:  BURN.

5.	Optionally, type:  MATCH to verify the burn.

6.	Repeat Steps 4 and 5 for multiple copies.

### 5.6.3	Burning Partial PROMs

You can burn part of a PROM provided that the part you burn is erased. Set START to the starting offset (in bytes) from the beginning of the disk image and LONG to the number of bytes to burn.  For example, to burn the first half of a 2716 PROM from an image starting in Block 300, you would need to type:

1 >2716  300 IMAGE  1024 LONG  BURN

To burn the second half from the same image, you would type:

1024 START  BURN

The START command affects LONG, adjusting it to equal the PROM length minus the START value.  START also sets AT to be equal to START, so that the second half of the disk image will be burned into the second half of the PROM.  If you want to burn the second half of the disk image into the first half of the PROM, you need to explicitly reset AT:

1024 START  0 AT

The START, LONG, and AT commands, in combination, let you burn any part of a disk image into any part of a PROM.

### 5.6.4	ODD and EVEN PROMs

Some computers, such as the IBM AT, use PROMs in pairs, with the even-numbered bytes of memory in one PROM and the odd-numbered bytes in another.  The data bus width is effectively 16 bits (2 bytes) and so has a bus width of 2; for example:

2 >2716

In this case you use the EVEN command to burn the even-numbered bytes of a disk image into one PROM and the ODD command to burn the odd-numbered bytes into another:

Insert the first PROM then type:	EVEN BURN

Insert the second PROM then type:	ODD BURN

Remember that LONG specifies the number of bytes in the disk image rather than in the PROM itself.  If you need to burn partial PROMs, there are two bytes of disk image associated with each byte of PROM, so LONG must be twice the length of either PROM.

### 5.6.5	Images Larger Than One PROM

Often you may need to burn a large program into several smaller PROMs.  The CHIP command specifies which smaller PROM to burn.  For example, suppose you need to burn an 8192-byte program into four 2716 PROMs. You would type, in sequence:

0 CHIP BURN
1 CHIP BURN
2 CHIP BURN
3 CHIP BURN

The disk image size should be an integer multiple of the PROM size (some parts of the image may be zero).  If the bus width is 2, CHIP can be combined with ODD and EVEN:

0 CHIP EVEN BURN	(first PROM)
ODD BURN	(second PROM)
2 CHIP EVEN BURN	(third PROM)
ODD BURN	(and so on)

### 5.6.6	Other PROM Programmers

If your PROM programmer is not one of the GTEK models described here, you may well be able to adapt the GTEK code to some other type.  Most intelligent PROM programmers use the same general format and communicate with you over an RS-232 serial interface line.  A typical dialog looks like this:

1.	You send spaces or carriage returns to the PROM programmer, allowing it to synchronize with you.

2.	It sends a sign-on message and a prompt sequence, which you can discard.

3.	You send setup information, discarding all prompts.

4.	You send the PROM image.

You should be prepared to respond to any error codes you receive.

## 5.7	NETWORK Utility

The network utility is used to move ranges of blocks between two polyFORTH systems; they do not need to have the same block numbers on each system.  Binary data as well as source code and files may be sent. Before using this utility, you should establish that you have a working serial interface between the two computers.  If you have a “break-out box” you can verify that signal lines are not crossed and are at the proper voltage levels, and you should use simple TYPE and EXPECT commands to verify that characters can be sent successfully in each direction.  Each system should have the serial terminal task  REMOTE defined and active, typically running at 9600 baud.

The NETWORK utility has only three commands: TRANSMIT, RECEIVE, and C.  The TRANSMIT and RECEIVE commands each take a range of blocks in “SWAPped” DO ... LOOP form; for example:

100 200 TRANSMIT  means  TRANSMIT  blocks 100 to 199.

100 101 RECEIVE   means  RECEIVE   block 100 only.

100 100 RECEIVE   means  RECEIVE   block 100 only.

The C command returns the status of the transfer, and may be executed at any time on either the receiving or the transmitting computer.  For example, on the transmitting computer before transfer has begun, one would see:

C  0 -1

The first number returned (0) is the number of the last block successfully transmitted (or received, if the receiving computer).  In this case, no blocks have been transmitted yet.  The second number (-1) is the CTR of this computer’s REMOTE task.  In this case, the transmitter’s REMOTE is waiting to receive one “go-ahead” character from the receiving task.

The network protocol operates as follows:  First, the transmitting computer sets up the range of blocks to be sent by executing:

start  end+1  TRANSMIT

as described above.  This action sets transmitter’s REMOTE to expect one handshake character, an ASCII 4 (EOT).  Next, the receiving computer executes:

start  end+1  RECEIVE

where the length of the block range must match the transmitter’s range, but the specific block numbers do not need to match.  This action sends the EOT to the transmitter, which replies by sending the first block (1024 bytes) and then returning to EXPECT mode for another EOT (if more than one block is to be sent).  Upon receipt of 1024 bytes, the receiver generates the EOT needed for the second block transmission, and so on.

The transmitter’s CTR status (as reported by C), will start at -1, go to 0 when the EOT is received, then jump to 1024 and count down to 0 as the block’s bytes are transmitted. The receiver’s CTR status will start at -1024 and count up to 0 as the block’s characters are received.  This sequence repeats for each block that is sent.

For example, suppose you want to transmit Blocks 9-11 to another computer where they will be stored as Blocks 20-23.  The transmitting computer goes first:

9 12 TRANSMIT	(send 3 blocks:  9 then 10 then 11)

The receiving computer goes second:

20 23 RECEIVE	(receive 3 blocks: 20 then 21 then 22)

The transfer begins immediately. You can use C to monitor the progress of the transfer on each computer.  Typical values might be:

Receiving Side	Transmitting Side

C  20 -300	C   9 300
C  20 -100	C   9 100
C  21 -502	C  10 502

Once the transfer is complete, the receiver’s C will show the last block number received and that no further characters are expected:

C  22 0

NETWORK has no “handshaking” logic beyond the initial request to send for each block.  If characters are lost, the receiver’s C will show that characters were expected when transmission ceased, and which block they were for:

C  21 -2
C  21 -2
C  21 -2

All previous blocks were correctly sent (at least 1024 characters per block were received), and no blocks after this one were attempted.  If C is “stuck” this way, correct the line problem and re-transmit blocks starting with the first incorrect one.

.

# 6.0	THE ASSEMBLER

Forth is one of the fastest, most efficient high-level languages available and is used extensively in real-time programming and applications programming.  Such programs are usually written in the extensible Forth word set.  However, where particular time constraints exist, Forth can assemble machine-language definitions of Forth words.  Among the many examples of words defined by machine-language instructions in the Forth nucleus are the operations:

+  -  SWAP  DROP  2DUP

The assembler for your particular CPU is detailed in the CPU Supplement to this manual.  This section provides a general overview of the assembler on any Forth system.  Note that the assembler is not used in ordinary high-level Forth programming, only in CODE definitions.  Assembler code is, by definition, machine-dependent.  However, there are many characteristics of Forth assemblers that are relatively consistent across all the processors on which Forth has been implemented.  It is this set of common characteristics which this section addresses.  Examples will be given using code for some of the most popular processors; unfortunately, space will not permit providing versions of each example for all processors.  Hopefully, the specific examples will still be comprehensible.

## 6.1	Code Definitions

The Forth defining word CODE creates a standard dictionary entry whose code address field contains the address of the byte that follows, which is the first byte of the parameter field where machine instructions are assembled.  See Fig. 6.1 for a diagram of this dictionary entry.  The form of a CODE definition is:

CODE NAME ... instructions ... code ending

CODE creates the definition, whose name is NAME.  It also selects the ASSEMBLER vocabulary, in which the various instruction mnemonics, addressing modes, etc., are defined.  These are used to build actual machine instructions, which are laid down in subsequent locations in the dictionary.  The code ending is one of several macros, all of which ultimately return to Forth’s address interpreter.

Fig. 6.1

Diagram of a dictionary entry for a CODE entry.

Aside from the dictionary entry header there is no high-level language overhead in either space or time within a code definition.  All instructions are executed at full machine speed.

As a general rule, Forth programs are written first in high-level language.  Then a time analysis is performed to locate the most frequently executed words, which are then CODEd.  Two examples of such words might be:

1.	The portion of an interrupt routine that actually moves data to and from a device.

2.	The innermost loop of a routine where the computer spends a significant portion of its time (for example, the word NEXT in the Forth nucleus).

REFERENCES

Code Endings, Section 6.2
Macros, Section 6.7

## 6.2	Code Endings

Most Forth code routines end with a jump to the address interpreter, sometimes after modifying the stack. Exceptions are interrupt routines (which return to the code that was being executed before the interrupt occurred) and routines which initiate processes whose completion will be signaled by an interrupt.  These end with a jump to the part of the multiprogrammer that will deactivate the current task and begin searching for the next one.

The most common code ending is NEXT, a macro that assembles the address interpreter return.  This, plus the other most common code endings are summarized below.  On some processors these endings assemble code or branches to the appropriate code, whereas on others they return addresses which may be used as arguments to a JMP.  Refer to your CPU Supplement for a list of the code endings for your processor.

Word	Description

NEXT	Address interpreter return.

WAIT	Enters the multitasking loop.

' PAUSE JMP	Enters multitasking loop for one turn.  The code for PAUSE is prefaced by instructions that decrement I by 2 so that the word containing ' PAUSE JMP will be executed over and over again.  This is useful for waiting on polled devices

INTERRUPT	Returns from interrupt.

Check your CPU Supplement for the appropriate list of code endings for your processor.

## 6.3	Assembler Instructions

To compile a colon definition, the interpreter enters a special compile mode in which the words of the input string are not executed (unless designated as IMMEDIATE).  Instead, their addresses are placed sequentially in the dictionary. During assembly, however, the interpreter remains in execute mode.  The mnemonics of the processor instructions are defined as words which, when executed, assemble the corresponding operation code at the next location in the dictionary.  Operands (addresses or registers) precede instruction mnemonics in order to leave information on the stack that will be used by the mnemonic to assemble the instruction.

Depending on the processor, several kinds of instructions and addressing are possible.  These are defined in the polyFORTH assembler for each processor to assemble instructions in the appropriate format, given the mnemonic operation code and the additional parameters that are necessary to describe the instruction.  The instruction is assembled into the next available location in the dictionary.

For example, the Intel 8080 processor has an ALU reference instruction format for instructions that perform arithmetic computations.  The Forth assembler defines the command ALU, which is used to define mnemonics of the ALU class, which in turn assemble ALU reference instructions.  For example, the mnemonic ADD is defined on 8080 systems by:

80 ALU ADD

ADD is an operation which assembles an ALU-type instruction whose numeric code is 80H and whose operand will be on the stack.  In use,

L ADD

assembles an instruction which, when executed, will add the contents of Register L into the accumulator.

## 6.4	Notational Conventions

Although each Forth assembler uses the manufacturer’s mnemonics, there are some standard Forth notational conventions that are shared by all assemblers. Fundamental Forth pointers have standard names:

Name	Description

S	Address of the top of the parameter stack.

W	Address of the parameter field or code field of the current definition.

I	Interpreter pointer.

R	Address of the top of the return stack.

U	Beginning of the user area.

These are often registers, but may reside in memory in some computers.  Refer to your CPU Supplement for a discussion of their locations on your system.  Wherever these pointers reside, the standard names may be used in code to refer to them.

Registers are numbered in a way that reflects the manufacturer’s usage and the actual bits used in assembled instructions.  In addition, for convenience and readability, some registers are given names by using CONSTANT.  Thus, on the DEC PDP-11:

5 CONSTANT S

enables you to refer to Register 5 by number or by the name S (which identifies this register as containing the stack pointer).  Similarly, the Intel 8080 has named Registers H and L.

Forth code routines tend to be extremely short, averaging under a dozen instructions on 16-bit processors.  Moreover, Forth assembler code is entirely structured.  The conventional vertical format that includes comments on each line is not very helpful.  FORTH, Inc. uses a horizontal format, with three spaces between each instruction and one space between each component of an instruction (address specifiers, the mnemonic itself, etc.).  In this format, the average code definition occupies only two or three lines and is still readable.  Comments that are enclosed by the command ( and its delimiting ) may appear anywhere:

( in parentheses like this)

Shadow blocks are also helpful.

REFERENCES

Disk and Block Layout, Section 5.2.5
Documentation Facilities, Section 1.5

## 6.5	Use of the Stack in Code

It is necessary when using code to distinguish how the stack is used at assembly time and at execution time.  The words in a code entry are executed at assembly time to create machine instructions which are placed in the dictionary to be executed later.  Thus,

HERE 2- TST

at assembly time places the current dictionary location on the stack (HERE) and decrements it by two.  The resulting number is the parameter for TST, which assembles a machine instruction that is the equivalent of:

TST *-2

in conventional assembler notation. Similarly, such words as SWAP and DUP are executed at assembly time to manipulate the parameters being used by assembler words, although such stack words would be compiled into the dictionary in a : definition.  For example, in the 8080:

0  HERE SWAP  H LXI  JMP

assembles an endless loop that loads zero into the accumulator.  HERE pushes the address of the next free byte of dictionary space onto the stack.  The phrase H LXI takes the zero from the top of the stack (at assembly time) and assembles a “load index immediate” that will load zero into the HL register pair.  The JMP uses the address left on the stack to assemble a jump to the first byte of the load.

In high-level definitions the run-time use of the stack is implicit:  Numbers that you type are placed there automatically, routines naturally leave their results there, etc.  Code, however, requires that parameters be handled explicitly, using S (the parameter stack pointer) and the code-endings that push or pop the stack before the execution of NEXT.

## 6.6	Addressing Modes

Although in general, Forth’s assembler implements the processor manufacturer’s mnemonics, there are standard notational conventions used in all processors for specifying addressing modes.  Obviously, not all processors have all addressing modes or interpret terms such as “relative” identically. Nonetheless, certain basic concepts do exist and it’s helpful when you’re working with several processors to have these concepts expressed in standard ways.

Refer to your CPU Supplement for the specific addressing modes that are implemented in your system.

Standard Forth addressing notation includes the right parenthesis, which indicates either relative addressing (when it is by itself) or indexing (when it is combined with an index register designation).  Some examples:

Word	Function

S )	Addressing relative to the top of the stack.

S)	Indexed by S.

1)	Indexed by Register 1.

On machines with automatic incrementing or decrementing, the parenthesis may be combined with + or -.  On the DEC PDP-11 and the TI 9900, for example:

Word	Function

S )+	Refers to the number on top of the stack, “popping” it off at the same time; that is, incrementing the stack pointer.

S -)	Refers to the next available location on the stack, a “push” operation.

The position of the sign indicates when the increment or decrement takes place with respect to the development of the effective address, in this case postincrementing and predecrementing.

Immediate addressing is indicated by # and memory-indirect by the right parenthesis; the assembler can determine from the address whether ) means register-relative or memory-relative (indirect).  In addition, there are specific items of notation for each processor—these are described in the CPU Supplement.

Parameters may be taken directly from memory if this is permitted by the architecture of the processor.  The assembler will automatically check to determine whether the address of the argument permits a short format instruction.  If it will not, an extended format will be used.  Often parameters may be picked up without being named.  As long as an address is on the stack, it doesn’t matter how it got there:

HERE 55 , ... LDA

will enter the literal 55 in the dictionary and leave its address on the stack at assembly time.  (The operation puts the number that is on the stack into the dictionary at HERE and increments H by one cell.)  The instruction LDA encounters the address on the stack and assembles an instruction to move its contents to Register A.

REFERENCES

, (comma), Section 2.7.2

## 6.7	Macros

Macros are easily defined in Forth by using : definitions that contain assembler instructions.  For example, on the RCA 1802 one frequently uses the operations DEC and STR successively on the same register.  For convenience, the macro:

: DST ( r)   [ ASSEMBLER ] DUP DEC  STR ;

has been defined.  Then S DST assembles the two instructions:

S DEC  S STR

Note the way the DUP in the definition of DST allows the single parameter S to be used by both the DEC and STR mnemonics.

Macros are mainly a notational convenience; DST assembles two instructions, just as if the expressions had been written out in full.

The words used to implement the assembler structures (loops and conditionals) are defined as macros, as are the code endings.

## 6.8	Program Structures

Control of logical flow is handled by Forth’s assembler using the same structured approach as high-level Forth, although the implementation of the commands is necessarily different.  The commands even have the same names as their high-level analogues; ambiguity is prevented by use of separate vocabularies.  The following are implemented as standard macros:

Word	Function

BEGIN	Puts an address on the stack (HERE).

UNTIL	Assembles a conditional jump back to the address left by BEGIN.  It is preceded by a condition code.  The loop is ended if the condition is met.  Common condition codes are 0= and 0<, as appropriate to the various CPUs.

NOT	Inverts the action taken for a condition code.

IF	Assembles a conditional forward jump, to be taken if the preceding condition is false, leaving the address of this instruction on the stack.  It is also preceded by a condition code.

Word	Function

ELSE	Provides the destination of IF’s jump (whose address was on the stack) and assembles an unconditional forward jump (whose location is left on the stack).

THEN	Provides the destination for a jump instruction whose location is on the stack at assembly time (left by IF or ELSE).

The ELSE clause may be omitted entirely.  This construction is functionally analogous to the IF ... ELSE ... THEN construction provided by Forth’s compiler.  For instance,

0= IF {code for 0}	ELSE {code for not 0} THEN ...

0= IF {code for 0}	THEN ...

Please note, however, that whereas the IF and UNTIL in high-level Forth remove an item from the stack and test it, the corresponding assembler words assemble conditional branches whose action will depend on condition codes set by the result of a previous instruction.

Since the locations or destinations of branches are left on the stack at assembly time, the structures BEGIN ... UNTIL and IF ... ELSE ... THEN may be nested naturally.  By manipulating the stack during assembly, however, you can assemble any branching structure.

If you wish to branch forward, use IF to leave the location of the branch’s address field on the stack.  At the branch’s destination, bring the location back to the top of the stack (if it is not there already) and use ELSE or THEN to complete the branch (by filling in the branch’s destination at the location that is on the top of the stack).  If you wish to branch back to an address, leave it on the stack with BEGIN.  At the branch’s source, bring the address to the top of the stack and use UNTIL or a jump mnemonic to assemble a conditional or unconditional branch back.  Be sure to manipulate the branch address before the condition mnemonic since each condition code adds one item to the stack.

Suppose, for example, you wish to define a word LOOK, which takes two parameters (a delimiter on top of the stack with a starting address beneath it) and which scans successive bytes until it finds either the delimiter or a zero.  The number of characters scanned is returned.  Here is a definition of LOOK for the Motorola 6800:

~ CODE LOOK ( a c - n)   B PUL  A PUL  TSX
   0 ) LDX  BEGIN   0 ) TST   0= NOT IF
      0 ) A CMP   0= NOT IF   INX   B INC
      ROT JMP  THEN THEN   A CLR
   TSX   PUT JMP

Here the phrase 0= NOT IF (used twice) assembles two conditional forward jumps which will be executed if the character scanned is the same as one of the delimiters.  If the loop is to be repeated, after B INC a JMP is needed back to the BEGIN.  Since the intervening IFs have left their locations on the stack, the backwards branch must be assembled by ROT JMP.  The ROT (executed at assembly time) pulls the address left by BEGIN to the top of the stack where it is used as JMP’s destination.  Finally, the THENs fill in the destination of the IFs.

There are no labels in Forth.  Although you could define them, their functions are better performed by the words IF, ELSE, THEN, BEGIN, and UNTIL.  Since CODE definitions are usually extremely short, labels are not particularly desirable; they tend to encourage complicated flow patterns that are not appropriate in Forth.

## 6.9	Literals

Some processors allow you to define instructions to reference literals.  For these, the standard Forth word for identifying a literal is #.  Thus the instruction:

1000 # 0 MOV

would move the literal 1000 into Register 0.  A few processors allow a short instruction format for small literals and a long format for larger ones.  In such cases the Forth assembler automatically examines the literal and generates the appropriate format.

On processors that do not directly support literals, the main technique for supplying them is to compile a literal directly, and then pass the literal’s address to an instruction that references it by HERE.  For example:

HERE 1000 ,
CODE FIX   0 MOV ...

In this example the literal 1000 is placed in memory and its address left on the stack.  The MOV instruction assembles a reference to that address.  When executed, the effect will be to move 1000 into Register 0.

## 6.10	Device Handlers

Device handlers should be kept extremely short, including only the instructions required to pass a value to or from the stack or to issue a command.  Consider, for example, a self-scan character display that is interfaced to an RCA 1802 as Device 2.  This is all that is needed to output one character from the top of the stack:

CODE (EMIT) ( c)   S INC   S SEX
   2 OUT   NEXT

In this example, S INC increments the stack pointer (to get the low-order byte), S SEX sets S as the output register, and 2 OUT sends the character to the device, incrementing S again to complete a POP.

Given this hypothetical definition of (EMIT) you could define (TYPE) at high-level to display a string of characters whose byte address and length are on the stack:

: (TYPE) ( a n)   0 DO  PAUSE  DUP C@
   (EMIT)  1+ LOOP  DROP ;

To convert and display a number on the stack, you could define SHOW:

: SHOW ( n)   (.) (TYPE) ;

Here (.) performs the conversion, leaving the address and length of the resulting string for (TYPE).  The point here is that, given the simple code definition (EMIT), full control of the display is available in high-level Forth.

Device drivers are highly variable in nature, depending upon both the processor and the actual device.  You’ll find a discussion of drivers for your processor in your CPU Supplement and useful examples in the system listings.

## 6.11	Interrupts

The presence of Forth’s multiprogrammer (plus a few standard conventions) makes dealing with interrupts in Forth relatively simple.  The principle strategy is to perform only the most time-critical actions at interrupt time, to notify the task responsible for the interrupting device that the interrupt has occurred, and to defer all complex logic to high-level routines executed by that task.  “Notification” may take the form of setting a flag, incrementing or decrementing a counter, or modifying the task’s status such that it will become active at the next opportunity in the multiprogrammer cycle.

The basic form of an interrupt handler is as follows:

ASSEMBLER BEGIN {code instructions} dev# INTERRUPT

where ASSEMBLER selects the assembler vocabulary (CODE does this for you automatically but should not be used because it builds an unneeded head); BEGIN pushes onto the stack the address of the beginning of the code (which will be used by the word INTERRUPT); the code instructions perform the necessary work of the routine; dev# stands for the device code or interrupt vector to which the routine will respond, and INTERRUPT is a special code ending macro that assembles the appropriate “return from interrupt” instruction and sets the address of the code supplied by BEGIN in the interrupt vector.

The actual implementation of INTERRUPT is highly processor-dependent.  On machines with hardware-vectored interrupts, the implementation merely stores the address of the code in the specified vector address.  On such machines, interrupts incur no additional overhead:  only the instructions in the interrupt routine itself are executed.  On machines in which software must identify the interrupting device, the identification system is system specific—consult your CPU Supplement.  However, one popular method is to set the polling routines in a chain, with the CPU’s interrupt vector pointing to the first polling routine.  If the device served by the first routine did not generate the interrupt, the first routine executes a jump to the second routine.

On every system conventions are established for the use of registers at interrupt time.  On most systems, you may not use any registers without saving and restoring them.  Save and restore only the registers you are actually going to use!  The usual place to save registers is on the return stack.  On systems with only one hardware stack, the parameter stack becomes the place of choice.  On systems with software vectors and few registers, one or two registers are routinely saved and restored so that you may use them freely.  Consult your CPU Supplement for details.

## 6.12	Example

As an example of the action of the assembler, consider the definition of the high-level comparison operator 0=.  This word expects a value on the stack.  If the value is non-zero, it will be replaced by a zero (false); if it is zero it will be replaced by a negative one (true).  The code for this routine on the Intel 8086 is:

CODE 0= ( n - t)   0 POP   0 0 OR
   0 # 0 MOV   0= IF   0 DEC
   THEN   0 PUSH   NEXT

Fig. 6.2 and Table 6.2 show the processor during compilation and execution of this routine.  Table 6.3 shows, for the purpose of comparison, the definition of 0= for other processors.

DECODE 0=
	58D	58	0 POP
	58E	0B C0	0 0 OR
	590	B8 00 00	0 # 0 MOV
	593	75 01	596 JNE
	595	48	0 DEC
	596	50	0 PUSH
	597	AD 97 FF 25	NEXT

Fig. 6.2

Disassembly of 0= (8086 version) using the polyFORTH disassembler utility (see the CPU Supplement for details).

Table 6.2

Assembly of 0= on the 8086

Instr.	Action During Assembly	Action During Execution

0	Pushes 0 (Register AX) onto the stack.	Pops the top of the stack into

POP	Assembles a POP instruction, refer-	Register AX.

encing Register 0.

Instr.	Action During Assembly	Action During Execution

0	Pushes 0 onto the stack.	OR’s Register AX with itself,

0	Pushes 0 onto the stack.	which sets the status bits.

OR	Assembles an instruction to “or”

Register AX with itself.

0	Pushes 0 onto the stack.	Puts a zero in Register AX,
#	Pushes 10 onto the stack to indicate	without affecting the status
	immediate addressing mode.	bits.

0	Pushes 0 onto the stack.

MOV	Assembles an instruction to move
	a zero to Register 0 (AX).

0=	Pushes 74H onto the stack (code for ≠)	Branches if the condition bits

IF	Assembles a JNE instruction with	do not indicate a zero.
	a destination address of zero, and 
	leaves the address of the destination 
	field on the stack.

0	Pushes 0 onto the stack, for Reg. AX. 	Decrements Register AX.  
DEC	Assembles an instruction to decre-	Since AX contained 0, this

ment Register AX.	leaves a -1 in AX.

THEN	Stores the current value of H (the	No action.

top-of-dictionary pointer) in the

address left on the stack by IF.

Assembles nothing.

0	Pushes 0 onto the stack.	Pushes the content of Register

PUSH	Assembles an instruction to push 	AX (containing either 0 or -1)

the contents of Register 0 (AX) onto 	onto the stack.

the stack.

NEXT	Assembles the following sequence:	Returns to the address inter-

LODS W 0 XCHG W) LIP	preter.

Table 6.3

Definition of 0=

On the 68000:

CODE 0= ( n - t)   D1 CLR   S ) TST
   0= IF   1 #Q D1 SUB
   THEN  D1 S ) MOV   NEXT

On the PDP-11:

CODE 0= ( n - t)   0 CLR   S ) TST
   0= IF   0 DEC   THEN
   S ) 0 MOV   NEXT

# 7.0	TARGET COMPILATION

Target compilation was originally developed to cross-compile a Forth system from one type of computer to another.  Now target compilation is used to produce stand-alone applications that do not require the full Forth programming environment.

The target compiler is not a single program, but a collection of software tools to solve three problems:

1.	How can a system recompile itself?  This problem is solved by compiling the new system into an out-of-the-way place.  The assembler and compiler are redefined.  The process of using polyFORTH to recompile itself is discussed in Sections 7.3, 7.4, 7.5, 7.6, and 7.7.

2.	How can the old system locate objects in the new system?  This problem is solved with special dictionary entries, which are discussed in Sections 7.1, 7.2, 7.7, and 7.8.

3.	How does the old system differentiate between the dictionary entries of itself and the new system? This problem is solved by using vocabularies, as discussed in Sections 7.1, 7.2, 7.7, and 7.8.

Other sections discuss diagnostics, debugging and other useful techniques.

The target compiler is a utility which is loaded by the command:

COMPILER LOAD

Subsequently, you load your application using a load block which, in turn, loads the source blocks describing the nucleus primitives and other system support words needed, and finally the application itself.  More detailed information about recommended target compilation procedures is provided in Sections 7.1 and 7.13.

There is a facility for interactively testing target compilable applications before they are target compiled.  This facility is in a block called “The target compatibility block.”  You may find it helpful to look at its commands to understand the general meaning (but not the actual process) of certain target compiler words.

REFERENCES

The Target Compatibility Block, Section 7.12

## 7.1	Resident, Host, and Target Words

Target compilation presumes that two machines exist.  The machine that already runs polyFORTH and acts as a host for the target compilation is called the “host” machine.  The machine which is the target of the compilation effort is called the “target” machine.

Three sets of words exist when performing target compilation.

1.	“Resident” words are the ordinary, native Forth words which reside on the host system.

2.	“Host” words compile definitions for the target system, but are written using resident words.  Host words provide the means by which the host offers its resources to compile the new target system.

3.	The words which will execute on the new target system are called “target” words.

Target definitions differ from normal Forth definitions in that they have two dictionary entries:  one in the dictionary being compiled for the target, and one in the dictionary of the host.  A target word’s code is in the target dictionary, and will be executed by the target computer.  The host’s definition for the target word is basically a constant, with special run-time behavior which compiles the constant’s value into the target’s dictionary (see the reference to EMPLACE, below).  This value is the address of the target definition’s code field in the target dictionary.

The host entry is necessary because during target compilation the resident system’s dictionary is the only one that can be searched.  Target dictionary entries may not have names and dictionary links, and may have run-time code which cannot execute correctly on the host.

The definitions that make up the new compiler and assembler used to produce definitions for the target system are host words.  In this section they are called the host compiler and host assembler.

Resident, host, and target words all have their own vocabularies in the host’s dictionary.

REFERENCES

Defining Words, Section 2.7
EMPLACE, Section 7.7
Vocabulary Conventions, Section 7.2
Vocabularies, Section 3.4

## 7.2	Vocabulary Conventions

The host system differentiates between resident, host, and target words by using vocabularies.  If you are not familiar with polyFORTH’s use of vocabularies, you should review Section 3.4.

All of the vocabularies used in the target compiler reside on the host machine.  The original polyFORTH system uses three vocabularies:

FORTH	0001
ASSEMBLER	0013
EDITOR	0015

The last digit is the first vocabulary searched.  The numbers are in hexadecimal with leading zeros shown.  Your machine may have the digits in reversed order.

Two new vocabularies are used for host words:

Search
Vocabulary	Order	Description

HOST	0017	The HOST compiler, sometimes called the “target” compiler.  Contains the words which produce the target program.

ASSEMBLER	0179	The HOST assembler, sometimes called the “target” assembler..  Contains the assembler words used to produce the target program’s nucleus.

In addition to these, FORTH is re-defined to have a search order of 0071H (adding HOST to the search order).  HOST and this version of FORTH are also IMMEDIATE words, unlike most vocabularies in polyFORTH, meaning that they will be executed immediately when they are invoked in colon definitions.

If the target system is itself programmable, three target vocabularies must reside in the host with index numbers similar to the resident vocabularies FORTH, ASSEMBLER, and EDITOR, but are offset by 10 decimal (0AH).  The respective hexadecimal assignments for the target versions of the standard vocabularies are:

FORTH	000B
ASSEMBLER	00BD
EDITOR	00BF

The vocabulary conventions for the target compiler are intended to provide syntactically equivalent access to both the resident and target vocabularies.  The target vocabularies are accessed by the host version of -'.  Since the host compiler only uses the host -', the host compiler will only compile words which have entries in the host’s target vocabularies.  The host version of -' works by temporarily substituting a vocabulary search pattern from the variable VOC into CONTEXT, and then performing a resident -'.

VOC serves as the target compiler’s CURRENT, in that it controls the linking of new words.  It also functions as its CONTEXT (when it is used by the host version of -').  The normal value of VOC is 000B.

The word TARGET unlinks the most recently defined word from a resident vocabulary of the host dictionary and relinks the new word into the primary vocabulary of VOC (usually target-FORTH).  TARGET is used immediately following the definition of a word to create words which will be executed by the target compiler rather than just compiled.  TARGET is analogous to the resident word IMMEDIATE.  A good example of TARGET’s use is in the definition of the target compiler’s compiler directives (see Block 249 of a system listing).  The target compiler usually searches only the target vocabularies (according to VOC), and thus its compiler directives must be in the target vocabularies.

REFERENCES

-', Section 2.6.1
Host (CREATE), Section 7.5.2
Vocabulary Definitions, Section 3.4.2

## 7.3	Dictionary Conventions

The target compiler was designed to be usable for as many types of applications as possible.  The polyFORTH nucleus is designed to work in either read-only memory or read/write memory.  The target compiler can therefore compile applications for two major environments:

1.	The “conventional” environment in which most system memory is read/write memory.  The system probably has a disk and terminals.

2.	The “dedicated” environment, in which the program resides permanently in read-only-memory, with a small block of scratch-pad read/write memory.  All types of memory may need to be minimized if the dedicated system will be mass-produced.

The dedicated environment is more demanding of the target compiler, because two types of memory must be managed.  Read-only memory can contain programs and constants, but no variable data.  The presence or absence of read-only memory is the major difference between conventional and dedicated environments.

### 7.3.1	Dictionary Conventions for Read-Only Memory

Forth creates code which is naturally non-self-modifying and can be placed in non-volatile read-only-memory.  All Forth facilities, including multitasking and the target-compiler, are designed so that they will be usable to produce a system with a minimum amount of read/write memory.

The majority of dictionary entries in Forth contain executable routines.  The rest are labels to data areas.  In a Forth system using ROM (Read Only Memory) the permanent dictionary resides in ROM.  Data areas in RAM (Random Access Memory) are labeled by constants in the dictionary in ROM.  The value of the constant is a pointer into RAM.

Because the target compiler must be able to allocate both RAM and ROM, the target compiler maintains two dictionary pointers.  'H (pronounced “tick-H”) is the dictionary pointer most similar to the normal, resident pointer H.  'H points to the next available byte of read-only-memory.  'R is the dictionary pointer to the next available byte of read/write (RAM) memory.

These are the words for managing memory.  Note that sometimes pairs of words are defined which perform analogous functions for ROM and RAM:

Word	Stack	Function

WINDOW	( a)	The vocabulary to which the target system’s FORTH definitions are emplaced.

HDS	( - a)	Returns the address of the cell containing the number of bytes in a dictionary head, excluding the cfa.  HDS 2+ is the address of the default head length.  The head length in HDS is reset to HDS 2+ by (CREATE).  (On 32-bit processors, HDS 4+ is the address of the default head length.)  HDS and HDS 2+ are set by DICTIONARY.

DICTIONARY	( n)	Sets the maximum number of bytes in a dictionary name to n.  A value of zero will let (CREATE) compile just the code field, and results in a minimum size dictionary of headless definitions.  An interactive target uses 34 bytes, which yields a dictionary link field, a character count, and up to 31 characters.  A code field is always included.  Individual definitions can be made headless by preceding them with a | (“bar” ASCII 124 decimal), or they can be given nontruncated names by preceding them with a ~ (“tilde”).  DICTIONARY also clears the target space to a fill value (typically -1).

HERE	( - a)	Returns the address of the next byte of the target system’s ROM.  This word is similar to the resident HERE but refers to ROM storage only (not variable data areas, which must be in RAM).

THERE	( - a)	Returns the address of the next byte of the target system’s RAM.  This word is the RAM version of HERE used to refer to variable data areas.

ORG	( a)	Sets the ROM dictionary pointer ('H) to a.

GAP	( n)	Advances 'H by n to reserve n bytes of ROM. Similar to ALLOT.

Word	Stack	Function

ALLOT	( n)	Advances 'R by n to reserve n bytes of RAM.  Using ALLOT after WINDOW sets 'R to an initial non-zero RAM address.  Similar to GAP.

EQU name	( n)	Creates a host constant without a corresponding entry in the target dictionary.

LABEL name		Labels a ROM target address location in the host system only, using HERE and EQU.  When name is executed, it returns the target address.

|		Sets the number of bytes in HDS to 0, so that the following dictionary entry has only a code-field address.  HDS is automatically reset to a default after the target head is created by (CREATE).  Pronounced “bar” (ASCII 12410).

~		Sets the number of bytes in WIDTH to 31, so that the entire name is compiled for the following dictionary entry.  WIDTH is automatically reset to a default.  Pronounced:  “tilde”

RECOVER		Reclaims a cell in the dictionary.  Often used after infinite loops or words ending in STOP, ABORT or QUIT to reclaim the two-byte address of EXIT compiled by ;.

REFERENCES

~, Section 1.1.1

### 7.3.2	Dictionary Conventions for Read/Write Memory

A program compiled for ROM using the tools and conventions described above will work satisfactorily in a RAM system as well.  If you wish to avoid having to allocate pseudo-ROM and RAM separately, however, running with data and definitions intermingled in the nucleus as they are in a resident dictionary, you may easily modify the polyFORTH target compiler to accommodate this.

The definitions that must change are those for VARIABLE (and its relatives) and ALLOT.  In addition, you must change references to THERE, 'R, etc.

Fig. 7.1

Chart showing Forth words used to control memory space in resident and host compilers.

## 7.4	Compilation to a Virtual Dictionary

The target compiler produces an entirely new dictionary tailored by a programmer for the target computer.

The new dictionary is constructed in a space apart from the host dictionary, in RAM or on disk.  If the target dictionary is being built in RAM, it will be copied to disk at the end of the target compilation process.

The size of the target dictionary is controlled by the constant #K, which gives its size in units of 1024 (1K) bytes.  It will be put on disk starting at the block whose number is given by the constant NEW.

### 7.4.1	Words that Differ for Different Types of Target Space

When the target compilation takes place, the code produced is not “mixed in” with the resident dictionary.  Instead, space is set aside on disk or in RAM and then accessed by the target compiler through a small, device-independent vocabulary of fetch, store, and compile words.  The code is usually compiled to an 8K-byte buffer called RAM.  This space is set aside for performing target compilation and will be called “target space.”  The first byte of target space coincides with the first byte of the target application program.  In many (but not all) systems, the first byte of target space will be the target CPU’s memory location zero.

The target compiler is designed so that it can compile to either a RAM buffer or an area on disk.  The target compiler load block contains a line which will be commented:

( Compile to RAM)   or   ( Compile to disk)

The special words that must be defined for each type of compilation are loaded by that line.  You may change that line so that the target compiler compiles to either disk or RAM.  To find the exact block numbers refer to the disk index at the beginning of your program listing to find the ( Compile to RAM) and ( Compile to disk) blocks in the target compiler section of your system disk.

Certain words are defined or redefined so as to operate in target space:

Word	Stack	Function

>T	( a - a)	Translates a target space address into a host address.  See the following subsections for details.

TC@	( a - b)	Fetches a byte from target space.

TC!	( b a)	Stores a byte to target space.

T@	( a - n)	Fetches a cell from target space.

T!	( n a)	Stores a cell to target space.

T+!	( n a)	Adds to a cell in target space.

C,	( b)	Compiles a byte in the next byte of ROM in target space.

,	( n)	Compiles a cell in the next two available bytes of ROM in target space (next four available bytes on 32-bit processors.

CMOVE	( s d n)	Moves n bytes starting from a host-system source address s to a target space destination d.

DUMP	( a n)	Dumps n bytes to the display, starting at target space address a.

DICTIONARY	( n)	Sets the maximum number of bytes that will be placed in a dictionary name to n.  A value of zero will let (CREATE) compile just the code field address and results in a minimum size dictionary of headless definitions.  The default size is 34 bytes, which gives a dictionary link field, a character count, and 31 characters.  A code field is always included.  Individual definitions can be made headless by preceding them with a | (“bar”), or they can be given non-truncated names by preceding them with a ~ (“tilde”).  DICTIONARY also clears the target space to 0 or -1.  The fill value should be -1 to allow patching in most ROM-based systems.

Depending on where the target space is kept, some other words, notably FLUSH, are redefined.  FLUSH occurs just once in the target compilation, at the end.  FLUSH saves the target space on disk.  On systems which are compiling to RAM, FLUSH is redefined to copy the target space to the range of blocks whose start is given by the constant NEW and length by #K.

### 7.4.2	Compiling to RAM

One of the places into which the target compiler can compile code is a memory buffer #K bytes in length called RAM.  Compiling to RAM is much faster than compiling to disk.  For extensive target-compiled applications, RAM may need to be larger on some computers.

The compiled program is always saved on disk at the end of the target compilation; usually in the same place as with the “compile-to-disk” option (change Block 240 to alter the “compile-to-RAM” to “compile-to-disk”).

Since target compilation compiles to different places, the definition of the word >T (pronounced “to-T”) changes.  >T translates a memory address for the target machine into an address for the host machine’s target compilation space.  The definition for >T differs considerably between a target compilation to disk and a target compilation to the buffer called RAM.  When target compiling to RAM, >T basically adds an offset to the target address to displace it into the buffer called RAM.  All physical addressing in the target compiler passes through >T.

Some other words are defined in special ways for RAM compilation:

Word	Function

DICTIONARY	Clears the RAM buffer, and sets HDS, the maximum dictionary length.

FLUSH	Moves the RAM buffer to disk, to the range of blocks starting at NEW.  FLUSH should occur only once in a target compilation:  at the very end.

DUMP	Redefined to dump target space to the display.

### 7.4.3	Compiling to Disk

If memory is limited (for example, if you are one of a number of programmers sharing a system with limited partition sizes), you can compile your program directly to the disk region described in the previous section.

The target compiler can compile to either disk or RAM; the choice is up to the user, based on how much RAM is available.  Change Block 240 to alter the “compile-to-disk” option to the “compile-to-RAM” option.

Compiling to disk is slow, but does not require a large region of memory.  Compiling to disk is sometimes necessary when several programmers are using one machine.

As target compilation compiles to different places, the definition of >T changes.  >T translates a memory address for the target machine into an address for the host machine’s target compilation space.  When compiling to disk, >T uses BLOCK to treat disk as virtual memory.  All physical addressing in the target compiler passes through >T.

Some other words are defined in special ways for disk compilation:

Word	Function

DICTIONARY	Clears the disk blocks of the compilation area (sets all the values to -1 or 0), and sets HDS, the maximum dictionary name length.

CMOVE	redefined in high level.  Moves from host memory to target space (on disk) using TC!.

DUMP	Redefined to dump target space to the display, using T@.

### 7.4.4	Compiling to a Remote Target

It is possible to target compile directly into a target machine, provided that the machine has sufficient read/write memory (with the correct addresses at RAM), a serial link to the host, and a “talker” program.

The talker program must be able to: read a byte from target memory to the serial link; write a byte from the serial link to target memory; and begin execution at an address given over the link.

An advantage of this form of target compilation is that a  target-compiled system may be interactively debugged on the target machine.  Another major advantage is that the target-compiled system may use the host system’s peripherals, such as disks, printer and display.  The disadvantages are that the system interconnection hardware may be clumsy to arrange and in most situations, target compilation to RAM gives almost the same or more interaction.

This is the method of choice for cross compiling Forth to another machine.  Typically, a core system is first defined which performs terminal and disk functions over a serial line connected to the host.  Then the system I/O functions are added until a free-standing system exists.  The final step is the installation of a disk bootstrap.

The words are defined as follows:

Word	Function

>T	Adds an offset (which may be zero).

TC@	Is a serial I/O routine which sends an operation ID and an address, and receives a byte (usually using STRAIGHT, see references).

TC!	Is a serial I/O routine which sends an operation ID, an address and a byte.

Target memory reference is written to use TC@ and TC!.

Word	Function

EXECUTE	Is a serial I/O routine.  Sends an operation ID and an address.  The target machine jumps to the address.

FLUSH	Copies the target system to the host’s disk by using TC@ to fetch data over the serial link.

CMOVE	Moves bytes from the host to the target using TC!.

DUMP	Is defined to dump target space to the host’s display, using TC@.

REFERENCES

STRAIGHT, Section 3.7.1

## 7.5	HOST Defining Words

A HOST defining word ( see Fig. 7.2 and 7.3) creates dictionary entries in both the HOST dictionary and the target dictionary.  The target dictionary contains the defined word’s run-time behavior and parameter fields.  The host dictionary contains constants that point to the target dictionary entries.

Fig. 7.2

Fig. 7.3

Fig. 7.2 is a diagram of the structures used and generated by a host defining word using ;CODE:

FORTH : 2CONSTANT ( n)
   DUP TWIN  (CREATE),  ;CODE
   2 W) PUSH   NEXT

(8086 version).  An example of its use is:

0 01000 2CONSTANT LIMITS

Fig. 7.3 is a diagram of the structures used and generated by a host defining word using the host DOES>:

FORTH : VARIABLE   THERE CONSTANT
   2 ALLOT  DOES>  @ ;

An example of its use is:

VARIABLE RATE

REFERENCES

Defining Words, Section 2.7

### 7.5.1	Using HOST Defining Words

In many instances, an application can be written more conveniently and compactly if defining words are used.  A defining word for a target compiled application must be loaded after the Forth nucleus and after the host defining words in Blocks 246-250.  The nucleus must be loaded so that target run-time code is available for the definitions of CREATE and CONSTANT in Block 250.

A fairly common type of application defining word defines arrays.  A resident definition to define byte arrays would be:

: ARRAY ( n)   CREATE ALLOT ;

What the above word does is label the beginning of an area of memory (with CREATE), and then ALLOT memory to form space for the array. To create an array of 30 bytes named BUF, the word ARRAY would be used:

30 ARRAY BUF

Note that after this definition BUF can be used in colon definitions to provide an address:

: @BUF ( n - n)   BUF +  C@ ;

A host version of the word ARRAY should be usable in exactly the same way as a resident version.

If ARRAY were target compiled without change, several things would go wrong:

1.	The host : would compile ARRAY into the target dictionary, making it unavailable for interpretive execution in the host.

2.	If ARRAY were available in the host dictionary, the host’s CREATE in ARRAY would label a position in target ROM, rather than target read/write memory.

Fixing these two problems might yield the definition:

FORTH : ARRAY ( n)   THERE EQU ALLOT ;

The word FORTH ensures that the resident colon compiler compiles ARRAY into the resident dictionary (so that ARRAY can be executed).  THERE returns the address of the next byte of target RAM.  EQU, generates only a resident dictionary entry.  Thus, the following sequence is in error:

FORTH : ARRAY ( n)   THERE EQU  ALLOT ;
30 ARRAY BUF

: @BUF ( n - n)   BUF + @ ;

The error would occur because BUF (as created by EQU) has no entry in the target dictionary, and thus no address for BUF can be compiled into @BUF.  No error would occur if @BUF and all other words that reference BUF were defined in code, because in a code definition BUF would be executed interpretively at target compilation time.  Also, the erroneous definitions in the example above are one of very few situations in which code will load correctly with the target compatibility block and incorrectly with the target compiler.

The definition of the host word ARRAY which has exactly the same usage as the resident word is:

FORTH : ARRAY ( n)   VARIABLE 2-  ALLOT ;

Note that the target compiler’s version of VARIABLE for programs in ROM is a CONSTANT which returns the address of a cell in RAM.  Thus the phrase VARIABLE 2- is equivalent to the phrase THERE CONSTANT.

Some host defining words need to have special run-time behaviors.  For these instances, the target compiler provides versions of the words ;CODE and DOES>.  These words may be used exactly as in a resident system, except that ;CODE must be followed by code in the target’s assembly language.  The following is an example of a word using DOES>:

FORTH : COEFFICIENT ( n)   CREATE ,
   DOES> ( a)  @ * ;

The host version of DOES> works differently from the resident version; DOES> is executed because DOES> is IMMEDIATE.  The host DOES> compiles a run-time word for itself (see Fig. 7.3) in the host definition of COEFFICIENT, then compiles an entry point in the target dictionary.  The target code at the entry point jumps to the target’s run-time definition for DOES> in the target nucleus.  After assembling the target entry point, DOES> begins compilation, compiling the remainder of the defining word’s source definition into the target dictionary, following the defining word’s target machine code entry point.  The resident portion of the defining word has a compiled value at the end of its resident dictionary entry that is the address of the target run-time portion’s machine code entry point.

New target words are defined when the host CREATE makes a Target Compilation dictionary entry in the target’s dictionary, and the code compiled by the host DOES> changes the new target word’s code field address to point to the defining word’s run-time machine code entry in the target dictionary.

Frequently the most compact solution to a problem uses many defining words because defining words of the same type share code.

REFERENCES

Defining Words, Section 2.7

### 7.5.2	The Operations of HOST Defining Words

The first host defining word loaded with the target compiler is (CREATE).  (CREATE) is used by all host defining words to construct dictionary entries.  Most of the complexity of (CREATE) occurs because (CREATE) builds entries in the target dictionary which are either headless or have names and dictionary links.  (CREATE) has the following steps:

1.	It uses EMPLACE to make a constant in one of the host’s target vocabularies (see References below—EMPLACE).

2.	It compiles a dictionary link in the target dictionary and links the new entry between HEAD (the target’s GOLDEN array) and the previous value in HEAD.

3.	It compiles the new word’s name in the target dictionary, preceded by a length.

4.	The value in HDS (which controls the size of the word’s head in the target dictionary) is reset to the default value in HDS 2+ (HDS 4+ on 32-bit processors).

5.	The code field address of the target dictionary entry is set to point to the first byte following the code field address.  This value of the code field address implies that machine code will follow the target’s head immediately, unless the code field address is changed (as it is after the nucleus is compiled, when target run-time behaviors are available).

When HDS has a value of 0, (CREATE) skips Steps 2 and 3, thus compiling only a code field address in the target dictionary.  HDS may be set temporarily (see Step 4) to 0 by the word | (pronounced:  “bar”).  HDS is reset to the value in HDS 2+ at the end of (CREATE).  HDS 2+ was initially set by the value into DICTIONARY (HDS 4+ on 32-bit processors). (See Block 181 of your system listing and the reference below).

Some target constants, addresses, and variables need to be accessed during target compilation.  Since the values for these words are in the target’s dictionary, they would not normally be available.  The word TWIN creates a twin dictionary entry of these constants, addresses etc. in the host’s dictionary, in a resident vocabulary.

Some host defining words only make entries in the host dictionary.  The two most notable are EQU and LABEL.

Command	Action

n EQU name	Defines a resident constant of value n whose name is name.  Saves space in the target dictionary.  Note that a word created by EQU may not be used in a target colon definition.

LABEL name	Defines a resident constant with the value of HERE.  Useful for labeling assembly language entry points.  Note that a word created by LABEL may not be used in a target colon definition.

REFERENCES

DICTIONARY , Section 7.3.1
EMPLACE, Section 7.7

## 7.6	The HOST Assembler

The host assembler has been called the “target” assembler because it assembles code for the target machine.  Do not confuse this with the assembler that will run on the target machine.  We will use the following definitions in this section:

Word	Description

Resident Assembler	Resides in the resident vocabulary called ASSEMBLER in the host computer, and assembles code for the resident system.

Host Assembler	Resides in the host vocabulary called ASSEMBLER , assembles code for the target system during target compilation.

Target Assembler	Resides in the target system, and is not executable in the host.  Will be the target system’s resident assembler.

If the host and target have the same CPU, the host assembler source is identical to the source for the target and resident assemblers.  The host assembler behaves differently because , and C,, the lowest level compiling words in the assembler, are HOST definitions which compile to the target’s address space.

At the beginning of target compilation, before the target nucleus is assembled, no target behaviors exist, not even NEXT.  The host assembler is the part of the target compiler which creates the fundamental target behaviors.  The word which first creates the heads (name, length, dictionary link, and code field address) for the host assembler is (CREATE).  (CREATE) is used in the host assembler’s definition of CODE, as well as the later host defining words.  (CREATE) sets the code field address of the target dictionary entry so that it points at the entry’s parameter field (see Fig. 7.2 and 7.3).

Note that if a different assembler is loaded by Block 240, and that if the source code is correct for the new assembler, the type of CPU is unimportant to the target compiler.  Target compilation forms a very powerful cross compilation facility (not surprising, as cross compilation was the target compiler’s original purpose).

REFERENCES

(CREATE), Section 7.5.2
CODE, Section 6.1

## 7.7	The HOST Compiler

The host compiler has been called the “target” compiler because it compiles colon definitions for the target machine.  Do not confuse this with the compiler that will run on the target machine.

Word	Description

Resident Compiler	Resides in the resident vocabulary called FORTH. Compiles high-level colon definitions for the resident system.

Host Compiler	Resides in the host vocabulary called HOST during target compilation, and compiles colon definitions for the target system.

Target Compiler	Resides in the target system and is not executable in the host.  Will be the target system’s colon compiler.

The host compiler (the word ], pronounced: “right bracket”) is simpler than a resident colon compiler.  The host compiler looks up words and executes their host dictionary definitions.  The result is usually to compile something (such as the target address of the word) in the target dictionary.  If a word cannot be found, the host compiler attempts to convert it to a number, and compile the number in the target dictionary. The host compiler cannot be loaded until after the target’s nucleus is assembled, because the host compiler needs target addresses for the run-time behaviors of literals, :, ;, and all compiler directives such as DO, IF, etc.

When the host compiler executes the host definition of a word, the run-time behavior of the word actually compiles values into target address space.  The most common example of such a run-time behavior is the word EMPLACE.  EMPLACE compiles the addresses in every high-level word in a target compiled system.  EMPLACE contains the run-time behavior of (CREATE) (which creates target dictionary heads, with a parallel entry in the host’s target vocabularies).  At compile time, EMPLACE uses the resident version of CONSTANT to compile a named value in a target vocabulary of the host’s dictionary.  When the word defined by EMPLACE is executed (by the host compiler, for example) the run-time behavior of EMPLACE compiles the word’s value into the next available cell of read-only memory in target address space.  The words defined by EMPLACE always have values which are the target addresses of target parameter field addresses.

A typical definition of EMPLACE is:

: EMPLACE ( a)   LOG CONSTANT TARGET
   DOES> @ 2- , ;

LOG produces an entry in the compilation log.  CONSTANT compiles the target address on the stack into a resident vocabulary of the host (probably HOST).  TARGET relinks the dictionary links to the new word, so that the new word may be found in the host’s target vocabularies.  Note that the comma after DOES> compiles to target address space because the comma is from the  HOST vocabulary.

Compiler directives and similar immediate words are placed directly into a target vocabulary of the host, to be executed by the host compiler.

REFERENCES

(CREATE), Section 7.5.2
IMMEDIATE Words, Section 2.8.8

## 7.8	Target Defining Words

There are three levels of system that can be produced by means of target compilation:

1.	Non-interactive—Usually a dedicated controller of some sort.  All dictionary entries are headless.

2.	Interactive—Some dictionary entries have heads.  The text interpreter and at least one terminal exist in the system.  An interactive system is a superset of a non-interactive system.

3.	Extensible—Words exist in the target dictionary which can define more words in the target dictionary.  An extensible system is a superset of an interactive system.

Target defining words are the words which make the third level possible.  Target defining words are essential parts of your polyFORTH development system.  This section is about “closing the loop”; how the target’s defining words are defined during target compilation.

Target defining words may have both a compile-time and a run-time behavior.  The basic target defining words have run-time behavior which is code in the nucleus.  To see an example of such a word, see the run-time definition of CREATE in Block 184, and the compile-time definition in Block 226.  The labeled gap which precedes the run-time code (in Block 184) will contain the high-level colon definition of CREATE’s compile-time behavior (from Block 226).

The compile-time and run-time behaviors must be physically adjacent in memory because of the method used to transfer the address of the run-time behavior to the code-field-address of the defined word.

Target defining words are an example of one way to handle forward references in Forth.  Before the compile-time definition is compiled 'H is set to the beginning of the associated labeled gap with an ORG. The compilation fills in the gap left when the nucleus was assembled.  When all forward-reference defining words have been compiled, the compiler’s 'H is reset to where it pointed before the ORG of the first forward-referencing defining word.

Forward referencing is not recommended because it creates unnecessary complexity.  Although techniques have been developed to eliminate forward references for the nuclear defining words, the techniques are still controversial because they consume slightly more space and time in critical system functions.  Only six forward references exist in most polyFORTH systems, they are:  : (colon), CREATE, CONSTANT, USER, X, and WORD.

REFERENCES

'H, Section 7.3
Defining Words, Section 2.7

## 7.9	Target Compilation of Tasks

The target compiler and multitasker are individually complex, so that target compilation of a system with multiple tasks is not a simple process.

A task is set up in three steps.  First, the task must be defined and its memory allotted.  Second, the task must be initialized and linked into the round-robin.  Third, the task must be activated with a program to run.

A FORTH task occupies two regions of memory.  One region contains:

1.	A dictionary head,

2.	A pointer to the task’s STATUS cell,

3.	An address of the bottom of the task’s parameter stack, and

4.	In terminal tasks only, a table of device-dependent initial values for user variables.

The above part is unchanging throughout execution, and may reside in read-only memory.  The other part, which must reside in RAM, contains:

1.	Both stacks,

2.	The status cell,

3.	The user variable area, and

4.	On terminal tasks, the dictionary and input message buffer.

Since this part resides in read/write memory, it must be initialized every time the target system powers up.  Task definition and memory allotment must occur during target compilation.  Memory is allotted for background tasks by the word BACKGROUND, and for TERMINAL tasks by the word TERMINAL.  The target nucleus and host defining words must be compiled before a target task can be defined with BACKGROUND.  For TERMINAL tasks, not only the nucleus and defining words, but also any device-dependent routines must also be loaded before a target teminal task is defined.  All useful multitasking functions must also be loaded.

The easiest method is to modify the multitasking definitions in Blocks 30-32 to make the definitions target compilable.  Versions of the task defining words that are suitable for target compiling are given in Blocks 253-254.

The word PROMPT needs to be changed so that SYSTEM is replaced with a greeting or behavior appropriate to the application.

REFERENCES

Application Defining Words, Section 7.5.1
Multitasking, Section 4.0

## 7.10	Conserving Memory

It is particularly important to ROM-based applications that they occupy a modest amount of memory.  The cost of each ROM—including burning, testing, and general handling—can strongly impact both the production cost and field reliability of high-volume ROM-based products.

Several techniques for conserving memory are covered in this section, notably: pruning, code-sharing, use of RECOVER, and head minimization.

Pruning is the process of removing parts of Forth unneeded by an application.  The system source code for Forth is grouped into blocks by function, so that related words are together.  When an entire facility is unnecessary (e.g., the target system’s compiler), the blocks simply are not loaded by the load block of the target-compiled application.  Several thousand bytes may be recovered in this fashion.  The facilities most commonly deleted are:  assembler, compiler, editor and disk support.  Non-interactive systems may also prune the interpreter, dictionary management and possibly the terminal drivers.  Note that if a system is sold which contains the compiler or assembler, FORTH, Inc. is entitled to royalties (see your Licensing Agreement).

If the memory size is critical (as in a system which will be mass-produced), pruning can be done within blocks.  One way of doing this is to copy the source blocks of the nucleus, and then delete all unused Forth primitives from the copies or move their definitions below an EXIT to reserve them for future use.

Pruning is the last step of producing a target compilable program—as much debugging as possible should be done first.  Pruning usually only requires a few hours, even on large projects.

Code sharing is another powerful conservation technique.  Code can be shared in three ways: by subroutines, by code which jumps into another word’s code, and by defining words.  Forth is a language of subroutines, so vast amounts of code sharing occur naturally.  Sharing code by jumping to the common section is routine Forth practice—use LABELs to mark entry points; they are more convenient than using HERE.  For example, in an 8086-based system, the two following pieces of code will target compile into the same code:

CODE EXECUTE ( a)   W POP  LABEL execute
   W DEC   W DEC  W ) LIP

CODE @EXECUTE ( a)   W POP   W ) W MOV
   W 1 MOV  1NZ IF  execute JMP
   THEN  NEXT

With HERE, the code is somewhat less readable:

CODE EXECUTE ( a)   W POP   HERE   W DEC
   W DEC  W ) LIP

CODE @EXECUTE ( a)   W POP   W ) W MOV
   W 1 MOV  1NZ IF   SWAP JMP
   THEN  NEXT

If the application has many words which end with a jump to a particular entry point, a custom code ending (a colon definition in the assembler vocabulary containing the words to assemble the jump) may be appropriate.

The use of defining words is a powerful, frequently neglected technique that allows words to share code.  In a control application which uses a few standard delays in many places, the following defining word would save memory:

FORTH  : DELAY ( n)   | CREATE  ,
   DOES> ( a)   @ MS ;

1000 DELAY SECOND
60000 DELAY MINUTE
  300 DELAY 300MS
   40 DELAY 40MS

Compare the source above with the following:

| : SECOND   1000 MS ;
| : MINUTE   60000 MS ;
| : 300MS   300 MS ;
| : 40MS   40 MS ;

Not only is the first solution easier to read and use, it also saves 12 bytes, and gives an equivalent result.  The savings increase as the number of similar words increases.

To see why this technique saves space, look at the defining word, DELAY.  The word compiles a target code field address followed by a single-precision delay.  Only two cells per delay word exist in the target dictionary.  The | (pronounced “bar”) automatically sets HDS temporarily to zero, ensuring that a name is not compiled to the target for each delay word.

Each time a standardized delay is used in a colon definition, it will require only two bytes in the target, instead of the six bytes required for a literal followed by MS.  In addition, each new delay word only requires four bytes (a CFA and a value), instead of the ten bytes (CFA, LITERAL, value, MS, EXIT) required by an ordinary colon definition.

Defining words are also sometimes useful for defining standardized output sequences to particular devices.  If defining words are used for output, each defining word should define a very particular output style for a particular class of devices, rather than trying to do everything.  The word MSG is an example of a specialized I/O defining word.

The bar used in the example above may be placed before the colon or CODE of any definition which does not need to be accessed interactively at run-time in the final target.  If all the definitions of an application can be headless (as in a calculator, for example), the argument into DICTIONARY should be zero.  DICTIONARY sets the default name length and clears the target compilation space.

Some colon definitions are an endless loop (e.g., QUIT, and INTERPRET).  On these, the EXIT compiled by ; is superfluous.  The last cell compiled may be “unallotted” by the word RECOVER.  Words containing ACTIVATE may also use RECOVER.  RECOVER may be used in resident applications as well.

REFERENCES

ACTIVATE, Section 4.5
Application Defining Words, Section 7.5
Code Endings, Section 6.2
LABEL, Target Compatibility Block, Section 7.12
MSG, Section 2.7.6.3

## 7.11	Power-up Initialization

The point of the power-up initialization routine is to force all I/O devices and critical memory areas (e.g., the Forth system variables and block buffers) into a known state before enabling interrupts and starting the application program.

Power-up code is sometimes difficult to debug.  Therefore, the most common strategy for initializing a Forth system is to move from machine code to a high-level definition as quickly as possible.

The following steps are most often followed:

1.	Disable interrupts (if necessary).

2.	Set up the Forth registers (S, I, R and on a multitasking system, U).

3.	Perform any necessary hardware initialization.

4.	Execute NEXT.

NEXT executes the “next” address at I, so I should be set to the address of the parameter field of a high-level power-up routine. The high-level power-up routine is often application-dependent.  The high-level routine should be debugged as thoroughly as possible before it is run on the target machine or as a power-up routine.

For an example of a power-up routine, see the initialization/power-up block of your polyFORTH system listing and the CPU Supplement.

REFERENCES

Diagnostic and Debugging Techniques, Section 7.13
The Address Interpreter, Section 1.1.6

## 7.12	Resident Testing of Target Applications

An important principle in Forth is to keep the programming and debugging cycle as interactive as possible.  When a target machine and the host machine have the same CPU, it is usually most convenient to do as much debugging as possible in the host.  When applications are target compiled, certain words are available which are not normally available in a resident system.  Therefore, to allow target compiled applications to be debugged in the resident system, there is a block called “target compatibility block.” On most systems this is Block 252.  The target compatibility block is loaded at the beginning of the application load block during resident system testing.

The target compatibility block contains definitions for words which are often used by target-compiled applications but which are not normally defined in a resident system.  The definitions in the target compatibility block are minimal definitions that will allow almost all target-style source code to be loaded in a resident system, and still emulate as closely as possible the target system behavior.

The principal difference between the resident and target environments is that during target compilation there are three distinct memory areas available, with allocation pointers to each.  These are host RAM, target RAM and target ROM.  In a resident system, there is only one memory area available for compiling:  host RAM.  The target compatibility definitions provide a separate region of host RAM analogous to the target RAM area, where the values of VARIABLEs, etc. will reside.

The size of this RAM region must be on the stack when the compatibility block is loaded.  Thus, to reserve 2K bytes of RAM, the appropriate phrase would be:

2048 252 LOAD

This memory will be taken from your user area, so you must ensure that your user area is large enough to contain the application plus the RAM buffer.

The target compatibility block provides a very useful level of interactive debugging not usually available on cross compilation systems.  It is not difficult to produce source code that works correctly with both the target compatibility block and the target compiler.  The following table gives the words usually present in a target compatibility block, along with a description of their functions and the significant differences, if any, from the corresponding target compiler definitions.

Note that these words behave this way only when loaded from the target compatibility block.  The target compiler versions are different, but equivalent.  You may wish to refer to your system listing of the compatibility block and the target compiler as you read the following table.

Word	Stack	Function

'R	( - a)	Emulates the target compiler’s pointer to the next available byte of RAM.  Note that a buffer is allotted to emulate the RAM in a system.  The size of the buffer must be on the stack when the target compatibility block is loaded.

THERE	( - a)	Pushes the address of the next available byte of simulated target RAM onto the parameter stack.  HERE is assumed to put the address of the next available byte of ROM on the stack.

ALLOT	( n)	Allots n bytes in the simulated RAM.  The corresponding word for ROM is GAP.  GAP is rarely used, as variable arrays cannot be defined in ROM, and tables of constants are defined using commas to compile each value.

ORG	( a)	Sets the value of H which is the pointer to the next available byte of simulated ROM.  Note that one must be careful using absolute addresses for ORG, because addresses during resident testing will be different from those in the target compiled system.  Absolute addresses may also lead the compiler to over-write the resident system.  Alternatives to absolute addresses are to use labels (defined later in this table) or offsets from HERE.

TARGET		Emulates the vocabulary change performed by the target compiler’s TARGET.  TARGET changes the precedence of the most recently defined word so that the word is immediate and will be executed at compile time.

HOST		A dummy definition that sets CONTEXT to be FORTH, the vocabulary containing the resident definitions.  HOST is IMMEDIATE so that it will be executed when the compiler sees it in a colon definition instead of compiling its address.

FORTH		Differs from the resident version in that it is IMMEDIATE.

VARIABLE name	Creates a constant whose value is the address of a cell in RAM.  Used exactly like the resident equivalent; the difference is that the parameter field and the value are segregated in ROM and RAM.

CVARIABLE name	Creates a constant whose value is the address of a byte in RAM.  CVARIABLE is not available on processors such as the LSI-11 which require alignment on even byte addresses.

LABEL name		Creates a constant whose value is an address in program memory.  LABEL is often used to provide entry points in assembler routines.  In target compiled code LABEL adds nothing to the code because LABEL creates an entry in the host dictionary rather than in the target dictionary.  In the target compatibility block, LABEL produces a dictionary entry whenever it is used—even in the middle of assembly code!  To make this transparent, the compatibility version of LABEL assembles a jump just before the dictionary entry to the point immediately after it.

n EQU name	( n)	Creates a constant with a value of n.  In the target compiler, EQU creates a constant in the host dictionary which is only available during target compilation.  The target compatibility block version is a normal CONSTANT.  The target version of EQU may be referenced in code or interpretively, but may not be referenced inside a colon definition, because there is no target definition to compile a reference to.  This is concealed by the compatibility version, which, being a normal constant, may be referenced anywhere.

Words defined by EQU and LABEL can not be referenced inside target compiled colon definitions.  Words defined by EQU and LABEL may be used interpretively and in assembler code, but not in colon definitions, because their target compiler equivalents don’t create a target dictionary entry.

If you make use of vectored execution, please note that the resident version of ASSIGN must be changed in order to be correctly target compiled.

An example of a resident version of ASSIGN is:

: assign ( a)   R> 2+ SWAP ! ;

: ASSIGN ( a)   BEGIN  COMPILE assign
   COMPILE [ 2- @ , ] ; IMMEDIATE

The resident version must circumvent the fact that in a resident system, the word : “colon” is headless, by using the phrase in brackets.  When target compiling, an equivalent definition of ASSIGN is:

: assign ( a)   R> 2+ SWAP ! ;

FORTH : ASSIGN ( a)   COMPILE assign
   colon , ; TARGET

ASSIGN stores the address of the behavior following ASSIGN into the address on the stack.  ASSIGN must be used in a colon definition.

REFERENCES

Vectored Execution, Section 2.4.8

## 7.13	Diagnostic and Debugging Techniques

The basic principle behind speedy debugging is to keep the debugging process as interactive as possible, as long as possible.  When the target and host have the same CPU, it is easy to test code interactively.

A facility which is sometimes useful is the target compilation log.  Every time that an entry in the target dictionary is created, the log types the entry’s name and the address of the first byte of the entry’s parameter field.  The log can be turned on by taking the LOAD command for the log out of its parentheses in the target compiler load block (Block 240).  The log is not usually used for routine target compilations because the extra I/O slows compilation and clutters the screen.  To use the log with printer output it is necessary to define a TERMINAL task for the printer with adequate memory to target compile the application.  The printer task is made to perform the target compilation.

The target compatibility block is included in the system specifically to allow a target-compilable application to be compiled and run by a resident system.  The resident system should have appropriate interfaces to allow thorough program debugging.  Once the application works correctly in the resident system, it can be target-compiled and run in the host again.  If it is possible to cycle power (or simulate cycling power with a reset button) without memory loss, the power-up initialization routine can be tested directly.  Alternatively, the target-compiled application can be moved to the proper (or the least non-interfering) places in the host’s read/write memory to simulate the target system.  The application can be initialized by jumping in machine code to the first instruction of the power-up initialization routine.  Be sure to keep the “move” routine in a place where it will not be copying on top of itself.

If the system refuses to do anything at all when the power-up code is executed, almost certainly there is a problem in the power-up initialization routine (after all, you’ve tested the other code already).  Invent some simple visible signal, for example:  toggling a console light or form-feeding a page on the printer.  Place this signal code so that it will be executed as early as possible in the power-up sequence.  If the signal does not occur, there is something seriously wrong with either the machine, or your understanding of it.  Read manuals carefully before calling a technician.  If the signal does occur, you can move the signaling routine to verify that each phase of power-up initialization occurs correctly and isolate trouble spots.  A good signal for high-level code is to send consecutive numbers to the already-initialized terminal.  Insert in your code:

...  1 .  words  2 .  words  3 .  ...

and your screen will indicate the latest spot successfully reached.

Once you have tested as much of the software on the host as you can, it is time to mate the software to the target hardware.  The most difficult situation is a read-only memory application running on a target which has a different type of CPU than the host.

Desk-check the program until no errors are found, then target-compile it with the log turned on and program the read-only memory.  Using the log with a hard-copy device will make it possible to determine where to insert patches, or what the logic analyzer means (if you’re using one).

Test the software in the target.  If the system does not come up, then go through the same steps described above for bringing the application up on the host (moving a signal through the program).  Remember that each iteration reduces the number of errors tremendously.  Be sure that you find an error or verify program correctness on every iteration.

One of the problems with read-only memory is that it takes time to erase.  It helps to have one EPROM chip set under the ultraviolet lamp and another in the computer (remember:  as interactive as possible).  If you routinely have read-only memory applications, you may find it worthwhile to construct a RAM simulator which uses read/ write memory but can be programmed by the host.

Alternatively, rather than iterating with EPROMs, you may find it easier to insert a “talker PROM” and compile to the system through a serial link—choose your strategy and stick to it.  Serial link compilation is much easier for large systems containing mostly RAM.

When the system comes up and seems to be working, remember to test the application as exhaustively as you did on the host.  The small problems that remain will not be found otherwise.

REFERENCES

RELOAD, The Forth Bootstrap, Section 3.8
Serial Link Compilation, Section 7.4.4

# 8.0	DATA BASE SUPPORT

The polyFORTH Data Base Support option is a set of tools with which you can design efficient data base applications or components for general applications.

The Data Base Support option includes:

•	A simple disk-oriented file manager.

•	Commands for defining records within files and fields within records.

•	Tools for generating columnar reports.

•	Utilities for producing totals and subtotals.

•	Techniques for linking subfiles to main files and for chaining records within files, and;

•	A set of words for creating ordered indexes (for keeping sorted lists).

## 8.1	Overview

polyFORTH presents its Data Base Support option in the form of a “kit,” leaving complete flexibility for you, the developer, to create a data base design that reflects the natural organization of the data itself.

Before you begin constructing a data base application, you must understand a few simple premises that underlie the design of the Data Base Support option.  First, let’s review common data base terminology.

A data base is the complete set of organized data that is available to the computer.  A data base is divided into related groups of data called files. For example, a file might contain the names, addresses, and phone numbers of all your clients.

A file, in turn, is divided into records.  A record might contain the name, address and phone number for a single client.  For every client, there would be one record in the file.

A record is itself divided into a collection of fields.  For instance, one field might be called “STREET.”

In a data acquisition environment, a file might contain a set of readings taken during one experiment.  Each record could contain the set of measurements taken at a single point in time during the experiment; each field could contain the reading of a different measurement.  In this case, you might have numerous files, each containing the data obtained during one run of the experiment; however, the records in each file would be laid out identically.

Many applications require multiple types of files that relate one to another.  Suppose you want to record all your invoices, using an “Accounts Receivable” program. In the course of your business, you bill several invoices to the same client. Rather than duplicate the name and address of the clients every time you bill them, it makes sense to have one file for the client data, and another file for invoices.  Each invoice record can point to one of the clients in the client file.  In this way, one file can “use” another file.

### 8.1.1	Contiguous Files and Performance

A premise of the Data Base Support option is that you are a knowledgeable programmer concerned about performance.  Its approach allows you to design the data base for optimum efficiency.

In contrast, typical data base packages are intended to simplify data base construction for non-programmers.  These packages do not require that you think about how your data is organized.  On the down side, you lose the ability to structure your data base in the optimal way.  The price for greater generality is impaired performance and increased size of compiled code.

In a disk-oriented data base application, the key determinants of performance are:

1.	How many physical disk accesses are required to access a logical data item?

2.	How much head motion occurs during normal operation?

If you can minimize the number of physical accesses required and the disk head motion, you can maximize performance.

Fig. 8.1

In polyFORTH, a file occupies a contiguous range of blocks on disk.  A file may be any size (using whole blocks) up to the capacity of the disk.  There is no need for a file allocation table.

The polyFORTH Data Base Support option addresses both issues simultaneously by imposing a single restriction: a file is a contiguous region of blocks on disk.*  This means the system does not automatically “manage” files by interweaving them on disk as they expand and contract.  Files are not fragmented across the disk, and there is no need for a file allocation table to point to the fragments.

Instead, you specify the maximum size of each file when you create the file, and assign it a contiguous range of blocks on the disk.  Although this requires some thought, there are several advantages:

1.	Since files, and therefore records, are contiguous, the exact location of any data element can be calculated. Thus any data element can be accessed in a single physical disk access. In traditional operating systems and file managers, an application cannot know from a record number where that record lies physically.  The location must either be looked up in a directory, or found by following a set of chains. Reportedly, one popular operating system requires up to six disk reads to access a single logical record.

2.	While accessing various elements within a single file, the disk’s magnetic head need only travel within the distance occupied by the file.  Head motion is minimized.

3.	You have control over the arrangement of multiple files in relation to one another. For instance, if one file serves as an index into another file, you can place these two files adjacent to each other on the disk. Again, head motion is minimized.

These benefits assume that you are running on a native polyFORTH system.  If your polyFORTH runs under another operating system, performance depends on the way that O/S treats the disk.

Just as files are contiguous and of fixed length, so too are records and fields.  Again, although variable-length fields require less thinking on the part of the user, they necessarily degrade performance.  Fixed-length fields do not necessarily imply fixed-length amounts of data, because a variable number of subordinate records may be chained together as necessary.  (We’ll explore this technique further in Section 8.7.)

Since the primary bottleneck in disk-based file systems is disk-access time, minimizing this bottleneck achieves surprising efficiency.  For example, one company sells a data base system which uses the polyFORTH Data Base Support option to handle 300 Mbytes of data and support 64 simultaneous users with under one-second response time even at peak load, on a single 68000 microprocessor.

REFERENCES

Disk Block I/O, Section 1.2.2, 3.2

### 8.1.2	Current Files and Records

Another concept that is fundamental to the Data Base Support option is the notion that at any given moment, exactly one file is current and one record is current.  Let’s first describe what is meant by a file being current.

We mentioned that a file is simply a contiguous, fixed-length range of blocks on the disk.  There is no file allocation table on the disk, nor is there any other indicator of which blocks constitute which file.  The knowledge of where each file begins and ends resides within the application code, specifically in a small table that you define for each file (using the defining word FILE, Section 8.3).  This table is called a File Definition Area (FDA).

The name you give this table is the name of the file itself. The table contains the starting block number, along with sufficient information about the number and size of records for the Data Base Support option to be able to calculate the absolute location of any record in that file.

When you invoke the name of a file, the file definition places the address of its parameter field in a user variable called F#.  All record-accessing operations in the Data Base Support option use this pointer to indicate the current FDA, which in turn points to the blocks where the desired record resides.

Thus, at any given moment, one and only one file is current.  Changing files is a simple matter of invoking the file name, which places a new address in F#, taking only microseconds.

Contrast this with the process of “opening” and “closing” files in traditional operating systems.  In these systems, each open and close operation requires noticeable disk activity to read in the file directory and write it out again.  For this reason, the question of how many files can be open simultaneously is a concern in such systems.  This concern disappears with polyFORTH’s Data Base Support option.

Fig.  8.2

For each file on disk there is a file definition in the dictionary.  This definition contains four parameters describing the location and dimensions of the corresponding file.  In this figure, the second file is “current.”

We can summarize the above discussion by saying, “Files are pointed to, not opened.”  Analogously, we can say that “Records are pointed to, not read.”

Just as there is always a current file, so there is also a current record, the number of the current record found in the user variable R#.  All the data-access operators refer to specified fields within the current record in the current file.

The polyFORTH Data Base Support option is once again unique in this con-cept.  Many data base packages actually read in an entire record from the disk, then allow access to the fields within it.  polyFORTH merely makes a record current; disk access only occurs when a field name is invoked in combination with a field access operator.

This design takes advantage of the behavior of BLOCK (Section 1.2.2).  Whenever a single field is accessed, BLOCK reads the entire block in which that field resides. If multiple fields in the record are required in the same operation (such as displaying all fields in the record for a report), it is unlikely that the block buffer will be reused before all the fields can be displayed.  (Should this happen, BLOCK will automatically read the block again.)  Moreover, it is even likely that references to neighboring records in the same block will also not require physical disk accesses.

An important advantage to not reading the record physically is the certainty that at any given moment only one copy of each record exists.  Systems that read a record into memory face the problem of two users accessing the same record, and having different copies of it.  Solving this conflict entails various “lockout” schemes, all of which complicate the system and reduce performance.

The file and record pointers are entirely independent of each other.  Not only can you select records without re-selecting the file, you can also change files without affecting R#.

From time to time in your application you may want to leave your current file and record temporarily (perhaps to examine or display a field from a related file) and return.  The following words enable you to “remember” F# and R# temporarily:

Word	Action

SAVE	pushes R# and F# onto the return stack.

RESTORE	pops those items off the return stack and places them in R# and F#.

Naturally, you must use SAVE and RESTORE as a paired set within the same definition, just as you would use >R and R>.  Similarly, you must use both words within or outside of any DO ... LOOP structure in that definition.

Following a SAVE, R# is on top of the return stack; if you need a copy of it you may get it by using R@.

REFERENCES

Return Stack, Section 2.1.3

### 8.1.3	How Data is Stored

The Data Base Support option allows storage of data in either numeric or alphanumeric form.  For instance, a U.S. telephone number, including area code, requires 14 bytes when stored in alphanumeric form:

(213) 372-8493

This same phone number can be stored in only 6 bytes per record, if it is recorded as a 16-bit area code and a 32-bit local number:

213  3728493

The appropriate punctuation symbols can easily be inserted when the number is displayed, using pictured numeric output.

The contents of numeric fields travel between the data stack and the disk; the contents of alphanumeric strings travel between the PAD and the disk.

For instance, if we have a double-length field named SALARY, we can fetch the value of the field (from the current record in the current file) by invoking the phrase:

SALARY D@

which places its value on the stack in the same way that the word 2@ fetches a double-length value from an ordinary variable.  Similarly, the phrase,

SALARY D!

removes a double-length value from the stack and places it in the current SALARY field.

Alternatively, the word B@ fetches the contents of an alphanumeric field, and copies it to the PAD.  The word B! stores an alphanumeric string at PAD into a given field.

Fig. 8.3

Shows how data travels between disk and memory.  Numeric data travels between disk and the parameter stack, and text strings travel between the disk and PAD.

REFERENCES

Field Reference Operators, Section 8.5.3
Pictured Numeric Output, Section 2.5.2.1
String Storage in PAD, Section 2.3.1

### 8.1.4	Working Storage

Two features of the Data Base Support option make use of a region of memory called working storage.  Working storage is allocated at the beginning of a task partition, and serves as a place where record data may remain which is easily accessible, but less volatile than PAD or the parameter stack.  Since each task has a working storage area, tasks running concurrently may use the same code referring to working storage without conflict.

One use of working storage appears in automatic totaling (Section 8.8.8).  Here working storage holds the accumulating registers for each column of data to be added as the report is generated.

The second use of working storage is as an “image” of a record.  The same relative positions are maintained both in the record on disk and in working storage.  For example, working storage is used to hold the key during a binary search of an ordered index (Section 8.6), in the field in which it will be found in records being searched.

The same field names that let you access fields on disk also may be used to reference the corresponding fields in local working storage.  There is only one “record” in the working storage area.  This technique lets you map data items as though they were contained in records although they are temporarily in resident memory instead of on the disk.

Fig. 8.4

Field names may be used to reference either the individual field in the current record (on disk), or the corresponding field in working storage.

The double use of working storage for both subtotaling and key searches rarely causes a conflict, since the two activities occur at separate times.  To be on the safe side, however, the statistics component uses the word REGISTER to return the address of the accumulators, which in turn is defined in terms of WORKING.  If you find that you will encounter a conflict, you may resolve it by simply redefining REGISTERS to point to some other area.

How much working storage is necessary?  If you are using the subtotaling feature, the amount depends on the number of accumulating registers you need.  In total, you will need the sum of:

16	register area management

4	header variables for registers

#registers * 8	8 bytes per accumulator

_____________

total

For instance, three accumulators will require 44 bytes* (16 + 4 + 3*8).

If you are using ordered indexes, working storage must be as large as the largest record in any ordered index file.

Remember that any task which performs an application that uses working storage must have sufficient room allotted for it—including the printer task.  Section 8.1.6 describes methods for allocating working storage.

REFERENCES

Accessing Fields in Working Storage, Section 8.5.5
Ordered Indexes, Section 8.6
Subtotaling, Section 8.8.8

### 8.1.5	Installing The Data Base Support Option

Before you begin to use the Data Base Support option, you must first decide whether you will be using it in your personal task only, or whether other terminal tasks may need to use it simultaneously.

To load the option into a your private terminal task, list its load block with the phrase,

FILES LIST

The number of this load block is returned by the constant FILES.  Make sure that the block begins with the phrase:

EMPTY   n ALLOT

where n is the amount of working storage required for your files application (see Section 8.1.4).

At the end of the load block, a null definition of TASK should appear.  This word will be the last word in the dictionary when file applications are loaded, and will mark the point at which overlays will occur.

Now issue the command:

FILES LOAD

to load the Data Base Support option.

As you create data base applications, each of these should begin with the phrase:

FORGET TASK   : TASK ;

This makes each application an overlay, which will discard other overlays that use this convention.  For instance, if you have an accounts-receivables application, its load block should begin with the above phrase, to forget any other applications without forgetting the Data Base Support option itself.

Finally, if you wish to output a report to your printer, you must allot a working storage area in the printer task.   This may be done with the phrase:

n TYPIST H HIS +!

This phrase advances the dictionary pointer H for TYPIST—the printer task—by the amount n.  It is most convenient to edit this phrase into the FILES load block.

Alternatively, if several terminals require use of the Data Base Support option, the package should be loaded with the system electives by Block 9.  In this case, remove the:

EMPTY   n ALLOT

at the top of the FILES load block; also remove the definition of TASK and the word EXIT at the bottom (by placing parentheses around them).  By allowing the final word GILD to execute, the Data Base Support definitions will become available to all tasks.

Edit the phrase:

FILES LOAD   n TYPIST HIS +!

where n is the amount of working storage required for your files application (see Section 8.1.4) into the last line of Block 9 (just above the EXIT).  If you have already loaded the electives before making this addition, type:

FLUSH RELOAD

then type:

HI

Using this approach, each files application must begin with the phrase:

EMPTY   n ALLOT

instead of FORGET TASK, where n is the amount of working storage needed by that application.

REFERENCES

ALLOT, Section 2.8.1
EMPTY, Section 3.3.4.1
FORGET, Section 3.3.4.2
HIS (User variables), Section 4.6
LIST, Section 5.1.1
Overlays, Section 3.3.4
Parentheses Used for Comments, Section 1.5.1
System Electives, Section 1.4.2

## 8.2	Creating a Simple File

This section introduces the procedures for creating a simple file by way of an example, and provides a contextual framework for the detailed sections that follow.

In this section we are assuming that we already know how to structure our data; we are concerned here only with the mechanical aspects of file creation and field layout.  For a more general discussion of data base design, see Section 8.9.

Our simple example will be a file of names and addresses.  To avoid extra detail, we will only use alphanumeric fields.  No attempt will be made at keeping a sorted file (ordered indexes are discussed in Section 8.6).

Step 1

Define the fields:

0  20 BYTES NAME   20 BYTES STREET
   14 BYTES CITY    2 BYTES STATE
    6 BYTES ZIP    14 BYTES PHONE   DROP

In the above lines we have defined six Forth words which will reference the individual fields in each record.  The initial zero is the relative position within the record.  The defining word BYTES creates an alphanumeric field of the specified width (the width must be an even number on cell-aligned processors).  The final DROP is necessary to discard the final relative position within the record (see Section 8.5.1).

In addition to BYTES, several other defining words are available for creating different types of fields (see Section 8.5.2).

Step 2

Determine how many records and blocks the file will need.  Two words that are not generally resident are available in the “file initialization block.”*  #R computes the number of records of given size that would fit in a given number of blocks; #B computes the reverse:  the number of blocks needed to hold a given number of records of given size (see Section 8.3.1).

Step 3

Define the file:

74 500 39 400 FILE PEOPLE

This statement defines a file called PEOPLE which contains records that are each 74 bytes in length.  The file will hold a maximum of 500 records, consuming a range of 39 contiguous blocks.  The starting block will be 400 (see Section 8.3.1).

Invoking the filename PEOPLE makes this file current.

Step 4

Initialize the file.  Load the file initialization block (if it’s not already loaded) and execute the phrase:

PEOPLE INITIALIZE

to fill all blocks in the file with zeroes (see Section 8.3.3).

Step 5

Enter data.

Here is the definition of a word that will allow data entry for a single record (person):

: enter   PEOPLE  SLOT READ
CR ." Name? "     NAME ASK
CR ." Address? "  STREET ASK
CR ." City? "     CITY ASK
CR ." State? "    STATE ASK
CR ." Zip? "      ZIP ASK
CR ." Phone? "    PHONE ASK ;

By invoking PEOPLE, we select the PEOPLE file as the current file.

The word SLOT allocates a new record in the current file, and leaves its number on the stack (see Section 8.4.3).  The word READ sets the current record according to the number on the stack (see Section 8.4.1).

Next, the definition prompts the user to enter the “name” field.  The word ASK is like EXPECT, except that it places the expected text in the given field.  The same process is followed for the remaining five fields.

Step 6

Display the data.

We define the following word to display the current record:

: person   NAME B?  CR STREET B?  CR
   CITY B?  STATE B?  ZIP B?  PHONE B? ;

The word B? displays the contents of the given BYTES field (see Section 8.5.3).

To display the contents of all records that have been entered, we define:

: everyone   PEOPLE RECORDS DO CR
   I READ  person  LOOP ;

By invoking PEOPLE, makes the PEOPLE file current.  The word RECORDS returns the appropriate arguments for a DO ... LOOP, including all records that have been allocated by SLOT in the current file (see Section 8.4.3).

Within the DO loop, READ makes each record current in turn, and person displays the information for that record.

Here is a sample of the output of everyone:

Andrews, Carl
1432 Morriston Ave.
Parkerville PA 17214 (717) 555-9853

Boehning, Greg
POB 41256
Santa Cruz CA 95061 (408) 666-7891

Chapel, Doug
75 Fleetwood Dr.
Rockville MD 20852 (301) 777-1259

Cook, Dottie
154 Sweet Rd.
Grand Prairie TX 75050 (214) 642-0011

To produce columnar output, we would use the “Report Generator” (Section 8.8).

For deleting records, we would use the word SCRATCH (see Section 8.4.3).

## 8.3	File Definition and Access

A polyFORTH file is a contiguous region of Forth blocks.  On native versions of polyFORTH this means that the file will be physically contiguous, and that you can arrange for files that are accessed together to be physically near one another.  This can significantly speed up a data base application.

Versions of polyFORTH that are co-resident with a “host” operating system (such as MS-DOS or RSX) are identical from the point of view of the programmer, but since allocation of physical disk space is performed by the host operating system you haven’t the actual level of control you do on the native versions.

This section discusses how files are defined and referenced on all polyFORTH systems.

### 8.3.1	The FILE Definition

The word FILE is used to define files, given the attributes of the file.  The format is:

length limit blocks origin FILE name

where:

Word	Description

length	is the length of each record in bytes (maximum 1024);

limit	is the maximum number of records (on 16-bit processors the limit is 32767 records per file);

blocks	is the number of 1024-bytes blocks occupied by the entire file;

origin	is the first block number, expressed as a double-precision number on all polyFORTH ISD-4 systems (remember the decimal point!); and

name	is the user-assigned name of the file.

The defining word FILE creates a new name (dictionary entry) that, when invoked, will make this file current.  The dictionary entry contains the File Definition Area (FDA) for the file being defined.

We recommend that you define all your files in a single block, making it easy to see which ranges of blocks have been allocated for other files.  If a disk will contain source or other data along with files, it’s a good idea to indicate these other uses in comments on the same block.

Here is an example of good file definition layout in a block:

( Bytes	Records	Blocks	Origin		Name)
	26	801	22	500.	FILE	(GLOSSARY)
	340	800	268	522.	FILE	GLOSSARY
	4	10	244	795.	FILE	HITS
	24	42	1	799.	FILE	SECURITY
	38	2600	100	800.	FILE	TESTS

Note that the number of blocks may be computed from the number of bytes/ record and number of records.  In fact, the blocks argument is not actually used by the Data Base Support words; it appears in this list to help you maintain the layout of your disk space.

Generally you will choose an appropriate maximum number of records, based on a reasonable estimate of the needs of the application and allowing for expansion.  You will also have worked out the approximate size of each record based on the width and type of fields needed.  Then derive the number of blocks from the number of records and size of each record.  The word #B in the file initialization block is a helpful tool for computing the number of blocks.  After loading this block, type:

#records #bytes/record #B .

For instance, if your application requires 2000 records, and each record is 42 bytes wide, type:

2000 42 #B .     84 ok

Alternatively you can compute the number of records based on the number of blocks.  The word #R in the file initialization block does the arithmetic.  Type:*

#blocks #bytes/record #R .

For example:

84 42 #R .     2016 ok

This shows that you can actually fit an extra sixteen records in the same number of blocks.

By using these tools, you can iterate on various sizes until you get the optimal combination.  Sometimes you can increase the size of a record without increasing overall file size. For instance, if your record width is 94 bytes, it takes 200 blocks to store the same number of records; however 200 blocks will store 2000 records even when each record is 102 bytes wide:

2000 94 #B .     200
2000 102 #B .    200

It’s a good idea to leave extra space in records, in case you need to add fields later.  Beware, however, of grossly oversizing either your record width or file length, as both of these will increase head motion.  Strive for generous but reasonable estimates.

### 8.3.2	File Definition Area and Access

The word FILE establishes a File Definition Area (FDA) for each file in the system.  The user variable F# always points to the current FDA.  Execution of the filename sets F# to address the associated FDA.

Each file’s FDA contains four values to specify the file.  Each of these values may be accessed by the following names, each of which returns the address of the associated value in the current FDA.

Name	Description

ORG	Starting disk-block number of the first disk block allocated to the file (Forth logical block number as a double-precision number).

LIM	Number of records, of declared record length, that the file can contain.

B/B	Number of bytes used per block.

B/R	Number of bytes per record.

While these words are used by the Data Base Support option, they are rarely referenced directly in applications.  ORG and LIM can be useful in debugging, however.  For instance, the phrase:

ORG 2@ D.

indicates which file is current; in case of an abort, you can tell which file you were in at the time.

### 8.3.3	File Initialization Utility

A file that has just been created must be initialized before it can be used.  A special utility is available for this purpose.  It may be loaded with the phrase:

FILES 5 + LOAD

This block* is not an overlay; you may load it any time after you load the block in which the files to be initialized are defined.  Since initialization is done rarely (only during application development), we recommend that you do not load this block routinely.

To initialize a file, once the initialization block is loaded, type:

filename INITIALIZE

The word INITIALIZE performs the following functions:

1.	Writes binary zeros throughout the entire file (including AVAILABLE).

2.	Writes a -1 in the second two bytes of Record 1 of the file.  This serves as a “stopper” for the binary search in an index file.  In other kinds of files this has no effect.

Two other words defined in this block—#R and #B—are useful when designing file layouts.

### 8.3.4	Shared Files

In polyFORTH files may be either shared or unshared. Shared files are those that are defined in the common dictionary available to all users (loaded by the electives load block).  If a file is defined in an overlay, it will be available only to the task or tasks in whose partition it is defined.

As we discussed in Section 8.1.3, all users may freely access the file without having to worry about simultaneous access problems, as long as standard polyFORTH accessing methods are used.  This is because BLOCK ensures that there will be only one copy of a record at a time, and each task does not have its own private copy.

Certain situations require an extra measure of control.  For example, one terminal might delete a record that is needed for processing at another terminal at a later point.  In such as case, you may use a “status” byte in the record to control access.

REFERENCES

BLOCK, Section 3.2
Installing the Data Base Support Option, Section 8.1.5

## 8.4	Record Management

The process of record management includes selecting records, finding the next free record when a new record is needed, and marking deleted records as available for future use.

Not all applications require special record allocation techniques.  For instance, if a file contains 100 records and each record contains information on a permanent piece of equipment which is identified by a two-digit number, there is no need to allocate or deallocate records.  You may just use the equipment number as the record number.  This is called “direct access.”

In an application in which the number of active records changes dynamically, it may be appropriate to use the record allocation techniques described here.

### 8.4.1  Record Selection

Field reference operators (Section 8.5.3) access fields in the current record.  The word READ makes a record current.

READ	( n -- )	Makes record n current, having verified that n is a valid record within the current file.

The name READ is slightly misleading, in that it doesn’t perform an actual disk operation, but merely sets a pointer to the current record.  READ checks that n is not less than zero and not greater than the value of LIM.  If n fails this range test, READ aborts.

READ stores the number of the current record in the user variable R#.

REFERENCES

Current Files and Records, Section 8.1.2

### 8.4.2	Available Records

To distinguish allocated records from available records, the Data Base Support option uses the convention that if the first two bytes in a record contain binary zeroes, the record is available for use.  When a file is initialized, all records are filled with zeroes.  Thereafter, active records may keep any non-zero data in the first two bytes; when a record is released, zero is stored in this area.

Fig. 8.5

The search for an available record performed by SLOT “wraps around” if necessary at the end of the file.

Record 0 of each file contains, in its first two bytes, the record number of the most recently allocated record in that file.  The word AVAILABLE returns the address of this pointer.  When the file is initialized, AVAILABLE is zero.

To allocate a new record, the system begins with the record immediately following the “available” record and searches forward for the first free record.

If the search should reach the end of the file without finding a free record, it “wraps” around to the beginning again, so that deleted records will be used.  For instance, in Fig. 8.5, AVAILABLE points to Record 196; however, there are no more free records between there and the end of the file.  But Record 5 is free.  By “wrapping around” to the beginning of the file, the search finds the available record.

### 8.4.3	Record Allocation/Deallocation Operators

Only two words are required for allocating and deallocating records:

Word	Stack	Description

SLOT	( -- n)	Allocates a new record in the current file and returns the number of the allocated record.

SCRATCH	( n)	Deallocates record n from the current file, making it available.

SLOT searches the file for the first free record, starting with the record following the one pointed to by AVAILABLE.  If a free record is found, SLOT sets the file’s AVAILABLE to point to it.  SLOT then stores a -1 into the first cell of the record to indicate that it is no longer free, and clears the remainder of the record to zeros.  If the file is full, an error message occurs and processing is terminated.

SLOT does not make the new record current, it only returns the selected record number on the stack.  The reason for this factoring is that you often want to do something with the record number before consuming it with READ (which makes this new record the current record).  For example, we may wish to cause a link in the current record to point to the new record, as in the phrase

SLOT  DUP LINK N!  READ

Here the phrase LINK N! must come first because after the READ we’ll be in a different record.

SCRATCH does not change the contents of the record beyond the first two bytes.

REFERENCES

N!, Section 8.5.3
READ, Section 8.4.1

### 8.4.4	Accessing Files Sequentially

The following words return appropriate stack arguments for a loop which will access the records in a file sequentially:

Word	Stack	Description

RECORDS	( available+1 1)	Typically used before DO, returns the content of AVAILABLE (the record number of the last record allocated) incremented by one and starting index (1) for a file that has never wrapped around.

WHOLE	( limit 1)	Typically used before DO, returns the content of LIM and starting index (1) for the entire file.

Since these words return the parameters for the current file, it’s a good habit to invoke the name of the file just before them, as in PEOPLE RECORDS.

When using WHOLE, you will probably want to check inside the loop whether each record is currently active.  This is normally done by the phrase:

LINK N@ IF ...

where LINK is the generic numeric field comprising the first two bytes of each record.  If these bytes contain zero, the record is available for use.

REFERENCES

Available Records, Section 8.4.2
DO Loops, Section 2.4.4
LIM, Section 8.3.2

## 8.5	Field Definition and Access

A record description is the list of defined fields that appear in the record.  Each field is an entry in the Forth dictionary, containing the displacement of the field from the beginning of the record in its parameter field.

A record description is not formally associated with any particular file.  It is more like a mask, which is used whenever it is appropriate to access data.

There are several kinds of fields:  numbers of various sizes and byte strings of specified length.  The following sections discuss the various types of fields and the related operators that are used to access the data stored in them.

### 8.5.1	Record Description

A record description defines the fields that make up each record in a file.  A record description has the following general format:

0	field-type field-name
	field-type field-name
	...
DROP

The various field types are described in Section 8.5.2.

A value is carried on the stack throughout the above process to give the relative displacement of the beginning of a record.  This value is initialized by the zero at the beginning of the record description, incremented appropriately by each field definition, and finally discarded at the end.

In a complex application the fields in a record description may be defined vertically, like this:

0 ( PEOPLE file records)

1

2 0  24 BYTES NAME	( Last name first)

3    24 BYTES STREET	( Street address)

4    10 BYTES CITY,STATE

5    DOUBLE ZIP	( zip code, US only)

6    AREA	( area code)

7    DOUBLE PHONE	( phone number )

8    NUMERIC >DETAIL	( link to DETAIL file)

9 	( For employees:)

10    DOUBLE SS#	( Social sec. number)

11    DOUBLE FICA

12    DOUBLE GROSS	( Gross income ytd)

13 DROP

14

15

As a quick check to verify that the number of bytes used for each record matches the expected value (as specified in the file definition), replace DROP with . (“dot”).  This format allows you to use the shadow block for a general discussion of the file and record.

The field-names defined in the example above—NAME, STREET, etc.—are now entries in the Forth dictionary.  When executed, these words return an address, except for field-names defined with BYTES, which return a count and address (see Section 8.5.2).

A record description is not formally attached to a particular file and has no name.  Use of a field name references the relative location given by that field name in the current record of the current file.

A record in EMPLOYEES:

Thus it is possible to use the same field names for two different files, even if the record size used in each file varies.  For instance, all the above-defined field names could be used with a file called EMPLOYEES, while the first six could be used with another file called CUSTOMERS.

A record in CUSTOMERS:

NOTE:  Due to the Data Base Support option’s record-allocation scheme, the first field of an active record may never contain a zero in its first two bytes.  In our example, this is not a problem because the first field is alphanumeric (even blanks are stored as decimal 32’s).  Otherwise, we would have to rearrange the order of the fields so that one which will never contain zero is first.

REFERENCES

Available Records, Section 8.4.2
Field Types, Section 8.5.2

### 8.5.2	Field Definitions

The following field types are defined:

Word	Description

1BYTE	This field is for an 8-bit value (range 0-255).  On processors that do not tolerate odd byte addresses (such as the PDP-11 and 68000), 1BYTE fields must be used in pairs to avoid wasting space.

Example:	1BYTE AGE

Words that are defined by 1BYTE return an address, suitable for use with the one-byte memory access operators 1@, 1!, and 1?.

NUMERIC	NUMERIC fields occupy two bytes of storage (on 32-bit systems also).  On cell-aligned processors, NUMERIC fields are automatically aligned on even-byte boundaries.

Example:	NUMERIC WEIGHT

Words defined by NUMERIC return an address, suitable for use with the numeric field access operators N@,  N!, N?, and N?.

Word	Description

DOUBLE	This field is for a 32-bit (4-byte) value.

Example:	DOUBLE SALARY

Words defined by DOUBLE return an address, suitable for use with the double field access operators D@, D!, and D?.

BYTES	This field is for alphanumeric text.  A count is required to specify the number of bytes in the field.

Example:	24 BYTES NAME

Words defined by BYTES return a length and address, suitable for use with the byte field access operators B@, B!, B?, and ?B.  The width of a BYTES field must be even.

FILLER	This field reserves space in the record, typically used for future expansion or to skip regions of a record that are to be accessed by other means.  FILLER requires the number of bytes to be reserved.

Example:	6 FILLER

FILLER creates no dictionary entry.

At compile time, the numeric field defining words (1BYTE, NUMERIC, DOUBLE) expect the current displacement in the record on the stack.  A copy of the displacement is compiled in the parameter field of the definition, and its value on the stack is incremented by the size of the field in bytes.  A BYTES field also expects the size of the field on the stack.  This value is compiled along with the displacement, and used to increment the displacement accordingly.

When a field-name defined by one of these words is executed, it pushes onto the stack the address of working storage, incremented by the displacement of the field to give the address of the field in the record image in working storage.  In the case of BYTES fields, the size of the field is beneath the address on the stack.  The working storage address (and size, in the case of BYTES fields) is the appropriate input to the field access operators described in the next section.

REFERENCES

Access to the Record Image in Working Storage, Section 8.5.5
Available Records, Section 8.4.2
Direct Access to Fields, Section 8.5.4
Field Reference Operators, Section 8.5.3
Working Storage, Section 8.1.4

### 8.5.3	Field Reference Operators

Fields in files are referenced with special words.  The following operators assume that the desired file and record have been selected.  They refer to fields in the current record (as indicated by the value of user variable R#; see Section 8.4.1)  In all cases, the name of the field precedes the operator; the field-name returns the appropriate address (and length, in the case of BYTES fields) to be used by the access operator.

Word	Stack	Action

1@	( a -c)	Fetches the contents of a 1BYTE field to the top of the stack.

1!	( c a - )	Stores a byte into the 1BYTE field whose address is on top of the stack.

1?	( a)	Fetches and displays the contents of a 1BYTE field.

?1	( a)	As for 1?, except the results are right-justified by the report generator.

N@	( a - n)	Fetches the contents of a NUMERIC field to the top of the stack.

N!	( n a - )	Stores a number into the NUMERIC field whose address is on top of the stack.

N?	( a)	Fetches and displays the contents of a NUMERIC field.

?N	( a)	As for N?, except the results are right-justified by the report generator.

D@	( a - d)	Fetches the contents of a DOUBLE field to the top of the stack (two cells).

Word	Stack	Action

D!	( d a - )	Stores two cells into the DOUBLE field whose address is on top of the stack.

D?	( a)	Fetches and displays the contents of a DOUBLE field.

B@	( n a)	Reads a BYTES field, according to the declared length, into PAD.

B!	( n a)	Stores a BYTES field, according to the declared length, from PAD.

B?	( n a)	Fetches and displays the contents of a BYTES field, according to the declared length.  PAD is used as intermediate storage of the field data.

?B	( n a)	As for B?, except the results are right-justified by the report generator.

Example of usage:

GROSS D@	Fetches the contents of the DOUBLE field GROSS onto the stack.

Two other words are included for storing data into BYTES fields:

Word	Stack	Action

PUT	( n a)	Copies the remainder of the input stream into a BYTES field.  For example:

NAME PUT Fred Ferguson ok

A string that is too long will be truncated when it is stored.  If it is shorter than the field size, it will be blank-filled.  A copy of the entire string is left in PAD.

ASK	( n a)	Awaits (via EXPECT) input from the keyboard, and copies it into a BYTES field using PUT.

The word ENTIRE may be used in place of a field name:

ENTIRE	( -- n a)	Returns parameters for the “pseudo-field” that occupies the entire record in BYTES format.  For example:

Word	Stack	Action

ENTIRE	(Cont..)			ENTIRE B?

types the contents of the current record as though it were a single BYTES field.

REFERENCES

EXPECT, Section 3.7.1
Fetching Input to PAD, Section 2.3.6.3
PAD, Section 2.3.1
Report Generator, Section 8.9

### 8.5.4	Direct Access to Fields

The Data Base Support option is set up so that field names may be used with field access operators in a transparent way, although in fact more is going on with these words than meets the eye.  In the event that you need to directly access fields in a file (for instance, if you wish to use MOVE, ERASE, etc. instead of N!, etc.), you should understand the details explained in this section.

The addresses returned by user-defined field names are intended to be consumed by the field reference operators (Section 8.5.3).  These addresses, however, are not the addresses of the actual data in a block buffer, but rather addresses within working storage (Section 8.1.4).  The field reference operators perform the necessary offset correction, call the appropriate block and access the data.  In the case of “fetch” operators, the operators move the data elsewhere (numbers are pushed onto the stack; strings are moved to PAD).  This allows the field-name words, which return the address, to be used transparently with either working storage or the file data itself; the difference depends solely upon the operator that fetches or stores the data.

Each field reference operation is an implied disk access, since it calls BLOCK.  It is important not to carry the address of a field in a block buffer on the stack across any I/O operation (such as displaying the content of a field or accessing another field), since in a multitasking environment another task may perform disk activity that changes the content of the disk buffer.

Occasionally it may be useful to bypass the protection of the field reference operators, and determine the actual address of a field in a disk buffer.  This can be done by the following phrase:

field-name ADDRESS

This phrase places the actual memory address of the field on top of the stack.  For example, the following phrase will move an array of 100 2-byte data elements from working storage to disk much faster than it would take to calculate addresses repeatedly using N!:

DATA  DATA ADDRESS  200 MOVE  UPDATE

The first use of DATA returns the address of the image of the field in working storage.  The phrase DATA ADDRESS returns the location of the field in virtual memory.  200 MOVE moves the image in working storage to the disk buffer.  UPDATE is necessary after writing to a disk buffer.

For BYTES fields (since invoking the name of a BYTES field pushes both the location and length onto the stack), the phrase:

field-name ADDRESS

returns the length and virtual memory address (note that the order is reversed from the standard “address, count” order).

If direct addressing is used, you must remember that the content of the buffer can change at any time the task either requests I/O from any source or causes execution of PAUSE or WAIT.  Furthermore, if you modify the contents of any field directly (without using N!, B!, etc.), you must invoke UPDATE after the modification.

REFERENCES

Disk Buffer Management, Section 3.2.1
MOVE, Section 2.3.4
Multitasking Overview, Section 1.2.3

### 8.5.5	Access to the Record Image in Working Storage

Because field names return addresses within local working storage, you can directly access the working storage image of a record.  This lets you map data items as though they were contained in records, although they are kept in resident memory instead of on the disk.  There is only one “record” in the working storage area.

Using ordinary memory-access operators in conjuction with field names provides access to working storage locations:

Word	Action

C@	Fetches an 8-bit number.
Example:  AGE C@

C!	Stores an 8-bit number.
Example:  39 AGE C!

@	Fetches a single-length number.
Example:  LINK @

!	Stores a single-length number. 
Example:  16 LINK !

2@	Fetches a double-length number.
Example:  PRICE 2@

2!	Stores a double-length number.
Example:  196.75 PRICE 2!

S@	Fetches a string from working storage to PAD.
Example:  NAME S@

S!	Stores a string from PAD into working storage.
Example:  NAME S!

The word WORKING returns the address of the beginning of the task’s working storage.

REFERENCES

Memory-Stack Operations, Section 2.1.2
PAD, Section 2.3.1
Working Storage, Section 8.1.5

## 8.6	Ordered Index Files

An ordered index file is one in which the records are kept in ascending order depending upon the ASCII values of a key.  A key is an item of data that is used in a match or comparison.

There are two purposes for an ordered index file.  First, it greatly speeds up searches based on the key data.  Second, it allows you to display the main file alphabetically without having to sort it.

Each record in the index file contains a key together with a  link address to an associated main file.  This link resides in a 16-bit field called LINK.  In Fig. 8.6, the index file (NAMES) contains the names of people, ordered alphabetically, along with links to the main file.

Fig.  8.6

An ordered index file (on the left) showing links to the corresponding records in the main data file (right).

You may have several index files addressing the same main file.  For example, a file of scientific data could be indexed by both sample name and observation number, using two separate index files.  In Fig. 8.7, a second index file (COMPANIES) points to the same main file PEOPLE, but uses the company field as a key, and keeps the records ordered alphabetically by company.

Searches on an ordered index are performed using a “binary search,” which locates a record (or the place that it should go if it is not in the file) with only log2 n steps rather than n/2 (which is the average for a “brute force” or sequential search).

A binary search works by taking the occupied part of the file and dividing it by two, then comparing the desired key with the field in the middle record.  If the key is larger, then the high half of the file is halved again.  This process is repeated until the size of the remaining set of records is one.  This remaining record must match the key, if the key is in the file; otherwise, it is the record before which the key would be inserted.  For a file of 128 records, a binary search requires only seven comparisons, as compared with an average of 64 for a sequential search.

Fig.  8.7

Another index to the same main file shown in Fig. 8.6, this time using the company name as key.

An ordered index is a “dense file.”  That is, there are no gaps between active records.  Therefore, AVAILABLE always reflects the number of records in the index file, and all records in the index file can be accessed with a DO LOOP with the knowledge that all records are active.  With files maintained using SLOT and SCRATCH, you must check the LINK field (first two bytes of every record) to see whether each record is active.

### 8.6.1	Index File Records

At minimum, an ordered index file must contain the key and the link that associates the key with its main data record.  The link is a 16-bit record number residing in the first two bytes of the record, and the key field immediately follows.

You can keep data other than keys in an index file and process this data in the same manner as data in other types of files.  Such a technique should be avoided, however, if more than one user will have simultaneous access to the file, because record numbers may change due to insertion or deletion by other users.

The time required to search an index depends upon the length of each record as well as the number of records, because longer records will require more blocks to store the file, and hence more disk accesses to search it.  Therefore, you should keep these records as small as possible.

The first two bytes of each record in an index file contain the link to the associated record in the main file.  polyFORTH 2012 predefines this field as LINK.  The phrase:

LINK N@

reads the link field of the currently selected record and returns it on the stack.

When creating the record description (Section 8.5.1) for an index file, you must skip over the LINK field by using the phrase 2 FILLER at the beginning of the layout, or by starting with a displacement of two rather than zero.  For example:

0  2 FILLER	( Link to PEOPLE file)
   10 BYTES NICK	( Last name key)
DROP

The key may be ASCII or binary.  In order to make it possible to use binary integers as keys, as well as to speed up the search, the comparison made in the search routine compares cell-by-cell, rather than byte-by-byte.  To accommodate this, you must make your key fields an even number of bytes in length.  On machines which use a byte order that would render the most significant byte the second one in a string, the operators B@ and B! reverse bytes when fetching and storing from disk such that the data on disk is in a compatible order.

Be aware that the order of the records in the index file is subject to frequent change as a result of file insertion or deletion.  Because the record number of an index record may change, it should not be used directly for any purpose.

You must also take special care when sharing ordered files.  We suggest you limit the index file to keys, and keep all other data in an associated main file record.  Otherwise, a task may be pointing at a current record in an index, but before it accesses the data in the record the index record changes position.

### 8.6.2	Ordered File Maintenance

An “ordered index” file in polyFORTH is one in which the keys are maintained in ascending ASCII sequence.  For instance, an index to a file of records of people might be ordered by last names.

An ordered file allows quick searching on key fields.  For instance, given a name, we can search the index file looking for a match.  From the index record where the match was found, we can obtain the link to the main file.

#### 8.6.2.1	Searching an Ordered Index

In polyFORTH, this routine is called BINARY (named because it performs a binary search).  Here’s how it works:

As we’ve seen (Section 8.5.5), field names return the address of the field in the “image” of the record in working storage.  BINARY expects to find the match criteria for the desired field in this image (Fig. 8.8).

Fig. 8.8

BINARY searches the ordered index for a match to the key in working storage.  It returns the content of the LINK field of the matching record, and aborts if there is no match.

Make sure that you have allocated enough room in working storage for all tasks (including the printer task) to hold the image of any record on which you use this technique (Section 8.1.6).

For instance, suppose we want to take a name from the input stream, then search for it in the NICK (short for “nickname”) field of our (NAMES) file.  The phrase:

1 TEXT  nickname NICK S!

captures the name from the input stream and stores it into the image of the NICK field in working storage.

Now we use BINARY to search the index file for this name, first ensuring that the index file is current.  BINARY expects on the stack the arguments returned by a BYTES field:

(NAMES)  NICK BINARY

Here’s what BINARY does:

Word	Stack	Action

BINARY	( n a - n)	Searches the current file looking for a match between the criteria in working storage and the given field in the data.  Issues a system abort if it cannot find the record requested.  On the stack is the record number of associated record in the main field (that is, the contents of the link field of the matching index record).  The matching index record number is in R#.

(A related word, -BINARY, is discussed in Section 8.6.2.2.)

REFERENCES

Access to the record image in working storage, Section 8.5.5
Binary search principles, Section 8.6
R#, Section 8.4.1
TEXT, Section 2.3.6.3
Working Storage, Section 8.1.4

#### 8.6.2.2	Inserting a Record in an Ordered Index

Inserting a new record in an ordered file involves two steps.  First, we must determine the location in the index file for a new key to be inserted.  This ensures that the index file will always be properly “sorted.”

Second, we must be able to insert the new key into the file at the appropriate place, moving all subsequent records one notch down in the file.

Using the example in Fig. 8.6, let’s consider what must happen when we add a new person to our data base.  First, must insert a new index record
 into the (NAMES) file in the appropriate place, then allocate a new record in the PEOPLE file for the data itself.  Finally, we must point the LINK field in the index record to the data record in the main file.

We’ve already seen in Section 8.4.3 that the word SLOT is used to allocate new records in data files.  Adding a record to the index file is more complicated, because we must insert the new record at the appropriate place to keep the keys ordered.  For this purpose, we use the words -BINARY and +ORDERED.

Word	Stack	Action

-BINARY	( n a - t)	Searches the current file looking for a match between the criteria in working storage and the given field in the data.  A zero result (‘false’) means that a match was found; a non-zero flag means that no record in the file contains the indicated key.  On exit, if a match is found R# contains the number of the first matching index file record; otherwise R# contains the number of the index record before which an insertion will be made.  Pronounced “not-binary,” because it returns ‘true’ if a match is not found.

+ORDERED	( - )	Inserts the record whose image is in working storage into the current record in an ordered index.  Subsequent records in the index file are advanced one position relative to the start of the file.

-BINARY expects the same conditions as BINARY (Section 8.6.2.1):

1.	The current file is the ordered index to be searched.

2.	The match criterion is in the key field in working storage.

3.	The arguments produced by a BYTES field name are on the stack.

+ORDERED expects the following conditions:

1.	The current file is the index to be modified.

2.	The record before which the insertion is to take place has been previously selected by -BINARY.

3.	The key and LINK fields to be inserted are in their respective fields in working storage.

Using our example, then, the standard procedure is:

1 TEXT	(scan the input stream for the name)

NICK S!	(store it into the image of NICK)

NICK -BINARY	(search the index file, using the NICK field as the key)

IF	(no match:)

SAVE PEOPLE SLOT RESTORE	( obtain available record number in main file)

LINK !	(store the record number into working storage)

+ORDERED	(insert the new index record)

ELSE	(duplicate entry)

ORDERED RELEASE

1 ABORT" Already in file "

THEN  ...

Because your code must provide the location into which the insertion will take place (using -BINARY), you have the option of determining how to handle duplicate keys if -BINARY returns a false (zero) indication.  This is normally handled as an abort condition, as shown above.

During execution of the -BINARY ... +ORDERED sequence, the index file should not be accessed by any other task, since the record numbers of all records following the insertion point are changing.

To prevent conflicts, the Data Base Support option includes a facility management variable called ORDERED.  -BINARY issues an ORDERED GET.  This phrase protects the file from being accessed by other tasks on the system until the current task releases it.  In this way, file integrity is maintained.  +ORDERED issues an ORDERED RELEASE.  If you exit from the operation in any other way, you must do this yourself.  The intent is for the task that performed the search to retain control of the file from the moment when the insertion point has been found until the expected insertion has taken place, or until it has decided not to do one.

The word BINARY also performs an ORDERED GET, so that searches cannot be performed while another task is using this facility.  BINARY performs an ORDERED RELEASE immediately after the search, however, so it “holds” the facility only during the period of the search itself.

REFERENCES

Binary Searches, Section 8.6.2.1
Facility Variables (GET and RELEASE), Section 4.7
SAVE and RESTORE, Section 8.4.2
TEXT, Section 2.3.6.3

#### 8.6.2.3	Deleting a Record From an Ordered Index

-ORDERED is used to delete a record from an index file.  It may only be issued immediately after the record has been selected (normally by a prior use of BINARY).

Word	Stack	Action

-ORDERED	(  )	Deletes the current record (R#) from an ordered index which is the current file.  Subsequent records move back one position, relative to the start of the file.

Because the actual space that was occupied by the deleted record will be occupied by the record that until now followed it, the record is completely obliterated by this operation (unlike SCRATCH, which only changes the first two bytes of the record).

Here is an example using -ORDERED.

1 TEXT	(scan the input stream for the name)

NICK S!	(store it into the image of NICK)

NICK BINARY	(search the index file, using the NICK field as the key; return main file record number)

ORDERED GRAB	(regain control of ORDERED, which BINARY released)

-ORDERED	(delete the index record)

PEOPLE SCRATCH	(de-allocate the record in the main file whose number is on the stack from BINARY.)

In this example we had to GRAB the facility variable ORDERED to prevent another task from accessing the file during the moving of records that will occur during the -ORDERED operation.  GRAB is used instead of GET because GET releases the CPU so other tasks can run (and potentially alter the file).  -ORDERED performs an ORDERED RELEASE when it is finished.

REFERENCES

Binary Searches, Section 8.6.2.
Facility variables (GRAB, GET, and RELEASE), Section 4.7
SCRATCH, Section 8.4.3

### 8.6.3	An Example—A Simple Mailing List

The following pages show an example of a simple mailing list application.  It demonstrates the use of an ordered index to provide easy access into a file based on a key, such as last name and first initial, and a report which is in alphabetic order based on that key.

This application is a good example of the layout of a polyFORTH application, with a “help screen” at the top of a triad, followed by the application load block and relevant file definitions.  The help screen is automatically displayed when the application is loaded, and may be displayed any time by the command HELP.

Ensuing blocks are shown with their corresponding “shadow blocks” on the facing page.  This was necessary due to the reduced size of this book; on most computer systems you would print the source and shadow blocks side-by-side on the same page, using the PAIRS command in the polyFORTH PRINTING utility.  The last pair of blocks are shown vertically, due to space limitations.

REFERENCES

Disk Organization, Section 5.2.5
PRINTING Utility, Section 5.2.3
Shadow Blocks, Sections 1.5.1, 5.2.3

828

0 ( Sample file system application)   EMPTY

1 (Working storage)   128 ALLOT

2

3 : HELP   828 HELPS ;   HELP

4

5 ( File allocation)   830 LOAD

6 ( Record description)   831 LOAD

7 ( Data entry)   832 833 THRU

8 ( Data display)   834 LOAD

9

10

11

12

13

14

15

829

0 This application assumes FILES is loaded in Block 9
1

2 HELP  Display these PERSONNEL instructions.

3 enter name	Enter a new person into the file with 
4	access key of 'name'

5

6 remove name	Delete 'name' from the data base

7

8 fix name	Enter new information replacing all 
9	current data for 'name'

10

11 see name	Display a person whose key is 'name'

12

13 s	Display current person

14

15 all	Display all records in the file.

830

0 (Bytes records blocks origin      name   )

1    16      300     6   1500 FILE  (PEOPLE)

2   128      300    43   1530 FILE  PEOPLE

3

4

5

6

7

8

9

10

11

12

13

14

15

1233

1 The record layout for both the PEOPLE and (PEOPLE)

2    files.  The LINK is predefined, and subsequent

3    fields are offset from the previous 4 fields.

5    For example, the NICK name is 14 bytes long

6    starting in the 2nd byte.

7 ZIP  is a 32-bit number, as is PHONE.

8 AREA  code is single precision.

9

10 The offset for the field types is carried on the

11    stack so that it may be either displayed or

12    dropped at the end of the load.  We use it in

13    this case to display the record size.

14

15

1234

0 PERSON  parses the input stream following it for the

1    NICK field.  It leaves us pointing at the NICK

2    field in the (PEOPLE) file.

3

4 DIGITS  Prompts the terminal for input and converts

5    it to binary on the stack.

6

7

8 >DOUBLE  Takes the output of DIGITS and converts the

9    number to 32-bit.  PTR is negative if the

10    NUMBER conversion was 16-bit, in which case the

11    high-order part of the number may be found at

12    'NUMBER 2+ .

13

14 !LABEL  Prompts for each field in order.

15

1235

0 enter  creates a new entry for the person whose

1    nickname follows in the input stream, prompting

2    for entry of additional data. If there is already

3    an entry for that nickname, an error message is

4    issued.  In either case, the record remains the

5    current one for future editing.

6

7 fix  accepts new data for the pre-existing entry

8    whose nickname follows in the input stream.

9

10 remove  deletes the person whose nickname follows

11   from the data base.

12

13

14

15

831

0 ( Record descriptions)

1 2 ( LINK)

2 14 BYTES NICK     ( Nickname, used as the key.)

3 32 BYTES NAME     ( Full name, first name first.)

4 32 BYTES STREET   ( Street addr. or PO Box, etc.)

5 32 BYTES CITY

6    DOUBLE ZIP     ( Note:  can only handle US zips)

7    NUMERIC AREA

8    DOUBLE PHONE

9

10  CR .( Main file: ) .  .( Bytes )

11

12

13

14

15

832

0 ( Data storage)

1 : PERSON ( - n a)   1 TEXT  NICK S!  (PEOPLE) NICK ;

2

3 : DIGITS ( - d/n)   QUERY  32 WORD  NUMBER ;

4

5 : >DOUBLE ( n/d - d)   PTR @ 0< IF 'NUMBER 2+ @

6    THEN ;

7 : !LABEL   CR ." Name: "   NAME ASK

8     CR   ." Street: "   STREET ASK

9     CR   ." City, State: "   CITY ASK

10     CR   ." Zip: "   DIGITS  >DOUBLE  ZIP D!

11     CR   ." Area: "   DIGITS AREA N!

12          ." Phone: "   DIGITS >DOUBLE  PHONE D! ;

13

14

15

833

0 ( Record management)

1 : enter   PERSON -BINARY IF  SAVE  PEOPLE SLOT DUP

2       READ  NICK S@ NICK B!  RESTORE  DUP LINK !

3       +ORDERED  PEOPLE READ !LABEL 
4       ELSE  ORDERED RELEASE  ABORT" Already known "

5    THEN ;

6 : fix   PERSON BINARY  PEOPLE READ !LABEL ;

7

8 : remove   PERSON  BINARY -ORDERED  PEOPLE SCRATCH ;

9

10

11

12

13

14

15

1236

0 .PHONE  displays the AREA and PHONE numbers as one

1    would expect to see them.

2

3 .ZIP  forces the zip code to be displayed in

4    nnnnn format.

5 n .PERSON  displays the data from the nth record in

6    the PEOPLE data file.

7 see  Parses the input stream and displays the proper

8    record.  s does the same thing using R#

9    (the current record).

10 all  uses the RECORDS word which returns the

11    initial value and number of records+1 in the

12    data file.  The loop counter is used to access

13    each record in the ordered index (PEOPLE),

14    where the LINK field points to the data in the

15    PEOPLE file.

834

0 ( Data Display)

1 : .PHONE   AREA N@ 0  <# 41 HOLD # # #  40 HOLD #>

2    TYPE SPACE  PHONE D@ <# # # # # #  45 HOLD ( -)

3    # # # #> TYPE ;

4 : .ZIP   ZIP D@ <# # # # # #>  TYPE ;

5

6 : .PERSON ( n)   PEOPLE READ  CR  NAME B?  5 SPACES

7    ." (" SPACE   NICK B?  ." )"  CR   STREET B?

8    CR  CITY B? CR   .ZIP  10 SPACES  .PHONE  SPACE ;

9

10 : see   PERSON BINARY  .PERSON ;

11

12 : s   R# @ .PERSON ;

13

14 : all   (PEOPLE) RECORDS DO  I (PEOPLE) READ

15    LINK N@ .PERSON  CR  LOOP  SPACE ;

### 8.6.4	Hierarchical Ordered Files

polyFORTH’s ordered indexes have the property than whenever a record is inserted or deleted all records following the point at which the action occurs are physically moved to accommodate the change.  Although this form of maintenance is somewhat slower than maintaining order by updating chains or pointers (as some data bases do) it is substantially more reliable.

The assumption is that in most applications an index is searched frequently, and insertions and deletions occur relatively infrequently.  As a result, we have optimized search time and reliability above maintenance time.

The actual time an insertion or deletion will take depends upon the position in the file at which the action occurs (if it is near the beginning of the file more records must be moved), the number of records in the file, and the size of each index record.  In practice, indexes of several thousand records may be maintained on a hard disk without unacceptable delays.

Some applications, however, involve tens of thousands of records that must be searched and maintained in order. In order to deal with such applications, the recommended approach is to divide the total index into several sub-indexes, each of which will be a manageable size.  For example, a company with 40,000 employees might separate them into departments.  The department code can index a table in memory giving the appropriate origin block number for the index of employees in each department.  This block number may be put into the ORG field of the FDA of a private copy of a generic file definition for the index.  Or, the first letter of the employee’s last name may be used to select one of 26 indexes.

Such a multi-layered approach is called a hierarchy.  If you are designing a hierarchical file structure, the important considerations include keeping the decision-making process simple and independent of any frequently changing conditions.  If possible, try to base the initial choice on something that can be evaluated without need for a special file search.  Above all, you should avoid keeping record numbers of records in an ordered index in a higher-level index, as ordered index record numbers are subject to change.

REFERENCES

File Definition Areas, Section 8.3.2

## 8.7	Chaining

Chaining is the linkage of one record to another, whether in the same or a different file.  Generally, chaining is appropriate when an unknown amount of data must be associated with a piece of information.

There are as many ways to chain records as there are varieties of applications.  In this section, we’ll cover most of the situations that require chaining, and present general solutions to each case.

### 8.7.1	Chaining Techniques

Before you begin coding, make sure that you study the exact requirements carefully.  Reviewing this section for considerations will be helpful.

Here are some design considerations to take into account:

1.	Will the chaining occur within the same file, or to an auxiliary file?

2.	Must there always be at least one auxiliary record chained to a main record, or may a main record have no auxiliary records?

3.	When you traverse the chain, should it be in the order in which its elements were added (first-in, first-out), or in reverse (last-in, first-out), or should the chain be maintained in order by a key (such as date and time)?

Let’s explore these issues one by one.

In some applications, it is possible to chain records within a single file.  Naturally, this is easier than chaining to another file.

For example, suppose that we have a file of customer names and addresses.  Some of our customers have several addresses:  one for invoicing, one for shipping, and so on.  Because multiple addresses are the exception, not the rule, and because address fields are large, we’d prefer not to allow room for multiple address fields within each customer record.

So, we use chaining instead.  At this point, we must examine how much information each auxiliary record must contain.  It turns out that each auxiliary record must contain almost as much information as the main record.  If we create a separate file for the auxiliary records, each record would need to be nearly as large as a record in the main file.

If there is relatively little in the main record (the one all customers have) beyond the primary address, you may as well use additional records in the same file to contain additional addresses.  As Fig. 8.9 shows, this approach lets us re-use the field layout structure that we created for the main file records, even though there are some fields in the primary record that we don’t use in the auxiliary records.

For another example of chaining within a single file, we turn to the DOCUMENTOR application included with polyFORTH ISD-4 (see Section 8.10).  This application lets you enter descriptions of the commands in your applications and produces alphabetized glossaries.

For each word that you enter into the system, the DOCUMENTOR saves its name, vocabulary, stack effects (before and after) as text strings, the source block, the date this entry was created or updated, plus as many lines of descriptive text as you care to include.

Fig. 8.9

Example of a chain with all records in the same file.

Fig. 8.10

Record chaining in the polyFORTH DOCUMENTOR utility.

Fig. 8.10 shows the record structure for the DOCUMENTOR.  All data except the text is stored in the main record for each command.  This record points to an auxiliary record that contains the text description.  This record may in turn point to a second text record, and so on.  A separate index file contains the alphabetized keys that point to main records in this file.

Although the main records and auxiliary records share no fields in common (except LINK), they are the same size.  Thus it is most efficient to keep both types of records in the same file.

Fig. 8.11

A variable number of serial number records for products purchased by each customer.  Note that each serial number record contains a pointer back to the “owner” record.  This is important for maintaining file integrity.

A third example illustrates the opposite situation.  Suppose we have a list of customers who have purchased our products.  For each customer, we also have a list of the serial numbers of the units they received.  For some customers, there are no serial numbers; for others, as many as twenty.

You can see in Fig. 8.11 that a serial number record takes much less space than a customer record.  Because of this size variance, it’s better to create two separate files, one called CUSTOMERS and the other SERIALS.  Each main record in the CUSTOMERS file may chain to one or a series of records in the SERIALS file.  A record in CUSTOMERS can also contain an empty link, which would be represented by a value of -1 in the LINK field.  A -1 LINK also identifies the last serial number for a particular customer.

This last example raises the second consideration: whether the application must be able to handle the case of no auxiliary records, or whether the minimum number of auxiliary records attached to a main record must be one.

In the first case, when a main record is created, its link can be left alone (-1) and no auxiliary record need be SLOTted. However, the routine that appends a new auxiliary record to the chain must check whether it is linking from the main record or an auxiliary record.

In the second case, when a main record is created, an auxiliary record must also be slotted, and its number saved in the main record’s pointer.  Furthermore, the routines for advancing through the chain will differ, as we’ll see in the next section.

A third consideration is whether chaining must be last-in, last-out; last-in, first-out; or both. In the case of the DOCUMENTOR described earlier, obviously chaining must be first-in, first-out.  In such cases, the process of adding a new record to the chain involves:

1.	Finding the end of the existing chain;

2.	Allocating a new record;

3.	Setting the link in the last record of the existing chain to point to the new record.

An example of the opposite situation is a bookkeeping database in which each customer record chains to a series of auxiliary records containing transactions.  Because we are almost always more interested in recent transactions than ancient ones, we chain in a last-in, first-out manner.  In this case, the process of adding a new record to the chain involves:

1.	Allocating a new auxiliary record;

2.	Setting the main record to point to it;

3.	Placing the main record’s previous link number into the link field of the new record.

If the application demands that both directions of chain-following be allowed, then each auxiliary record must contain two link fields:  one to the next record in the chain, and one to the previous.

Each chained record should contain a pointer back to the record that is the head of the chain (which may or may not be in the same file as the chain).  Some applications use this directly.  For instance, suppose in our serial number example we keep an ordered index file using the serial number itself as the key.  If records in SERIALS contain a pointer to the owner of the chain as shown in Fig. 8.11, then by entering a serial number the user can see which customer has received that instrument.

The most important reason for including a pointer to the owner, even if the application doesn’t otherwise demand it, is for ensuring integrity of the data.  If through some mischance of hardware failure a link in the main file becomes lost, the chains can be reconstructed and attached to the main records.

### 8.7.2	Chaining Commands

As we have seen in the previous section, the choice of chaining techniques depends on application needs and on performance tradeoffs.  Rather than attempt to decide for you, the developers of polyFORTH 2012 provide a collection of commonly-used chaining tools.  You may leave them as is, or you may modify them.  The table below gives the general set of commands in the chaining toolbox.  Some words appear more than once; this is because several implementations may be useful, depending on how you’ve answered the design questions in Section 8.7.1.  The version shipped with the system is marked with an (*).  The others are minor variants; code for some of the alternate versions is given elsewhere in this chapter.

Word	Stack	Action

HEAD	( -- a)	A user variable that points to the first record (head) of the current chain.

LINK	( -- a)	A pre-defined field (the first two bytes of any record) which may be used for chaining.  This same field is used in ordered index records to link to the main file records.

Word	Stack	Action

FIRST	(  )	(*) READs the HEAD record in the chain.  This version is used in applications in which there is always at least one auxiliary record and all are within the same file.

FIRST	( -- t)	Returns a flag indicating whether the main record is chained to any auxiliary records, and if it is, READs the record. This version is used in applications in which the HEAD record may have no auxiliary records, and when auxiliary records are in a different file.

-NEXT	( -- t)	(*) Reads the next record, assuming that the chain is linked through the field called LINK.  Returns ‘true’ if there is not a next record in the chain.  Pronounced “not-next.”

-NEXT	( -- r/0)	Alternate version of -NEXT; returns the record number of the next record in the chain, if any, 0 (‘false’) otherwise.  Does not read the record.

-LOCATE	( n - t)	Searches the chain, starting from HEAD, for the nth record, returning true if the chain isn’t that long. Otherwise, it returns false, having left R# pointing to the specified record.

CHAIN	( n)	Inserts a new record at the nth position.  If n is larger than the length of the chain, inserts the new record at the end.  Alternate versions might take no argument and chain at the beginning (last-in, first-out), end (first-in, first-out) or according to a key.

UNCHAIN	( n)	Removes the nth record from the chain.

SNATCH	( a r - r)	Given a field address and record number, fetches the record number from that field and replaces it with the record number given.  It is used to update chains.

The arguments for -LOCATE, CHAIN, and UNCHAIN count from zero, where zero is the first record in the chain, and count sequentially down the chain.  An argument of -1 is conventionally used to specify the end of the chain (since you don’t necessarily know how long the chain is).

The standard versions of FIRST and -NEXT assume there is always at least one record in the chain, and it’s also the HEAD record (i.e., it will be subject to the same processing as the others).  The chain may be processed in a BEGIN ... UNTIL loop:

FIRST BEGIN ... -NEXT UNTIL ...

The alternate versions allow for the possibility that there are no auxiliary chains, and are optimized for a BEGIN ... WHILE ... REPEAT loop:

FIRST BEGIN  ?DUP WHILE  READ ...
   NEXT REPEAT ...

If you have only one set of chained records and the top of the chain is in a different file from the members, you may incorporate the selection of the file in the words FIRST, etc.  If you have several sets, you will need to select the file externally.  Still another set of variations might allow for the fact that you have more than one chain attached to your main file, and therefore not all chains start with the LINK in the main file record.  Moreover, there may even be multiple chains through the auxiliary records.  In these cases, you would remove the references to LINK in these words and specify the field externally.

The intent here is to present a design concept that has worked in many applications, but which presumes that you will tailor a basic vocabulary to your specific application needs—a practice that is consistent with the overall design of Forth in general.  Assuming you are adding custom versions of the chaining words for your application, don’t forget to remove from the FILES load block the reference to the standard ones.

REFERENCES

BEGIN ... UNTIL, Section 2.4.2
BEGIN ... WHILE ... REPEAT, Section 2.4.3
FILES Load Block, Section 8.1.5

### 8.7.3	Application Examples

This section offers coded solutions to two application problems.

We introduced the DOCUMENTOR program, which is included with your polyFORTH system, in Section 8.7.1.  The use of this utility is more thoroughly documented in Section 8.10.

The word (SHOW) includes this sequence:

... 	( display data from the main record)
BEGIN +L  -NEXT NOT WHILE
   10 SPACES  PHRASE B?
REPEAT ;

The word +L is similar to CR; see Section 8.8.4.

The word (SHOW) displays all information about a command.  The code fragment shown above displays the list of description lines for the command.  When it begins, the main record is still current.

As we saw in Section 8.7.1, the main record’s link field points to the first descriptive record, if there is one, which resides in the same file.  When the loop begins, -NEXT determines whether the main record is linked to an auxiliary record.  If not, the loop ends and nothing is displayed.  If so, the WHILE portion is executed, which displays the first line of text and repeats the loop.  Now -NEXT indicates whether there is another auxiliary record.

When the last record is reached, -NEXT indicates this and the loop ends.

The word ?LINES is defined as:

: ?LINES   1 BEGIN 1+  -NEXT UNTIL  ?PAGE ;

The purpose of ?LINES is to determine whether the current command’s description will fit entirely on the page, or whether it is necessary to advance the page first to keep all of its lines together.  The loop counts the number of lines (the head plus an unknown number of auxiliary records, at one line each).  The word ?PAGE, introduced in Section 8.8.4, takes an argument from the stack, starting a new page if that many lines will not fit on the current page.

Here is a definition using SNATCH:

: DELETE ( r#)   ...   BEGIN READ
   LINK 0 SNATCH  DUP 0< UNTIL  DROP ;

The part of DELETE shown here removes both the main record and all auxiliary records chained to it. The code begins on the main record.  The phrase LINK 0 SNATCH fetches the record’s link field, then replaces it with zero.  This has the effect of “scratching” the record, but also provides a pointer to the next record to scratch.

The phrase DUP 0< tests whether the pointer indicates that the record just scratched was the last in the chain.  If so, the loop ends; otherwise, it reads the next record, and so on.

You may also wish to study the definitions of T, P, and U, which use -LOCATE, CHAIN, and UNCHAIN in straightforward ways.

Our second coding example is another that we introduced earlier in this section: the customer file and associated serial numbers.  Here we will present two versions of the application.  The first, in Fig. 8.12, uses the versions of FIRST, -NEXT, and -LOCATE that are provided with your polyFORTH system.

In the first block we’ve defined the record structures for the two files.  In the second block, we have words for entering new customers and serial numbers. The word add makes use of chaining.

As we saw in our earlier discussion of this application, it is legitimate for a CUSTOMERS record to have no serial number attached to it.  In this case, the CUSTOMERS record will contain –1 in its LINK field.  If auxiliary records are chained, they will reside in a separate file called SERIALS.

The process of adding a new serial-number record is not as simple as it would be if all records were contained in the same file.  Here, add must make a decision.  If there is no chaining yet, it must go to the SERIALS file and use SLOT to allocate a record.

Since this is the first record in the chain, it must also store this in the main record’s LINK field.  But if a chain has already been started, it will go to SERIALS and use CHAIN to add a new record.

The problem is that we cannot use CHAIN unless a chain exists already.  If all records existed in the same file, then the main record would be the first record in the chain; and we could simply use CHAIN in all cases.  We would not need a conditional.  Or, even if records existed in separate files, but a minimum of one auxiliary record was always present, we could use CHAIN and avoid the conditional.

The phrase -1 CHAIN is a cliché that means “attach a new record onto the end of the chain.”  The -1 serves as a number that never gets reached, and CHAIN is defined so that if it never reaches n it adds the new record to the end of the chain.

0 ( Customers and Serial Numbers)

1 ( CUSTOMERS records:)

2 0  2 FILLER  ( LINK to 1st serial#)

3   20 BYTES COMPANY

4   16 BYTES CONTACT

5   30 BYTES STREET

6 DROP

7

8 ( SERIALS records:)

9 0  2 FILLER  ( LINK to next serial#)

10   10 BYTES SERIAL#

11    NUMERIC PRODUCT  ( product code)

12    NUMERIC OWNER  ( link to owner CUSTOMERS record)

13 DROP

14

15

0 ( Customer/serial number file)

1 : edit   CR  ." Company name? "  COMPANY ASK

2    CR  ." Contact? "  CONTACT ASK

3    CR  ." Address? "  STREET ASK ;

4 : new   CUSTOMERS  SLOT DUP . READ  edit ;

5

6 ( Assumes a serial# chain linked thru HEAD)

7 : (add)   CR  ." Serial# ? "   SERIAL# ASK ;

8 : add   SAVE  LINK N@ DUP 0< IF  ( empty chain) DROP

9          SAVE  SERIALS SLOT  RESTORE

10          DUP LINK N!  SERIALS READ

11       ELSE  HEAD !  SERIALS -1 CHAIN ( add at end)

12   THEN (add)  RESTORE ;

13

14 : edit ( n)   SAVE SERIALS  1- -LOCATE ABORT" Can't"

15    CR  Serial# B?  (add)  RESTORE ;

0    ( Customer/serial number display)

1 : .company   COMPANY B?  CONTACT B?  STREET B? ;

2

3 : .companies   CUSTOMERS  RECORDS DO

4      CR  I .  I READ  .company  LOOP ;

5

6 : .serials   0  SERIALS  FIRST BEGIN

7   CR 1+ DUP .  Serial# B?  -NEXT UNTIL ;

8

9 : serials   CR  .company  LINK N@  0>  IF

10    LINK N@  HEAD !  SAVE  .serials  RESTORE  THEN ;

11 : show ( n)   CUSTOMERS READ  serials ;

12

13 EXIT   Usage:

14 To enter a new customer:  new  then  add  as needed.

15 To edit an old one:  n show  then  add  or  n edit .

Fig. 8.12

An application example using the standard polyFORTH chaining operators.

The word edit may be used to change an existing serial number.  From its purpose we can assume that a chain exists, and therefore it doesn’t have to check the main record’s LINK to make sure it points to a valid auxiliary record.  It simply goes to the SERIALS file and uses -LOCATE to make the desired record current (aborting if the argument is not valid and -LOCATE terminates before reaching it).  Then it displays the current contents of the field and lets the user re-enter it.  Finally it restores the file pointers to the main file.

In the next block, the word serials displays the current company data, followed by a list of all associated serial numbers.  Again, since there may be no chain at all, serial must make a decision.  The test LINK N@ 0> returns ‘true’ if the link is positive (that is, not -1 or 0), indicating the first record in the chain.  In this event, serials saves this link in the variable HEAD, selects the SERIALS file, and invokes .serials which uses FIRST and -NEXT to loop through all records in the chain.

To give you an idea of some of the many possibilities, we’ve coded the same application using different versions of the words FIRST, NEXT, and -LOCATE.  While these definitions themselves are more complicated, they reduce the complexity of the application words that use them.  These versions are sensitive to the possibility that a main record may not have any auxiliary records attached to it.

Here are the re-definitions, followed by the new versions of the affected application commands:

: VALID ( n - t)   0 OVER < DUP  IF
   SWAP READ  ELSE  SWAP DROP  THEN ;

: FIRST ( - t)   HEAD @ VALID ;

: NEXT ( - t)   LINK N@ VALID ;

: -LOCATE ( n - t)   FIRST IF BEGIN  DUP WHILE
      1-  NEXT NOT IF  DROP -1 EXIT  THEN
   REPEAT  ELSE DROP -1 THEN ;

: add   LINK N@ HEAD !  SAVE  SERIALS
   FIRST IF  -1 CHAIN  ELSE ( no chain)
      SLOT DUP RESTORE  LINK N!  SAVE
      SERIALS READ  THEN
   (add)  RESTORE ;

: serials   LINK N@ HEAD !  SAVE  0
   SERIALS  FIRST BEGIN  WHILE  CR  1+ DUP .
      SERIAL# B?  NEXT REPEAT  RESTORE  DROP ;

In the first block, FIRST returns a flag that is true if a chain exists at all.  If so, the first record in the chain is made current.  The word NEXT returns a flag that is true if another record exists in the chain.  If so, that record is made current.

As you can see, both words make use of the same code, which we have factored into the definition called VALID.

We have also re-coded -LOCATE in this block.  As usual, -LOCATE returns a “true” flag if the requested element of the chain cannot be found.  In this version, it also returns a “true” flag if no chain exists.

These changes simplify our application definitions. add  still has to make a decision, but it uses FIRST for the test.

Because of the way we have rewritten FIRST, serials no longer needs an IF statement at all.  The only conditional is WHILE, which gets its argument the first time around from FIRST, and henceforth from NEXT.  Thus, if a first record is absent, the WHILE phrase never gets executed.  We eliminated the need for a subordinate word .serials completely.

REFERENCES

Data Base Design, Section 8.9

## 8.8	Report Generator

The polyFORTH Report Generator is a set of words that assist you in the preparation of formatted output reports.  Once you have specified the page format and column headings, and indicated the layout of a single record as a row of data, the Report Generator performs all required output formatting and also controls paging, the heading of each page and related operations.

An optional feature of the Report Generator allows subtotals and grand totals to be accumulated in a simple manner; these totals can then be printed on a separate line with a minimum of effort.

The following example will serve as a quick introduction to the Report Generator.  It assumes the fields defined in the example in Section 8.1.  Here is the code:

: .person   NAME ?B  STREET ?B  CITY ?B
   STATE ?B   ZIP ?B ;

[R People\ Name          Address
       City          St. Zip ]

: all   LITERAL LAYOUT  +L
      PEOPLE RECORDS DO  I READ  .person
   +CR  LOOP ;

This produces:

FORTH, Inc.	Page 1  26 AUG 1986

People

Name	Address	City	St	Zip

Andrews, Carl	1432 Morriston Ave.	Parkerville	PA	17214
Boehning, Greg	POB 41256	Santa Cruz	CA	95061
Chapel, Doug	75 Fleetwood Dr.	Rockville	MD	20852
Cook, Dottie	154 Sweet Rd.	Grand Prairie	TX	75050

In the example above, the word .person is defined similarly to the version given in Section 8.1, except that the field reference operator ?B is used instead of B?.  ?B is the Report Generator version of B?, and takes the same stack arguments. The difference is that it performs “tabbing” based on a table of columns created by the word [R (third line of example).  Section 8.8.3 lists all the output operators that use this table.

The word [R specifies both a title (the word “People,” centered) and the column headings (the row of labels above each column).  It also creates the column table mentioned above, leaving this address on the stack.  See Section 8.8.2 for more on [R.

The final word, all prints the tabulated report.  It begins by invoking LITERAL so that the address passed from [R will become part of this definition, then calls LAYOUT, which consumes this address.

+L (short for “plus-line”) forces an extra carriage return into the report above the first row of data.  Next, PEOPLE guarantees that the PEOPLE file is current whenever we display this report.  RECORDS supplies the appropriate arguments for DO.  Each time through the loop, the next record is made current with READ, and the row is displayed with .people.  Then, +CR forces a new-line and checks whether the page is full—if so, the report generator automatically heads the next page.

The report also contains a page banner which includes some text at the upper-left hand corner of the page and the page number and date in the upper right.  These are formatted automatically by LAYOUT, but are user-configurable.

REFERENCES

Controlling Paging, Section 8.8.4
Page Banner, Section 8.8.5

### 8.8.1	Specifying a Title/Column-Heading Pair

A single word, [R, lets you specify both the title and the column headings.  The set-up phrase must appear in the source block, usually just preceding the definition of the report for which they are designed.  This source text must be available for the application when it is running (i.e., mounted on a disk drive); if you need a ROMable version, contact FORTH, Inc.

The format for a title/column heading pair is:

[R title-text \column-heading-text ]

The entire title/heading pair statement may extend over multiple lines, since the Forth interpreter treats source blocks as contiguous 1K-byte strings.

All characters up to the backslash, except the first blank that follows [R, are used for the title text.  The title text will be displayed approximately at the center of an 8-1/2" x 11" page, assuming that the title-text is about 30 characters in length (actually, the system outputs 20 spaces before typing this string to approximate the centering; this occurs in the word +PAGE).

All characters that follow \, ending with the delimiter ], are used for the heading.  The first character (blank or non-blank) that follows the \ corresponds to the first column of the report page.

In addition to being displayed, the heading text is parsed at the time the block that contains the heading is loaded, to produce a table of column widths and locations of the text to be displayed.  This table is used by the set of words that output the contents of fields for the Report Generator; this wordset includes:  ?B, ?N, and ?1.  Thus, each column “knows” where it should appear on the page and how wide it should be.

When displaying BYTES fields, it is necessary to ensure that the width of the heading text for that field matches the width of the storage field, plus a few extra spaces as desired for column separation.  Any fewer spaces, or significantly more spaces, will result in a skewed output.

With numeric fields, caution should be exercised that the length of the field to be printed does not exceed the width of the column to be used.  Should the actual size of a string exceed the column width, it will nonetheless be printed in full and the remaining columns permanently shifted right to accommodate it.

Also, make sure that no blank space intervenes between the last column heading and the delimiting ]; this will cause an extra column entry in the table and may skew the output.

Finally, make sure that each heading does not contain a space, such as “Acct. No.” because this will be interpreted as two column headings.

The address of the columns table is left on the stack at load time by [R; this is the address that must be passed to the word LAYOUT.  LAYOUT initiates the printing of a report and specifies the type of page heading routine to be invoked.  It also saves the address of the title/column heading table (in user variable RPT) so that each page of the report will display the same header information.

If the title/column-heading pair is to be used in several reports, the address of the table for the title/heading pair may be used as the value for a CONSTANT, thus giving a name to the title/heading pair:

[R A Report \Col1 Col2 Col3 ]
   CONSTANT 'SHOW'
: SHOW   'SHOW' LAYOUT  ... ;

Otherwise, it is more efficient to just keep this address for a LITERAL to compile as a literal in the definition that uses this report:

[R A Report \Col1 Col2 Col3 ]
: SHOW   LITERAL LAYOUT ;

This address may, of course, be DUPed if more than one reference is required, provided the DUP appears outside any definition (and thus is executed):

[R A Report \Col1 Col2 Col3 ] DUP
: SUMMARY   LITERAL LAYOUT ... ;
: SHOW   LITERAL LAYOUT ;

REFERENCES

+PAGE, Section 8.8.4

### 8.8.2	Formatting Lines

To the report generator, a line consists of a series of columns, each of which has as fixed width.  These columns are used to align the data to be printed, with all data right-justified in the current column.

The following words are provided by the Report Generator to display fields within the columns determined by the title/column heading pair:

Word	Stack	Action

.N	( n)	Displays the single-length integer n right justified in the next column, in the format used by . (dot).

?N	( a)	Displays the contents a address as a single-length integer n right justified in the next column, in the format used by . (dot).

?1	( a)	Displays the contents of the specified 1BYTE field, right justified in the next column.

.D	( d)	Displays the double-length integer d right justified in the next column, in the format used by D.

?B	( n a)	Reads and displays a BYTES field, according to the declared length, left-justified in the next column.  PAD is used as intermediate storage of the field.

.M/D/Y	( n)	Given a Julian date, displays it in the next report column.  Since this routine invokes (DATE).  it will work with either calendar.  Most data base applications prefer to use the mm/dd/yy calendar (Block 43).

Each of these operators advances the columns table to the next column, determines the width of the new field, then right-justifies the output string in this column.

You may also build your own formatting words to display columns, using the word RIGHT.

RIGHT	( a n)	Displays an alphanumeric string of length n, beginning at address a, right-justified in the next column.

The stack arguments are identical to those of TYPE.

In fact, .N, ?N, ?1, .D, and ?B are defined using RIGHT and behave according to its rules:

1.	If the length of the output string exceeds the width of the column, the results are unpredictable but will include loss of format control.

2.	If the length of the output string equals the width of the column, the string is displayed and the column pointer is advanced.

3.	If the length of the output string is less than the width of the column, the difference is output as blank spaces, so that the string will be right-justified.

4.	Text strings are also right-justified; however their trailing blanks are included, making them appear left justified.

Here is an example:

0 ( Accounts example)     FORGET TASK : TASK ;

1

2 0  10 BYTES NAME   NUMERIC ACCT#   DOUBLE BALANCE

3    
 4 : (.$) ( d - a n)   SWAP OVER DABS

5    <#  # #  46 HOLD  #S  SIGN  #> ;

6 : .ACCOUNT   ACCT# ?N  NAME ?B

7    BALANCE D@ (.$) RIGHT ;

8

9 [R Account Balances\   Account# Name          Balance]

10 : balances   LITERAL LAYOUT

11    ACCOUNTS RECORDS DO  I READ  .ACCOUNT  LOOP ;

12

13 : enter ( n d)   ACCOUNTS  SLOT READ  BALANCE D!

14    ACCT# N!  NAME PUT ;

15  ( Example:  456 100.00 enter John Doe <RETURN> )

The word BALANCES produces:

FORTH, Inc.	Page 1  26 AUG 1986

Account Balances

Account#	Name	Balance

456	John Doe	100.00

489	Mary Smith	2970.00

620	Ed Poore	2.59

Notice that the first column heading, “Account#” appears in the title/ column-heading pair three spaces after the backslash. This causes the heading on the output report to be indented three spaces (the first space after the backslash counts).  On the corresponding formatted lines, the first field is formatted with ?N, which right-justifies the string against the end of the “Account#” heading.

The middle column is formatted with ?B; as a text string this field is effectively left-justified.  To make the output more pleasing, we have forced the “Name” column heading to be flush left to match.  The use of RIGHT in ?B causes the column table to be adjusted the first time a row is displayed (see Section 8.8.6).

In the third column, the data is once again right-justified under the last character of the “Balance” column heading.  In this case, we wished to display the double-length field in dollars-and-cents format, requiring the use of a pictured numeric output routine (Lines 4 and 5 of the listing).  On Line 6, this pictured numeric output string is displayed, but with RIGHT rather than TYPE.

If the previous column displayed was the final column on a line, RIGHT automatically advances to the next line and resets the column table to begin with the first column on the line.

The following words are available for special formatting requirements:

Word	Stack	Action

0COL	(  )	Resets the column table pointer to point to the first column width.  Exercise care with this word, since it can cause the output to be misaligned if it is not issued when the actual output print position is at the beginning of a line.

COLS	( -- n)	Advances the column pointer and returns the width of the new column.

Word	Stack	Action

SKIP	(  )	Skips one column.

SKIPS	( n)	Skips n columns.

### 8.8.3	Controlling Paging

The report generator counts each output line; when no available lines remain, it forces a page heading cycle by placing standard titling information at the top of each page.

The maximum number of lines which may be placed on a page is stored in the user variable L/P (lines/page).  For the printer task, its value is normally sixty-six, which corresponds to the standard page size of 8-1/2" x 11".  In areas of the world where the standard page is not eleven inches, this value must be set appropriately.

For terminal task, the value of L/P is usually twenty-four.

To change this value for a terminal task, simply store the new value into L/P:

60 L/P !

To reset the value for the printer task, you may either place a similar phrase inside a definition that is executed by the printer task (as in the definition of a report that you are printing), or with the following phrase (see Section 4.6):

60 TYPIST L/P HIS !

When using the Report Generator, it is not necessary to explicitly invoke a “new-line” function at the beginning of each row of data.  As the field-display operators cycle through the columns table, after the last column has been displayed, the next operator resets the column pointer to the beginning of the column table again and issues a “new-line.”

The following words control pagination:

Word	Stack	Action

+PAGE		Starts a new page, incrementing the page count in P# and displaying the headings for the new page.

Word	Stack	Action

?PAGE	( n)	Starts a new page if there are fewer than n spaces remaining; otherwise issues a “new-line.”

+L		Issues a CR and increments the line count in L#.  Also resets the column pointers using 0COL.

+CR		Checks whether the page is full (using 1 ?PAGE), then issues a +L.

You can ensure that a set of related lines will appear together on the same page, by using ?PAGE before each set to guarantee that they will all fit.  For example, if the report consists of groups of three lines that should always appear together, issue the following phrase immediately before displaying each three-line unit:

3 ?PAGE

Within the three lines, you would use +L rather than +CR to avoid a possible page break within the group.

You can unconditionally force a new page by providing an argument that appears to be greater than the largest possible number of lines per page:

-1 ?PAGE

If you wish to suppress automatic paging totally (for example, in cases where the output is directed to a terminal that is not a printer, where page headings are not desired), you may use the word +L in place of +CR.  The words are the same except that +L does not check for page full.

Consider the code example in Section 8.8.  Here +L is used at the beginning of the report, but +CR is used after each line inside the loop, to enable automatic pagination.  Neither +L nor +CR is inside .person, because this definition (which simply displays the current record) may be used for a single-record terminal inquiry, as well as the many-record report all.  The terminal inquiry might look like this:

: show ( n)   PEOPLE READ  .person +L ;

Here +L is preferred because we do not want to paginate every few inquiries!

REFERENCES

0COL, Section 8.8.3

### 8.8.4	The Page Banner

At the top of each page of the report appears the “page banner” which includes:

<optional text>	Page nn  <date>

where nn is the current page number, and <date> is the current system date.

If you wish to modify or eliminate the optional text, simply change the ." string in the definition of +PAGE (Block 133).

It is possible to eliminate the page banner entirely by replacing the word LAYOUT with HEADING.  Like LAYOUT, HEADING takes as an argument the address of a title/column heading table as provided by the word [R, and establishes this table as current.  It then displays the “title” line, without attempting to center it, and on the next line displays the column headings.

HEADING	( a)	Saves the address of a title/heading table, and outputs the title and column headings.

### 8.8.5	How the Columns Table Works

The format of the columns table is:

Byte	Contents

address + 0, 2	Disk location of the title/heading pair (character position and block number).

+ 4	Address of the page heading vectored routine.

+ 6	First column width.

8 et seq.	Subsequent column widths.

A zero entry in the table indicates the end.

A heading line can contain up to 128 characters.  These lines are used to establish a table of column widths at load time in the following manner.

Starting from the backslash in the title/column-heading pair, [R scans forward using 32 WORD in a loop.  Each time it encounters a heading, it continues to scan to the first blank character following it, computes the difference from the starting point or previous heading (the width of the field), and compiles this into the table.  This loop repeats until the ] delimiter is encountered.  At this point, the indicator for the end of the line (a column width of zero) is inserted and the scan is complete.

For example, suppose the following is the set-up string for a set of column headings (the numbers across the top are your guide to indicate column positions):

0         1         2         3         4

01234567890123456789012345678901234567890123456789

\   Account# Name      Balance]

The first blank after the “Account#” heading occurs at relative position 12; thus the number 12 is compiled into the table as the width of the first column.  The first blank after the “Name” heading occurs at relative position 17; the difference, 5, is compiled as the width of the second column. The delimiting ] occurs at 34, and the difference of 16 is compiled as the width of the third column.  Finally, a zero is compiled to indicate the end of the table.

The finished column table, as constructed by [R, contains:

12  5  16  0

The total width of all columns equals the position number of the last non-blank character.

Notice that some hidden “sleight-of-code” is taking place with the middle column.  In our BALANCES report, the full 10 characters of the name field appear.  What happens is that the first time BALANCES invokes ?B, the word RIGHT (which is invoked by ?B) determines that a width of five is not sufficient for the 10-character BYTES field plus the single space between columns.  Accordingly, RIGHT adds six to the five, to achieve the necessary eleven spaces needed. Finally, to keep the rest of the columns lined up, RIGHT subtracts six from the next column (“Balance”).

Thus, by the time the first row of data has been displayed, the column table contains these entries:

12  11  10  0

If you crowd the headings too close together in the title/column-heading pair, RIGHT will “steal” so many spaces from the next column that it will contain zero.  If this happens, the column table will appear to be terminated at that point, and there will be more output operations on each line than there are columns in the table.  The result will be a skewed output.

A line may contain as many columns as required for the output format.  Due to the method of establishing columns, the minimum width of a column is two characters.

### 8.8.6	Non-standard Report Headings

By default, the “new-page function” performs the following steps at the beginning of each page, including the first page:

1.	Displays the page banner as described in Section 8.8.5;

2.	Performs a +L and skips 20 spaces (to approximately center the title);

3.	Executes a word called TITLE.  TITLE is defined as:

: TITLE   RPT @ HEADING ;

RPT is the user variable that points to the current title/column heading table.  HEADING displays the title and column-heading lines from the given table (Section 8.8.5).

However, the Report Generator lets you vector the third function above.  This feature lets you execute your own definition instead of, or in addition to, TITLE.  For instance, you might add other lines of information below the page banner.

This vectoring is possible without recompiling the FILES utility because the third cell of the title/column-heading table contains the address of the routine to be executed at the top of each page.  When [R generates this table, it copies in the address of the routine TITLE by default.  By re-setting this address to point to your own definition, you can change the output of the new-page function.

Here is an example:

0 ( A Non-standard Report Heading)
 1 VARIABLE WHICH

2 [R   \Col1   Col2   Col3]  DUP  4 +

3 : 'ITEM'   LITERAL ASSIGN  ." Report on Item No. "

4    WHICH ?  +CR  TITLE ;

5 : SHOW ( n)   WHICH !  'ITEM'  LITERAL LAYOUT ;

This example shows a report for some particular item that is selected numerically, like this:

2500 SHOW	Stores 2500 into WHICH so that you can see a report for Item 2500.

The report generator will print the item number at the top of each page, with headings:

Acme Manufacturing Co.	Page 1  26 AUG 1986

Report on Item No. 2500

Col1     Col2     Col3

where the top line is the standard page banner, and the text “Report on Item No.___” is formatted by user-defined code.

Here are the steps used in the above example to vector the user-defined code into the new-page routine:

1.	Create a title/column heading pair as usual (in this case, we have left the “title” blank).

2.	Following the creation of the title/column heading pair, invoke the phrase DUP 4+.  This computes the address of the execution vector in the table, in addition to the starting address of the table.

3.	Define a word that vectors this address to the code you wish to execute.  In the definition of 'ITEM' above, the word LITERAL makes the vectored-execution address part of the definition.  (LITERAL consumes the “address 4 +” during compilation.)  ASSIGN causes that address to point to the code following ASSIGN itself (Section 2.3.8.2).

4.	The rest of the definition is the code that will be executed as the third step of the new-page routine.  It includes the message “Report on Item No.,” followed by the display of the chosen item number.  It then invokes +CR to move to the beginning of the next line.  Finally it invokes TITLE, which displays the title/column-heading pair.  In this case, we have specified a blank “title,” so the title line is blank.

5.	Define the report-generating word (the word SHOW) in the usual way, using the address remaining on the stack (the beginning of the table) as the argument to this second occurrence of LITERAL.

Having once executed 'ITEM' in the above code, the user-defined routine is now a permanent part of the SHOW report.  However, other reports may be co-resident; since each has its own title/ column-heading table, each has its own new-page execution behavior.

### 8.8.7	Totals and Subtotals

The Data Base Support option includes a simple utility for computing subtotals and totals of numeric fields as the report is being displayed.  In general, the following steps must be followed:

1.	Allot enough “working storage” for the registers. Working storage is created by invoking n ALLOT immediately after the word EMPTY at the beginning of the FILES load block (Section 8.6).  The value of n is calculated by this Forth phrase:

( # of registers needed) 8 *  4 +  16 +

See the shadow block associated with the word REGISTER on your system.

2.	At the beginning of your report word, simultaneously define and clear as many accumulator-pairs as there are fields you wish to total, using the word ZERO (see below).

3.	As the fields are being displayed, accumulate the values in the subtotal registers by using either SUM or FOOT.

4.	When you wish to display the subtotals (if at all), use the word SUB, followed by an appropriate numeric output command.

5.	When you wish to display the grand totals, invoke GRAND.  This copies the grand totals to the subtotals registers. Then use SUB as in Step 3.

Here are the relevant words in detail:

Word	Stack	Description

ZERO	( n)	Defines n subtotal accumulators, and n grand-total accumulators, and sets all to zero.  Each accumulator is double-length.

For example, if you are totaling three fields, the phrase:

3 ZERO

creates three subtotal accumulators and three grand-total accumulators, and sets all to zero.  ZERO must be used at the beginning of a report if any of the following words are used.

SUM	( d n)	Adds d to the subtotal accumulator for the nth relative field.

FOOT	( d - d)	Advances to the next subtotal register and adds d to it.  If at the last register, wraps around to the first.

For instance, suppose you have a DOUBLE field called SALARY that you want to both display and add to the running total.  The phrase:

SALARY D@ FOOT .D

fetches the contents, adds it to the corresponding subtotal register, then displays it.

SUB	( -- d)	Advances to the next subtotal register and fetches its contents.  Also adds the contents into the corresponding grand-total accumulator and clears the subtotal register.  If at the last register, wraps around to the first.

GRAND	(  )	Copies the grand totals to the subtotal accumulators.  For example, the phrase:

SUB .D

will display the subtotal of the next field.  The phrase:

GRAND  SUB .D

will display the grand total of the next field.

The following example shows how subtotals and grand totals can be easily computed and displayed:

Wine Inventory by Store

Location	Chablis	Rose	Champagne

Northern California

Palo Alto	25	42	78

San Jose	16	32	50

Mill Valley	31	29	36

San Francisco	70	59	82

142	162	246

Southern California

Chatsworth	35	48	29

Woodland Hills	32	40	60

67	88	89

Grand Total:	209	250	335

Here is the code that produced this display:

0 ( Totals and Subtotals)   FORGET TASK   : TASK ;

1 0  16 BYTES Location   NUMERIC Chablis   NUMERIC Rose

2     NUMERIC Champagne   DROP

3

4 : .amounts   Location ?B  Chablis N@ 0 FOOT .D

5    Rose N@ 0 FOOT .D  Champagne N@ 0 FOOT .D

6 : .subs   SUB .D  SUB .D  SUB .D  +CR ;

7 [R Wine Inventory by Store\Location         Chablis

8 Rose  Champagne]

9 : INVENTORY   LITERAL LAYOUT  3 ZERO  +CR

10    ." Northern California"  +CR

11    WINES RECORDS DO  I READ  .amounts  I 4 = IF  +CR

12      SKIP .subs +CR  ." Southern California " +CR

13      THEN LOOP  +CR

14    SKIP  .subs  ." Grand Total:     " COLS DROP

15    GRAND .subs ;

The phrase 3 ZERO appears in the definition of INVENTORY on Line 9.  This creates and clears three sets of accumulators, one set for each field we wish to total.

The word FOOT appears in the definition of .amounts on Line 4:

Chablis N@ 0 FOOT .D

In this case, the field is NUMERIC (single-length), so we fetch it with the operator N@.  FOOT, however, expects a double-length number; the zero preceding it supplies the high-order part.  FOOT will add the value to the first subtotal accumulator.  FOOT also returns a copy of the value (as a double-length number).  Finally .D displays the value in Report Generator format.

The second invocation of FOOT in:

Rose N@ 0 FOOT .D

will cause the value of the Rose field to be added to the second accumulator, and so on.

The word SUB appears in the definition of .subs on Line 6.  This definition displays the contents of the three subtotal accumulators in turn.  Notice that the three uses of .D correspond to the second, third, and fourth columns in the report generator; thus we can only invoke .subs when we are about to display the second column (after having output or SKIPped the first column).

In INVENTORY, we display the standard header with LAYOUT, below which we display the category heading “Northern California.”

Inside the loop we display the fields in the usual way, except that we check to see if the index is 4.  If so, then it is time to display the subtotals for Northern California and the category heading for Southern California; the code for this appears on Line 12.  Here we SKIP the first Report Generator column, then issue .subs.

After the loop has been completed and the second set of records displayed, the phrase:

SKIP .subs

(Line 14) displays the subtotals for Southern California, and issues a +CR.

Finally we display the text “Grand Total.”  The trick here is that we also want to display the grand totals on the same line.  We cannot use SKIP, because it outputs the necessary number of spaces to get to the next report column; after printing the text, we’re half the way there already.  Our solution is to pad the message with trailing blanks so that the message is 17 characters long (the width of the first field plus one); this leaves us in position to display the second column.

However, the columns table must also be advanced to point to the second column.  The phrase:

COLS DROP

is the same as SKIP without issuing the spaces.

Finally, GRAND copies the grand-total accumulators to the subtotal registers, and .subs displays these.

## 8.9	Data Base Design

Before building a house, it is best to have a blueprint.  So too, before defining files and records, it is best to map-out the overall data base needs.

In general, we can formulate two simple rules for planning your data base:

1.	Look at the kinds of information you have.

2.	Arrange like kinds of information into files.

### 8.9.1	A Hospital Patient Management Data Base

Our goal in this example is to create a data base for tracking patients in a large hospital.  For each patient there is a set of information: items such as address, height, weight, date-of-birth, and so on.  (Note that we save date-of-birth and not age.  We can always compute age if that’s what we need in a report, but a date-of-birth is never obsolete.)

Clearly, this information all belongs in a single record, one per patient.  However, there is also a variable number of information items that may be associated with each patient. For instance, each patient may have a different number of tests, and each type of test may have a different amount of information that it produces.  In short, the amount of information that we need to keep for each patient is variable in length.

At this juncture, many data base designers would opt for variable-length records and fields.  But variable-length records are complex and slow, as we saw in Section 8.1.  With nearly the same convenience we can achieve the same results by using a fixed header plus a variable number of subordinate records.

Fig.  8.13

Patient records chained to test results.

Fig. 8.14

Three “test results” chains for each patient.

This is where chains come in.  Fig. 8.13 shows that a field in each PATIENTS record can point to the first in a series of RESULTS records, each of which is chained to the next.  We achieve the same effect, but at much less expense.

Now suppose that we need to record particular test results for three different tests for each patient.  We can accomplish this by providing three fields in each PATIENTS record, each pointing to a different result record or chain of records (Fig. 8.14).

Fig. 8.15

Patient numbers directly index a record in (PATIENTS), which contains a link to the main PATIENTS file.

Here’s another intriguing problem.  The application demands that a patient record can be found on the basis of a “patient number.”  A patient number is a very large number issued in sequence; in other words, the patient number last issued reflects the total number of patients that have ever been admitted to the hospital in its history.  This number could reach 200,000 during the lifetime of our system.  However, the department for which we are designing this application expects to see only 30,000 patients during the lifetime of this system.  Unfortunately, maintaining an ordered index even of 30,000 records, indexed on “patient numbers,” is unmanageable.

Is there any way we can translate a patient number directly into a record number for our PATIENTS file?  Let’s try this:  we’ll create a file of 200,000 records, each record being only two bytes long.  This gives us one record per potential patient number.  The two-byte field will contain a record number, pointing to the record in the PATIENTS file corresponding to the patient number (Fig. 8.15).

This elegant scheme requires 400 blocks for the look-up file, and yet gives immediate access to a patient record, with only one intermediate disk access.  No searching is needed.  Furthermore, when new patients are added, SLOT is not needed in the look-up file.

In general, direct access is much faster than searching, and should be used whenever appropriate.

### 8.9.2	An Integrated Business System

Our goal in this example is to create a package that will track income (sales and accounts receivable) and expenses (purchase orders, accounts payable, and payroll), and from these inputs will produce general ledgers, income statements and balance sheets.

Although many commercial business packages treat these functions as separate programs, our goal is to integrate them into a single system.  By doing so, we will make the system simpler to use and reduce the opportunity for error.  For instance, when a sales order is entered, the order should be forwarded to the accounts receivable component, and the sale automatically posted to the general ledger without further manual entry.

How shall we organize our data base?  Let’s begin by identifying the entities and operations that are part of our business, and the reports that we wish to obtain:

Entities:	Operations:	Reports:

customers	sales:	general ledger:

vendors	  order entry	  income statement

employees	  accounts receivable	  balance sheet

payments received

purchases:

purchase orders

accounts payable

checks written

payroll:

Looking first at the left column, clearly we will need to store information about the entities in a file structure.  The question to ask is, “What do we need to know about these entities?”  It turns out that for each of our three types of entity, the answer is remarkably similar.  In each case we need to know:

name
address (street, city, state, zip)
phone number

This observation suggestions the possibility of using shared code, an opportunity for program simplification.  At the very least, this means we can use the same field definition names (NAME, STREET, etc.) for three different files.

In fact, though, we never have more than several hundred people and companies that we do business with in any year.  As a result, we can mix all people and companies in single file, called PEOPLE, and add an extra field called KIND to indicate whether the entity is a customer, vendor, or employee.

This reduces the number of files for “entities” from three to one, and simplifies the program accordingly.

Because we will need to search and order this file on an alphabetical basis, we must also create an index file, called (PEOPLE).  This index will contain simply a link field—to point to the corresponding record in PEOPLE—and a “nickname” field, which contains the name in a form that we want it alphabetized by.

We can establish the following rules for entry of the “nickname” field:

for human beings:  last name, first name, initial

for companies (customers or vendors):  company name
	(sometimes somewhat abbreviated)

As for the additional fields that employees need, we find it simplest to create an additional file called AUXILIARY.  Each employee record contains a pointer to a record in AUXILIARY.

Now let’s turn to the operations.  Each operation results in a transaction that must be saved.  These transactions will become records in a file of events.  What do we need to know about these events?  In the case of a sale, we have:

customer
date of sale
amount 
check number

In the case of a purchase order, we have:

vendor
date of order
amount
purchase order number

In the case of payroll, we have:

employee
date of paycheck
amount
check number
commissions (for commissioned salespeople)
tax contributions, etc.

Once again, it appears that many fields exist in common.  With the exception of the extra information needed for payroll, we can summarize the above requirements as:

WHO
WHEN
AMOUNT
NO.

We decide to keep all events in a single file.  We will call this file DETAIL.  Besides the fields described above, we will add a field called KIND to indicate whether the event is a sale, an order, etc.

When we organize our data needs in this way, we see that entities and events can be organized together for simplicity.  With this understanding, it will be easier to integrate the entire system.

Let’s look at the WHO field.  What should it contain?  Perhaps the name of the person or company.

On the other hand, we know there will be many more DETAIL records than anything else, so we want to make each record as small as possible.  Were we to keep a name field in the DETAIL file, it would take up considerable space and require that we look up the name in an index in order to get the address or other information on the name.

Instead, we will keep the record number of the related person or company in the WHO field.  This occupies only two bytes, and requires no searches.

Now let’s study some of the operations we’ll want to perform.  Suppose it is the end of the month and time to write checks.  This is easy.  We simply look through the DETAIL file looking for accounts payable entries that are due now.  From the record in DETAIL we can follow the pointer into PEOPLE to get the name and address of the payee.

Let’s take another example.  We want to be able to determine the current balance owed by a particular customer or to a vendor.  But we have not included a “Balance” field in the PEOPLE records. All we have to do is let each PEOPLE record point to the most recent transaction, then let each transaction record point to the next-most-recent transaction, etc.  Here we are using chains.

ACME Widgets, Inc.	Page 1  31 OCT 1986

So. Bay Office Supply Account Status

#	Job	Ref	Due	DR#	CR#	Amount	Paid	Balance
	3344		1086	29 NOV	5220	2100	189.24	0.00	189.24
	3343	47	626	29 NOV	1210	2100	10.74	0.00	10.74
	3205		2270	10 OCT	2100	1030	779.74	779.74	0.00
	2773	61	930	30 SEP	1210	2100	59.04	59.04	0.00

Fig. 8.16

Portion of a report showing a vendor account.  The first column shows the number by which each detail item is referenced; it is actually the number of the record in the DETAIL file.  The report title is a ‘custom’ one, showing the subject account.  Custom report titles are described in Section 8.8.7.

Chaining is appropriate in cases such as this, in which there is no way to predict how many elements there will be, and it makes it easy to generate reports of activity for a vendor such as the one in Fig. 8.16.

There are at least three ways that chaining can be done:

1.	Chaining from most recently entered transaction to least recent.

2.	Chaining from least recent transaction to most recent.

3.	Chaining by something other than order of entry, such as date field, etc.

In this case, we prefer to list transactions starting with the most recent events.  This makes possible reports such as shown in Fig. 8.16.  As we saw in an Section 8.7, the polyFORTH Data Base Support option includes a block of chain manipulation words that you can customize for your particular application.

So far we have a PEOPLE file and a DETAIL file.  Now let us look at our desired reports.

The general ledger is produced monthly, organized by account.  Under each account are itemized all transactions, both credits and debits involving that account during the month.  In the balance sheet (Fig. 8.17), we show year-to-date summaries for each account.

ACME Widgets, Inc.	Page 1  31 OCT 1986

Balance Sheet

CURRENT ASSETS
	CASH
	Continental Bank	24,165
	Amalgamated Bank	104,965
	Short Term Investments	248,000
	Petty Cash	5,000	382,130

Fig. 8.17

Portion of a Balance Sheet report.

The traditional data base approach to General Ledger might involve running, once each day, some program that looks through the latest events and posts them to another file containing the general ledger data.  To produce the general ledger at month’s end, this approach would require sorting the transactions file by accounts.

But daily posting not only requires creating another file, it also involves an extra step for the bookkeeper.  And sorting would take longer because it requires handling each record several times.  As we’ve mentioned, the polyFORTH Data Base Support option offers many techniques that reduce the need to sort.

Still, each transaction record must indicate the account it affects.  When we produce the general ledger and income statements, we will need to display accounts by name, e.g. “Advertising.”  It doesn’t make sense to place an account name in each transaction record.  Following our dictum, “Arrange like kinds of information into files,” we create a new file called ACCOUNTS, containing the name of each account.  Now our transaction records can simply use a record number to point to an account record.

Actually, each transaction affects two accounts, one as a debit and one as a credit.  So, each transaction really needs two pointers.  We will call one DR# (debit record number) and the other CR# (credit record number).  As each transaction is made, we know what accounts are involved.  For example, when we enter an invoice, the amount will be credited as a sale and debited as an account receivable.

Fig. 8.18

Chains can be used to link transactions affecting each general ledger account.

Since each transaction points to a pair of account records, when displaying a transaction we can also display the account names.

What about a general ledger file?  Upon further analysis, we realize that numbers don’t need to be transferred to another file, when they are in one file already.  It will be easier to run a program once a month that computes the account balances.

To run the general ledger, we need to start with each account, then look at the transactions that affected that account to produce an account summary.  We could loop through the ACCOUNTS file, and for each account record, step through each transaction that affected the current account during the month.  This approach could also use chains.  Each ACCOUNTS record would point to the most recent transaction affecting it, and each transaction would point to the previous transaction affecting the same account (Fig. 8.18).

However, chaining is somewhat complicated, and better avoided whenever possible.  Rather than chaining from ACCOUNTS to DETAIL, we can simply loop through our DETAIL file for this month.  Each DETAIL record points to a pair of ACCOUNTS records.  For each transaction, we can add the amount to an accumulator for a credit account, and subtract the amount from an accumulator for a debit account.  In this way, we can tally all our account totals by looping through the DETAIL file only once.

But where do we keep these accumulators?  Since we need one and only one for every account, it makes sense to add a field called BALANCES to our ACCOUNTS records.

Is this idea really better than following chains? By following chains from ACCOUNTS to DETAIL, we would have to handle each transaction record twice:  once while following a credit-account chain, and once for a debit-account chain. By keeping balances, we can loop through our transactions only once.

By using a one-pass posting algorithm with no chaining, we improve performance a great deal by avoiding sorting, and by about a factor of two by not using chains.

Our ACCOUNTS file can use some embellishments.  In addition to the two fields it already has:

Account No.
Balance

we can add HISTORY, which is an array of balances for the past 12 months.

In addition, the ACCOUNTS file needs an index, which we will call (ACCOUNTS).  At first it would appear that we could use the account numbers themselves to sort the accounts when preparing the balancing statement.  In fact, however, accountants prefer to subclassify accounts into groups for their own reasons.  For instance, taxes are an expense account, but they are usually listed at the end of the list of expense accounts.  For this reason, the (ACCOUNTS) file is numbered according to the order in which we want accounts to appear on the balance sheet.

Our next step is to write words that reflect the kinds of high-level actions the bookkeepers want to record.  Let’s start with the operation of placing an order.  How must this order affect our data base?  What do we need to know?

Clearly we are going to create a new DETAIL record.  This record will include a WHO field to indicate the company from which we are ordering.  Since we have our vendors in the PEOPLE ordered index file, we need supply only the name of the company.  The program can then look up the company, find the record number and place it in the WHO field of the new transaction record.  The program must also link this new transaction into the chain for that vendor.

We also need to supply the amount of the purchase, and our purchase-order number.

The program itself can place the current system date into the WHEN field, and by default, place the date 30 days hence into the DUE field.  Since this is an order, the program must place the code for a purchase in the KIND field.

So what should our “program” for entering an order look like to the bookkeeper?  We know the bookkeeper must supply:

1.	The amount.

2.	The purchase-order number.

3.	The name of the vendor.

The simplest, most Forth-like solution is to call the word BOUGHT, precede it with the two numeric data items and follow it with the string data.  This gives us the syntax:

200.00 5134 BOUGHT ACME

We can now take a similar approach with a program to record a sale:

3998.00 7409 SOLD CROFT

The word SOLD is preceded by amount and their purchase-order number, and followed by the name of the customer.

We can record the receipt of a check with the word FROM:

amount check# line# FROM Conway

In the above, line# is a number that identifies the sale for which this is a payment received.  The bookkeeper finds this number on a report of outstanding balances (see Fig. 8.16).  While this is simple for the bookkeeper, it is also simple for the program because line# just happens to be the record number of the DETAIL record showing the sale.

The same syntax can be used for writing a check:

amount check# line# TO ROSS

Thus, each “program” is simply a Forth word.  This approach allows our application to use the Forth interpreter.  The problem of how the bookkeeper selects a given operation is effectively eliminated.

To appreciate the significance of this, consider the typical alternative.  Most business applications are menu-based.  From the main menu, the bookkeeper might select Accounts Payable. Then, from the Accounts Payable menu the bookkeeper might choose Purchase.  From there, an entry form might appear, wherein the bookkeeper can select or enter the customer, then fill in the data.

While popular, this menu-based approach can be more laborious for the user.  To avoid the switching application modes, the bookkeeper may separate all the purchases from the sales, etc., and do each group one at a time.  This requires more paper shuffling.

Our approach, with no hierarchy, lets the user enter various transactions in any order, leading to a more pleasant, efficient working environment.  A “help screen” can display the syntax of the commands on request during the learning curve.

In retrospect, we seem to have designed the data base very efficiently.  The file with the most records, DETAIL, also has the smallest records.  Each record in DETAIL is only 16 bytes long, and contains no text at all.  (This means that 64 such records will fit in a block.)

### 8.9.3	A Facility Management System

In this example we will see how to organize and simplify a massive data problem by studying the data and looking for a natural hierarchy.

The example involves the problem of controlling digital and analog input/ output with a distributed computer system, where there are several thousand I/O points in dozens of buildings and other locations at a large industrial plant.

Digital “points” include switches, buttons, pressure-sensitive floor plates, pulses to unlock doors, and so on.  Analog points include thermocouples, meters on control panels, heating levels, lighting levels, and so on.  Our task is to install a distributed computer system to control all these points.

We begin by studying the points as the architects and engineers designated them.  The ID for an individual point has the form:

ABC-123-1234

Experience has taught us that numbers such as this are usually encoded, and that usually the coding scheme presents a goldmine of information on how to organize the system.  Upon further investigation we discover these relationships:

ABC     -     123     -     1234

a facility	a control panel	a point number on a
control panel in a facility.

This information provides the key for our establishing a hierarchical data base, a necessary strategy when dealing with thousands of anything.  Another example in which coded numbers can reveal hierarchy is with inventory or parts numbers.

Fig. 8.16

Diagram of the data base for the Facility Management example.

One of the things we must do is allow the control of individual points from the central computer.  The operator can enter a point ID and indicate some action.  So one of our problems is to associate a point ID with a physical hardware location.  One approach would be to have a points index contain-ing the ID of each point in the system.  The nature of the coding scheme makes it a candidate for an ordered index, but with over 20,000 records even a binary search is more cumbersome than we can afford.  The logical hierarchy will help deal with the volume, and thus the performance.  It will also improve the information content; for example, in reports we can make use of the implicit meaning of the data.

The first part of the code is a “facility:”  A physical building or location on the plant, like the parking garage, the fire station, etc.  A “panel” may be a manually-attended console, a switchbox in a closet, or it may be imaginary, as in the case of points attacked directly to the computer.

A single panel may control many points.  And a computer may control many points in many buildings (for instance, the fire station computer needs fire alarms in all buildings).  But what is the relationship between computers, facilities, panels and points?

Further digging uncovers fact that a panel is attached to only one computer.  This means that each computer can be responsible for its own private data base of panels.  Each panel can be chained to any number of points.  The master computer can have a file of facilities.  Each facility can be chained to any number of panels.

We can now identify four files:

FACILITIES
COMPUTERS
PANELS
POINTS

We have solved the problem of chaining from computers down to points.  Now let’s consider the reverse problem.  Inputs generate “events.”  An event has to be dealt with quickly; for instance, the event may be an alarm.  If a fire alarm is triggered, a computer will need to display information about the point, such as which building it is in.  In other words, we must have linkage from point to panel to building.

The first step of this linkage is the association between an electrical event within the computer and the corresponding point in the POINTS file.  For instance, the pressing of a button might cause execution of an interrupt routine.  This routine must be able to determine which point caused the interrupt.

At first it may seem logical to keep a table that associates hardware addresses with point names.  But this would require an extra search.  It is more direct to create a table that associates points’ hardware addresses with record numbers within the POINTS file.  Another benefit is that the point record number only requires two bytes, so the table is small.

With this scheme, an electrical event is associated with a record in POINTS, which in turn contains the information we need to know about the point, including its code name. Because the code name contains meaningful information, we can now determine which building the point is in.

We can now rest assured that we have found a good solution to the problem, since we have achieved good performance while at the same time reducing complexity.  This sort of win/win situation provides the positive feedback that tells us we’re on the right track as we iterate through our design.

### 8.9.4	A Filing Scheme for Image Processing Applications

Our final example illustrates the flexibility the Data Base Support option provides—including the freedom to not use some of its features when the application dictates otherwise!

Conceptually, a filed image has two elements, a header and the pixel data.  The header indicates what the image is, when it was recorded, who made the image, the dimensions of the image (in pixels), and so on.

There are a variety of ways to index into images. But the real problem is managing the pixel data.  Image processing is a prime example of an application in which speed is critical, because there is simply so much pixel data to handle. An array of 512x512 points contains 262,144 pixels, which at 8 bytes per pixel occupies 256 blocks.  Just reading this many blocks will take some time.

Now imagine trying to access these pixels one at a time using 1@ (or N@).  This approach involves the invocation of BLOCK plus the record and field accessing computations for each and every pixel.  This will be unacceptably slow.

An approach that has proven effective is an interesting hybrid of the Data Base Support option tools, plus ordinary direct disk-access techniques.  In this approach, we use the Data Base Support commands for header information, but we keep the pixel data elsewhere on the disk.  In other words, we reserve three regions on the disk:  a file for headers, another for an index to our headers, and a region of blocks that are not files for pixel data.  Within the header, a field points to the block number where the pixel data begins for that image.  Another field indicates how many blocks are used.

We also recommend keeping the data in the form used by the image processing device (usually binary integers).  You may want to process an image using floating point (although in the absence of a hardware floating point processor the fixed-point routines supplied with polyFORTH will be much faster).  But a 64-bit floating point number is eight bytes long, which means an image will require eight times as many blocks and take eight times as long to read and write off the disk.  It is faster to float the numbers after fetching them.

Some users believe that saving pixel data in floating point form retains better resolution.  In fact, however, the typical A/D converter on a Vidicon camera (for instance) does not possess many bits of resolution. In industrial vision applications, these devices rarely provide more than one byte of precision.  The extra bits that floating point provides simply represent noise.

On the other hand, some applications do utilize greater precision, but store a much smaller number of pixels.  In astronomy, for example, an image size may be only 64x64. But the image might be recorded with a highly sensitive detector over a four-hour period with atmospheric correction. Thus, each pixel has already been integrated and may contain as 16 bits or more of information.

## 8.10	DOCUMENTOR Utility

The DOCUMENTOR is a Forth utility that allows maintenance of a file that contains descriptions of Forth words.  This provides a convenient way to document polyFORTH programs.  It is also an excellent example of the use of all data base management features.

Each word defined in the glossary has the following information associated with it:

1.	The block in which the word is defined.

2.	The glossary vocabulary.

3.	Stack usage.

4.	One or more lines of text that describe the word and its use.

The DOCUMENTOR utility provides commands to maintain this file and to print reports that include either selected glossary vocabularies or the complete file of words.

### 8.10.1	File Structure

The DOCUMENTOR uses one or more glossary files that are specified by the user.  Each glossary file is physically composed of two separate files; a data file and an index file.  Index file support in the polyFORTH system is required to implement the documentor.

You must preallocate the two polyFORTH files required for a glossary.  The data file is composed of 64-byte records, several of which may be chained together to provide multiple lines of text, 64 characters per line.  It is named GLOSSARY.

The index file used for the glossary is compiled of 26-byte records, with a 24-byte key length comprised of the word-name (12 bytes) and a vocabulary-name (12 bytes). One index record is required for each glossary entry. The name of the index file is (GLOSSARY).  It is an ordered index, ordered by word-name and vocabulary; this has the effect of maintaining the glossary in alphabetic order.

Here is a sample definition for a glossary file that contains 450 entries.

( BYTES	RECORDS	BLOCKS	ORG	NAME)

26	 450	 12	160	FILE (GLOSSARY)

64	2300	144	172	FILE GLOSSARY

### 8.10.2	Loading Instructions

The DOCUMENTOR requires a 1000-byte partition for execution.  It is loaded with the following command:

DOCUMENTOR LOAD

The DOCUMENTOR will empty the user’s partition, replacing any other overlay.

The procedure for entering a word into a DOCUMENTOR glossary consists of making the block number and glossary vocabulary current, entering the stack usage and the word-name, and then entering associated text.  You can change block number, stack usage, and text lines easily.  The following sections are interdependent; reading through them at one sitting will provide a helpful overview.

### 8.10.3	Source Block Identification

When you begin to document your application, you will usually specify a source block to be documented and then enter all the words that are defined in that block.

To specify a source block, use the following phrase:

blk# SOURCE !

Until changed by re-use of the phrase above, this current block number will automatically be stored with each succeeding word entry.

REFERENCES

Entry Changes, Section 8.10.8

### 8.10.4	Glossary Vocabulary Identification

Along with each word, the system stores the name of the entry’s application vocabulary.  This usually means the name of the portion of the application in which the word is used, such as the name of its load block.  These vocabularies are not necessarily the same as program vocabularies.  Glossary vocabularies exist only for logical grouping of words and to enable the same word to be variously defined several times in different blocks.

Before you begin entering words for a new glossary vocabulary, make it the current vocabulary by typing:

VOCAB vocabulary-name

Note that the name cannot be longer than ten characters.  Until changed, this name is kept in memory and copied into each succeeding data record entered.

In order to search for a previously entered word, you must make its vocabulary the current one.

The glossary vocabulary name serves as a secondary key for searches.  This means that the same word may be entered in numerous vocabularies, with each entry unique.

The vocabulary is also set by the report command /VOCABULARY.

REFERENCES

/VOCABULARY, Section 8.10.7
Finding Previously Entered Words, Section 8.10.8

### 8.10.5	Glossary Entries

Words are entered into the glossary through the ENTER command.  This command sets the basic entry into the file.  It has the following format:

ENTER word-name  (e.g., ENTER NAME)

The program will prompt you for brief (16-character) descriptions of stack entries before and after execution.  Any valid Forth word name may be used; the maximum length recognized by the DOCUMENTOR is twelve characters.  If a longer word name is entered, its length will be truncated to twelve characters.

Following ENTER, the new word is made the current word, with which will be stored the current block number, current glossary vocabulary name, and up to four lines of associated text.  Immediate subsequent use of AT, STACKS, T, U, or P will affect this entry.

REFERENCES

Entry Changes, Section 8.10.8
T and P, Section 8.10.6

### 8.10.6	Text Specification

The DOCUMENTOR provides commands which allow up to four lines of text to be associated with each entry and also allow modification of previously entered text.

The following command is used to enter a line of text that is associated with a definition:

U new text line

The command U inserts “new text line” under the current text line (which begins at 0 after a new entry).  The new text line may be composed of one to 64 characters, including embedded blanks.

Following the use of ENTER or FIND, the current text line is initialized to zero.  Use of U not only inserts a new text line, it also increments the current line number.  Thus subsequent usage of U adds additional text lines.

The command P is used to modify existing text.  You do this by displaying the line to be changed and then using P to replace the old text with new text.  Remember that you can only work on the current word in the current vocabulary.  You display the appropriate line of text (lines are numbered starting from zero) by typing:

line# T  (e.g., 3 T to display the fourth line)

After a line of text has been displayed, you can modify it by using the following command:

P replacement-text-line

The command X is used to delete a text line previously selected by the T command.  Thus, to delete Line 2 you would type:

2 T
X

Lower-case versions of U, T, P, and X are provided for convenience.

REFERENCES

ENTER, Section 8.10.5
FIND, Section 8.10.8

### 8.10.7	Definition Display

To display the current entry, type:

F

To print all definitions in all the vocabularies in the glossary, in ASCII alphabetical sequence, use the word SUMMARY.  The same information as for FIND is printed for each word entered in the glossary.  The printed report is paged and numbered.

The command:

/VOCABULARY

will print definitions as for SUMMARY but only in the glossary vocabulary whose name is specified.

REFERENCES

Glossary Vocabularies, Section 8.10.4 
Making an Entry Current, Section 8.10.8

### 8.10.8	Changes

Changes always affect the current word.  Words are made current in two ways.

1.	 A word just entered is the current word.

2.	A previously entered word in the current vocabulary may be made current by using the command:

FIND word-name

This displays the requested word, with its vocabulary name, block number, stack usage, and text description.

Following the use of ENTER or FIND, the current line number is initialized to zero.

The current word’s stack entries may be changed by typing:

#in-#out STACK

The current word’s source-block# may be changed by typing:

new-blk# AT

You may not change vocabulary and word names except by deleting and re-entering the entry, since these two items form the index keys.

To redisplay the complete entry for the current word, type:

F

REFERENCES

Changing Description Lines, Section 8.10.6
Making a Vocabulary Current, Section 8.10.4

### 8.10.9	Text and Definition Deletion

The following command is used to remove all text lines associated with the current definition and then to delete the current definition from the glossary:

DELETE word-name

© 2010 GreenArrays, Incorporated.

Specifications are subject to change without notice.

GreenArrays, GreenArray Chips, arrayForth, and the GreenArrays logo are trademarks of GreenArrays, Inc.  All other trademarks or registered trademarks are the property of their respective owners.

*	In the second edition; p. 242 in the first edition.

*	On 32-bit sytems such as the M68000, WIDTH is a 16-bit cell.  Therefore, to change both bytes at once you would use the phrase:

HEX   1F1F WIDTH W!   DECIMAL

*	On 8-bit and 16-bit computers, a “single-precision integer” is 16 bits wide, and “double-precision” implies 32 bits.  On 32-bit CPUs such as the 68000, widths are 32 and 64 bits, respectively.

*	The interpreter pointer I should not be confused with the I which fetches a copy of the top of the return stack.

* 32-bit systems also include 4+, 4-, and 4*.  These words are most valuable for converting cells to bytes, and incrementing and decrementing by cell widths.

* For users of previous versions of polyFORTH that used 1 as the standard ‘true’ value, compatibility definitions may be obtained from FORTH, Inc.

*	On DEC (PDP-11), Intel, and Zilog computers the first byte is in the low-order position of each cell.

* For 32-bit machines, use 60 ALLOT, 4+, and 56 MOVE.

** For 32-bit machines, use CFA 4+ and 4*.

*	Remember that -TEXT compares cell by cell.  This is fine in this case, since “no” is of even length.  If your string is of odd length, you must add a space to make it even length and ensure that your test string is also blank-filled if necessary.  This is usually easy to do; for example, if the test string is input from the keyboard, TEXT will move it to a blank-filled PAD.

*	The command U. was used to type out the number as an unsigned integer because block buffers are usually in high memory.  Otherwise, this address would appear as a negative number.

*	In polyFORTH’s text editor, the commands changing a block being edited perform UPDATE automatically.  Similarly, the DISKING utility, Target Compiler, and other standard utilities serve as good examples of how UPDATE should be used in applications.

*	Many “intelligent” disk controllers perform retries automatically.  On these, as well as on versions of polyFORTH running as a “guest” under another operating system, there is nothing to be gained by attempting retries from within polyFORTH.

*	This section applies only to 8-bit and 16-bit CPUs.  On 32-bit CPUs all block numbers are 32 bits, allowing for a total capacity of 2 Gbytes.

**	The actual range is ± 32767.  A single precision block number may be negative, for accessing blocks in the previous partition.

* This means they’re physically contiguous on a disk supported by a native polyFORTH.  Versions of polyFORTH that are co-resident with another OS use its files to contain blocks, and thus rely on the host OS to manage disk.

* On 32-bit machines these sizes should be doubled.

* This block is not generally resident, because it is used only in the initial creation of the data base.  It may usually be found at FILES 5 +.

* Responses shown in light type.

* The specific block number may vary on different versions of polyFORTH.  It is usually the fifth block after the FILES load block.